3GPP TSG-RAN WG3 Meeting #123	R2-2307155

Toulouse, France, August. 21st – August. 25th, 2023

Agenda Item:	7.11.2.1

Source: 	Canon Research Centre France

Title:	Discussion on security issue with multicast MCCH

Document for:	Discussion

# Introduction

In this paper, we discuss the PTM configuration delivery through SIB and MCCH for MBS multicast reception in RRC_INACTIVE state. The discussion takes into account the following agreements from the previous RAN2 meetings reported in [1] and [2]:

At RAN2#121: - UE shall join in the multicast session before receiving multicast in RRC INACTIVE. - We introduce a new MCCH logical channel for multicast in INACTIVE (different from broadcast MCCH). - Multicast MCCH configuration is provided via new SIB. - Optionally, Multicast MCCH configuration for the serving cell can also be provided in dedicated signalling. Understanding is we are not optimizing mobility case because of this. At RAN2#121bis: - Similar to Rel-17 broadcast reception procedure, UE acquires new SIB and multicast MCCH to get PTM configuration after cell reselection.

# Discussion

## Multicast MCCH security issue

The access to MBS multicast data is subject to prior authorization for each UE during the join procedure. Then, the MBS multicast configuration should be available only to the UEs who have acquired the proper authorization during the join procedure.

It was agreed that, similar to Rel-17 broadcast reception procedure, a UE acquires new SIB and multicast MCCH to get PTM configuration after cell reselection. In practice, the UEs that read the multicast MCCH channel to get PTM configuration for a MBS multicast session should have previously joined this MBS multicast session. However, as the multicast MCCH configuration is provided via new SIB, the MCCH can be accessible to all UEs without restriction which, while acceptable for broadcast services, represents a security breach for MBS multicast services. The security protection of MBS multicast data may be provided by the application layer but this is not guaranteed.

Observation 1: The MCCH can be accessible to all UEs without restriction which represents a security breach for MBS multicast services.

## Multicast MCCH scrambling

A mechanism should be provided to restrict the access to a PTM configuration through MCCH to the UEs that have previously joined the corresponding MBS multicast session.

One solution may be to scramble the multicast MCCH content and to provide the key to retrieve the content to the authorized UEs. The key would be provided to the authorized UEs when they are in RRC_CONNECTED stated, for instance at the join procedure.

Proposal 1: The multicast MCCH content may be scrambled so that the content can only be retrieved by authorized UEs.

As the multicast MCCH may convey several PTM configurations for different MBS sessions, it should also be avoided that a UE, authorized to receive a PTM configuration for a MBS multicast session it has joined, can also retrieve PTM configurations for MBS multicast sessions it has not joined.

For this purpose, the content of the multicast MCCH may be scrambled with different keys, each key being associated to one MBS multicast session. Then, each portion of MCCH containing a PTM configuration is scrambled with a key different to the keys used to scramble the other portions containing other PTM configurations. To identify each portion, the MBS session ID (or TMGI) may be included in MCCH content but not scrambled.

Proposal 2: The multicast MCCH content may be scrambled with different keys, each key being associated to one MBS multicast session.

# Conclusion

In this contribution, we have described a potential security issue when delivering MBS multicast configuration through SIB and MCCH. Then, we have proposed a solution based on MCCH content scrambling. Besides, we have made the following observation and proposals:

Observation 1: The MCCH can be accessible to all UEs without restriction which represents a security breach for MBS multicast services.

Proposal 1: The multicast MCCH content may be scrambled so that the content can only be retrieved by authorized UEs.

Proposal 2: The multicast MCCH content may be scrambled with different keys, each key being associated to one MBS multicast session.

# References

[1] RAN2#121e meeting MBS session notes

[2] RAN2#121-bis-e meeting MBS session notes