3GPP TSG RAN2 Meeting #123		R2-2307167

Toulouse, France, August 21-25, 2023

Source:	Telit

Title:	Considerations on TN-NTN cell re-selection

Agenda Item:	7.7.4.1.1 NTN-TN enhancements

Document for:	Discussion and Decision

# Introduction

In the previous RAN2 meeting, RAN2#122 also TN-NTN cell re-selection enhancement was briefly discussed, also in [1]. However, finally it was not concluded whether TN cells could provide neighbor cell information related to NTN coverage.

According to our understanding without the possibility to signal assistance information for TN-NTN transition support, there will always remain an unbalance between NTN-TN and TN-NTN idle/inactive mobility.

# Discussion

When comparing the definition and agreements related to NTN-Tn and TN-NTN transition it becomes obvious that as it is agreed/specified today there is an unbalance.

NTN-TN idle/inactive mobility is supposed to be supported by signaling of available TN coverage area. Hence, based on said information a UE can decide whether it makes sense to perform TN measurements and on which frequencies to perform them. Hence depending on parameters configured the UE can accordingly perform cell re-selection from NTN to TN.

Obersvation1: For NTN -> TN RRC_idle/RRC_inactive the 3GPP specification intends to provide means to provide all required information to perform cell re-selection towards TN network when UE is camping on NTN.

When looking at TN-> NTN idle or inactive mobility so far there is no information available related to NTN coverage. Hence in whatever scenario, i.e. when leaving Tn coverage due to seaside scenario or entering mainly unpopulated/ Tn uncovered area there is no information provided by the network related to NTN coverage.

As a consequence, the TN->NTN mobility is rather a cell selection procedure than cell re-selection. Means a UE leaving the TN coverage area would first try to scan for all TN neighbor frequencies that were indicated and further technologies it is capable of, prior starting to search for NTN network.

This process is for two reasons very time consuming and power demanding. The UE will continue for searching TN neighbors for quite a while unless it gives up as it hasn’t found a suitable cell and afterwards the UE will start searching for NTN without any assistance information provided.  Hence it will take a long time unless the UE has found a suitable NTN cell.

Observation 2: When moving from TN-> NTN the UE will first search for all possible TN neighbor cells prior it start NTN cell search without any assistance information. Hence this process will take long and is power consuming.

When assistance information would be provided for NTN search the TN-> NTN RRC_idle/RRC_inactive mobility could be improved. And is in general required for RAN4 to specify corresponding requirements.

Furthermore leaving TN coverage area and having the need to connect to NTN are in certain geographical scenarios a very common use case, i.e. goods or persons on board of a ship leaving an harbor.  Means they will leave the TN coverage area and to remain reachable there is a need to connect to NTN.

Observation 3: For certain geographical scenarios i.e. when persons or goods leave an harbor TN-> NTN idle mode mobility is a very common scenario and should be feasible by re-selection.

During discussion related to [1] various options related to what NTN neighbor cell information should be provided in the TN cell (e.g. SIB19 or ephemeris or common TA, ntn-Config).

Giving the actual status of the overall NTN mobility enhancements discussion and the fact that NTN neighbor cell information provisioning is especially beneficial in certain localized scenarios where corresponding information would be required, and only for dual-mode devices also supporting NTN, we would suggest to enable SIB19 also to be signaled by TN cells and all cells contained in SIB19 are regarded as NTN neighbor cells. This would require some clarification in the specification but no changes to the SIB structure/content itself. The impact to UEs could be minimized and SIB19 would only be acquired by those devices supporting NTN. Furthermore

Observation 4: SIB19 broadcasting in TN cells is a suitable way to provide the NTN neighbor cell information in cells/areas where these is beneficial.

# Conclusion

RAN2 to discuss the following proposals related to TN-NTN RRC_idel/RRC_inactive mobility enhancements:

Proposal1: Support to provide NTN neighbor cell information in TN cells to allow for TN-> NTN cell re-selection.

Proposal 2: Allow for broadcasting of SIB19 in TN cells, in order to provide the NTN neighbor cell information.

# Reference

[1] R2-2306643	[offline-103] NTN-TN cell reselection Nokia	discussion	Rel-18	NR_NTN_enh-Core