3GPP TSG-RAN WG2 Meeting #123	R2-2307178

Toulouse, France, August 21-25, 2023

Agenda item:		7.3.2

Source:		Lenovo

Title:		Various (RRC Procedure, Measurement, SR, CG etc.) alignment aspects

Document for:		Discussion and Decision

- Introduction

One of the Techniques to improve network energy savings are the Techniques in time domain e.g., Cell DTX/DRX. Cell DTX/DRX is applied to at least UEs in RRC_CONNECTED state. A periodic Cell DTX/DRX (i.e., active, and non-active periods) can be configured by gNB via UE-specific RRC signalling per serving cell.

RAN2 made some agreements on this in RAN2#122:

Agreements

1	UE monitors PDCCH for RAR during Cell DTX non-active time. The ra-ResponseWindow could be started as legacy.

2	UE monitors PDCCH for msg4 during Cell DTX non-active time. The ra-ContentionResolutionTimer could be started as legacy.

3	Working assumption:  When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy.  It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

4	Once gNB recognizes there is an emergency call or public safety related service (e.g. MPS/MCS), the NW should ensure there is no impact to the emergency call (e.g. may deactivate Cell DTX/DRX).  The behavior is captured in stage 2 spec

5	When an DG grant is received, by the gNB during cell DRX/DTX, the UE follows the grant assignment (i.e. like in legacy).  This includes DL HARQ feedback.

Previously, RAN2 made some following agreements on this in RAN2#121bis:

Agreements

- A periodic cell DTX/DRX configuration is explicitly signalled to the UEs.

- A periodic cell DTX/DRX pattern is configured by UE specific RRC signalling.

- The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration.

- As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released.

- From RAN2 point of view, majority companies see a benefit with L1 signalling for Cell DTX/DRX activation/deactivation, send a LS to RAN1 (email 308) with our preference and ask about feasibility and design details.   Ask about feasibility and reliability of using L1 signaling.  Clarify that the question is about activation/deactivation copy the agreement from last meeting that we are focusing on single configuration.  Extract a few key benefits of dynamic signaling from email discussion and online discussions

- As baseline, UE doesn’t monitor SPS occasions during Cell DTX non-active period. As baseline, gNB is assumed to be not transmitting PDSCH to that UE on such SPS occasions during the Cell DTX non-active period

- As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods

- As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period

FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to to support high priority traffic

- (for the SRs that will be dropped) If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH.  For the FFS case there may be some exceptions.

- The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.   UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.   FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

Since, Cell DTX and Cell DRX modes can be configured and operated separately, we are making independent proposals for Cell DRX and Cell DTX.

- Discussion

  - Alignment to Cell DRX

RAN2 should discuss possible UE behaviour when the cell/ Network is (going to be) in DRX i.e., will not receive. The below Figure 1 shows two RRC Connected UEs with their corresponding C-DRX configurations, as well as their serving cell in energy saving mode utilizing a DRX configuration. A UE configured with a C-DRX configuration can transmit at any time, but it can receive only during certain limited time periods designated as “active time” and shown in the below Figure 1 as “UE reception ON” period. In uplink, cell is not expected to receive during “Cell Reception OFF” periods. With this background let’s discuss some specific aspects.

Figure 1

## RRC Procedures:

Certain RRC procedures need a response from the UE indicating successful completion. An NES Network may attempt to minimize start of RRC procedure that may not be completed in time until the DRX at the network side is started. And if a procedure is started, network may keep receiving for a limited time beyond the start of the intended DRX time to ensure that a possible response from a UE is not missed. But since the UE does not know if the network is receiving in a supposedly “Cell Reception Off” period, we need to have a deterministic UE behaviour to avoid any protocol errors.

As one sensible handling, if the network triggers a RRC Procedure while the network is in DRX (or about to enter DRX), the UE after successfully executing the procedure (e.g., applying new configuration) should not immediately transmit the procedure completion message (e.g., RRC Reconfiguration Complete message) but transmit this when the network is in reception again i.e., at the start of the next reception time at the network (end of a network DRX period) and, UE assumes that the procedure is successfully completed. Following RRC procedure related completion message can be sent in this “delayed” manner:

- RRC Reconfiguration Complete,

- RRC Setup Complete,

- RRC Reestablishment Complete,

- RRC Resume Complete,

- Security Mode Complete,

- UE Information Response etc.

Accordingly, the following proposal is being made:

The UE after successfully executing a RRC procedure (e.g., applying new configuration) assumes the procedure is successfully completed if the network triggers a RRC Procedure while the network is in DRX (or about to enter DRX). The procedure Complete message is transmitted when the network is in reception again.

UE initiated procedure/ information cases: The UE initiated procedure(s) or RRC messages e.g., Dedicated SIB Request, Idle mode SIB Request, UE Assistance Information, UL Information Transfer, Location Measurement Indication, UE Capability Information, Counter Check Response, MBS Interest Indication, Measurement Report App Layer, UE Positioning Assistance Info etc. shall be postponed until the network starts reception (DRX time is over). The UE shall always report the latest values applicable at the time of actual transmission of these messages.

UE initiated RRC procedure/ information cases shall be postponed until the network starts reception (DRX time is over). The UE shall always report the latest values applicable at the time of actual transmission of these messages.

Further, Re-establishment procedure can be triggered for a number of reasons including reconfiguration failures, RLF etc., where upon initiation of the procedure, the UE shall stop all running timers among T310, T312, T304, T311, T316, reset MAC etc., and perform cell selection in accordance with the cell selection process as specified in TS 38.304. Upon selecting a suitable NR cell, a UE ensures having valid and up to date essential system information.

When NES feature is deployed, a R18(+) UE should determine if the selected cell is in DRX, or about to enter DRX (say in next 10 ms. or so) – if so, it shall look for another suitable NR cell that is not in DRX and not going to enter DRX anytime soon, otherwise the UE can’t obtain essential SIBs. Alternatively, this criterion can be included as part of the suitable cell selection procedure such that a cell is considered suitable only when the cell is not in DRX and not going to enter DRX anytime soon – at least when timer T311 is running. The proponent thinks that the latter allows a more generic UE behaviour and ensures that the UE is camped on a cell where it can receive System information and enjoy cell services, when the need comes. Therefore:

A cell is considered suitable only when the cell is not in DRX and not going to enter DRX anytime soon, at least when timer T311 is running. FFS: How/ if to define “soon”.

## RRM Measurements

Another problem with NES feature seems to be a scenario wherein a RRC Connected makes (un-necessary) measurements but the same can’t be reported successfully because of serving cell being in DRX. The UE measurements consume a substantial share of UE battery among total power consumed by the UE Modem.

If RRC informs physical layer of the DRX time of the network, Physical layer need not perform measurements when a potential triggered measurement event might fall under the network’s DRX time. Indeed, the physical layer of the UE can start radio measurements for configured/ active measurement identities not before parameter TTT (TimeToTrigger) time before the start of the reception time (end of a DRX period) at the network. To this end, the highest value of the parameter TimeToTrigger among all configured measurement events is used. Further, it might take the UE finite time to start measurement (warmup/ tune RF, scan the radio frequency even do cell search/ synchronization etc.), the UE may indeed need to perform measurements a little earlier than just TTT ms from the next Cell Reception ON period, the exact time depending on UE implementation.

UE does not perform RRM measurement until TimeToTrigger time before the start of the reception time (i.e., end of a DRX period) at the source cell.

## UL Transmission

In the last meeting#122, RAN2 made the following agreements:

1	UE monitors PDCCH for RAR during Cell DTX non-active time. The ra-ResponseWindow could be started as legacy.

2	UE monitors PDCCH for msg4 during Cell DTX non-active time. The ra-ContentionResolutionTimer could be started as legacy.

3	Working assumption:  When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy.  It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

4	Once gNB recognizes there is an emergency call or public safety related service (e.g. MPS/MCS), the NW should ensure there is no impact to the emergency call (e.g. may deactivate Cell DTX/DRX).  The behavior is captured in stage 2 spec

5	When an DG grant is received, by the gNB during cell DRX/DTX, the UE follows the grant assignment (i.e. like in legacy).  This includes DL HARQ feedback.

- SR transmission:

With respect to SR transmission, two conclusions could be made. Some companies argued that some SR transmission exceptions may allowed which are under network control and when configured, the UE may transmit SR regardless of if the NES cell might be sleeping (i.e., in cell DRX/ DTX - not receiving and/ or not transmitting).

Other companies argued that such SR exceptional configuration may not be necessary since as most “urgent/ important” cause – even the emergency call establishment – can tolerate certain delay (e.g., due to CSFB required in legacy deployment). The proponent disagrees since:

- The delay beyond what technology must withstand, should be seen as “wilful”.

- The limit to such “delay tolerance” will impose a restriction to network’s energy saving possibility.

- There are other reasons (even) disregarding the emergency call situation which might occur frequently:

  - BFR MAC CE: Failure to recover from beam failure and may lead to Scell disruption, or SpCell disruption if the Beam Failure Recovery procedure determines that at least one BFR for only one BFD-RS set has been triggered, leading to bad user experience – when a triggered SR (highlighted in yellow) for a BFR MAC CE transmission can’t be transmitted. At the same time, the proponent note that the situation is not critical since the beam failure recovery of the SpCell and in situation where if BFR is triggered for both BFD-RS sets of the SpCell, a RACH procedure is initiated (which may or may not be contention free).

## 5.17 Beam Failure Detection and Recovery procedure The MAC entity may be configured by RRC per Serving Cell with a beam failure recovery procedure which is used for indicating to the serving gNB of a new SSB or CSI-RS when beam failure is detected on the serving SSB(s)/CSI-RS(s). Beam failure is detected by counting beam failure instance indication from the lower layers to the MAC entity. If beamFailureRecoveryConfig is reconfigured by upper layers during an ongoing Random Access procedure for beam failure recovery for SpCell, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure using the new configuration. When the SCG is deactivated, the UE performs beam failure detection on the PSCell if bfd-and-RLM is set to true. RRC configures the following parameters in the beamFailureRecoveryConfig, beamFailureRecoverySpCellConfig, beamFailureRecoverySCellConfig and the radioLinkMonitoringConfig for the Beam Failure Detection and Recovery procedure: - beamFailureInstanceMaxCount for the beam failure detection (per Serving Cell or per BFD-RS set of Serving Cell configured with two BFD-RS sets); - beamFailureDetectionTimer for the beam failure detection (per Serving Cell or per BFD-RS set of Serving Cell configured with two BFD-RS sets); - beamFailureRecoveryTimer for the beam failure recovery procedure; - rsrp-ThresholdSSB: an RSRP threshold for the SpCell beam failure recovery; - rsrp-ThresholdBFR: an RSRP threshold for the SCell beam failure recovery or for the beam failure recovery of BFD-RS set of Serving Cell; - powerRampingStep: powerRampingStep for the SpCell beam failure recovery; - powerRampingStepHighPriority: powerRampingStepHighPriority for the SpCell beam failure recovery; - preambleReceivedTargetPower: preambleReceivedTargetPower for the SpCell beam failure recovery; - preambleTransMax: preambleTransMax for the SpCell beam failure recovery; - scalingFactorBI: scalingFactorBI for the SpCell beam failure recovery; - ssb-perRACH-Occasion: ssb-perRACH-Occasion for the SpCell beam failure recovery using contention-free Random Access Resources; - ra-ResponseWindow: the time window to monitor response(s) for the SpCell beam failure recovery using contention-free Random Access Resources; - prach-ConfigurationIndex: prach-ConfigurationIndex for the SpCell beam failure recovery using contention-free Random Access Resources; - ra-ssb-OccasionMaskIndex: ra-ssb-OccasionMaskIndex for the SpCell beam failure recovery using contention-free Random Access Resources; - ra-OccasionList: ra-OccasionList for the SpCell beam failure recovery using contention-free Random Access Resources; - candidateBeamRSList: list of candidate beams for SpCell beam failure recovery; - candidateBeamRS-List-r16: list of candidate beams for SCell beam failure recovery or beam failure recovery of BFD-RS set one of Serving Cell; - candidateBeamRS-List2-r17: list of candidate beams for beam failure recovery of BFD-RS set two of Serving Cell. The following UE variables are used for the beam failure detection procedure: - BFI_COUNTER (per Serving Cell or per BFD-RS set of Serving Cell configured with two BFD-RS sets): counter for beam failure instance indication which is initially set to 0. The MAC entity shall for each Serving Cell configured for beam failure detection: 1> if the Serving Cell is configured with two BFD-RS sets: 2> if beam failure instance indication for a BFD-RS set has been received from lower layers: 3> start or restart the beamFailureDetectionTimer of the BFD-RS set; 3> increment BFI_COUNTER of the BFD-RS set by 1; 3> if BFI_COUNTER of the BFD-RS set >= beamFailureInstanceMaxCount: 4> trigger a BFR for this BFD-RS set of the Serving Cell; 2> if BFR is triggered for both BFD-RS sets of the SpCell and the Beam Failure Recovery procedure is not successfully completed for any of the BFD-RS sets: 3> initiate a Random Access procedure (see clause 5.1) on the SpCell; 2> if the Serving Cell is SpCell and the Random Access procedure initiated for beam failure recovery of both BFD-RS sets of SpCell is successfully completed (see clause 5.1): 3> set BFI_COUNTER of each BFD-RS set of SpCell to 0. 3> consider the Beam Failure Recovery procedure successfully completed. 2> if the beamFailureDetectionTimer of this BFD-RS set expires; or 2> if beamFailureDetectionTimer, beamFailureInstanceMaxCount, or any of the reference signals used for beam failure detection is reconfigured by upper layers or by the BFD-RS Indication MAC CE associated with a BFD-RS set of the Serving Cell: 3> set BFI_COUNTER of the BFD-RS set to 0. 2> if a PDCCH addressed to C-RNTI indicating uplink grant for a new transmission is received for the HARQ process used for the transmission of the Enhanced BFR MAC CE or Truncated Enhanced BFR MAC CE which contains beam failure recovery information of this BFD-RS set of the Serving Cell: 3> set BFI_COUNTER of the BFD-RS set to 0; 3> consider the Beam Failure Recovery procedure successfully completed for this BFD-RS set and cancel all the triggered BFRs of this BFD-RS set of the Serving Cell. 2> if the Serving Cell is SCell and the SCell is deactivated as specified in clause 5.9: 3> set BFI_COUNTER of each BFD-RS set of SCell to 0; 3> consider the Beam Failure Recovery procedure successfully completed and cancel all the triggered BFRs of all BFD-RS sets of the Serving Cell. 1> else: 2> if beam failure instance indication has been received from lower layers: 3> start or restart the beamFailureDetectionTimer; 3> increment BFI_COUNTER by 1; 3> if BFI_COUNTER >= beamFailureInstanceMaxCount: 4> if the Serving Cell is SCell: 5> trigger a BFR for this Serving Cell; 4> else if the Serving Cell is PSCell and, the SCG is deactivated: 5> if beam failure of the PSCell has not been indicated to upper layers since the SCG was deactivated or since the deactivated SCG was last reconfigured with BFD-RS: 6> indicate beam failure of the PSCell to upper layers. NOTE: After beam failure is indicated to upper layers, the UE may stop the beamFailureDetectionTimer and lower layer beam failure indication while BFI_COUNTER >= beamFailureInstanceMaxCount for the deactivated SCG. 4> else: 5> initiate a Random Access procedure (see clause 5.1) on the SpCell. 2> if the beamFailureDetectionTimer expires; or 2> if beamFailureDetectionTimer, beamFailureInstanceMaxCount, or any of the reference signals used for beam failure detection is reconfigured by upper layers associated with this Serving Cell: 3> set BFI_COUNTER to 0. 2> if the Serving Cell is SpCell and the Random Access procedure initiated for SpCell beam failure recovery is successfully completed (see clause 5.1): 3> set BFI_COUNTER to 0; 3> stop the beamFailureRecoveryTimer, if configured; 3> consider the Beam Failure Recovery procedure successfully completed. 2> else if the Serving Cell is SCell, and a PDCCH addressed to C-RNTI indicating uplink grant for a new transmission is received for the HARQ process used for the transmission of the MAC CE for BFR which contains beam failure recovery information of this Serving Cell; or 2> if the SCell is deactivated as specified in clause 5.9: 3> set BFI_COUNTER to 0; 3> consider the Beam Failure Recovery procedure successfully completed and cancel all the triggered BFRs for this Serving Cell. The MAC entity shall: 1> if the Beam Failure Recovery procedure determines that at least one BFR has been triggered and not cancelled for an SCell for which evaluation of the candidate beams according to the requirements as specified in TS 38.133 [11] has been completed and if none of the Serving Cell(s) of this MAC entity are configured with two BFD-RS sets: 2> if UL-SCH resources are available for a new transmission and if the UL-SCH resources can accommodate the BFR MAC CE plus its subheader as a result of LCP: 3> instruct the Multiplexing and Assembly procedure to generate the BFR MAC CE. 2> else if UL-SCH resources are available for a new transmission and if the UL-SCH resources can accommodate the Truncated BFR MAC CE plus its subheader as a result of LCP: 3> instruct the Multiplexing and Assembly procedure to generate the Truncated BFR MAC CE. 2> else: 3> trigger the SR for SCell beam failure recovery for each SCell for which BFR has been triggered, not cancelled, and for which evaluation of the candidate beams according to the requirements as specified in TS 38.133 [11] has been completed. 1> if the Beam Failure Recovery procedure determines that at least one BFR for any BFD-RS set has been triggered and not cancelled for an SCell for which evaluation of the candidate beams according to the requirements as specified in TS 38.133 [11] has been completed; or 1> if the Beam Failure Recovery procedure determines that at least one BFR for only one BFD-RS set has been triggered and not cancelled for an SpCell for which evaluation of the candidate beams according to the requirements as specified in TS 38.133 [11] has been completed; or 1> if the Beam Failure Recovery procedure determines that at least one BFR has been triggered and not cancelled for an SCell for which evaluation of the candidate beams according to the requirements as specified in TS 38.133 [11] has been completed and if at least one Serving Cell of this MAC entity is configured with two BFD-RS sets: 2> if UL-SCH resources are available for a new transmission and if the UL-SCH resources can accommodate the Enhanced BFR MAC CE plus its subheader as a result of LCP: 3> instruct the Multiplexing and Assembly procedure to generate the Enhanced BFR MAC CE. 2> else if UL-SCH resources are available for a new transmission and if the UL-SCH resources can accommodate the Truncated Enhanced BFR MAC CE plus its subheader as a result of LCP: 3> instruct the Multiplexing and Assembly procedure to generate the Truncated Enhanced BFR MAC CE. 2> else: 3> trigger the SR for beam failure recovery of each BFD-RS set for which BFR has been triggered, not cancelled, and for which evaluation of the candidate beams according to the requirements as specified in TS 38.133 [11] has been completed; 3> trigger the SR for SCell beam failure recovery for each SCell for which BFR has been triggered, not cancelled, and for which evaluation of the candidate beams according to the requirements as specified in TS 38.133 [11] has been completed. All BFRs triggered for an SCell shall be cancelled when a MAC PDU is transmitted and this PDU includes a MAC CE for BFR which contains beam failure information of that SCell. All BFRs triggered for a BFD-RS set of a Serving Cell shall be cancelled when a MAC PDU is transmitted and this PDU includes an Enhanced BFR MAC CE or Truncated Enhanced BFR MAC CE which contains beam failure recovery information of that BFD-RS set of the Serving Cell.

  - C-LBT failure: Like BFR case, we do not see a critical problem when consistent uplink LBT failures are detected on SpCell, since here the UE switches to another UL BWP with configured RACH resources on that cell, initiates RACH, and reports the failure via MAC CE.

However, when consistent uplink LBT failures are detected on SCell(s), the UE reports this to the corresponding gNB (MN for MCG, SN for SCG) via MAC CE on a different serving cell than the SCell(s) where the failures were detected. If no resources are available to transmit the MAC CE, a Scheduling Request (SR) can be transmitted by the UE. If SR can’t be transmitted due to serving cell being in DRX sleep, the user experience may be adversely affected.

Based on the findings from BFR MAC CE and C-LBT failures, we make the following observation:

Observation: Inability to transmit beam failure recovery MAC CE (or transmit these much later when the network starts receiving again) and LBT failure report for a Scell, can cause bad user experience.

To tackle the issue, we propose that an SR can be configured to cater to the critical situations (Emergency calls, beam failure recovery of a BFD-RS set MAC CE). Since this is configurable, an operator might choose to not use it.

SR can be configured to cater to the important situations (Emergency calls, beam failure 
 recovery of a BFD-RS set MAC CE) only.

Another case is that for configured grant, for a transport block, two or more repetitions can be in one slot, or across slot boundaries in consecutive available slots with each repetition in one slot. If the uplink grant is a configured uplink grant with bundling enabled, and the configured grant transmission occasion overlaps with the cell Reception OFF period, following the similar consideration, we propose:

RAN2 to discuss whether UE initiates the first transmission within a bundle when the cell is in cell Reception OFF duration for making the first transmission within a bundle.

  - Alignment to Cell DTX

RAN2 should discuss possible UE behaviour when the cell/ Network is (going to) in DRX i.e., will not receive. The below Figure 1 shows two RRC Connected UEs with their corresponding DRX configurations, as well as their serving cell in energy saving mode utilizing a DTX configuration. A UE configured with a DRX configuration can transmit at any time, but it can receive only during certain limited time periods designated as “active time” and shown in the Figure 1 as “UE reception ON” period. In downlink, cell is not expected to transmit during “Cell Transmission OFF” periods. To which signals/ channels does the transmission-off is applied can vary from scenario to scenario and is simply network implementation – but in any case, it needs to inform UEs being served in the corresponding cell about the current energy saving configuration being (or to be) used in the cell.

Figure 1

## How to align UE DRX and Cell DTX:

UE DRX can be aligned to the Cell DTX by UE itself. To this end, as one option, UE DRX configuration needs to be masked (= AND operation) with the Cell DTX configuration, as shown below. During the NW ON periods UE follows the legacy DRX procedure/behaviour according to its DRX configuration. In DTX/OFF periods, UE is also in DRX, i.e., not monitoring PDCCH:

Figure 2: Masking/ ANDing of the UE DRX and Cell DTX

As another option, the Reference point for the UE DRX (DRX offset) is aligned with the “on-duration” of the cell DTX; this is shown in the lower part of the Figure 3, where the new reference point for the UEs is “brought forward” to align with the switching ON of the nearest transmission time of the cell. In addition to the shift of the reference point, masking can be performed as well, as in the Figure 3 as shown below:

Figure 3: Shows “shift” of the reference point as well as masking of the two configurations – only one of these might suffice.

These are mainly UE based options and need no particular signalling once one of these is specified as a way to align Cell DTX and UE DRX configuration. It should be noted that for all of these options during the NW On periods UEs Active Time can be dynamically extended as required until the beginning of the next NW DTX period.

Alternatively, the network can itself determine a shifted/ superimposed DRX-config for each UE. A potential NES serving cell can reconfigure all RRC Connected UEs’ DRX Cycles by individually sending a dedicated reconfiguration to each of these. This is however taxing for the network, wastes network energy in transmitting the reconfiguration messages, and at the same time prolongs network’s transmission-reception state in trying to reach each RRC Connected UE successfully. To avoid this, each RRC Connected UE is signalled a first DRX-Config (per MAC entity) and a second DRX-Config applicable for case when/ if energy saving will be activated. Both first and second DRX-Config are sent together to the UE using dedicated RRC signalling. Network determines a DTX configuration in advance (i.e., before activating the NES mode) and the second DRX-Config is to be used when network energy saving is activated i.e., cell is using DTX.

Further, in the ongoing Email Discussion [3] it is being discussed if common L1/L2 signalling can be considered for activating/deactivating the Cell DTX/DRX mode, or rather the RRC message used to signal the cell’s DRX/ DTX configuration can be used for activating/deactivating the Cell DTX/DRX mode. When activated, UE applies one of the above or similar means to align its cell DRX configuration with cell DTX configuration when it received L1/ L2 signalling indicating that the network is transitioning to energy saving mode. In our opinion, the first solution from Figure 2 is the simplest, does not create any ambiguity or signalling overhead: UE DRX configuration is masked (= AND operation) with the Cell DTX configuration

To align UE cell DRX configuration with cell DTX configuration when the network is transitioning to energy saving mode, UE DRX configuration is masked (= AND operation) with the Cell DTX configuration.

## UL Data Transmission

Cell DTX configuration is sent to the UE using RRC signalling (broadcast or dedicated). UE is configured with CG resources. UE keeps transmitting e.g., using CG (Configured Grant) configuration even when the cell is in DTX for making initial transmission and one or more autonomous retransmissions can be made similar to NR-U i.e., using configuredGrantReTxTimer and configuredGrantTimer. Upon configuredGrantTimer expiry, the NDI is considered toggled, HARQ buffer flushed. UE makes retransmission when configuredGrantReTxTimer is not running; configuredGrantReTxTimer is restarted when a re(Tx) is made. Additionally, UE should stop making autonomous retransmissions when cell is about to leave DTX (and enter Cell Transmission ON time).

During Cell Transmission Off periods, a UE configured with CG resources makes initial transmission. In addition, autonomous retransmissions can be made using configuredGrantReTxTimer and configuredGrantTimer like timers.

UE stops making autonomous retransmissions when cell is about to leave DTX (and enter Cell Transmission ON time).

- Repetitions of a configured uplink grant are HARQ retransmissions which are triggered without waiting for feedback from previous transmission. If the transmission within a bundle of the configured grant overlaps with the cell Transmission OFF duration, it is not necessary to stop the retransmissions within a bundle of the configured grant when the cell enters Cell Transmission ON duration to avoid unnecessary complexity. The UE shall initiate the transmission within a bundle and complete the configured repetition transmissions as in the legacy even if the cell enters cell Transmission ON duration.

UE performs the transmission within a bundle of the configured grant regardless the cell is in Cell Transmission ON or OFF duration.

# Conclusion

This document discussed some UE alignment behaviour for Cell DRX only. Following proposals are made:

Alignment to cell DRX – Control Plane aspect:

Proposal 1: The UE after successfully executing a RRC procedure (e.g., applying new configuration) assumes the procedure is successfully completed if the network triggers a RRC Procedure while the network is in DRX (or about to enter DRX). The procedure Complete message is transmitted when the network is in reception again.

Proposal 2: UE initiated RRC procedure/ information cases shall be postponed until the network starts reception (DRX time is over). The UE shall always report the latest values applicable at the time of actual transmission of these messages.

Proposal 3: A cell is considered suitable only when the cell is not in DRX and not going to enter DRX anytime soon, at least when timer T311 is running. FFS: How/ if to define “soon”.

Alignment to cell DRX – Measurements aspect:

Proposal 4: UE does not perform RRM measurement until TimeToTrigger time before the start of the reception time (i.e., end of a DRX period) at the source cell.

Alignment to cell DRX – UL data transmission:

Proposal 5: SR can be configured to cater to the important situations (Emergency calls, beam failure recovery of a BFD-RS set MAC CE) only.

Proposal 6: RAN2 to discuss whether UE initiates the first transmission within a bundle when the cell is in cell Reception OFF duration for making the first transmission within a bundle.

Alignment to cell DTX:

Proposal 7: To align UE cell DRX configuration with cell DTX configuration when the network is transitioning to energy saving mode, UE DRX configuration is masked (= AND operation) with the Cell DTX configuration.

Proposal 8: During Cell Transmission Off periods, a UE configured with CG resources makes initial transmission. In addition, autonomous retransmissions can be made using configuredGrantReTxTimer and configuredGrantTimer like timers.

Proposal 9: UE stops making autonomous retransmissions when cell is about to leave DTX (and enter Cell Transmission ON time).

Proposal 10: UE performs the transmission within a bundle of the configured grant regardless the cell is in Cell Transmission ON or OFF duration.

# References

[1]: R2-2213040	3GPP TR 38.864 v0.4.0