3GPP TSG-RAN WG2 Meeting #123	R2-2307179

Toulouse, France, August 21-25, 2023

Agenda item:		7.3.5

Source:		Lenovo

Title:		 CHO Procedure in NES Mode

Document for:		Discussion and Decision

- Introduction

Following agreements were made with regards to CHO procedure in the last meeting (RAN2#122):

Agreements

1.	We will have a CHO solution that considers NES mode of at least source cell.

2.	We can have a specific NES CHO execution condition based on source cell NES mode.   FFS how the UE determines is in NES mode.   FFS on how this is achieved in RRC

3.	We will not introduce new L1 signalling for the purpose of CHO

4.	Event A3, A4, A5 can be configured as a CHO execution condition in the NES scenario.   We will study the time based mechanism

This document progresses the NES mobility procedure from source as well as target cell’s aspect.

- Discussion

The source side:

Based on the agreements from the last meeting, a source cell must send CHO to the UE early enough (as is also the main motivation generally for “conditional” handovers), allowing the RRC Connected UEs in the cell sufficient time to evaluate handover conditions contained in the CHO.

-	For source cell CHO framework, RAN2 assumes a reference scenario where the UE has already performed CHO conditions evaluation by the time the source cell starts some “NES-mode”

-	As a baseline, UE initiates CHO evaluation upon receiving the CHO configuration.  FFS what trigger is used for execution of CHO

Further, configuring relaxed condition (e.g., lower event Threshold) increases the UE’s chances of finding a target cell but, on the other hand, a target qualifying threshold(s) from relaxed condition may not be in a good enough radio to service UE with required QoS.

Artificial mobility

UE is in good radio of the source cell, even at cell center

Radio condition of a target cell found based on relaxed condition may not be good enough and UE just lies at the cell edge of such a target cell. If the UE executes handover immediately after the fulfillment of relaxed condition, some bearer’s QoS may not be fulfilled in the target side and UE’s stay in the target may be short-lived. [Too-early handover case].

Therefore, UE should stay with the source cell as long as possible i.e., until the source cell is about to switch off (or use a NES technique). But if the stay becomes too long after the fulfillment of relaxed condition, by the time UE executes handover, the old measurements leading to evaluation of the condition may not be valid anymore. [Too-late handover case].

Too early case can be remedied by ensuring that UE does not start handover execution until the source cell is about to get into Cell DRX/ DTX sleep. When the source cell would start sleeping can be determined by the UE itself based on the received start offset Cell DTX/ DRX in UE dedicated RRC signalling, agreed earlier by RAN2. RAN2 agreed to not use a L1 signal for this end.

Observation 1: The time when the source cell would start sleeping can be determined by a UE itself based on the received start offset of Cell DTX/ DRX configuration received in UE dedicated RRC signalling.

Proposal 1: If relaxed measurement evaluation conditions are configured and subsequently handover condition is met, UE starts handover execution only when the source cell is about to get into Cell DRX/ DTX sleep.

Proposal 2: The time when the source cell starts (or about to start) Cell DRX/ DTX sleep can be determined by the UE based on the received start offset of Cell DTX/ DRX configuration received in UE dedicated RRC signalling.

To tackle, too late handovers we need to ensure that the condition evaluation (relaxed measurement event fulfillment) remain relevant (i.e., not stale) when the handover is to be executed to the corresponding target cell. This can be done using a timer that defines how long a handover condition when considered met remains valid. The timer is started when handover condition(s) is met, is stopped when handover execution is started and upon its expiry the handover condition(s) are no more considered as fulfilled and therefore UE needs to perform fresh evaluation, if required.

Proposal 3: A network configured timer is used to supervise the freshness of fulfilled relaxed handover condition(s). When timer expires, handover condition(s) are no more considered as fulfilled and therefore UE needs to perform fresh evaluation, if required.

Further, in traditional CHO, one execution condition including one or two events is configured by source cell for each CHO candidate cell. As a useful extension, it should now be possible to configure for a CHO candidate cell both a “traditional” CHO condition and a “NES” CHO condition. Therefore, two execution conditions (each containing up to two Measurement Ids) should be possible to be configured to one CHO candidate cell. The handover execution for the fulfilled “traditional” CHO condition will be like in legacy and should be preferred over a fulfilled “NES” CHO condition.

Proposal 4: Up to two execution conditions can be configured by the source cell to the UE for a single CHO candidate cell; one for “traditional” CHO condition and another for relaxed “NES” CHO condition.

The target side:

A handover target is a candidate cell for which all the handover condition(s) are fulfilled. UE however needs to determine if a candidate cell fulfilling handover conditions is or will be in sleep by UE’s arrival in the target (e.g., during T304 is running). If this is not done so then immediately after completion of RACH procedure either the target needs to get active to serve this UE, thus sacrificing energy saving, or UE will be released/ redirected/ handed over immediately upon arriving on the target cell (e.g., as soon as the UE transmits HO Complete on the target side in RACH Msg3), this impacting “user experience”. To avoid this, the serving cell can provide DRX/ DTX configuration for each candidate cell included in CHO. A UE determines if a candidate is using NES and if so, will it be sleeping around the time the UE arrives at the target. Another solution being proposed by some companies is that some prioritization per candidate cell can be indicated by the source cell in the CHO, but it is not clear how this ensure that the UE executes handover exactly when the target is not sleeping. Of course, it is best if the source would include only candidates that are not using NES technique, but it is unclear how far is this possible.

Observation 2: It may not always be possible to avoid including NES candidate cells in CHO.

Proposal 5: If NES candidate cell(s) are included in CHO, the serving cell can provide DRX/ DTX configuration for each candidate cell included in CHO.

Proposal 6: A UE determines if a candidate is sleeping around the time the UE arrives at the target.

Finally, what happens when there’s still no non-sleeping candidate/ target cell when the source cell is about to enter sleep?

- Option 1: UE selects a best amongst worst candidates

- Option 2: UE informs the serving cell about un-availability of a target cell just before the serving cell will sleep. After receiving this information from the UE, gNB can delay the state transition to sleep.

- Option 3: UE meets RLF since there’s no cell fulfilling handover conditions and source sleeps.

It may be useful to have RAN2 discussion on this, but proponent think that allowing RLF to happen is not the best choice since RAN2 has strived since release 8 to minimize RLFs, in its various studies/ work undertaken.

Observation 3: Executing handover to a sleeping cell might lead to handover failures and RLF.

Observation 4: RAN2 has strived since release 8 to minimize RLFs, in its various studies/ work undertaken. And it is sensible if we do not introduce a new reason for RLF.

Proposal 7: UE informs the serving cell about un-availability of a target cell when there’s still no target and the source cell is about to enter sleep.

# Conclusion

This document discussed NES mobility procedure from source as well as target cell’s aspect. Following proposals are made:

Observation 1: The time when the source cell would start sleeping can be determined by a UE itself based on the received start offset of Cell DTX/ DRX configuration received in UE dedicated RRC signalling.

Proposal 1: If relaxed measurement evaluation conditions are configured and subsequently handover condition is met, UE starts handover execution only when the source cell is about to get into Cell DRX/ DTX sleep.

Proposal 2: The time when the source cell starts (or about to start) Cell DRX/ DTX sleep can be determined by the UE based on the received start offset of Cell DTX/ DRX configuration received in UE dedicated RRC signalling.

Proposal 3: A network configured timer is used to supervise the freshness of fulfilled relaxed handover condition(s). When timer expires, handover condition(s) are no more considered as fulfilled and therefore UE needs to perform fresh evaluation, if required.

Observation 2: It may not always be possible to avoid including NES candidate cells in CHO.

Proposal 4: Two execution conditions can be configured to a single CHO candidate cell; one for “traditional” CHO condition and another for relaxed “NES” CHO condition.

Proposal 5: If NES candidate cell(s) are included in CHO, the serving cell can provide DRX/ DTX configuration for each candidate cell included in CHO.

Proposal 6: A UE determines if a candidate is sleeping around the time the UE arrives at the target.

Observation 3: Executing handover to a sleeping cell might lead to handover failures and RLF.

Observation 4: RAN2 has strived since release 8 to minimize RLFs, in its various studies/ work undertaken. And it is sensible if we do not introduce a new reason for RLF.

Proposal 7: UE informs the serving cell about un-availability of a target cell when there’s still no target and the source cell is about to enter sleep.

# References

[1]: R2-2304353	Summary of EMAIL DISC [121bis#xxx] on AI 7.3.5 Connected Mode Mobility Lenovo