3GPP TSG-RAN WG2 Meeting #123	R2-2307180

Toulouse, France, August 21-25, 2023

Agenda item:		7.4.2.3

Source:		Lenovo

Title:		 Cell Switch details

Document for:		Discussion and Decision

- Introduction

LTM based Cell Switch procedure requires early TA acquisition as well as new MAC CE based mobility procedure. This paper goes in the details of cell switch procedure and builds on RAN2 progress from the last meetings.

Following agreements were reached in RAN2#122[1]:

Dynamic grant can be used for RACH-less LTM, for the first UL data transmission to the target cell: - - the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch. - - upon cell switch decision, R2 assumes that the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant. Configured grant can be used for RACH-less LTM, for the first UL data transmission to the target cell, the UE selects the configured grant occasion, which is associated with the beam indicated in the LTM MAC CE (as set by source cell). FFS further optimization - For early TA acquisition for candidate Cells For PDCCH ordered early TA acquisition without RAR, there is no need for UE to maintain the TA timer for candidate cell (i.e. it is NW implementation to determine the TA validity), TA is given in the cell switch MAC CE (when available in the network). RAN2 doesn’t see a need for a solution with RAR in for Rel-18. Observation: Without RAR (without UE maintaining TA), the UE will need to do RACH for link recovery and/or conditional (if supported), which is acceptable in Rel-18 The UE determines to trigger RACH-less cell switch in MAC layer, if the LTM cell switch MAC CE provides the TA value (no RAR is assumed). We send LS to R1 - Can use legacy behaviour: P2: RAN2 assumes that network implementation allows speedy data recovery for RLC AM bearer at intra-DU LTM cell switch without specification impact. P3: The PDCP data recovery procedure can be applied to the RLC AM bearers for inter-DU LTM cell switch. - After discussion Will not support HARQ continue at LTM cell switch in this release.

- Discussion

Preparation for Cell Switching

Regarding early TA acquisition without RAR, after UE receives the PDCCH order from source cell, UE transmits preamble to the corresponding candidate cell based on the RO resource for early TA acquisition. Once the candidate cell receives the preamble, the candidate cell will calculate TA value and transmit early TA value to source cell/DU via CU. Like normal handover procedure where once UE detaches from source cell, synchronizes to target’s DL and initiates PRACH transmission, a separate MAC entity should be established.  LTM is used for PCell change and PSCell change. a separate MAC entity should be established for preamble transmission for Early TA acquisition.

There may be cases e.g., for early TA acquisition is to be started while there is an ongoing RA in serving cell. In this case we see that two RACHs may be allowed in parallel. Whether this left to UE capability or can be considered mandatory needs to be discussed at a later stage of RAN2 work.

A separate MAC entity is used for Early TA acquisition.

For early TA acquisition while there is an ongoing RA in serving cell, two RACHs are allowed in parallel. FFS: UE capability.

- Contents of Cell switch command

- According to one previous agreement, LTM candidate cell also can be used for recovery. Namely, once LTM cell switching fails and the suitable cell is LTM cell during cell selection of re-establishment procedure, UE also can perform LTM procedure. To increase the chances of mobility, LTM may contain more than one target cells that may be used for handover execution if the mobility to a previous one fails.

RAN2 to discuss if cell switching command may contain more than one target cells.

- After Cell Switching

When UE performs cell switch based on LTM command MAC CE, UE may reset MAC or partially reset MAC. If the source cell is still a candidate cell after cell switching, UE can continue to maintain TA for the source cell. Namely, keep the TA value and maintain timeAlignmentTimers (TAT).

UE continues to keep TA and maintain corresponding timeAlignmentTimers (TAT) for the original source cell if the source cell is still a candidate cell after cell switch.

- Conclusion

This paper went in the details of cell switch procedure and builds on both RAN1 and RAN2 progress from the last meeting. Following proposals are made:

- Proposal 1: A separate MAC entity is used for Early TA acquisition

- Proposal 2: For early TA acquisition while there is an ongoing RA in serving cell, two RACHs are allowed in parallel. FFS: UE capability

- Proposal 3: RAN2 to discuss if cell switching command may contain more than one target cells.

- Proposal 4: UE continues to keep TA and maintain corresponding timeAlignmentTimers (TAT) for the original source cell if the source cell is still a candidate cell after cell switch.

- References

[1]: Chairman notes for RAN2#122 meeting