3GPP TSG RAN WG2 Meeting #123	R2-2307187

Toulouse, France   22-26 May 2023

Agenda Item:	7.2.2 - Sidelink positioning

Source:	Fraunhofer IIS, Fraunhofer HHI, Ericsson

Title:	Preconfigured Assistance Data for UE Positioning in Hybrid Uu and PC5 scenarios

Document for:	Discussion & Decision

# Introduction

The WID [1] for study on expanded and improved NR positioning enhancements in Rel. 18 lists among others, the following as RAN2 objectives:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2]

In RAN2#122 Meeting [2], we reached the following agreements, which are relevant for session-based sidelink positioning.

Agreements: - SLPP over PC5-U/Uu will support reliable transport for at least unicast. FFS groupcast. Inform SA2 about our agreements on sidelink positioning, with “take into account” action. SLPP carried over NAS is used between UE and LMF. FFS on how to manage the session/transaction. - Anchor UE selection is supported by information about the candidate anchor UEs. At least the following list can be discussed for use in anchor UE selection: 1. UE roles 2. Supported positioning method 3. In coverage or not 4. RSRP 5. LOS/NLOS 6. Location 7. PLMN A normative requirement on which anchor UEs to select (e.g., ranking) will not be specified. RAN2 impact of this information to be determined. FFS which information would be determined statically/dynamically

In this contribution, we provide our views on next steps for sidelink positioning.

# Preconfigured AD with validity criteria for sidelink sessionless operation

In Rel. 17, RAN2 agreed on providing multiple instances of preconfigured AD to a UE, where each instance of AD is applicable to an area in the network. In case of positioning for V2X scenarios based on Uu interface, then the resulting scenario could be simplified as shown in Figure 1.

Figure 1: Example of preconfigured assistance data using RSU using Rel. 17 Uu-based positioning methods.

One of the advantages of utilizing UE as fixed anchors in vehicular scenarios (roadside units) or as fixed anchors in UE positioning indoor factory scenarios (fixed anchors) is to improve positioning accuracy through densification of anchors.

Observation 1: Densification of anchors help improve accuracy by improving DOP, availability of NLOS links and so forth.

If we consider self-localization of a UE, as depicted above in Figure 1 and anchor UEs were to be added to enable sidelink positioning, the UE would be able to receive the sidelink reference signal emitted by the anchor UEs and compute its position. The configuration of sidelink anchor UEs (road-side units) could be provided as preconfigured assistance data. In a hybrid scenario, this would look like Figure 2.

In a fast changing vehicular scenario, the assistance data can be provided to the UE ahead of time, and can be utilized by the UE for measurement when the UE enters the validity area, without having to request assistance data.

Figure 2: Example of application of preconfigured assistance data to a hybrid modus with RSU consisting of TRPs and anchor UEs (Sidelink).

If a UE is preconfigured with AD instances applicable to Area 1 and Area 2, where the UE either computes its own location or provides the measurements to the network via LPP, then active sessions between the sidelink UEs are not necessary. The location of sidelink UEs may be stationary (e.g. fixed RSUs) or semi-stationary (e.g. stationary devices deployed as anchors). Such assistance data can be preconfigured in the same manner as the assistance data for UL/DL positioning.

Observation 2: Sidelink positioning can be supported in sessionless mode when preconfigured assistance data is provided to the target UE, as long as anchors are at least semi-stationary.

Proposal 1: Providing sidelink reference signal configuration using  preconfigured assistance data (similar to Rel. 17) mechanism shall be supported.

In Rel. 17, we discussed validity criteria for preconfigured assistance data as a means for reducing latency. For Uu-interface, the TRPs which served as positioning anchors were fixed in space. The anchors in sidelink scenario may be static, semi-static or mobile. Accordingly, the assistance data shall reflect that some components of assistance data shall have a time-validity.

Proposal 2: Elements of preconfigured assistance data may be configured with time validity in addition to area validity.

# Signalling Aspects

In Rel. 17 preconfigured assistance data is provided by adding the assistanceDataValidityArea-r17 in the IE NR-DL-TDOA-ProvideAssistanceData-r16.

-- ASN1START

NR-DL-TDOA-ProvideAssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-AssistanceData-r16		NR-DL-PRS-AssistanceData-r16		OPTIONAL,	-- Need ON

nr-SelectedDL-PRS-IndexList-r16		NR-SelectedDL-PRS-IndexList-r16		OPTIONAL,	-- Need ON

nr-PositionCalculationAssistance-r16

NR-PositionCalculationAssistance-r16

OPTIONAL,	-- Cond UEB

nr-DL-TDOA-Error-r16				NR-DL-TDOA-Error-r16				OPTIONAL,	-- Need ON

...,

[[

nr-On-Demand-DL-PRS-Configurations-r17

NR-On-Demand-DL-PRS-Configurations-r17

OPTIONAL,	-- Need ON

nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17

NR-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17																				OPTIONAL,	-- Need ON

assistanceDataValidityArea-r17	AreaID-CellList-r17					OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

The same mechanism can be reflected in SLPP protocol, where the SL PRS configuration of one or more UE, together with the information about the area where the assistance data can be used.

-- ASN1START

NR-SL-TDOA-ProvideAssistanceData-r16 ::= SEQUENCE {

nr-SL-PRS-AssistanceData-r18		NR-SL-PRS-AssistanceData-r16		OPTIONAL,	-- Need ON

sl-AssistanceDataValidityCriteria	SL-ValidityCriteria					OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

In sidelink, the sidelink devices may have reachability less than that of a cell. To signal this, we add a possibility of signalling the validity condition not only by the list of cells (like in Rel. 17), but also consider an option that measurement made on one SL-PRS (for example, if the RSRP on a certain SL-PRS is above threshold) could indicate that the target UE is in the vicinity of one or more other anchor UEs and consider the configuration data from such UEs to be valid for measurement.

-- ASN1START

SL-ValidityCriteria-r18 ::= SEQUENCE {

validityCriteria 			CHOICE {

assistanceDataValidityArea-r17	AreaID-CellList-r17

sl-PRS-Measurement-r18		SEQUENCE {

sl-PRS-ID-Info-r18		SL-PRS-ID-Info-r16

rsrp-Threshold 			INTEGER(0..127)

}							  								  	OPTIONAL,		-- Need ON

}

expirationTime-r17				UTCTime							 		OPTIONAL, 	-- Need ON

...

}

-- ASN1STOP

In case of hybrid Uu + PC5 positioning, the LMF may have information about the location of fixed anchors and would be able to signal the target UE, the assistance data corresponding to sidelink anchors.

Proposal 3: In hybrid Uu + PC5 scenarios, the LMF provides preconfigured assistance data corresponding to UE anchors using preconfigured assistance data, where each instance of sidelink preconfigured assistance data is applicable in different portion of the network (e.g. V2X zone).

# Conclusions

In this contribution, we have made the following observations:

Observation 1: Densification of anchors help improve accuracy by improving DOP, availability of NLOS links and so forth.

Observation 2: Sidelink positioning can be supported in sessionless mode when preconfigured assistance data is provided to the target UE, as long as anchors are at least semi-stationary.

Based on the above observations, we make the following proposals:

Proposal 1: Providing sidelink reference signal configuration using  preconfigured assistance data (similar to Rel. 17) mechanism shall be supported.

Proposal 2: Elements of preconfigured assistance data may be configured with time validity in addition to area validity.

Proposal 3: In hybrid Uu + PC5 scenarios, the LMF provides preconfigured assistance data corresponding to UE anchors using preconfigured assistance data, where each instance of sidelink preconfigured assistance data is applicable in different portion of the network (e.g. V2X zone).

# References

| [1] | RP-223549, "New WID on Expanded and Improved NR Positioning," 3GPP, Online, 2022-12. |
| --- | --- |
| [2] | 3GPP RAN WG2#122, "Chairman's Notes on Positioning and Relay Session," 3GPP, Incheon, 2023-5. |