3GPP TSG-RAN WG2 Meeting #123	 R2-2307189

Toulouse, France, Aug 21-25, 2023

Agenda Item:	7.6.2.1

Souce:	MediaTek Inc.

Title:	On Disabling HARQ Feedback in IoT-NTN

Document for:		Discussion and Agreement

# Introduction

Rel-18 enhancements for IoT-NTN Work Item was approved at RAN Plenary #96e [1]. The corresponding work item has identified IoT-NTN Performance Enhancements to address remaining issues from Rel-17 as one key agenda. The work item objective (WID) has mentioned “Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates” [1]. Previous agreements in Rel-17 NR-NTN Work Item has agreed semi-statically disabling/enabling of downlink HARQ using RRC Signalling.

During RAN2#119, the following agreements have been made:

Agreements:

- Disabling DL HARQ feedback is supported for NB-IoT and eMTC NTN. FFS on UE capability

- For UL HARQ operation, introduce two HARQ modes, i.e., HARQ mode A and HARQ mode B in IoT NTN (both NB-IoT and eMTC NTN), similarly to NR NTN

- From RAN2 perspective, at least for eMTC, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signalling. FFS for NB-IoT (and especially for CP solution for NB-IOT).

In RAN2#119bis-e, RAN2 had the following agreements:

Agreements:

- For NB-IoT, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signaling (e.g. RRCConnectionSetup). This does not preclude other options (e.g. DCI-based). We can also revert this decision if requested by RAN1.

- Disabling HARQ feedback is supported for NB-IoT with single HARQ process, and it is up to eNB implementation whether to disable the HARQ feedback

- HARQ mode A/B for uplink transmission may be configured per UL HARQ process at least via UE specific RRC signalling for eMTC and NB-IOT NTN. We can also revert this decision if requested by RAN1

- RAN2 agree to take R17 NR NTN DRX solution as baseline for IoT NTN, e.g. for HARQ process with DL HARQ feedback disabled, the UE will not start the corresponding DL HARQ RTT timer.

- For NB-IoT NTN with single HARQ process when the HARQ feedback is disabled, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PDSCH reception (can still check whether the alternative to set the HARQ RTT timer to 0 also works)

- RAN2 agree to take R17 NR NTN DRX solution as baseline for IoT NTN, e.g. for HARQ process in HARQ mode B, the UE will not start the corresponding UL HARQ RTT timer.

- For NB-IoT NTN with single HARQ process in HARQ mode B, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PUSCH transmission (can still check whether other alternatives also work)

- The solutions of LCP restriction on allowed HARQ mode in NR NTN can be reused for eMTC NTN.

Working Assumption:

- Blind retransmission can be used in IoT NTN when HARQ feedback is disabled and when HARQ mode B is used (RAN2 assumes there is no spec change for this)

In RAN2#120, RAN2 had the following agreements:

Agreements:

1.	For NB-IoT NTN with single HARQ process when the HARQ feedback is disabled, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PDSCH reception plus 12 subframes.

2.	RAN2 understands that something needs to be added to consider the processing time also for inactivity timer of HARQ mode B. Continue the discussion on the details in the next meeting

3.	For eMTC, the following LCH to HARQ process mapping rules are supported:

1) LCH is mapped only to a HARQ process configured with HARQ mode A;

2) LCH is mapped only to a HARQ process configured with HARQ mode B;

3) If an LCH is not configured with a mapping rule, it may be mapped to any HARQ process (HARQ mode A or B).

4) If UL HARQ mode is not configured, LCH mapping rules are not supported (legacy behaviour)

4.	For eMTC, introduce allowedHARQ-mode for each logical channel, e.g. included in LogicalChannelConfig IE. 1.	An optional UE capability is introduced to indicate whether the UE supports disabling HARQ feedback for downlink transmission.

5.	An optional UE capability is introduced to indicate whether the UE supports HARQ Mode B and, for eMTC, the corresponding LCP restrictions for uplink transmission

In RAN2#121, RAN2 had the following agreements:

Agreements:

- For UE to report GNSS position fix time duration for measurement during the initial access, at least the following Msg5 message can be used:

RRCConnectionSetupComplete, RRCConnectionSetupComplete-NB,

RRCConnectionResumeComplete, RRCConnectionResumeComplete-NB,

FFS for RRCreestablishmentComplete and RRCConnectionReconfigurationComplete.

FS for Msg3

- FFS whether the UE can stay in RRC_CONNECTED state when current GNSS position becoming out-of-date if the UE has initiated a new measurement

- The value range {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity} introduced in R17 is reused for connected UE GNSS validation duration report, unless modified by RAN1.

- UE reports GNSS validity duration after GNSS measurement. FFS whether the UE reports every time or only if the validity duration changes. FFS if the duration is the remaining validity duration or the whole duration

In RAN2#121-bis-e, RAN2 had the following agreements:

Agreements:

1.	RAN2#121’s agreement is revised to “For NB-IoT NTN with single HARQ process when the HARQ feedback is disabled, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PDSCH reception plus 12 subframes plus deltaPDCCH” (Can further check in the NB-IoT session if anything needs to be done for legacy NB-IoT as well, as some timers don’t take deltaPDCCH into account)

2.	Wait for RAN1’s decision on the RRC signalling of enabling DCI-based solution to indicate HARQ feedback enabled/disabled, and the signalling granularity, e.g. per UE or per HARQ process

3.	On DCI indication overriding RRC configuration for the HARQ feedback enabled/disabled, wait for RAN1’s progress on DCI-based solution before discussing related DRX impact in RAN2.

4.	On DL multiple TB scheduling, wait for RAN1’s progress before discussing related DRX impact in RAN2.

5.	P4 in R2-2302557 is not agreed, i.e. no special handling for single HARQ process for eMTC.

6.	For eMTC NTN, a parameter harq-FeedbackEnablingforSPSactive could be configured for a UE. If harq-FeedbackEnablingforSPSactive is configured to enable HARQ feedback, UE reports ACK/NACK for the first SPS PDSCH after activation, regardless of if HARQ feedback is enabled or disabled corresponding to the first SPS PDSCH after activation.

7.	For a NB-IoT UE configured with a single HARQ process in HARQ mode B, send LS to RAN1 and ask for the “processing time for starting drx-InactivityTimer (i.e. start to monitor NPDCCH)”. (can further check the detailed wording of the question)

8.	Network implementation resolves the issue of ambiguity on start of DRX inactivity timer after the PUSCH transmission by not scheduling the NPDCCH back-to-back during the ambiguity period (i.e., Koffset – UE’s TA)

9.	Send LS to RAN1 to check for UL multiple TB scheduling, which UL HARQ mode combination(s) are to be supported.

10.	In the LS to RAN1, we don’t include a question on whether RAN1 intends to introduce the DCI-based solution for the UL HARQ mode

11.	UL transmission using SPS can be configured with HARQ mode B

12.	P1 in R2-2303713 is not agreed, i.e. do not enhance the LCP restriction based on uplinkHARQ-Mode for different RLC PDU types

13.	Send LS to RAN1 informing RAN2’s agreements and also including potential questions to be checked with RAN1

14.	Add one more question in the LS to check with RAN1 which of the below understandings is correct for the RAN1 agreement.

Understanding 1: For a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH for the same HARQ process in a period of Y=12(ms) from the end of reception of the NPDSCH.

Understanding 2: For a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH for all the HARQ processes in a period of Y=12(ms) from the end of reception of the NPDSCH.

15.	RAN2 further discuss whether UL transmission using PUR can be configured with HARQ mode B.

In this contribution we continue to discuss the FFS aspects related to HARQ enhancements.

# 2 Discussion

DL HARQ enhancement

RAN2 has made the following agreement in RAN2#121bis:

RAN2#121’s agreement is revised to “For NB-IoT NTN with single HARQ process when the HARQ feedback is disabled, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PDSCH reception plus 12 subframes plus deltaPDCCH” (Can further check in the NB-IoT session if anything needs to be done for legacy NB-IoT as well, as some timers don’t take deltaPDCCH into account)

The RAN2 agreement specified the condition of starting/restarting drx-inactivity timer is NB-IoT NTN with single HARQ process. Per my understanding, it indicates the case that only one DL HARQ process is configured.

RAN1 also has made a similar agreement:

Agreement For a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH in a period of Y=12(ms) from the end of reception of the NPDSCH.

Furthermore, RAN1 has confirmed in the LS [3] that the understanding 2 is the correct understanding:

Understanding 2: For a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH for all the HARQ processes in a period of Y=12(ms) from the end of reception of the NPDSCH.

Hence, we think the two HARQ processes case and multiple TBs case can also be defined for starting or restarting drx-InactivityTimer.

Proposal 1: When the NB-IoT UE is configured with two HARQ process or if the multiple TBs are scheduled, which HARQ feedback is disabled, start or restart drx-InactivityTimer in the subframe containing the last repetition of the corresponding PDSCH reception + 12 subframes + deltaPDCCH, where deltaPDCCH is the interval starting from the subframe containing the last repetition of the corresponding PDSCH reception plus 12 subframes to the first subframe of the next PDCCH occasion.

Two RRC parameters for HARQ feedback disabling has been defined in the 36.331 running CR [4]. downlinkHARQ-FeedbackDisabled-Bitmap is used to disable the DL HARQ feedback per HARQ process. downlinkHARQ-FeedbackDisabled-DCI indicates that DCI indication is used to directly indicate / override RRC configuration for disabling HARQ feedback. These two RRC parameters were carried by dedicated RRC signaling, like Msg4. When these parameters was decoded by RRC layer, the HARQ feedback should has been assembled by lower layer. The configuration of HARQ feedback disabling should not affect the assembled HARQ feedback.

And it is always beneficial to acknowledge the RRC signaling, especially for HARQ feedback disabling configuration.

Proposal 2:	The downlinkHARQ-FeedbackDisabled-Bitmap and downlinkHARQ-FeedbackDisabled-DCI does not apply to the dedicated RRC signaling(e.g., Msg4) which carries these configurations.

In rel-15, quick release of RRC connection is introduced for NB-IoT and eMTC:

36.331 5.3.8.3: NOTE 0: For BL UEs, UEs in CE and NB-IoT, when STATUS reporting, as defined in TS 36.322 [7], has not been triggered and the UE has sent positive HARQ feedback (ACK), as defined in TS 36.321 [6], the lower layers can be considered to have indicated that the receipt of the RRCConnectionRelease message has been successfully acknowledged.

Quick release of RRC connection depends on the positive HARQ feedback. If the HARQ feedback is disabled, it cannot work. According to the current agreement, the DCI-based HARQ feedback overridden mechanism is optional, so it cannot be counted on to enable HARQ feedback for a particular signaling. To keep it functional, HARQ feedback for RRCConnectionRelease should be always enabled despite the HARQ feedback disabling configuration. Furthermore, RRCConnectionRelease is a key signaling, it is worth to protect by HARQ feedback.

Proposal 3: At least for NB-IoT quick connection release case, HARQ feedback for RRCConnectionRelease is always enabled despite the HARQ feedback disabling configuration.

For the multi-TBs case for HARQ feedback disabling, RAN1 has specified that for DCI based configuration, HARQ is enabled or disabled for all TBs.

RAN1#113 Agreement for NB-IoT and LTE-MTC in CE Mode B, if multiple TBs is configured, for DCI-based HARQ enabling/disabling direct indication in multiple TBs scheduled by single DCI, the same indication is applied to all scheduled TBs, i.e. HARQ is enabled or disabled for all TBs.

For RRC signaling based configuration, there is also no point to support the case that some HARQ processes disabled corresponding to the scheduled multiple TBs are configured with enabled HARQ feedback and the others are configured with disabled HARQ feedback.

Proposal 4: If multiple TBs is configured, for RRC configuration of HARQ feedback disabling, HARQ is enabled or disabled for all TBs.

The RRC configuration for HARQ feedback disabling is per HARQ process. For multiple TBs, if HARQ is enabled or disabled for all TBs, one configuration for HARQ processes is enough. For NB-IoT, to keep the solution simple, RRC configuration for the HARQ process 0 can be used.

Proposal 5: For NB-IoT, when the multi-TB is configured, RRC configuration of HARQ feedback enabling/disabling for HARQ process 0 applies for both HARQ processes.

As indicated in the LS[2], RAN1 has specified a working assumption for the DCI based overridden mechanism, the NB-IoT UE does not wait for an RTT+3ms (i.e., till subframe n+Kmac+3 in TS36.213 section 16.6) before monitoring NPDCCH for the same HARQ process.

Agreement For single TB scheduled by DCI, - Working assumption 2 For Option 1 + Option 3 DCI based overridden mechanism, for a HARQ process configured as HARQ feedback disabled by per-HARQ process bitmap signaling and further reversed to HARQ feedback enabled by DCI, the NBIoT UE does not wait for an RTT+3ms (i.e., till subframe n+Kmac+3 in TS36.213 section 16.6) before monitoring NPDCCH for the same HARQ process (or monitoring any NPDCCH for the case of single HARQ process configuration). - Send an LS to RAN2 with the following contents: - RAN1 respectfully ask RAN2 for the feasibility of Working assumption 2 (taking into account potential RAN2 spec impact).

RAN1 also ask RAN2 for the feasibility of this working assumption.

This working assumption implies for the DCI override case, the network can directly re-scheduled another DL/UL transmission right after the HARQ ACK of the first DL transmission with a RTT+3ms delay. The impact on RAN2 spec will be the HARQ RTT timer. Since The UE does not wait for an RTT+3ms before monitoring PDCCH, RTT +3ms can be removed from HARQ RTT timer length. But at least 1ms should be keep for the TX-RX transition.

Proposal 6: Working assumption: For NB-IoT, a HARQ process configured as HARQ feedback disabled by per-HARQ process bitmap signaling and further reversed to HARQ feedback enabled by DCI, the HARQ RTT timer is set to k+1+N +deltaPDCCH.

UL HARQ enhancement

As indicated the LS [3] from RAN1, for HARQ mode B, the minimum interval between the last NPUSCH transmission and the next NPDCCH is 1ms.

RAN2’s question 1a: For an UL HARQ process with HARQ mode B for NB-IoT UEs, what is the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring for the same HARQ process? RAN1 response: For a NB-IoT UE operating with two HARQ processes, for an UL HARQ process with HARQ mode B, the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring for the same HARQ process is 1 ms. - Note: this implies a RAN1 specification change in Rel-18. For a NB-IoT UE operating with one HARQ process, for an UL HARQ process with HARQ mode B, the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring is 1 ms. - Note: this implies a RAN1 specification change in Rel-18.

Based on the information, we can propose the drx-inactivity timer for HARQ mode B in NB-IoT.

Proposal 7: For NB-IoT UE, an UL HARQ process in HARQ mode B, start or restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PUSCH transmission + 1 subframes + deltaPDCCH.

In the RAN2#121bis, companies have consensus on SPS configured with HARQ mode B.

UL transmission using SPS can be configured with HARQ mode B

There is almost no TU left for RAN1 to discuss DCI based UL HARQ mode, hence we propose at least in Rel-18, RAN2 confirms its previous agreement on UL HARQ mode configuration based on RRC signalling (similar like NR NTN) .

Proposal 8: RAN2 confirms the previous agreement, i.e., UL HARQ mode configuration is based on RRC signalling (similar like NR NTN).

For NB-IoT, according to 36.321, a configured uplink grant shall be used only for BSR or SPS confirmation transmission. For configured uplink grants for BSR, the HARQ Process ID is set to 0.

If the HARQ mode B is configured for the SPS, network can schedule a retransmission right after NPUSH before receiving the UL data. By this way, network can ensure a reliable transmission of BSR and SPS confirmation.

For NB-IoT, according to 36.321, for configured uplink grants for BSR, the HARQ Process ID is set to 0.

When the HARQ process 0 has been configured with HARQ mode B, and SPS has been configured, it can be considered as the SPS has been configured with HARQ mode B.

Proposal 9: For NB-IoT, if HARQ process 0 has been configured with HARQ mode B, and SPS has been configured, it can be considered as the SPS has been configured with HARQ mode B.

For eMTC, the UL SPS can be used to transfer user data. Like NB-IoT, if HARQ mode B is configured for the UL SPS, network can schedule a retransmission right after the PUSCH before receiving the UL data. Besides, network can also schedule a new transmission right after the PUSCH before receiving the UL data. By this way, network can ensure a reliable transmission of UL SPS data, or network can improve the UL data rate.

For eMTC, according to 36.321, the HARQ process ID varies in time. If HARQ mode B configuration for SPS follows the HARQ processes, and if not all the HARQ processes is configured with HARQ mode B, then HARQ mode of SPS varies in time. To keep the HARQ mode of SPS consistent, a sperate configuration of HARQ mode for SPS can be introduced.

Proposal 10: For eMTC, a new separate configuration of HARQ mode is used to control the HARQ mode of the UL SPS.

According to 36.321 5.4.2.1, for UL transmission with UL grant in RAR and for transmission using PUR, HARQ process identifier 0 is used. When the UL HARQ mode B has been configured to HARQ process 0, it is unclear whether HARQ mode B is feasible for Msg3 of ransom access in RRC connected mode.

If the HARQ mode B is not feasible for Msg3 in RRC connected mode, after the UE has sent the Msg3, according to 36.213 16.6, UE will stop monitor PDCCH for the same HARQ process for Kmac + 3ms. However, during this period, the network which has not received the Msg3, is possible to send DCI N0/N1 to this UE. To avoid losing that potential DCI, UE can monitor the PDCCH during the Kmac+3ms. HARQ mode B is beneficial for the Msg3 of random access in RRC connected mode.

Proposal 11: UL HARQ mode B for HARQ process 0 is applicable for the Msg3 of random access in RRC connected mode.

As indicated the LS [3] from RAN1, all 3 cases are transparent to RAN1.

RAN2’s question 2: For UL multiple TB scheduling, which of the following HARQ mode combinations does RAN1 intend to support for eMTC and NB-IoT? - Case 1: all HARQ processes corresponding to the scheduled multiple TBs are configured with HARQ mode A - Case 2: all HARQ processes corresponding to the scheduled multiple TBs are configured with HARQ mode B - Case 3: some HARQ processes corresponding to the scheduled multiple TBs are configured with HARQ mode A and the others are configured with HARQ mode B RAN1 response: Whether/how to support case 1, case 2 and/or case 3 for UL multiple TB scheduling for eMTC and NB-IoT is transparent to RAN1 and can be left to RAN2 decision.

Obviously, UE should support the case 1 and case 2.

For NB-IoT which does not has LCP restriction based on uplinkHARQ-Mode, UE will not distinguish which HARQ process is used for the UL data. Therefore, at least for NB-IoT, there is no use case for case 3.

Proposal 12: At least for NB-IoT, some HARQ processes corresponding to the scheduled multiple TBs are configured with HARQ mode A and the others are configured with HARQ mode B is not support

Since if multiple TBs is configured, UL HARQ mode is A or B for all TBs, similar to DL HARQ, only one HARQ mode configuration is needed. To keep the solution simple, for NB-IoT, the RRC configuration for the HARQ process 0 can be used.

Proposal 13: For NB-IoT, when the multi-TBs is configured, RRC configuration of UL HARQ mode for HARQ process 0 applies for both HARQ processes.

# 3 Conclusions

Proposal 1: When the NB-IoT UE is configured with two HARQ process or if the multiple TBs are scheduled, which HARQ feedback is disabled, start or restart drx-InactivityTimer in the subframe containing the last repetition of the corresponding PDSCH reception + 12 subframes + deltaPDCCH, where deltaPDCCH is the interval starting from the subframe containing the last repetition of the corresponding PDSCH reception plus 12 subframes to the first subframe of the next PDCCH occasion.

Proposal 2:	The downlinkHARQ-FeedbackDisabled-Bitmap and downlinkHARQ-FeedbackDisabled-DCI does not apply to the dedicated RRC signaling(e.g., Msg4) which carries these configurations.

Proposal 3: At least for NB-IoT quick connection release case, HARQ feedback for RRCConnectionRelease is always enabled despite the HARQ feedback disabling configuration.

Proposal 4: If multiple TBs is configured, for RRC configuration of HARQ feedback disabling, HARQ is enabled or disabled for all TBs.

Proposal 5: For NB-IoT, when the multi-TB is configured, RRC configuration of HARQ feedback enabling/disabling for HARQ process 0 applies for both HARQ processes.

Proposal 6: Working assumption: For NB-IoT, a HARQ process configured as HARQ feedback disabled by per-HARQ process bitmap signaling and further reversed to HARQ feedback enabled by DCI, the HARQ RTT timer is set to k+1+N +deltaPDCCH.

Proposal 7: For NB-IoT UE, an UL HARQ process in HARQ mode B, start or restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PUSCH transmission + 1 subframes + deltaPDCCH.

Proposal 8: RAN2 confirms the previous agreement, i.e., UL HARQ mode configuration is based on RRC configuration (similar like NR NTN).

Proposal 9: For NB-IoT, if HARQ process 0 has been configured with HARQ mode B, and SPS has been configured, it can be considered as the SPS has been configured with HARQ mode B.

Proposal 10: For eMTC, a new separate configuration of HARQ mode is used to control the HARQ mode of the UL SPS.

Proposal 11: UL HARQ mode B for HARQ process 0 is applicable for the Msg3 of random access in RRC connected mode.

Proposal 12: At least for NB-IoT, some HARQ processes corresponding to the scheduled multiple TBs are configured with HARQ mode A and the others are configured with HARQ mode B is not support

Proposal 13: For NB-IoT, when the multi-TBs is configured, RRC configuration of UL HARQ mode for HARQ process 0 applies for both HARQ processes.

# 4 References

- RP-221554, MediaTek, WID on IoT NTN Enhancements, RAN#96e, June 2022.

- R1-2306245, LS on NPDCCH monitoring restriction for NB-IoT NTN RAN1	LS in	Rel-18	IoT_NTN_enh-Core	To:RAN2

- R1-2306182, Reply LS on HARQ Enhancements RAN1	LS in	Rel-18	IoT_NTN_enh-Core	To:RAN2

- R2-23d06954 Running CR - Introduction of IoT NTN enhancements