3GPP TSG-RAN WG2 Meeting #123	R2-2307190

Toulouse, France, August 21-25, 2023

Agenda Item:	7.6.2.2

Souce:	MediaTek Inc.

Title:	Enhancements on GNSS operation

Document for:		Discussion and Agreement

Introduction

In RAN#98e [1], the revised WID on IoT NTN enhancements has been endorsed for Release 18. The work item aims to specify further enhancements for E-UTRA (LTE-RAN) based NTN (non-terrestrial networks).

The WID considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objective related to improved GNSS operations as follows:

Study and specify needed improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1, RAN2]

- NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements.

This contribution aims to discuss RAN2 aspects related to enhanced GNSS operations for IoT NTN.

2 Discussion

GNSS measurement reporting

Whether to introduce specific SR to GNSS validity duration MAC CE has been discussed in the RAN2#122, but no consensus has been reached.

For the network aperiodical trigger GNSS measurement, the measurement gap is assigned by the network. The network has the knowledge of when the measurement gap ends. The network can assume the UE has successfully done the GNSS measurement and send UL grant to UE after the measurement gap. In this way, random access can be skipped, and power consumption can be improved.

For the network configured UE autonomously tigger, according to the RAN1 agreement, the GNSS measurement is expected to be done after GNSS validity duration expired + potential delay. The Network should also have the knowledge of when the autonomous GNSS measurement timer ends. The network can assume the UE has successfully done the GNSS measurement and send UL grant to UE after the measurement gap.

Proposal 1: For network aperiodical trigger and UE autonomously trigger, after GNSS validity duration expired, GNSS measurement reporting does not trigger SR.

For the UE autonomously trigger during the DRX inactivity time, the network would have any idea when the GNSS measurement has been done. If UE does not trigger SR to report the remaining GNSS validity duration to the network, the network would trigger another GNSS measurement which should be avoided.

Proposal 2: For UE autonomously trigger during the DRX inactivity time, UE may trigger SR for GNSS measurement reporting.

MAC CE for Remaining GNSS Validity Duration Report has been agreed in the last RN2 meeting. This MAC CE should have priority higher than regular data as it is a signalling. The priority can be lower than other MAC CE signalling since this is not time sensitive.

Proposal 3: MAC CE for Remaining GNSS Validity Duration Report has priority higher than data from any Logical Channel, except data from UL-CCCH and lower than MAC control element for DCQR and AS RAI, with exception of when DCQR is to be included in Msg3.

The GNSS measurement is done in the upper layer. In Rel-17, GNSS related control is done by RRC layer. In Rel-18, It should remain this way. When the GNSS measurement is done, upper layer will notify RRC. RRC will trigger MAC to report the remaining GNSS measurement validity duration.

Proposal 4: RRC triggers MAC to report the remaining GNSS measurement validity duration upon completing the GNSS measurement.

Successful GNSS measurement

Once the GNSS measurement is done, GNSS validity duration timer must be restarted since that is when the validity duration begins.

Proposal 5: UE re-starts the GNSS validity duration timer upon successful GNSS measurement.

In the last RAN2 meeting, it has been agreed to add a note to state some AS operations are suspended when UE is performing GNSS measurement during GNSS measurement gap. But it is not discussed when the AS operations to be resumed. One alternative is to be done when the GNSS measurement gap/timer ends. The other options is to be resumed upon UE completing GNSS measurement. The latter is better as once the UE completes GNSS measurement during the measurement gap/timer, the AS operation interruption can be minimized.

Proposal 6: The UE AS operation should be resumed upon UE completing GNSS measurement.

UE capability

It has been agreed that UE needs to report the GNSS position fix in Msg5. But when the network does not support GNSS operation enhancement, the reporting of GNSS position fix is redundant.  Network can indication support of GNSS operation enhancement in the broadcasting signaling. When cell does not enable GNSS measurement enhancement UE does not report GNSS position fix duration.

Proposal 7: Network indication GNSS operation enhancement in SIB2. UE does not report GNSS position fix duration when cell does not enable GNSS measurement enhancement.

In Rel-17 the UE reports the remaining GNSS validity duration in Msg5. When the reported value is infinity, it implies two cases. The first case is the UE is able of performing the GNSS measurement during the DL/UL transmission. When the GNSS validity duration ends, UE can autonomously get a new GNSS position fix and it would not break the connection. Therefore, network can consider the GNSS validity duration of UE as infinity. The other case is that the UE is a stationary device, which is unlikely to move during the connection. There is no need to obtain a new GNSS position fix during the connection, hence network can consider the GNSS validity duration of the UE as infinity.

When it comes to Rel-18, in both cases mentioned above, the UE does not need to obtain a new GNSS position fix during the connection process, so there is no need to report the GNSS position fix time duration.

Proposal 8: UE does not report GNSS position fix time duration when the remaining GNSS validity duration is infinity.

GNSS measurement trigger

Since Rel-17, GNSS related function is controlled by RRC layer, this way can be remained in Rel-18. When MAC layer receives the GNSS measurement command MAC CE, MAC will indicate to RRC. RRC will urge upper layer to perform GNSS measurement.

Proposal 9: MAC indicates to RRC to require the UE to perform GNSS measurement after receiving GNSS Measurement Command MAC CE.

RAN1#113 has the following agreement:

Agreement For NB-IoT and eMTC, at least for the case where the network configuration does not include a periodicity (if supported), for autonomous GNSS re-acquisition, the UE may re-acquire GNSS autonomously during GNSS measurement timer, the start time of the autonomous GNSS measurement timer is based on the original GNSS validity duration. - FFS: additional delay and details of delay (if any), e.g. delay can be zero or can be equal to/larger than the duration X where UL transmission can be allowed after original GNSS validity duration expires without GNSS re-acquisition. - Note1: Autonomous GNSS re-acquisition mechanism is enabled or disabled by network. - Note2: The length of GNSS measurement timer can be configured by network and the length of GNSS measurement timer is equal to the latest reported GNSS position fix time duration for measurement when the length of GNSS measurement timer is not configured. - Note3: The autonomous GNSS re-acquisition can be periodic in certain conditions without further spec impact.

The configuration of the UE autonomously trigger can be done via RRC dedicated signaling.

Similar like the measurement gap for network aperiodical trigger, a time duration for GNSS measurement should be configured by the network. RAN1 expects that is done by a timer. A new RRC timer should be introduced accordingly. The default value of time duration can be the UE reported GNSS position fix duration.

Proposal 10: UE autonomously trigger GNSS measurement can be configured via RRC dedicated signaling.

Proposal 11: A new RRC timer is configured by RRC signaling where UE can re-acquire GNSS position fix autonomously. FFS for details of the RRC signaling.

GNSS measurement during DRX inactivity duration

RAN1#111 has made the following agreement:

Agreement The UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement - FFS based on configured timing

RAN1#112bis has sent LS to RAN2:

| Conclusion From RAN1 perspective, UE is not forbidden to autonomously re-acquire GNSS position fix during inactive state of Connected DRX. - Note: The configured DL/UL transmissions during inactive state of Connected DRX should not be impacted - Note: details are up to RAN2 Send an LS to RAN2 for the conclusion. |  |
| --- | --- |

According to RAN1’s agreement, besides the network trigger, UE may re-acquire GNSS autonomously when configured by the network. The timing of UE autonomously trigger can be:

- DRX inactive time before GNSS validity duration expiry

- After GNSS validity duration expiry

- Time X delay after GNSS validity duration expiry

For the case 1, as long as the inactive time of C-DRX is longer than GNSS position fix duration, and there is no overlapping PDSCH/PUSCH transmission, UE can utilize this period to autonomously start GNSS measurement without any interruption on the connection. Therefore, it is better to start GNSS measurement before any possible network trigger and before GNSS validity duration expiry. But it is also not beneficial to start GNSS measurement too early. Ideally, UE should start GNSS measurement somewhere near the validity duration expiry. The exact time of UE start GNSS measurement during inactive state of C-DRX can be left for UE implementation.

Proposal 12: UE can autonomously start GNSS measurement during the inactive state of C-DRX.

Proposal 13: The exact time of starting GNSS measurement during the inactive state of C-DRX can be left for UE implementation.

The time point UE moves to idle

When should the UE leave RRC connected and move to RRC idle? In general, UE should move to RRC idle when failed to acquire new GNSS position fix. The exact UE behavior should depend on how the GNSS measurement is triggered.

- If the GNSS measurement is started by network aperiodically trigger, UE moves to RRC idle when UE failed to re-acquire GNSS position fix. Because when network send the trigger, it means network found that it is necessary to re-acquire GNSS at a certain point of time, otherwise the UL timing/frequency synchronization may not be maintained. If it is failed to reacquire the GNSS position fix, there is no point to keep the connection.

- If UE was configured with UE autonomously acquisition after GNSS validity duration expiry + X, UE moves to RRC idle when UE failed to re-acquire GNSS position fix after GNSS validity duration expiry + X. The time of GNSS validity duration expiry + X is the last minute that UE think the UL timing/frequency synchronization can be maintained. If it is failed to reacquire the GNSS position fix, there is no point to keep the connection.

- If UE failed to re-acquire the GNSS position fix during the inactive state of C-DRX, UL timing/frequency synchronization can still be maintained, thus another attempt of GNSS measurement triggered by network or UE can be allowed. It is possible that the GNSS satellites were temporarily blocked, another attempt of GNSS measurement is beneficial. UE does not move to idle when UE failed to re-acquire the GNSS position fix during the inactive state of C-DRX.

- If there is no network aperiodically trigger and network configuration of UE autonomously acquisition of GNSS position fix after GNSS validity duration expiry + X, UE moves to idle when GNSS validity duration expired.

Proposal 14: If network aperiodically trigger the GNSS measurement, and UE cannot re-acquire the GNSS position fix before the end of the GNSS measurement gap, UE moves to RRC_IDLE.

Proposal 15: If UE failed to autonomously re-acquire the GNSS position fix (GNSS measurement timer expired), UE moves to RRC_IDLE.

Proposal 16: If UE failed to autonomously re-acquire the GNSS position fix during the inactive state of C-DRX, UE does not move to idle.

Proposal 17: If there is neither network aperiodically trigger nor network configuration of UE autonomously GNSS measurement, UE moves to RRC_Idle when GNSS validity duration expired.

3 Conclusions

Proposal 1: For network aperiodical trigger and UE autonomously trigger, after GNSS validity duration expired, GNSS measurement reporting does not trigger SR.

Proposal 2: For UE autonomously trigger during the DRX inactivity time, UE may trigger SR for GNSS measurement reporting.

Proposal 3: MAC CE for Remaining GNSS Validity Duration Report has priority higher than data from any Logical Channel, except data from UL-CCCH and lower than MAC control element for DCQR and AS RAI, with exception of when DCQR is to be included in Msg3.

Proposal 4: RRC triggers MAC to report the remaining GNSS measurement validity duration upon completing the GNSS measurement.

Proposal 5: UE re-starts the GNSS validity duration timer upon successful GNSS measurement.

Proposal 6: The UE AS operation should be resumed upon UE completing GNSS measurement.

Proposal 7: Network indication GNSS operation enhancement in SIB2. UE does not report GNSS position fix duration when cell does not enable GNSS measurement enhancement.

Proposal 8: UE does not report GNSS position fix time duration when the remaining GNSS validity duration is infinity.

Proposal 9: MAC indicates to RRC to require the UE to perform GNSS measurement after receiving GNSS Measurement Command MAC CE.

Proposal 10: UE autonomously trigger GNSS measurement can be configured via RRC dedicated signaling.

Proposal 11: A new RRC timer is configured by RRC signaling where UE can re-acquire GNSS position fix autonomously. FFS for details of the RRC signaling.

Proposal 12: UE can autonomously start GNSS measurement during the inactive state of C-DRX.

Proposal 13: The exact time of starting GNSS measurement during the inactive state of C-DRX can be left for UE implementation.

Proposal 14: If network aperiodically trigger the GNSS measurement, and UE cannot re-acquire the GNSS position fix before the end of the GNSS measurement gap, UE moves to RRC_IDLE.

Proposal 15: If UE failed to autonomously re-acquire the GNSS position fix (GNSS measurement timer expired), UE moves to RRC_IDLE.

Proposal 16: If UE failed to autonomously re-acquire the GNSS position fix during the inactive state of C-DRX, UE does not move to idle.

Proposal 17: If there is neither network aperiodically trigger nor network configuration of UE autonomously GNSS measurement, UE moves to RRC_Idle when GNSS validity duration expired (same like legacy behavior).

4 References

- RP-221554, MediaTek, WID on IoT NTN Enhancements, RAN#96e, June 2022.