3GPP TSG-RAN WG2 Meeting #123	R2-2307196

Toulouse, France, Augest 21st-25th, 2023

Agenda item:	7.5.4.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on PDU set discarding for XR traffic

Document for:	Discussion and Decision

Introduction

Two kinds of PDU set discarding have been agreed for XR traffic, i.e., timer-based discarding [1] and PSI-based discarding [2] in case of congestion.

PDU set discard is modelled using the existing PDCP discard timer for the uplink. The timer is in network control.

[Agreed in RAN2#121 bis about the timer-based discarding]

Network indicates UE to apply PSI-based XR discard mechanism via dedicated signalling.

[Agreed in RAN2#122 about the PSI-based discarding]

However there are still many details that need to be resolved, e.g.:

- how to use the existing PDU level discard timer to realize the PDU set level discarding,

- is the NW indication about PSI-based discarding one-shot or also applies to the subsequent packets,

- do we need to consider 16 PSIs’ differentiated handling for PSI-based discarding?

In the remainder of this document, we will discuss these issues in detail.

Discussion

- 2.1 General points

An issue that was raised in the past was whether any enhancements to layers below PDCP are needed for XR PDU set discarding. If an XR RLC PDU/segment has already been delivered to MAC layer, since the XR RLC PDU/segment may be multiplexed into a TB together with other non-XR data, the MAC layer should not discard the TB even if the associated XR PDCP SDU/PDU has been discarded. For RLC layer, today the RLC entity discards the indicated RLC SDU only if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. In fact, for RLC UM, nothing would really prevent those segments which have not been submitted to the lower layers to be discarded at RLC layer. However, there is no such mechanism currently because the gain is limited while keeping UM and AM mode operation aligned simplifies the UE implementation. For this reason, we prefer to keep the existing principle.

- No additional triggers are introduced for PDU/SDU discarding at RLC and MAC layers due to PDCP PDU set discarding mechanism.

Another issue is about the receiving side. Sometimes the transmitting side may actively discard some PDUs and process the subsequent PDUs, which will lead to the SN gap. If the receiving side does not get any notification of the PDU discarding, but is still waiting for those discarded PDUs, then even the subsequent PDUs have been received successfully, those PDUs will be stalled in the receiving window.

- RAN2 should resolve the receiving window stall issue caused by PDCP PDU set discarding.

- 2.2 Timer-based PDU set discarding

A PDU set will not be valid forever and its validity is related to PSDB. Moreover, as RAN2#121 bis has agreed [1], the timer based PDU set discarding should reuse the existing PDCP discard timer.

PDU set discard is modelled using the existing PDCP discard timer for the uplink. The timer is in network control.

Then there are 2 possible solutions:

Option 1: The length of PDCP discard timer varies for different PDUs.

For example, suppose the PSDB is 10 ms, the 1st PDU arrives at time 0 and the associated PDCP discard timer is set to be 10 ms, 3ms later the 2nd PDU arrives, then its associated PDCP discard timer will be 7ms (i.e., the current value of the previous PDU’s PDCP discard timer), another 2ms later the 3rd PDU arrives and its associated PDCP discard timer is set to be 5ms, and so on. By doing so, all of the PDCP discard timers associated to the PDUs of the same PDU set will expire at the same time.

Option 2: Expiry of the first PDCP discard timer triggers the discard of the whole PDU set.

In this option, once the PDCP discard timer of the 1st PDU expires, all the PDUs within the same PDU set shall be discarded. In other words, once the 1st PDU’s discard timer expires, the discard timers of the other PDUs belonging to the same PDU set are regarded as expired.

These two options can achieve the same goal, but option 2 reuses the current PDCP procedure to the largest extent and is much simpler from both specification and implementation point of view.

- When PDU set level discarding is enabled, the expiry of the 1st PDU’s discard timer triggers discard of the whole PDU set.

- 2.3 PSI based PDU set discarding in case of congestion

SA4 agreed to include a 4 bit long PSI field in the RTP extension header [4] which means there are 16 levels of PSI available. However, as expressed by the majority of companies during the previous discussions, there is no need for RAN2 to consider such granular differentiation when designing PSI-based discarding mechanism for UL. It is unlikely that a certain application will differentiate packets into all of the 16 groups and it would be very complex for the gNB to manage the congestion mechanism with such granularity. For example, the gNB would have to be aware of the of the data volume distribution among the PSIs for each application to be able to decide which PSIs to discard at a certain time. Furthermore, there is no evidence that the finer PSI-based PDU set discarding will help in alleviating the congestion quicker. Hence in UL, it should be sufficient to specify a 2 PSI-groups based PDU set discarding in Rel-18. Moreover, as sub-clause 4.4.2.6.2 of TS26.522 [4] explained, application fills in the PSI field from the encoding and decoding perspective. For example, if a PDU set contains 100% information, the PSI will be filled as 0, if a PDU set contains 90% information, the PSI will be filled as 1, and so on. For different applications, if the same protocol is used (e.g., H.264, HEVC), they will follow the same principle to fill in the PSI field. However, different applications may have different understanding of high/low importance. For instance, for remote surgery video, PSI 0~14 are high importance, while for AR gaming, only PSI 0~5 are high importance. So in UL, how to classify 16 PSIs into the importance groups should be decided by the UE based on the implementation. In DL, how to use the PSI information conveyed in GTP-U header to perform the PDU set discarding is up to network implementation.

PDU Set Importance [PSI] (4 bits): The PDU Set Importance field indicates the importance of this PDU Set compared to other PDU Sets within the same QoS flow. Lower values shall indicate a higher importance PDU Set with the highest importance PDU Set indicated by 0 and the lowest importance PDU Set indicated by 15.

- For uplink, PDU Sets are classified into two PDU set importance groups (i.e., high and low importance) based on UE implementation.

When it comes to how the PSI-based mechanism should work in detail, the companies expressed their initial views in the offline discussion that took place during RAN2#122 meeting and was summarized in [5]. There were two main families of solutions:

- Direct discard of all PDU sets belonging to low importance group – in this option, when the network enables the PSI based discard mechanism, the UE discards all low-importance PDU sets until the network disables the mechanism.

- Timer-based discard – in this option, the PDCP discard timer can be reused for congestion control mechanism. High and low importance PDU sets are configured with different values of PDCP discard timer. According to the congestion status and other policies, the gNB can temporarily adjust the PDCP dicardTimer length of low importance PDU sets to be a shorter value.

We think there is a very significant advantage of option 2 over option 1. In option 1, when the mechanism is enabled, the UE discards all of its low importance packets which may lead to a very significant service quality deterioration which in fact may be equal to releasing the service completely. For example, if an application encodes only every 10-th frame as a high-importance PDU set, it would mean that 90% of the frames are discarded by the UE. With option 2, the network still has a possibility to deliver at least part of low-importance PDU sets and hence there is a much higher chance the service is still usable. Furthermore, if the situation really requires all low importance PDU sets to be discarded, in option 2, the PDCP discardTimer of low importance PDU set group can be set to 0, leading to low importance PDU sets being discarded immediately and thus achieving the same outcome as with option 1. Another advantage is that discard operation is based on an existing PDCP discard timer and hence the specifications and implementation impact is minimized.

Observation 1: “Direct PSI discard” mechanism will often lead to XR service being unusable and thus has no benefit over simply releasing the UE’s connection.

Observation 2: Timer-based PSI discard gives network much more flexibility and allows delivery of at least part of low-importance PDU sets making the service still usable.

Observation 3: Timer-based PSI discard allows reusing the existing PDCP specifications / PDCP discard timer operation to a large extent.

Considering these observations, we propose the following:

- Introduce PDU set importance based PDCP discard timer values, which the network may enable to alleviate the network congestion.

- Considering UEs are unable to detect the congestion by themselves, RAN2#122 has agreed that network indicates UEs to apply the PDU set discarding in case of congestion [2].

Network indicates UE to apply PSI-based XR discard mechanism via dedicated signalling

- Related to this, it needs to be discussed whether the reconfiguration of the discard timers should be done by the RRC protocol or rather in lower layers, e.g. using a MAC CE. Using the RRC protocol allows for maximum reuse of the current reconfiguration mechanism and minimizes the specifications impact. On the other hand, it may be impossible for the gNB to timely adjust the setting of the discard timer in line with the dynamically changing congestion situation. Hence, we think the most efficient way is to pre-configure multiple timer values by RRC protocol, and use a MAC CE to switch the currently used one.

- Network pre-configures multiple PSI group-specific PDCP discard timer values by RRC, and activates one via MAC CE based on the current congestion status.

- Another open issue is related to the FFS captured during RAN2#122 [2], i.e. if the network indication is one-shot or also applicable to the subsequent packets. We think the network indication cannot be one-shot as it would require the network to keep on sending dedicated signalling (to convey the indication) when the network is already congested.

- Once the network enables PDU set importance based discarding, it is effective until the UE is configured to stop this mechanism.

Conclusion

In this contribution, we have discussed the PDU set discarding for XR traffic and made the following observations and proposals:

Observation 1: “Direct PSI discard” mechanism will often lead to XR service being unusable and thus has no benefit over simply releasing the UE’s connection.

Observation 2: Timer-based PSI discard gives network much more flexibility and allows delivery of at least part of low-importance PDU sets making the service still usable.

Observation 3: Timer-based PSI discard allows reusing the existing PDCP specifications / PDCP discard timer operation to a large extent.

- No additional triggers are introduced for PDU/SDU discarding at RLC and MAC layers due to PDCP PDU set discarding mechanism.

- RAN2 should resolve the receiving window stalled issue caused by PDU set discarding.

- The expiry of the 1st PDU’s discard timer triggers discard of the whole PDU set.

- For uplink, PDU Sets are classified into two PDU set importance groups (i.e., high and low importance) based on UE implementation.

- Introduce PDU set importance based PDCP discard timer values, which the network may enable to alleviate the network congestion.

- Network pre-configures multiple PSI group-specific PDCP discard timer values by RRC, and activates one via MAC CE based on the current congestion status.

- Once the network enables PDU set importance based discarding, it is effective until the UE is configured to stop this mechanism.

Reference

- Chairs note of RAN2#121 bis, https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_121bis-e/Inbox/Chairs_Notes/RAN2-121bis-e%20LTE%20MUSIM%20QoE%20XR%20(Tero)_2023-04-26-EOM.docx

- Chairs note of RAN2#122, https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_122/Inbox/Chairs_Notes/RAN2-122%20LTE%20MUSIM%20QoE%20XR%20(Tero)_2023-05-26-EOM.docx.

- 3GPP TS 23.501 V18.2.2, System architecture for the 5G System (5GS).

- 3GPP TS 26.522 V0.1.0, 5G Real-time Media Transport Protocol Configurations.

- R2-2306565, Offline 204 on XR Discard, Nokia (Rapporteur)