3GPP TSG-RAN WG2 Meeting #123                                                                  R2-2307208

Toulouse, France, August 21-25, 2023

Source:	CATT

Title:	Remaining Issues on RACH-less LTM

Agenda Item:	7.4.2.1

Document for:	Discussion and Decision

Introduction

This contribution discusses remaining issues on RACH-less LTM.

The contribution is organized as the following. In section 2, we discuss open issues on the following aspects.

- Beam application time

- Scenarios for Beam selection

- Configured grant

- UE Determination of the successful reception of its first UL data by NW

The proposals are summarized in Section 3.

Discussion

  - Beam application time

In the received LS [1] from RAN1, Beam application time for LTM has been supported according to RAN1 agreements. And RAN1 ask RAN2 to provide information on components to be considered for defining the beam application time.

Agreement - For the beam application time for Rel-18 LTM, - Beam application time is supported, and starts after the last symbol of the PUCCH or PUSCH carrying the HARQ-ACK for the PDSCH which carries MAC-CE containing cell switch command with the beam indication for the target cell(s) - FFS: reference SCS, i.e. serving cell and/or target cell - At least the following components are further studied to define the beam application time - Whether TCI state activation is received before/together with cell switch command - Legacy values, i.e. and BeamAppTime-r17 - RF retuning time when inter-frequency switch is performed, which is up to RAN4 - Whether the target cell is one of the current serving cells - Cell switching time, which is defined by RAN2 and RAN4, may or may not include the potential components of beam application time above. RAN1 respectfully asks RAN2 and RAN4 to provide their feedback to complete the RAN1 work on beam switching time for LTM.

When UE receives the cell switch command, UE need to apply the target cell configuration including the L2/3 configuration and L1 configuration (i.e., mentioned in the Tprocessing,2 in 38.300 running CR),

Table X.1-1: Components of Mobility Latency

| Component | Meaning | Value |
| --- | --- | --- |
| TRRC | Processing time for RRCReconfiguration carrying candidate configurations | Up to [10] ms |
| Tprocessing,1 / Tprocessing,2 | Time for UE processing, before and after cell switch command, respectively. This may include L2/3 reconfiguration, RF retuning, baseband retuning, security update if needed, etc. | Up to [20] ms for same FR Up to [40] ms for different FR |
| Tmeas | Measurement delay (from target appears to cell switch command) | - |
| Tcmd | Time for processing L1/L2-command (HARQ and parsing) | Up to [5] ms |
| Tsearch | Time required to search the target cell | 0ms (if cell is known) Up to [60] ms (if cell is unknown) |
| TΔ | Time for fine tracking and acquiring full timing information | SMTC periodicity (typ. [20] ms) |
| Tmargin | Time for SSB or CSI-RS post-processing | Up to [2] ms |
| TIU | interruption uncertainty in acquiring the first available PRACH occasion in the new cell | Typ. [15] ms |
| TRAR | Time for RAR delay | Typ. [4] ms |
| Tfirst-data | Time for UE performs the first DL/UL reception/ transmission on the indicated beam of the target cell, after RAR | - |

The indicated beam of the target cell can only be applied when UE is ready on the target cell (i.e., the application of all the target cell configuration has been completed), so the beam application time should be long enough to allow UE to complete the application of all the target cell configuration.

Observation 1: Beam application time should be long enough to allow UE to complete the switching to the target cell.

Proposal 1: Beam application time covers the time for UE to apply the target cell configuration including L2/3 configuration and L1 configuration.

  - Scenarios for beam indication

RAN1 has agreed to indicate activated TCI state in the cell switch command as below,

Agreement A UE can be indicated and activated a single joint TCI state or a pair of UL/DL TCI state in the cell switch command.

It is straightforward that the TCI state indicated in cell switch command can be for RACH-less LTM case as UE will directly send the first UL on target cell according to the indicated TCI state when LTM triggers.

However, it is still not clear whether UE may receive the TCI state in cell switch command for RACH-based LTM case.

For CFRA, how to indicate the CFRA resource is under discussion. Considering the CFRA resource includes the SSB index, preamble index, RO. Even though the network wants to indicate the selected CFRA resource in the cell switch command, it is more direct to include the SSB index in the cell switch command but not the TCI state.

For CBRA, in legacy the beam used for the first UL is determined based on the association between SSB and the RACH occasion. UE could determine the RACH resource according to the measurement result itself, furthermore UE could choose the RACH resource considering the time of the usable RO arrival. It doesn’t need to include SSB index to restrict the UE behavior, and it doesn’t need to include the TCI state for CBRA either.

Therefore, it is needed to include TCI state in the cell switch command only for RACH-less LTM case

Proposal 2: The TCI state is indicated in the cell switch command only for RACH-less LTM case.

Configured grant

For the grant used for the first UL transmission in RACH-less LTM case, both dynamic grant and configured grant way were agreed in last RAN2 meeting. For configured grant case, there is a FFS on whether further optimization is needed as below,

Configured grant can be used for RACH-less LTM, for the first UL data transmission to the target cell, the UE selects the configured grant occasion, which is associated with the beam indicated in the LTM MAC CE (as set by source cell). FFS further optimization

Hence, we consider whether/what optimization is needed for configured grant case.

In RAN2#122 meeting, it was agreed that for dynamic grant case, source DU needs to informs the target DU about the selected beam as below,

Dynamic grant can be used for RACH-less LTM, for the first UL data transmission to the target cell: - - the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch. - - upon cell switch decision, R2 assumes that the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant.

It is not clear yet whether the beam indication between DUs is needed for configured grant case. One may argue that beam indication for configured grant case is not necessary as the target DU can monitor the configured grants on all the candidate beams of all the candidate cells and determine the selected beam if target DU receives the UL data from UE on one candidate beam. It may be workable but unnecessarily increase the load of target DU. Target DU can only monitor the configured grant on the selected beam if it is indicated by source DU.

Besides, source DU cannot distinguish between dynamic grant and configured grant case as configured grant is included in the candidate configuration container. To let source distinguish between dynamic grant and configured grant case, additional network interface is needed.

So beam indication between source DU and target DU should be supported regardless of dynamic grant case or configured grant case.

Therefore, it is proposed,

Proposal 3: For RACH-less LTM, beam indication between source DU and target DU should be supported regardless of dynamic grant case or configured grant case.

For configured grant, some companies propose to further indicate the grant to be used in cell switch command. There is no need as the agreed mechanism is already sufficient, i.e., NW preconfigured the configured grant which is associated with one candidate beam of a candidate cell. When LTM triggers, the UE can uniquely determine the configured grant according to the selected beam in the LTM cell switch command.

Therefore, we propose that,

Proposal 4: For configured grant case, there is no need to further indicate the grant resource in the LTM cell switch command.

UE determination of the successful reception of its first UL data by NW

In last meeting, it is agreed that,

For RACH-less LTM, the UE considers that LTM execution procedure is successfully complete when the UE determines the NW has successfully received its first UL data.

Then during the offline [R2-2304214], it also discussed how the UE determines the successful reception of its first UL data by NW, the following options are on the table already.

Option 1: RLC ACK of RRCReconfigurationComplete

Option 2: C-RNTI addressed PDCCH

Option 3: DL Contention Resolution MAC CE

Since Option 1 may cause extra delay, it should be excluded. For option 2, for dynamic scheduling grant mechanism, it couldn’t distinguish the C-RNTI addressed PDCCH is used for scheduling the first UL grant, or it is used for scheduling other transmission after the NW successfully received UE’s first UL grant. And a unified solution should be chosen for both dynamic scheduling grant and CG mechanism. Hence option 2 is not suitable either. So option 3(i.e., to reuse the LTE RACH-Less HO) is more suitable.

Proposal 5: For RACH-less LTM, for UE to determine the successful reception of its first UL data by NW, LTE RACH-less HO solution (i.e., the UE Contention Resolution Identity MAC CE) is reused.

Conclusion

Based on the discussion in section 2, the observations and proposals are summarized as follows:

Observation 1: Beam application time should be long enough to allow UE to complete the switching to the target cell.

Beam application time

Proposal 1: Beam application time covers the time for UE to apply the target cell configuration including L2/3 configuration and L1 configuration.

Scenarios for beam indication

Proposal 2: The TCI state is indicated in the cell switch command only for RACH-less LTM case.

Configured grant

Proposal 3: For RACH-less LTM, beam indication between source DU and target DU should be supported regardless of dynamic grant case or configured grant case.

Proposal 4: For configured grant case, there is no need to further indicate the grant resource in the LTM cell switch command.

UE determination of the successful reception of its first UL data by NW

Proposal 5: For RACH-less LTM, for UE to determine the successful reception of its first UL data by NW, LTE RACH-less HO solution (i.e., the UE Contention Resolution Identity MAC CE) is reused.

Reference

[1] R2-2302412/R1-2302194, LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM