3GPP TSG-RAN WG2 Meeting #123	R2-2307214

21 Aug – 25 Aug, 2023, Toulouse

Agenda Item	: 7.15.2 (NR_SL_enh2)

Source	: LG Electronics Inc.

Title	: Discussion on SL consistent LBT failure

Document for	: Discussion and Decision

1.	Introduction

This contribution presents our views on SL consistent LBT failure for SL-U.

2.	Discussion

2.1 SL Consistent LBT failure cancellation (mode 2, RRC ide/inactive UE)

RAN2 discussed SL Consistent LBT failure recovery for mode 2 UE (i.e. idle/inactive UE) in the last meeting, and discussed whether mode 2 UE can cancel triggered SL C-LBT failures under the following conditions, but did not reach an agreement.

RAN2 discussion for SL-C LBT failure cancellation in #122 meeting - Mode 2 (RRC idle/inactive UE) - Upon resource pool (re)selection (P11:5089) - SL consistent LBT failure recovery parameters are reconfigured (P18:4831) - PC5 MAC reset (P18:4831) - Reconfiguration of resource pool(s) that include SL RB set(s) with triggered but not cancelled SL consistent LBT failure (P18:4831) - Transition between RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode (P18:4831) - RA mode change (P7a:5227) - Reconfiguration of RB sets (P3:4934) - Based on timer (P17:4831) - Based on measured channel condition (P17:4831)

We present our view on this issue in this contribution.

Firstly, mode 2 UE (i.e. Idle/Inactive UE) can reuse the RB set that declared SL-C LBT failure based on the timer. For example, mode 2 UE starts a timer when it declares SL-C LBT failure, and when the timer expires, it can cancel triggered SL C-LBT failure and reuse the RB set that declared SL-C LBT failure.

In addition, depending on the channel condition, mode 2 UE can reuse the RB set that declared SL-C LBT failure. For example, mode 2 UE can cancel the triggered SL C-LBT failure and reuse the RB set that declared the SL-C LBT failure when the number of SL LBT failure is detected below the threshold for a certain period of time.

Proposal 1. Mode 2 UE (i.e. Idle/Inactive UE) can cancel triggered SL consistent LBT failure based on the timer approach.

Proposal 2. Mode 2 UE (i.e. Idle/Inactive UE) can cancel triggered SL consistent LBT failure based on measured channel status.

2.2 LCP order of SL LBT failure MAC CE

RAN2 agreed on UE behaviour of reporting an SL LBT Failure MAC CE to gNB when UE declares SL C-LBT Failure. Therefore, RAN2 should additionally define the LCP order of this SL LBT Failure MAC CE.

We suggest that the LCP order of SL LBT Failure MAC CE be defined lower than the LCP order of LBT Failure MAC CE as shown below;

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	MAC CE for LBT failure;

- 	MAC CE for SL LBT failure;

-	MAC CE for Timing Advance Report;

Proposal 3. LCP order of SL LBT Failure MAC CE be defined lower than the LCP order of LBT Failure MAC CE as shown below;

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	MAC CE for LBT failure;

- 	MAC CE for SL LBT failure;

-	MAC CE for Timing Advance Report;

3.	Conclusion

This contribution presented remaining issues on SL consistent LBT failure for SL-U, which can be summarized as follows:

Proposal 1. Mode 2 UE (i.e. Idle/Inactive UE) can cancel triggered SL consistent LBT failure based on the timer approach.

Proposal 2. Mode 2 UE (i.e. Idle/Inactive UE) can cancel triggered SL consistent LBT failure based on measured channel status.

Proposal 3. LCP order of SL LBT Failure MAC CE be defined lower than the LCP order of LBT Failure MAC CE as shown below;

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	MAC CE for LBT failure;

- 	MAC CE for SL LBT failure;

-	MAC CE for Timing Advance Report;

4.	Reference

[1] #122 meeting report from session on NR SL, Samsung