3GPP TSG-RAN WG2 Meeting #123	R2-2307215

21 Aug – 25 Aug, 2023, Toulouse

Agenda Item	: 7.15.3 (NR_SL_enh2)

Source	: LG Electronics Inc.

Title	: Discussion on SL resource (re)selection and LCP restriction

Document for	: Discussion and Decision

1.	Introduction

This contribution presents our views on SL resource (re)selection and LCP impact.

2.	Discussion

2.1 Resource (re-)selection in MCSt

In #122 meeting, RAN2 discussed resource (re)selection in MCSt case due to an LBT failure as follow, but did not reach an agreement.

RAN2#122 discussion MCSt (FFS whether SL LBT failure triggers resource (re)selection or not) - Yes (P3:4806/P4-5:5686) - Proposal 4: For MCST case, reselect consecutive resources for dropped PSSCH transmission due to an LBT failure indication from L1 - Proposal 5: For MCST case, the resource (re)selection is triggered when LBT succeed and there has dropped PSSCH transmission due to an LBT failure indication from L1 - No (P3:4793) - Proposal 3. When it comes to the MCSt, RAN2 does not pursue the UE behavior that UE triggers resource (re-)selection when the PSSCH transmission was not performed due to the LBT failure. [Xiaomi]: For MCSt, if LTB passes, all corresponding slots can be transmitted. It’s same principle as other case. [Ericsson]: MCSt already provides multiple chances for transmission. Agree with LG. [IDC, ASUSTek, Qualcomm]: Agree with LG and Ericsson. We may need further discussion on single TB or multiple TB cases. [Vivo]: Agree with LG and Ericsson for single TB case. But we may need further discussion for multiple TB case. [Lenovo]: In NR-U case, if the first TB is not successful due to LBT failure, the UE can move the TB into the second slot. [Vivo]: If MCSt does not include retransmission occasion and transmission fails due to LBT failure, the UE should trigger resource (re)selection. [Lenovo]: In NR-U, moving the first TB to the second slot is for the initial transmission. [Huawei]: Solutions are made on the fly and better to have more time to think until next meeting. To be concluded next meeting.

RAN2 can further discuss whether UE triggers a resource (re-)selection when PSSCH transmission was not performed due to LBT failure in MCSt case. In conclusion, we prefer that in MCSt, the UE does not trigger resource (re-)selection even if PSSCH transmission is not performed due to the LBT failure. Because, for non-MCSt case, RAN2 agreed on UE behaviour that UE triggers resource (re-)selection when the PSSCH transmission was not performed due to the LBT failure for the purpose of giving the UE more transmission opportunities. However, the main purpose of the MCSt is to reduce the Type 1 LBT overhead. If it allows the UE to trigger resource reselection when the PSSCH transmission was not performed due to the LBT failure, Type 1 LBT overhead can be increased which does not meet the purpose of the MCSt design. Therefore, in MCSt case, we prefer UE behavior that UE does not trigger resource (re-)selection when the PSSCH transmission was not performed due to the LBT failure.

If RAN2 agrees to trigger resource reselection due to LBT failure in the MCSt case, RAN2 should define resource re-selection procedures so that UEs can obtain the benefits of MCSt. For example, UE can re-select a resource from those located after the last resource among the reserved resources for MCSt.

Observation 1. For non-MCSt case, RAN2 agreed on UE behaviour that UE triggers resource (re-)selection when the PSSCH transmission was not performed due to the LBT failure for the purpose of giving the UE more transmission opportunities.

Observation 2. The main purpose of the MCSt is to reduce the Type 1 LBT overhead. If it allows the UE to trigger resource reselection when the PSSCH transmission was not performed due to the LBT failure, Type 1 LBT overhead can be increased which does not meet the purpose of the MCSt design.

Proposal 1. When it comes to the MCSt, RAN2 does not pursue the UE behavior that UE triggers resource (re-)selection when the PSSCH transmission was not performed due to the LBT failure.

Proposal 2. If RAN2 agrees to trigger resource reselection due to LBT failure in the MCSt case, RAN2 should define resource re-selection procedures so that UE can obtain a benefit of MCSt (i.e. UE can re-select a resource from those located after the last resource among the reserved resources for MCSt when a resource re-selection is triggered due to LBT failure.).

2.2 Resource pool selection for MCSt

If the UE's MCSt-based sidelink TB transmission consists of blind Sidelink TB transmission (i.e. SL TB transmitted without HARQ Feedback) and HARQ Feedback-based SL TB transmission, RAN2 can discuss which resource pool the UE should select.

For example, if UE's MCSt-based sidelink TB transmission consists of blind Sidelink TB transmission and HARQ Feedback-based SL TB transmission, and the preceding MCSt transmission is blind Sidelink TB transmission and the following MCSt transmission is HARQ Feedback-based SL TB transmission, the UE can select only the sidelink resource pool in which the PSFCH is configured, when selecting a sidelink resource pool for generating the selected sidelink grant related to blind Sidelink TB transmission.

Observation 2. If the UE's MCSt-based sidelink TB transmission consists of blind Sidelink TB transmission (i.e. SL TB transmitted without HARQ Feedback) and HARQ Feedback-based SL TB transmission, RAN2 can discuss which resource pool the UE should select.

Proposal 3. if UE's MCSt-based sidelink TB transmission consists of blind Sidelink TB transmission and HARQ Feedback-based SL TB transmission, and the preceding MCSt transmission is blind Sidelink TB transmission and the following MCSt transmission is HARQ Feedback-based SL TB transmission, the UE can select only the sidelink resource pool in which the PSFCH is configured, when selecting a sidelink resource pool for generating the selected sidelink grant related to blind Sidelink TB transmission.

2.3 Resource (re-)selection in MCSt & COT & LCP restriction

UE can perform the destination selection procedure in the LCP procedure considering the CAPC value of the SL data on the sidelink grant when generating multiple sidelink grants within the COT it has created. For example, when the UE generates multiple sidelink grant within the COT it has created if a CAPC value of the SL data related to the subsequent sidelink grant resource (e.g., sidelink grant which behinds in time domain) is greater than a CAPC value of the SL data on the preceding sidelink grant resource (e.g., sidelink grant which precedes in time domain), resource re-selection procedure of the subsequent sidelink grant can be triggered.

In addition, UE can perform the destination selection procedure in the LCP procedure considering the CAPC value of the SL data on the sidelink grant for the MCSt after selecting the MCSt resources for multiple sidelink TBs. For example, when the UE selected the MCSt resources for multiple sidelink TBs, if a CAPC value of the SL data related to the subsequent sidelink grant resource (e.g., sidelink grant which behinds in time domain) of MCSt resource is greater than a CAPC value of the SL data on the preceding sidelink grant resource (e.g., sidelink grant which precedes in time domain), resource re-selection procedure of the subsequent sidelink grant can be triggered.

Observation 3. UE can perform the destination selection procedure in the LCP procedure considering the CAPC value of the SL data on the sidelink grant when generating multiple sidelink grant within the COT it has created.

Proposal 4. When UE generates multiple sidelink grant within the COT it has created, if a CAPC value of the SL data related to the subsequent sidelink grant resource (e.g., sidelink grant which behinds in time domain) is greater than a CAPC value of the SL data on the preceding sidelink grant resource (e.g., sidelink grant which precedes in time domain), resource re-selection procedure of the subsequent sidelink grant can be triggered.

Observation 4. UE can perform the destination selection procedure in the LCP procedure considering the CAPC value of the SL data on the sidelink grant for the MCSt after selecting the MCSt resources for multiple sidelink TBs.

Proposal 5. When UE selected the MCSt resources for multiple sidelink TBs, if a CAPC value of the SL data related to the subsequent sidelink grant resource (e.g., sidelink grant which behinds in time domain) of MCSt resource is greater than a CAPC value of the SL data on the preceding sidelink grant resource (e.g., sidelink grant which precedes in time domain), resource re-selection procedure of the subsequent sidelink grant can be triggered.

2.4 Resource (re-)selection considering the shared COT

When the UE received a shared COT from a peer UE, if the generated sidelink grant is within the shared COT and there is logical channel data that satisfies the COT requirement, the UE can transmit sidelink data by performing Type 2 LBT within the shared COT.

In order to ensure the UE's Type 2 LBT gain, we can consider a new trigger condition for the resource (re-)selection when the generated sidelink grant does not belong to the shared COT. For example, a UE can trigger resource reselection to select a new resource(s) within the shared COT when the generated sidelink grant does not belong to the shared COT.

Observation 5. In order to ensure the UE's Type 2 LBT gain, we can consider a new trigger condition for the resource (re-)selection when the generated sidelink grant does not belong to the shared COT.

Proposal 6. A UE can trigger resource reselection to select a new resource(s) within the shared COT when the generated sidelink grant does not belong to the shared COT.

2.5 COT withdrawal & RAN2 impact

At the last RAN1 #113 meeting, the following working assumptions related to COT withdrawal were made;

Working assumption

For the case where a COT initiating UE uses Type 1 channel access procedure to initiate a SL transmission,

- it is supported that the COT initiating UE can transmit transmission(s) within the same channel occupancy that follows a COT responding UE’s SL transmission(s) according to the channel access procedures.

  - FFS details of the SL transmission(s) from responding UE

  - FFS whether the above should be based on NR-U DL-UL-UL (Clause 4.2.1.0.3 of TS37.213) or DL-UL-DL (Clause 4.1.3 of TS37.213) COT sharing principle and its corresponding transmission gap requirements

  - FFS any other condition and restriction

So, RAN2 can discuss whether there is a RAN2 impact regarding this WA.

For example, when the COT Responding UE detects that the COT initiating UE has withdrawn the shared COT, the COT responding UE can perform type 1 LBT-based channel access procedure by returning from enhanced LCP to legacy LCP in the remaining shared COT duration.  In addition, a generated sidelink grant can be discarded or continue to be used for type 1 LBT-based MAC PDU transmission.

And the COT Responding UE can determine that the COT Initiating UE has withdrawn the shared COT when receiving SL Data of the COT Initiating UE in the Shared COT duration. Alternatively, the COT initiating UE can transmit a COT withdrawn related message (i.e. SCI) to the COT responding UE.

Observation 6. RAN2 can discuss whether there is a RAN2 impact regarding the RAN1 working assumption for COT withdrawal.

Proposal 7. When the COT Responding UE detects that the COT initiating UE has withdrawn the shared COT, the COT responding UE can perform type 1 LBT-based channel access procedure by returning from enhanced LCP to legacy LCP in the remaining shared COT duration.

Proposal 8. When the COT Responding UE detects that the COT initiating UE has withdrawn the shared COT, if the COT responding UE has a generated sidelink grant, the generated sidelink grant can be continue to be used for type 1 LBT-based MAC PDU transmission.

2.6 Resource (re-)selection & multiple PSFCH occasions

When RX UE transmits the PSFCH on one PSFCH occasion among multiple PSFCH occasions, it starts the SL DRX HARQ RTT Timer at the time when the PSFCH is successfully transmitted. At this time, if selected sidelink grant generated by TX UE in advance is within in the HARQ RTT Timer of the RX UE, the TX UE cannot use the selected sidelink grant. Therefore, in SL-U, it is necessary to define a resource selection procedure considering the multiple PSFCH occasions. For example, retransmission resources considered by the TX UE when generating the selected sidelink grant should be limited to resources located after the last PSFCH occasions among the multiple PSFCH occasions of the RX UE.

Observation 7. When RX UE transmits the PSFCH on one PSFCH occasion among multiple PSFCH occasions, it starts the SL DRX HARQ RTT Timer at the time when the PSFCH is successfully transmitted. At this time, if selected sidelink grant generated by TX UE in advance is within in the HARQ RTT Timer of the RX UE, the TX UE cannot use the selected sidelink grant. Therefore, in SL-U, it is necessary to define a resource selection procedure considering the multiple PSFCH occasions.

Proposal 9. Retransmission resources considered by the TX UE when generating the selected sidelink grant should be limited to resources located after the last PSFCH occasions among the multiple PSFCH occasions of the RX UE.

2.7 Type 1 LBT block issue of intra-UE

In #121 meeting, RAN2 discussed Type 1 LBT blocking issue. Based on the discussion, RAN2 made the following agreement. Besides, RAN2 sent LS to RAN1 to check that RAN2's understanding of Type 1 LBT blocking issues is correct.

RAN2#121 Agreement RAN2 understands L1 handles LBT impact to/from other UEs’ reserved resources in SL candidate resource selection (inter-UE case). RAN2 will study how MAC performs resource (re)selection with the consideration of LBT impact to its own candidate resource (intra-UE case). Will send LS to RAN1 to check if there is any concern.

Based on the LS, RAN1 discussed Type 1 LBT blocking issue and made following working assumption including options to resolve the Type 1 LBT blocking issue.

RAN1#113 Working assumption For Type 1 LBT block issue (inter-UE case), the following option 2 and option 1 are supported separately based on UE capability - Option 2: If transmission in slot(s) before a reserved resource is able to share its initiated COT to the reservation [with high L1 SL priority], UE may prioritize/select resource(s) in the slot(s) for transmission. - FFS: details of applying this prioritization, which layer to perform above prioritization behaviour, and if the reserved resource belongs to a MCSt, the COT initiating UE should be able to share the COT to cover the whole MCSt - (pre)configuring enabling/disabling option 2 is supported - Option 1: - UE may avoid selection of N consecutive resource(s) before a reserved resource with high L1 SL priority. - The value of N can be selected from {0, 1, 2} - The selection of the value of N is up to UE implementation - FFS: unless (pre-)configured or indicated by UE reserved resource in SCI - UE may avoid selection of M consecutive resource(s) after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource. - M is determined based on UE implementation (at least including 0) - FFS: Which layer to perform above behaviour - FFS: any restriction of M - (pre)configuring enabling/disabling option 1 is supported - FFS: Whether the above high priority is determined according to a (pre)configured threshold - Note: both option1 and option2 are optional UE features

Similar to RAN1 discussion, RAN2 can discuss Type 1 LBT blocking issue for Intra-UE case. For example, similar to option 1 of above RAN1 working assumption, when TX resource (re-)selection is triggered as the result of the TX resource (re-)selection check, UE may avoid selection of N consecutive resource(s) before a reserved resource for a SL TB with high logical channel priority. And if TX resource (re-)selection is triggered as the result of the TX resource (re-)selection check, UE may avoid selection of M consecutive resource(s) after a reserved resource when the reserved resource overlap with LBT of the selected resource.

Observation 8. Similar to RAN1 discussion, RAN2 can discuss Type 1 LBT blocking issue for Intra-UE case.

Proposal 10. When TX resource (re-)selection is triggered as the result of the TX resource (re-)selection check, UE may avoid selection of N consecutive resource(s) before a reserved resource for a SL TB with high logical channel priority.

Proposal 11. If TX resource (re-)selection is triggered as the result of the TX resource (re-)selection check, UE may avoid selection of M consecutive resource(s) after a reserved resource when the reserved resource overlap with LBT of the selected resource.

2.8 Resource pool switching & RB set switching due to the C-LBT failure

If SL C-LBT failure is declared in RB set, additional clarification is required for UE behavior for RB set switching and Resource pool switching. That is, when the UE declares SL C-LBT failure only on some RB sets included in SL resource pool, and selected sidelink grant belongs to the RB set in which the SL C-LBT failure occurred, the UE's behaviour should be additionally clarified.

For example, when the UE declares SL C-LBT only on some RB sets included in the SL resource pool, and the selected sidelink grant belongs to the RB set in which the SL C-LBT failure occurred, the UE can perform the following operation;

Option 1) The UE can maintain the existing SL resource pool and trigger resource re-selection with an RB set in which SL C-LBT failure has not occurred in the SL resource pool.

Option 2) The UE can replace the SL resource pool and clear the sidelink grant on the existing SL resource pool. And then, UE can create a new sidelink grant from the replaced SL resource pool.

Observation 9. If SL C-LBT failure is declared in RB set, additional clarification is required for UE behavior for RB set switching and Resource pool switching. That is, when the UE declares SL C-LBT failure only on some RB sets included in SL resource pool, and selected sidelink grant belongs to the RB set in which the SL C-LBT failure occurred, the UE's behaviour should be additionally clarified.

Proposal 12. When the UE declares SL C-LBT only on some RB sets included in the SL resource pool, and the selected sidelink grant belongs to the RB set in which the SL C-LBT failure occurred, the UE can perform the following operation;

Option 1) The UE can maintain the existing SL resource pool and trigger resource re-selection with an RB set in which SL C-LBT faiure has not occurred in the SL resource pool.

Option 2) The UE can replace the SL resource pool and clear the sidelink grant on the existing SL resource pool. And then, UE can create a new sidelink grant from the replaced SL resource pool.

3.	Conclusion

This contribution presented our views on SL resource (re)selection and LCP restriction, which can be summarized as follows:

Observation 1. For non-MCSt case, RAN2 agreed on UE behaviour that UE triggers resource (re-)selection when the PSSCH transmission was not performed due to the LBT failure for the purpose of giving the UE more transmission opportunities.

Observation 2. The main purpose of the MCSt is to reduce the Type 1 LBT overhead. If it allows the UE to trigger resource reselection when the PSSCH transmission was not performed due to the LBT failure, Type 1 LBT overhead can be increased which does not meet the purpose of the MCSt design.

Proposal 1. When it comes to the MCSt, RAN2 does not pursue the UE behavior that UE triggers resource (re-)selection when the PSSCH transmission was not performed due to the LBT failure.

Proposal 2. If RAN2 agrees to trigger resource reselection due to LBT failure in the MCSt case, RAN2 should define resource re-selection procedures so that UE can obtain a benefit of MCSt (i.e. UE can re-select a resource from those located after the last resource among the reserved resources for MCSt when a resource re-selection is triggered due to LBT failure.).

Observation 2. If the UE's MCSt-based sidelink TB transmission consists of blind Sidelink TB transmission (i.e. SL TB transmitted without HARQ Feedback) and HARQ Feedback-based SL TB transmission, RAN2 can discuss which resource pool the UE should select.

Proposal 3. if UE's MCSt-based sidelink TB transmission consists of blind Sidelink TB transmission and HARQ Feedback-based SL TB transmission, and the preceding MCSt transmission is blind Sidelink TB transmission and the following MCSt transmission is HARQ Feedback-based SL TB transmission, the UE can select only the sidelink resource pool in which the PSFCH is configured, when selecting a sidelink resource pool for generating the selected sidelink grant related to blind Sidelink TB transmission.

Observation 3. UE can perform the destination selection procedure in the LCP procedure considering the CAPC value of the SL data on the sidelink grant when generating multiple sidelink grant within the COT it has created.

Proposal 4. When UE generates multiple sidelink grant within the COT it has created, if a CAPC value of the SL data related to the subsequent sidelink grant resource (e.g., sidelink grant which behinds in time domain) is greater than a CAPC value of the SL data on the preceding sidelink grant resource (e.g., sidelink grant which precedes in time domain), resource re-selection procedure of the subsequent sidelink grant can be triggered.

Observation 4. UE can perform the destination selection procedure in the LCP procedure considering the CAPC value of the SL data on the sidelink grant for the MCSt after selecting the MCSt resources for multiple sidelink TBs.

Proposal 5. When UE selected the MCSt resources for multiple sidelink TBs, if a CAPC value of the SL data related to the subsequent sidelink grant resource (e.g., sidelink grant which behinds in time domain) of MCSt resource is greater than a CAPC value of the SL data on the preceding sidelink grant resource (e.g., sidelink grant which precedes in time domain), resource re-selection procedure of the subsequent sidelink grant can be triggered.

Observation 5. In order to ensure the UE's Type 2 LBT gain, we can consider a new trigger condition for the resource (re-)selection when the generated sidelink grant does not belong to the shared COT.

Proposal 6. A UE can trigger resource reselection to select a new resource(s) within the shared COT when the generated sidelink grant does not belong to the shared COT.

Observation 6. RAN2 can discuss whether there is a RAN2 impact regarding the RAN1 working assumption for COT withdrawal.

Proposal 7. When the COT Responding UE detects that the COT initiating UE has withdrawn the shared COT, the COT responding UE can perform type 1 LBT-based channel access procedure by returning from enhanced LCP to legacy LCP in the remaining shared COT duration.

Proposal 8. When the COT Responding UE detects that the COT initiating UE has withdrawn the shared COT, if the COT responding UE has a generated sidelink grant, the generated sidelink grant can be continue to be used for type 1 LBT-based MAC PDU transmission.

Observation 7. When RX UE transmits the PSFCH on one PSFCH occasion among multiple PSFCH occasions, it starts the SL DRX HARQ RTT Timer at the time when the PSFCH is successfully transmitted. At this time, if selected sidelink grant generated by TX UE in advance is within in the HARQ RTT Timer of the RX UE, the TX UE cannot use the selected sidelink grant. Therefore, in SL-U, it is necessary to define a resource selection procedure considering the multiple PSFCH occasions.

Proposal 9. Retransmission resources considered by the TX UE when generating the selected sidelink grant should be limited to resources located after the last PSFCH occasions among the multiple PSFCH occasions of the RX UE.

Observation 8. Similar to RAN1 discussion, RAN2 can discuss Type 1 LBT blocking issue for Intra-UE case.

Proposal 10. When TX resource (re-)selection is triggered as the result of the TX resource (re-)selection check, UE may avoid selection of N consecutive resource(s) before a reserved resource for a SL TB with high logical channel priority.

Proposal 11. If TX resource (re-)selection is triggered as the result of the TX resource (re-)selection check, UE may avoid selection of M consecutive resource(s) after a reserved resource when the reserved resource overlap with LBT of the selected resource.

Observation 9. If SL C-LBT failure is declared in RB set, additional clarification is required for UE behavior for RB set switching and Resource pool switching. That is, when the UE declares SL C-LBT failure only on some RB sets included in SL resource pool, and selected sidelink grant belongs to the RB set in which the SL C-LBT failure occurred, the UE's behaviour should be additionally clarified.

Proposal 12. when the UE declares SL C-LBT only on some RB sets included in the SL resource pool, and the selected sidelink grant belongs to the RB set in which the SL C-LBT failure occurred, the UE can perform the following operation;

Option 1) The UE can maintain the existing SL resource pool and trigger resource re-selection with an RB set in which SL C-LBT failure has not occurred in the SL resource pool.

Option 2) The UE can replace the SL resource pool and clear the sidelink grant on the existing SL resource pool. And then, UE can create a new sidelink grant from the replaced SL resource pool.

4.	Reference

[1] #122 report from session on NR SL, 	Samsung