3GPP TSG RAN2 Meeting #123                                                              R2-2307220

21st Aug. – 25th Aug. 2023, Toulouse, France

Agenda item:	7.3.4

Source:	Samsung

Title:	Discussion on cell selection and reselection for NES

Document for:	Discussion & Decision

Introduction

During the previous RAN2 #122 meeting, RAN2 discussed the UE cell selection and reselection among NES supporting cells. And RAN2 have the following agreements:

Agreements:

- We will define UE capabilities with signaling.  Details are FFS and will be discussed later during the WI phase.

- Separate camping restrictions for NES-capable and non-NES UEs will be defined.  FFS if it is a single bit or more.

In this contribution, we discuss the remained FFSs of the UE cell selection and reselection regarding NES cells.

Camping for NES-capable and Legacy UEs

RAN2 discussed and agreed now that the NW may configure NES-capable UEs to select/reselect a specific NES cell or NES cells on a specific frequency, with a separate camping restrictions different with the legacy UEs. And from RAN2 perspective, such NES-capable UEs and legacy UEs can be handled via cell selection/reselection techniques in the presence of NES cells.

In this contribution, we would like to discuss the further details.

Considering cell selection, RAN2 now have two candidate solutions as follows:

Option 1)  Use IntraFreqExcludedCellList/ InterFreqExcludedCellList IEs

Option 2)  Use the cellBarred IE or cell reservation fields in MIB/SIB

With respect to barring legacy UEs from NES cells, we believe that the conventional solutions could be simply reused, e.g., Legacy UEs are barred from NES cell if ‘cellBarred’ is set to ‘barred’. Then the NES UE can just ignore this field, and instead, observe an additional field in MIB/SIB to determine whether the NES UEs are barred for this cell or not.

Proposal 1.  NES UEs shall ignore the legacy ‘cellBarred’ field.

Proposal 2. Introduce a ‘single bit' cell barring field in SIB1 for NES capable UEs. If this field is set to ‘barred’, NES capable UEs shall consider this cell as barred.

Then, if an NES UE is barred from an NES cell, there is an additional problem to be solved in relation to frequency barring. For the frequency barring, the following three options can be considered.

i.   Option 1: The UE can re-select to another cell on the same frequency as the barred cell

ii.  Option 2: The UE shall not re-select to another cell on the same frequency as the barred cell, if intraFreqReselection field in MIBs is set to ‘not Allowed’

iii. Option 3: Introduce a new intraFreqReselection_NES field for NES UEs in MIB. The UE shall not re-select to another cell on the same frequency as the barred cell, if intraFreqReselection_NES field in MIBs is set to ‘not Allowed’

Here we support Option 2 which is aligned with the legacy NR behaviour.

Proposal 3. If an NES UE is barred from an NES cell and if intraFreqReselection field in MIBs is set to ‘not Allowed’, the UE shall not re-select to another cell on the same frequency as the barred cell.

Considering cell reselection, according to [1] and [3], RAN2 have discussed the below candidates:

- Change the Frequency Priority

- Change the settings of offset values within Reselection Criteria, so that reselections would happen faster

- Provide/priorities particular frequencies within RRC Release

- Even the use of specific slicing for energy savings might be considered, resulting in reselections to a particular frequency layer

Having the above candidates and existing mechanisms, we believe that there are practical methods for gNB to prioritize/down-prioritize an NES cell for an NES UE (re)selection.

Proposal 4. No additional mechanism is needed to encourage NES-enabled UEs camping in NES cells

Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1. NES UEs shall ignore the legacy ‘cellBarred’ field.

Proposal 2. Introduce a ‘single bit' cell barring field in SIB1 for NES capable UEs. If this field is set to ‘barred’, NES capable UEs shall consider this cell as barred.

Proposal 3. If an NES UE is barred from an NES cell and if intraFreqReselection field in MIBs is set to ‘not Allowed’, the UE shall not re-select to another cell on the same frequency as the barred cell.

Proposal 4. No additional mechanism is needed to encourage NES-enabled UEs camping in NES cells

Reference

[1] 3GPP TR 38.864 V18.0.0 (2022-12), 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on network energy savings for NR (Release 18)

[2] R2-2212971 [PRE120][303][NES] Summary of Cell (Re)selection – 8.3.4 (Apple)

[3] R2-2210995, Report of [Offline-302][NES] Cell Selection/Reselection and SSB/SIB-less (Huawei)