3GPP TSG-RAN WG2 Meeting #123	 R2- 2307226

Toulouse, France, 21 – 25 Aug, 2023

Agenda Item:	7.9.3

Source:	Xiaomi

Title:	Discussion on service continuity enhancement

Document for:	Discussion

Introduction

In this contribution, we discuss how to support the service continuity enhancements for L2 UE-to-network relay.

Discussion

Event Z1 is introduced to trigger the report during i2i path switch according to following agreement,

For i2i path switch procedure, introduce a new measurement event based on individual thresholds i.e., Event Z1: Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2.

Both SL-RSRP and SD-RSRP can be used as the measurement quantity for serving U2N relay according to following agreement,

For i2i scenario, for serving U2N relay UEs, when SL-RSRP is unavailable, SD-RSRP is used as the measurement quantity. And for candidate U2N relay UEs, only SD-RSRP is used as the measurement quantity.

According to RAN1/RAN4 reply LS [1][2], SL-RSRP and SD-RSRP of the same UE may be different due to power control difference. Therefore, if only one threshold is used for SL-RSRP and SD-RSRP, the event Z1 fulfilment may be changed due to measurement quantity change. Similar issue was identified at U2U relay and following agreement is reached,

Separate thresholds for SL-RSRP and SD-RSRP are configured for the trigger of U2U relay (re)selection.

Therefore, separate thresholds should also be configured for threshold1 in Z1.

Proposal 1: Separate thresholds for SL-RSRP and SD-RSRP are configured for the threshold1 in Z1.

In R17, SL-PathSwitchConfig is used to indicate the target relay UE ID and T420 during direct to indirect path switch. It’s agreed T420 is reused during i2i path switch. Therefore, SL-PathSwitchConfig can be reused during i2i path switch.

Proposal 2: SL-PathSwitchConfig is reused during i2i path switch.

Another open issue on service continuity is how to decide the target node during inter-gNB path switch. Based on the measurement report, source node can be aware of the sidelink channel quality between remote UE and relay UE. But there are other important information to be considered in path switch, e.g. Uu channel quality of relay UE, relay UE’s RRC state, relay UE’s load. Source node is aware of such information. NW can decide to choose the target relay UE taking all information into account. Therefore, source node shall provide the candidate relay UE’s measurement result to source node. Following similar procedure as handover, target node generates the RRCReconfiguration including the SL-PathSwitchConfig and send it to source node. Source node forward the received RRCReconfiguration message to remote UE.

Proposal 3: Source gNB sends the measurement result of candidate relay UE to target gNB.

Proposal 4: Target node generates the RRCReconfiguration including SL-PathSwitchConfig and sends it to source gNB. Source gNB forwards the received RRCReconfiguration message to remote UE.

Conclusion

Based on the discussion in section 2, we have following proposals:

Proposal 1: Separate thresholds for SL-RSRP and SD-RSRP are configured for the threshold1 in Z1.

Proposal 2: SL-PathSwitchConfig is reused during i2i path switch.

Proposal 3: Source gNB sends the measurement result of candidate relay UE to target gNB.

Proposal 4: Target node generates the RRCReconfiguration including SL-PathSwitchConfig and sends it to source gNB. Source gNB forwards the received RRCReconfiguration message to remote UE.

Reference

[1] R2-2304617	Reply LS on comparison of SL-RSRP and SD-RSRP measurements (R1-2304211; contact: Nokia)	RAN1	LS in	Rel-18	NR_SL_relay_enh-Core	To:RAN2	Cc:RAN4

[2] R2-2304637	LS on Comparison of SL-RSRP and SD-RSRP measurements (R4-2306366; contact: Nokia)	RAN4	LS in	Rel-18	NR_SL_relay_enh	To:RAN2	Cc:RAN1