3GPP TSG-RAN WG2 #123	R2-2307233

Toulouse, France, August 2023

Agenda Item:	7.9.2

Source:	OPPO

Title:	Discussion on U2U Relay

Document for:	Discussion, Decision

Introduction

This paper will discuss the U2U Relay.

Discussion

Relay (Re)Selection and Discovery

For AS-layer criterion of discovery and relay (re)selection in U2U relay, the following table summarizes the comparison with R17 U2N relay based on TS 38.331[1], TS 23.304[2] and RAN2 agreements[3-8] till now

Table 1 comparison between R17 U2N and R18 U2U relay regarding discovery and relay (re)selection

|  | R17 U2N |  | R18 U2U |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Remote UE | Relay UE | S-End UE | relay UE | T-End UE |
| Relay (re)selection trigger | Uu RSRP< threshHighRemote for relay selection the current SL quality< sl-RSRP-Thresh for relay reselection |  | Direct link quality can be used to trigger relay selection First hop SL quality can be used to trigger relay reselection FFS on the other link quality | N.A. (no link quality condition is needed) | Direct link quality can be used to trigger relay selection Second hop SL quality can be used to trigger relay reselection FFS on the other link quality |
| Model-A discovery | Rx: select the relay if SD-RSRP>sl-RSRP-Thresh+sl-HystMin | Tx: No link quality condition is defined | RX: first link quality condition to select the relay | Relay: Link quality condition is agreed in RAN2 for including UEs in the list | RX: second link quality condition to select the relay |
| Model-B discovery Solicitation | Tx: is performed in order to select relay | RX: No link quality condition is needed | TX: Relay (re)selection will trigger discovery Tx | Relay: FFS on the link quality condition | RX: N.A. (no link quality condition is needed) |
| Model-B discovery Response | Rx: select the relay if SD-RSRP>sl-RSRP-Thresh+sl-HystMin | Tx: No link quality condition is defined | RX: first link quality condition to select the relay | Relay:FFS on the link quality condition | TX: second-hop quality can be used to trigger discovery response |
| Integrated discovery: DCR | - | - | - | Relay: first link quality condition can be used to determine on relaying | RX: N.A. (no link quality condition is needed) |
| Integrated discovery: DCR response | - | - | RX: N.A. (no link quality condition is needed since SA2 defined T-End UE to do the final selection) | Relay: N.A. (no link quality condition is needed since the relay has been selected) | TX: second-hop quality can be used to trigger DCR respond (to select the relay UE) |
| Condition of being a relay/remote | Uu RSRP< threshHighRemote -hystMaxRemote | threshLowRelay +hystMinRelay <Uu RSRP< threshHighRelay -hystMaxRelay | FFS on the AS condition to determine the direct link is available | N.A. (no link quality condition is needed) | FFS on the AS condition to determine the direct link is available |

Issue-1: Relay (re)selection vs. Discovery Tx/Rx

For this issue, i.e., besides the agreed trigger condition on relay (re)selection, whether there is a need for the separate trigger condition of solicitation message/model-A discovery message/DCR message transmission.

In RAN2 #122, RAN2 has agreed the discovery transmission/reception will be triggered together with relay (re)selection, and whether there will be the separate condition defined for discovery transmission/reception besides relay (re)selection is marked as [LowPriority]

Agreement:

For non-integrated U2U relay discovery model B, when relay (re)selection is triggered at the remote UE, the discovery transmission may be triggered at the same time to search for candidate relay UEs.

[LowPriority] Proposal 5b: RAN2 to discuss whether remote UE can perform Model B discovery while relay (re)selection is not triggered.

In R17, there is no such differentiation for the U2N relay and the spec also doesn’t differentiate the trigger of discovery transmission and relay (re)selection, i.e., the UE triggers discovery and relay (re)selection at the same time upon the defined conditions.

#### 5.8.15.3 Selection and reselection of NR sidelink U2N Relay UE A UE capable of NR sidelink U2N Remote UE operation that is configured by upper layers to search for a NR sidelink U2N Relay UE shall: 1> if the UE has no serving cell; or 1> if the RSRP measurement of the cell on which the UE camps (for L2 and L3 U2N Remote UE in RRC_IDLE or RRC_INACTIVE)/ the PCell (for L3 U2N Remote UE in RRC_CONNECTED) is below threshHighRemote within sl-remoteUE-Config: 2> if the UE does not have a selected NR sidelink U2N Relay UE; or 2> if the UE has a selected NR sidelink U2N Relay UE, and SL-RSRP of the currently selected NR sidelink U2N Relay UE is available and is below sl-RSRP-Thresh; or 2> if the UE has a selected NR sidelink U2N Relay UE, and SL-RSRP of the currently selected NR sidelink U2N Relay UE is not available, and SD-RSRP of the currently selected U2N Relay UE is below sl-RSRP-Thresh; or NOTE 1: U2N Remote UE uses SL-RSRP measurements for relay reselection trigger evaluation when there is data transmission from U2N Relay UE to U2N Remote UE, and it is left to UE implementation whether to use SL-RSRP or SD-RSRP for relay reselection trigger evaluation in case of no data transmission from U2N Relay UE to U2N Remote UE. If SD-RSRP is used, the discovery procedure will be performed between the U2N Remote UE and the selected U2N Relay UE. 2> if the UE has a selected NR sidelink U2N Relay UE, and upper layers indicate not to use the currently selected NR sidelink U2N Relay UE; or 2> if the UE has a selected NR sidelink U2N Relay UE, and upper layers request the release of the PC5-RRC connection; or 2> if the UE has a selected NR sidelink U2N Relay UE, and sidelink radio link failure is detected on the PC5-RRC connection with the current U2N Relay UE as specified in clause 5.8.9.3: 3> perform NR sidelink discovery procedure as specified in clause 5.8.13 in order to search for candidate NR sidelink U2N Relay UEs; …

R17 spec did not differentiate discovery Tx/Rx trigger condition and relay (re)selection trigger condition.

When comes to R18 U2U relay, there seems no reason to deviate from it, and it is not clearly justified the necessarity of defining separate discovery transmission besides the agreed one, so there is no need to discuss the discovery Tx/Rx trigger condition and relay (re)selection trigger condition separately.

As in R17 U2N, RAN2 not pursue separate AS-layer criterion for discovery message transmission/reception and for relay (re)selection.

Issue-2: AS condition in Model-B discovery

In model-B discovery, the condition for the relay to decide on relaying the solicitation/respond message is proposed to evaluate the per-hop link quality at the relay side, i.e., relay will do filtering based on the defined AS condition. In RAN2 121bis, the following proposal has been discussed

Proposal 4a: After a relay UE receives a discovery message from a source remote UE, the relay UE transmits discovery response message or forwards the discovery message for DCR message with integrated Discovery case only if the PC5 RSRP between the relay UE and the source remote UE is above a threshold.

Proposal 4c: After receiving discovery solicitation message from source remote UE, relay UE is triggered to transmit discovery solicitation message to target remote UE only if the PC5 RSRP between the relay UE and the source remote UE is above a threshold.

And in RAN2 #122 meeting, we have agreed that Source UE will perform relay selection based on the signal strength of discovery response message from relay UE

For Model B discovery, upon discovery response messages reception, the source remote UE considers a relay UE as a candidate relay UE if the SD-RSRP towards the relay UE is above a configured threshold.

We should note that in R17 U2N relay, there is no PC5 link quality evaluation defined at relay side to decide whether to respond the solicitation message.

In R17 U2N relay, there is no PC5 link quality evaluation defined for relay UE to decide whether to respond the solicitation message.

Besides, technically speaking, the benefit of this new relay-based filtering in Model-B is doubtful since,

- According to SA2 spec and RAN2 agreement, in Model-B, the target end UE will do the initial round of filtering based on the 2nd hop quality, and the source end UE will do the final selection based on the 1st hop quality[2], which means both hop qualities have been considered during the discovery procedure, no need for the duplicate filtering at relay side;

<Omit> … 3. The discoveree 5G ProSe End UE (UE-2) that matches the value of RSC and the User Info ID of the discoveree 5G ProSe End UE (UE-2) responds to the 5G ProSe UE-to-UE Relay with a 5G ProSe UE-to-UE Relay Discovery Response message. The 5G ProSe UE-to-UE Relay Discovery Response message contains the Type of Discovery Message, RSC, User Info ID of the discoverer 5G ProSe End UE (UE-1), and User Info ID of itself, and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4. If the discoveree 5G ProSe End UE (UE-2) receives multiple UE-to-UE Relay Discovery Solicitation messages from different 5G ProSe UE-to-UE Relays, it may choose to respond or not to a 5G ProSe UE-to-UE Relay (e.g. based on the PC5 signal strength of each message received). … <Omit> … Service authorization and provisioning has been performed for the 5G ProSe Layer-2 UE-to-UE Relay and the 5G ProSe End UEs as described in clause 6.2 before this procedure. 1. Model A or Model B 5G ProSe UE-to-UE Relay Discovery as described in clause 6.3.2.4 is performed and a source 5G ProSe End UE selects a suitable 5G ProSe Layer-2 UE-to-UE Relay for the communication with a target 5G ProSe End UE. 2. The source 5G ProSe End UE decides whether to use an existing PC5 link with the 5G ProSe UE-to-UE Relay for the required service. If an existing PC5 link is used then the Layer-2 link modification procedure as specified in clause 6.4.3.7 is used towards a 5G ProSe UE-to-UE Relay, otherwise a Layer-2 link establishment procedure is used towards a 5G ProSe UE-to-UE Relay. This procedure is towards the selected 5G ProSe UE-to-UE Relay, and for Layer-2 link establishment, the security establishment is performed before step 3 is initiated.

For Model-B discovery, after receiving a discovery message from a relay UE, a target remote UE transmits the discovery response message only if the PC5 RSRP between the target remote UE and the relay UE is above a configured threshold.  FFS if there is separate impact for this agreement from the relay selection functionality.

For Model B discovery, upon discovery response messages reception, the source remote UE considers a relay UE as a candidate relay UE if the SD-RSRP towards the relay UE is above a configured threshold.

- And relay UE(s) in different coverage areas may have different RSRP threshold configurations, so may over-kill some links which are fine for the End UEs and may impact the performance.

So considering the above technical concerns and the mechanism defined in R17, the same logic should be followed in R18 U2U relay, i.e., the relay filtering/selection should be done by the remote UE.

For model-B discovery, R2 not pursue AS-layer criterion for relay-UE to decide on whether to relay the solicitation/response message.

Issue-3: Others for relay-(re)selection / discovery

For whether there is a need for the relay UE to indicate other hop quality, RAN2 has discussed whether one hop link quality needs to be indicated to the other hop end UE to trigger relay (re)selection

Proposal 13: RAN2 to discuss if U2U relay can indicate one of the following information related to the second hop to the source remote UE after relay link between source remote UE and target remote UE has been established.

-	An indication to indicate that the link between the target remote UE and U2U relay is below a threshold;

-	PC5 RSRP of second hop between relay UE and target remote UE.

The link quality degradation is different from RLF since the signal strength degradation may not lead to failure, i.e., the hop can still work. Besides, we have agreed that the UE can trigger relay reselection based on the current hop quality, i.e., the link quality degradation is already handled. Therefore, the hop-1 AS link quality doesn’t need to be considered by the hop-2 remote UE during relay reselection.

Relay UE does not forward AS link quality degradation of one hop to the peer remote UE of the other hop.

For the AS condition to determine the direct link is available, during RAN2 #121b, the following proposals were discussed:

Proposal 14a: During relay reselection, reselection towards direct link is supported.

Proposal 14b: If P14a can be agreed, RAN2 to discuss whether AS criterion is needed for switching back from indirect to direct link.

The whole relay reselection procedure is defined by SA2, from RAN2 perspective, AS layer only needs to

- Evaluate the current relay link quality to trigger relay reselection;

- If configured by the upper layer, to monitor/evaluate the new link quality to set up a new PC5 connection;

There is no service continuity in U2U relay from AS layer perspective, and it is just release/setup of old/new link to AS layer. If relay reselection is triggered, it is upper layer who decides which discovery (from relay or target UE) to monitor or what discovery message (to relay or target UE) to transmit.

Besides, the direct link and relay link will use different L2 ID, and it is up to Prose layer to judge whether the two different L2 IDs associate with the same peer UE via User Info, which means AS layer is unable to know whether a L2 ID is from the same peer UE, and thus should not be involved in the determination of communication type (i.e., direct or U2U relay communication).

It is fully up to upper layer to decide which kind of communication (direct or relayed U2U) to initiate;

So based on the above analise, when we come back to Proposal 14a/P14b discussed in RAN2 #121b meeting, there is no need for AS criterion for direct link quality evalution.

R2 not pursue an AS-layer criterion for direct link reachability judgement for indirect to direct switching.

Adaptation Layer Design

Issue-1: U2U SRAP header field

For UE ID field in SRAP header, we have agreed that the ID is needed while leave the details FFS.

An ID mappable to the destination remote UE is needed in the first hop (Tx remote UE to relay), at least in case multiplexing of different destinations in the same RLC channel is supported.

An ID mappable to the source remote UE is needed in the second hop (relay to Rx remote UE).

FFS if the IDs are different (e.g., source and destination UE IDs) or common (e.g., a local ID for the pair).

FFS whether both UE IDs are included in the header or the relay UE does a mapping.

The FFS points include the following aspect:

- Whether the ID is short ID or L2 ID;

- Whether it is one ID or 2 IDs in the header;

- Whether the IDs on each hop are same or different;

For the first issue on UE ID and L2 ID, we have agreed to rely on Relay UE to do the assignment in case short ID is finally agreed. And based on the discussions, the coordination/configuration between UEs is needed which needs further discussion on the detailed mechanisms and if we consider multi-hop in future release, there will be multiple relay UEs, who/how to assign the IDs will become more complicated.

Therefore it is suggested to rely on L2 ID of each UE, to save the discussion on ID allocation mechanism design.

R2 discusses using 24-bit L2 ID as the UE ID to be included in SRAP header.

For short ID, the following discussion is for further clarification on the short ID in RAN2 #122, and the left issue is: 1) Whether the IDs on each hop is same or different; 2) Whether it is one ID or 2 IDs in the header;

Agreements:

For the possible use of a short ID in U2U relay, RAN2 will downselect between the following options for identifying the source and destination remote UEs at the SRAP layer:

a)	Single ID, identifying the source and destination remote UEs

b)	Source ID and Destination ID

For the possible use of a short ID in U2U relay, the U2U relay UE performs the ID assignment. FFS if this ID should be assigned hop-by-hop or globally.

These agreements do not imply agreement to use a short ID.

For the issue on whether the IDs on each hop is same or different, in R17, the UE ID is same on  both hops,and the relay UE doesn’t need to determine the UE ID field of SRAP header.

In R17 U2N relay, the UE ID is the same on both hops, and there is no UE ID field determination at the relay UE.

So the different UE ID field will bring additional specification impact since it is different from R17 U2N relay. Besides, it also brings additional burdern at the relay UE since the relay has to maintain the mapping of the different IDs and determine/change the ID field according to the mapping, which will further enlarge the processing delay.

Different UE ID field brings additional specification impact and burdern at relay UE.

So it is suggested to follow R17 mechanism, i.e., UE ID field should be the same for both hops considering the above impacts and the benefit for different UE ID field is not jusitified.

For U2U Relay, the IDs on the 2 hops are same, i.e., there is no ID replacement operation by relay UE.

Then for the issue on whether 1 or 2 IDs should be included, based on the RAN2 discussions, 1 ID means the ID is the connection ID and 2 IDs means the IDs are UE IDs.

In Rel-17, the ID field in SRAP is UE ID which is used:

- By relay UE for the purpose of bearer mapping;

- By gNB to identify the remote UE.

The ID field in SRAP header in R17 is UE ID.

When it comes to R18 UE-to-UE relay, there may be multiple source/target remote UEs connecting through one relay UE which is different from R17 UE-to-Network relay with only one remote UE and one gNB. According to SA2 specification[2], and existing link between source and relay UE can be reused for the new coming U2U relay service to a new target UE, as shown in the following example:

Figure 1 Example of source connects with multiple targets via single relay

As shown in the above example, the ID configuration procedure for single connection ID and both UE IDs are:

- For connection ID based option, the signaling exchange procedure between both source-relay and relay-target2 is needed to configure a new connection ID, and the source UE need to maintain multiple connection IDs with the same relay UE;

- For UE ID based option, the UE ID for source UE can be reused so for source UE, it only need to maintain a single UE ID with the same relay UE.

Therefore, using the UE ID

- on the one hand follows the R17 principle which has less specification impact;

- on the other hand saves the possible signalling exchange and UE burdern on maintaining multiple IDs.

If short ID is agreed, include both source UE ID and destination UE ID in SRAP subheader.

PC5-RRC procedure and QoS Enforcement

Issue-1: SL-SRB Configuration

In RAN2 #122 meeting, we have agreed the PDCP configuration for E2E SL-SRB is specified as in legacy but leave SRAP/RLC/MAC configuration FFS.

For the E2E SL-SRB configuration of U2U relay, specified PDCP configuration is used. FFS for the SRAP and PC5 RLC channel configuration for SL-SRB.

Based on S2 TR 23.700-33 conclusion

The following conclusions are specific for Layer-2 UE-to-UE Relay:

-	Per-hop links (i.e. PC5 link between Source UE and UE-to-UE Relay, as well as between UE-to-UE Relay and Target UE) needs to be established before E2E PC5 link establishment is performed. Sol#30 (clause 6.30.2.2) is used as basis for normative work.

In order to allow the E2E PC5-S signaling exchange (for DCR/DCA, and SMC messages) after the per-hop link establishment, the AS-layer setting for the related SL-SRBs have to be ready in advance. It seems natural to follow the legacy approach, i.e., specified configuration for the SRAP/RLC/MAC.

And for the SRAP configuration, as we have agreed that the SRAP is also there for the SRB, the same principle, i.e., the specified configuration for SRAP can be used for the SRBs.

Rely on specified SRAP/RLC/MAC configuration for E2E SL-SRB in L2 U2U Relay.

Issue-2: QoS Split

It is agreed in RAN2 #122 that AS layer is responsible for  QoS split in L2 U2U relay.

AS layer is responsible for QoS split in L2 U2U relay.

SA2 has already designed/specified the solution for L3 U2U relay in TR 23.700 and TS 23.304

Figure 6.4.2-1: QoS control for Layer 3 UE-to-UE Relay

To achieve this, the source 5G ProSe Layer-3 End UE initiates PC5 QoS Flows setup or modification during the Layer-2 link establishment or modification procedure, the source 5G ProSe Layer-3 End UE provides the QoS Info as described in clause 6.4.3.7.3 to the 5G ProSe Layer-3 UE-to-UE Relay. The received PC5 QoS parameters of the QoS Info (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.) are interpreted as the end-to-end QoS requirements by the 5G ProSe Layer-3 UE-to-UE Relay for the traffic transmission between source 5G ProSe Layer-3 End UE and target 5G ProSe Layer-3 End UE. The 5G ProSe Layer-3 UE-to-UE Relay, based on its implementation, decides the PQI for the first hop PC5 QoS control and the PQI for the second hop PC5 QoS control. The 5G ProSe Layer-3 UE-to-UE Relay provides the QoS Info (including PQI value chosen by the 5G ProSe Layer-3 UE-to-UE Relay for the second hop) to the target 5G ProSe Layer-3 End UE. After accepted QoS Info of the second hop QoS from the target 5G ProSe Layer-3 End UE is received, 5G ProSe Layer-3 UE-to-UE Relay provides the QoS Info (including PQI value chosen by the 5G ProSe Layer-3 UE-to-UE Relay for the first hop) to the source 5G ProSe Layer-3 End UE with considering the received second hop QoS. If the source 5G ProSe Layer-3 End UE performs the Layer-2 link modification procedure to add new PC5 QoS Flow(s) or modify the existing PC5 QoS Flow(s) for IP traffic or Ethernet traffic over PC5 reference point, the source 5G ProSe Layer-3 End UE may also provide the PC5 QoS Rule(s) for the PC5 QoS Flow(s) to be added or modified to the 5G ProSe Layer-3 UE-to-UE Relay. The 5G ProSe Layer-3 UE-to-UE Relay may generate the Packet Filters used over the second hop based on the received PC5 QoS Rule(s).

In SA2 defined solutions, the QoS split is done per-QoS flow,  it is straightforward to follow L3 mechanism, i.e., the split QoS in L2 U2U Relay should be performed per-QoS flow.

SA2 has specified the QoS split procedure for L3 U2U Relay, which is performed per-QoS flow.

For QoS split in L2 U2U Relay, the QoS split is performed per-QoS flow as in L3 U2U Relay.

Then for the criterion for QoS split, the L3 U2U mechanism defined by SA2 can be reused, i.e., Relay UE based on its implementation to decide the split-QoS for the 2 hops.

For QoS split in L2 U2U relay, it is up to relay UE implementation to decide the split-QoS for the 2 hops as for L3 U2U Relay.

The main step of L3 U2U QoS split can be summarized as follows:

- Step 1: Source UE provides the E2E QoS to Relay UE;

- Step 2: Relay UE based on its implementation to decide the split-QoS for the 2 hops;

- Step 3: Relay UE sends the split-QoS of hop-2 to Target UE;

- Step 4: Target UE accept the split-QoS of hop-2;

- Step 5: Relay UE sends the split-QoS of hop-1 to Source UE.

For L2 U2U Relay, the SA2 designed procedure can be reused as baseline by changing PC5-S signaling to PC5-RRC signalling since besides the signaling, the logic is the same. Therefore, the following PC5-RRC signalling can be defined,

- Between Source remote UE and Relay UE: for Source remote UE to indicate the E2E QoS (negotiated with Target UE) to Relay UE and for Relay UE to reject or accept by sending the hop-1 split-QoS result to Source remote UE (as shown in the following Figure 2);

Figure 2. PC5-RRC signalling between source UE and Relay UE for QoS splitting

- Between Relay UE and Target remote UE: for Relay UE to send the hop-2 split-QoS to Target remote UE and for Target remote UE to feedback accept/reject to the Relay UE (as shown in the following Figure 3).

Figure 3. PC5-RRC signalling between Relay UE and Target UE for QoS splitting

For QoS split in L2 U2U relay, RAN2 to define the PC5-RRC signalling between Source remote UE and Relay UE to support Source UE provides the E2E QoS (negotiated with Target UE) to Relay UE and Relay UE to reject, or accept by sending the hop-1 split-QoS result to Source remote UE.

For QoS split in L2 U2U relay, RAN2 to define the PC5-RRC signalling between Relay UE and Target remote UE to support Relay UE to send the hop-2 split-QoS to Target remote UE and Target remote UE to feedback accept or reject to Relay UE.

Furthermore, for the reject handling, i.e., how for the Relay UE/Source remote UE to react if Target remote UE reject the split-QoS for hop-2. For Relay UE, in L3 U2U Relay case, it is more of up to Relay UE implementation to handle this. In case the split-QoS is rejected by the Target remote UE, the relay UE can:

- Either reconfigure the split-QoS for the remote UE;

- Or consider the configuration is failed and feedback the rejection information to the Source remote UE;

For the above 2 operations, operation 1) gives more chances for the relay/remote UE to negotiate the split-QoS but also increases configuration delay, and it is more complicated to judge when (after how many times of reconfiguration) to trigger configuration failure; while operation 2) saves the signalling for negotiation but may have more failure happens. So one way forward is reuse the SA2 principle, i.e., leave the decision of re-split/consider split failure decision to relay UE implementation.

For QoS split in L2 U2U relay, if Target remote UE reject the split-QoS configuration, RAN2 to further discuss relay UE up to its implementation to decide whether to re-initiate split-QoS negotiation with the target remote UE or reject Source remote UE, i.e., feedback the failure message.

If the QoS split for a QoS flow failed, i.e., relay UE feedback the failure indication for a QoS flow to the Source remote UE, since the QoS flow handling (i.e., establishment/release/modification) is in scope of PC5-S layer, the failure information can be indicated from AS layer to upper layer.

For QoS split in L2 U2U relay, if split-QoS failure feedback is received at Source remote UE, the AS layer at Source remote UE indicate the failure to its upper layer.

For the detailed signaling, either we use existing PC5-RRC signaling (e.g., RRCReconfigurationSidelink) for QoS split procedure or introduce a new signaling.

In legacy Sidelink, the configurations are all per-direction, i.e., using RRCReconfigurationSidelink from Tx to Rx UE, while QoS in current sidelink is  not a per-direction parameter, which means the current PC5-RRC signaling cannot fit well with split-QoS procedure.

The current per-direction PC5-RRC configuration procedure cannot fit well with split-QoS procedure.

Therefore, it seems clearer/easier to use a new signaling for the QoS split procedure in L2 U2U Relay.

Define a new PC5-RRC signal for the QoS split procedure in L2 U2U Relay.

Then for which QoS parameter to split, in R17 U2N relay, only split-PDB is agreed to be configured to the U2N relay UEs, while the other QoS parameters are concluded as no need for relay UE to know. For the detailed QoS parameters

Table 2 PC5 QoS parameter

|  | R17 U2N Relay | Whether need to be split in R18 U2U Relay |
| --- | --- | --- |
| sl-ResourceType-r16 | No discussion on the split | No since whether it should be the same for E2E QoS and per-hop QoS on this aspect (i.e., whether it is GBR, delay critical GBR, non-GBR) |
| sl-PriorityLevel-r16 | No discussion on the split | No since the priority should be the same for E2E QoS and per-hop QoS on this aspect (i.e., a high priority data should be high priority on each hop) |
| sl-PacketDelayBudget-r16 | gNB configure the split PDB to the remote/relay UE | Similar to R17, the PDB impacts the resource selection/scheduling |
| sl-PacketErrorRate-r16 | Up to gNB implementation | May relate to SRAP/RLC/MAC setting, If split, (1-E2E PER)=(1-hop_1 PER)*(1- hop_2 PER) |
| sl-AveragingWindow-r16 | No discussion on the split | No since it indicates the averaging window for a QoS flow, which is not related to AS configuration. |
| Sl-MaxDataBurstVolume-r16 | No discussion on the split | No since MDBV denotes the largest amount of data that the 5G-AN is required to serve within a period of 5G-AN PDB, i.e., should be the same for per-hop and E2E QoS |

As analized in the above table, only the PDB and PER may need to be considered to split in U2U relay. For PDB, it is similar to R17 U2N relay, i.e., the PDB is used as the input for resource selection.

For QoS split of per-hop QoS, PDB is critical for resource allocation;

Include PDB into the PC5-RRC message for QoS split in L2 U2U Relay.

For PER, on the one hand, it may have some impacts on bearer configuration, e.g., the SRAP/RLC/MAC setting. But on the other hand, it is not clear how to split the PER and how to use the PER to derive the SRAP/RLC/MAC setting.

If RAN2 agrees that the split of PER is necessary, for how to split PER, the following alternatives are feasible:

- Alt1: Up to relay UE implementation to split the E2E QoS/PER;

- Alt2: Assuming PER is split equally between the 2 hops.

For the above 2 alts, the first one gives relay UE more flexibility on the split, but it also leaves more further issues on how to derive the SRAP/RLC/MAC setting, considering the setting is all controlled by network, and the non-standardized PER is not known by the network in advance for the OOC/IDLE/INACTIVE UE (where SIB/Preconfiguration is static/fixed). On the contrary, the second one restricts the split flexibility but it also avoids the SRAP/RLC/MAC setting issue. Besides ,since the 2 hops are both PC5 hops, the transmission condition are similar, so it is fine to split the PER equally at the 2 hops.

For QoS enforcement of per-hop QoS, there is a trade-off between flexibility and complexity with respect to how to split PER.

Not include PER into the PC5-RRC message for QoS split in L2 U2U Relay, by assuming that PER is split equally between the two hops.

Issue-3: SL-DRB configuration

For bearer configuration, in last RAN2 meeting, the following agreements have been made for OOC/RRC IDLE/RRC INACTIVE UEs

For OOC U2U relay/remote UE, pre-configuration is used for the E2E SL-DRB and per-hop PC5 RLC channel configuration.

For RRC_IDLE/INACTIVE U2U relay/remote UE, SIB is used for the E2E SL-DRB and per-hop PC5 RLC channel configuration.

It is agreed that both per-hop and E2E configurations are from preconfiguration or SIB for U2U relay/remote UEs. While it is not touched whether the configuration of Relay UE as Tx side is from Relay UE’s network or Remote UE’s network. The reason for the necessarity of the discussion on this issue is

- In legacy, a UE decide on the RLC/MAC/PHY configuration based on per-flow QoS and the SDAP configuration (as flow-to-bearer mapping), no matter by pre-configuration or network configuration.

- In U2U relay, at relay UE, it has no control on the flow to bearer mapping, which is controlled by SDAP entity located at end-UE, but can only adjust the mapping based on bearer ID within SRAP header. So at least the legacy SIB/pre-configuration taking QoS flow as input does not work here, i.e., relay UE is unable to map a PDCP PDU which was generated based on flow-to-bearer mapping-1 at source UE side, to SRAP/RLC/MAC configuration, which is based on flow-to-bearer mapping-2 at relay UE side

Legacy design cannot directly apply to the relay UE as Tx-UE of the second hop, since the QoS flow to bearer mapping is controlled by end-UE.

Due to this reason, we see several alternatives to solve this:

Alt1: Still follow the legacy framework, i.e., up to relay UE (or the serving gNB) as Tx UE to decide on the Tx configuration for the second hop, FFS how to revise the pre-configuration considering the difference caused by L2 U2U Relay.

Alt2: Rely on Tx end-UE to decide on the Tx configuration for the second hop, i.e., the Tx configuration of relay UE comes from Tx end-UE (or its serving gNB). So that still the pre-configuration can take QoS flow as input.

For L2 U2U relay UE as Tx-UE of the second hop, R2 discusses whether rely on relay UE itself (or the serving gNB) or the Tx end-UE (or the serving gNB) to decide on the Tx side related PC5 RLC channel parameters.

Besides, the agreements only indicate the UE obtain the configuration from SIB/Pre-configuration, but there was no detailed discussion on whether the configuration in SIB/Pre-configuration is based on E2E QoS as legacy or the split QoS also need to be considered

- In legacy SL, it is E2E QoS that SL bearer configurations rely on;

- In L2 U2N relay, it is up to gNB to decides the bearer configurations.

For the E2E configuration, i.e., PDCP/SDAP configurations, the legacy configuration for direct SL communication is feasible.

For OOC/IDLE/INACTIVE L2 U2U End-UE, PDCP/SDAP setting is obtained via Pre-configuration/SIB by referring to end-to-end QoS as in legacy.

For the per-hop configuration, i.e., SRAP/RLC/MAC configuration, as discussed in Proposal 16, assume only PDB is to be split by the relay UE, the SRAP/RLC/MAC configuration can also be setted by referring end-to-end QoS as in legacy since PDB is only used in resource selection procedure and determined based on UE implementation in legacy sidelink which is decoupled with the RLC/MAC configurations

NOTE 3C: How the MAC entity determines the remaining PDB of SL data is left to UE implementation.

For OOC/IDLE/INACTIVE L2 U2U End-UE, SRAP/RLC/MAC setting is obtained via Pre-configuration/SIB by referring to end-to-end QoS as in legacy.

For the SLRB/RLC configurations for L3 U2U Relay, in R17 U2N relay, RAN2 only discussed it for L2 U2N relay and considers the legacy configurations will be used for L3 relay/remote UEs. In R18 U2U relay, the same principle can be reused.

For L3 U2N relay/remote UE, only discovery configurations was discussed/introduced in R17.

RAN2 confirms L3 U2U remote/end-UE can derive SLRB configuration as non-relay UEs.

L2 U2U relay E2E security

For the ciphering/deciphering, it is agreed that E2E bearer ID is used as input for L2 U2U relay, and it is confirmed by SA3.

SA3 would like to thank RAN2 for their LS on the security for L2 UE-to-UE relay. SA3 discussed the working assumption of RAN2 and confirms that the end-to-end bearer ID can be used as input for the L2 U2U relay ciphering, deciphering and integrity protection at PDCP. In order to avoid keystream reuse, different end-to-end bearer IDs shall be used for each bearer of the same link using the same key. This applies for bearers used at the same time, as well as for reusing bearer IDs when setting up a new bearer.

The following issue is how to understand the E2E bearer ID. As discussed offline during POST 121bis, there are 3 different understandings:

- configuration index in the list of SLRB configurations in Uu interface, i.e. slrb-Uu-ConfigIndex;

- configuration index in the list of SLRB configurations in PC5 interface, i.e. slrb-PC5-ConfigIndex;

- Bearer ID in SRAP header;

For these 3 understandings, for the first one, i.e., slrb-Uu-ConfigIndex is not correct since the intention of using bearer ID instead of LCID is to align the inputs for ciphering/deciphering at the 2 peer end UEs, and slrb-Uu-ConfigIndex is the configuration from network, which means Tx UE may not need to reflect / expose that to the Rx UE.

slrb-Uu-ConfigIndex may be different between 2 peer end UEs.

Then for the other 2 understandings, before discussing the down selection, it’s better to check the Uu bearer ID used in the ciphering/deciphering procedure as in PDCP spec

For downlink and uplink ciphering and deciphering, the parameters that are required by PDCP for ciphering are defined in TS 33.501 [6] and are input to the ciphering algorithm. The required inputs to the ciphering function include the COUNT value, and DIRECTION (direction of the transmission: set as specified in TS 33.501 [6]). The parameters required by PDCP which are provided by upper layers TS 38.331 [3] are listed below: - BEARER (defined as the radio bearer identifier in TS 33.501 [6]. It will use the value RB identity –1 as in TS 38.331 [3]); - KEY (the ciphering keys for the control plane and for the user plane are KRRCenc and KUPenc, respectively).

As shown in the above spec, the bearer ID is configured by RRC in DL/UL cipehering/deciphering procedure.

In downlink and uplink ciphering/deciphering procedure, the bearer ID to be used by PDCP is the value configured by RRC.

So same principle should be followed in U2U relay since the decision on rely on bearer ID is try to reuse Uu mechanism.

While for the concern on RX remote UE always needs to know which e2e bearer the encrypted traffic belongs to: The Rx remote UE can know the E2E bearer of a SRAP PDU based on

- The bearer ID in SRAP header;

- And the configuration of SRAP to PDCP mapping;

I.e., the bearer identification is done at SRAP layer, but after the traffic is submitted to PDCP, the E2E bearer is already known, so the deciphering can be done – which is similar for Uu, i.e., for Rx the bearer-ID is essentially identified via LCID, so no need to mention that explicitly in PDCP spec for security handling, because the bearer identification has already been done before the PDCP PDU being submitted to PDCP Rx entity.

Actually, in R17 U2N, there bearer ID in SRAP header is determined based on the RRC configured bearer ID so somehow understanding 2 and 3 are aligned if the bearer ID in SRAP header in U2U relay is defined same as R17 U2N relay.

In R17 U2N relay, the bearer ID in SRAP header is aligned with the RRC configured bearer ID.

Therefore, it is feasible to obtain the PDCP ciphering/deciphering input from the configuration index in the list of SLRB configurations in PC5 interface.

Another concern for the PC5 configuration index is that in current spec, the SRB configurations are specified, which means there is no PC5 configuration index for SRB. And SA3 indicates the different end-to-end bearer IDs shall be used for each bearer of the same link, therefore, one solution is we avoid using index of 0/1/2/3/4 in the PC5 configuration index, assuming that is to be used for SRB0/1/2/3/4 based on specified configuration. (Whether 4 needs to be reserved can be further discussed, since discovery does not require AS-layer security).

Define BEARER used for PDCP ciphering/deciphering via: 1) specified values for SL-SRBs, and 2) slrb-PC5-ConfigIndex for DRB (excluding the specified values for SL-SRBs).

Network involvement in U2U relay

For the network involvement in U2U relay, RAN2 have made the following agreements:

Proposal 1.1 (modified):In UE-to-UE relay, the remote/relay UE in RRC_IDLE/RRC_INACTIVE or OOC can acquire discovery configuration as in Rel17 (i.e., cell-specific configuration/preconfiguration).  FFS if any restrictions specific to UE-to-UE relay are introduced for in-coverage UE in RRC_CONNECTED.

Proposal 2: RAN2 to agree that in U2U relay, OOC UEs obtain discovery configuration from pre-configuration and IDLE/INACTIVE UEs obtain discovery configuration from SIB.

RAN2 will strive to simplify the gNB involvement in U2U-relay-specific operation as compared to the U2N case.  Details are FFS, including whether some gNB control is needed for the in-coverage scenario and how/whether the gNB involvement can be simplified compared to U2N.

RAN2 has agreed that for OOC/IDLE/INACTIVE UEs, the same configuration mechanisms for discovery are reused, i.e., rely on SIB/Precnfiguration. And the left issues for the network involvement is:

- how for the UE to know whether the current network supports U2U relay service;

- whether/how for network know the UE role is U2U relay/remote UE.

In Rel-17 U2N, network indicates its capability on whether/which types of U2N service it can support towards U2N Remote UE and U2N Relay UE. Therefore, finally in the SIB, the following Boolean parameters are used for such indication:

SIB12-IEs-r16 ::=             SEQUENCE {

sl-ConfigCommonNR-r16         SL-ConfigCommonNR-r16,

lateNonCriticalExtension      OCTET STRING                   OPTIONAL,

...,

[[

sl-DRX-ConfigCommonGC-BC-r17         SL-DRX-ConfigGC-BC-r17                                                 OPTIONAL,    -- Need R

sl-DiscConfigCommon-r17              SL-DiscConfigCommon-r17                                                OPTIONAL,    -- Need R

sl-L2U2N-Relay-r17                   ENUMERATED {enabled}                                                   OPTIONAL,    -- Need R

sl-NonRelayDiscovery-r17             ENUMERATED {enabled}                                                   OPTIONAL,    -- Need R

sl-L3U2N-RelayDiscovery-r17          ENUMERATED {enabled}                                                   OPTIONAL,    -- Need R

sl-TimersAndConstantsRemoteUE-r17    UE-TimersAndConstantsRemoteUE-r17                                      OPTIONAL     -- Need R

]]

}

Similarly, when it comes to Rel-18 U2U Relay, such indication is also necessary to allow U2U Remote UE and U2U Relay UE to get aware of the network capability. The signalling details can be discussed in stage 3 discussion.

Introduce indication(s) in SIB message for the network capability on U2U service.

Besides, no matter which option is selected by RAN2, the network awareness of U2U relay is inevitable since we agreed mode-1 and discovery pool are supported for U2U relay and the the network awareness on the UE role as a U2U relay UE/remote UE is needed so that network knows the requested resources are for U2U relay service.

L2 U2U relay and remote UE reports the UE role to network.

R2 discuss the need for L3 U2U relay and remote UE to report the UE role to network, by considering the need for dedicated configuration of relay (re)selection and discovery.

At last, for the path switch, in R17 U2N relay, the path switch is handled by gNB, but in R18 U2U relay,

- It has been indicated in the SI discussion that the service continuity is not supported in U2U relay from RAN2 perspective;

- The gNB control of U2U relay path-switching requires a lot of network effort on measurement report reception/path-switch decision… which violates the intention of “simplify the gNB involvement in U2U-relay-specific operation”.

- SA2 has concluded the NAS procedure for path-switching and relay reselection negotiation which doesn’t involve network configuration[2];

#### 6.7.4.2 5G ProSe Layer-2 UE-to-UE Relay reselection Depicted in Figure 6.7.4.2-1 is the procedure for the negotiated 5G ProSe Layer-2 UE-to-UE Relay reselection. Figure 6.7.4.2-1: 5G ProSe Layer-2 UE-to-UE Relay reselection

Thus it is preferred that besides the AS criterion determination for U2U relay reselection as discussed in section 2.1, no further RAN2 discussion on the path-switch issue is needed, the higher layer procedure defined by SA2 is sufficient.

Besides the AS layer criterion for U2U relay reselection, R2 not pursue further AS-layer spec impact for relay reselection, but just rely on the higher layer procedure defined by SA2.

Conclusion

We have the following obersarvations:

Observation 1	R17 spec did not differentiate discovery Tx/Rx trigger condition and relay (re)selection trigger condition.

Observation 2	In R17 U2N relay, there is no PC5 link quality evaluation defined for relay UE to decide whether to respond the solicitation message.

Observation 3	It is fully up to upper layer to decide which kind of communication (direct or relayed U2U) to initiate;

Observation 4	In R17 U2N relay, the UE ID is the same on both hops, and there is no UE ID field determination at the relay UE.

Observation 5	Different UE ID field brings additional specification impact and burdern at relay UE.

Observation 6	The ID field in SRAP header in R17 is UE ID.

Observation 7	SA2 has specified the QoS split procedure for L3 U2U Relay, which is performed per-QoS flow.

Observation 8	The current per-direction PC5-RRC configuration procedure cannot fit well with split-QoS procedure.

Observation 9	For QoS split of per-hop QoS, PDB is critical for resource allocation;

Observation 10	For QoS enforcement of per-hop QoS, there is a trade-off between flexibility and complexity with respect to how to split PER.

Observation 11	Legacy design cannot directly apply to the relay UE as Tx-UE of the second hop, since the QoS flow to bearer mapping is controlled by end-UE.

Observation 12	For L3 U2N relay/remote UE, only discovery configurations was discussed/introduced in R17.

Observation 13	slrb-Uu-ConfigIndex may be different between 2 peer end UEs.

Observation 14	In downlink and uplink ciphering/deciphering procedure, the bearer ID to be used by PDCP is the value configured by RRC.

Observation 15	In R17 U2N relay, the bearer ID in SRAP header is aligned with the RRC configured bearer ID.

We have the following proposals:

Proposal 1	As in R17 U2N, RAN2 not pursue separate AS-layer criterion for discovery message transmission/reception and for relay (re)selection.

Proposal 2	For model-B discovery, R2 not pursue AS-layer criterion for relay-UE to decide on whether to relay the solicitation/response message.

Proposal 3	Relay UE does not forward AS link quality degradation of one hop to the peer remote UE of the other hop.

Proposal 4	R2 not pursue an AS-layer criterion for direct link reachability judgement for indirect to direct switching.

Proposal 5	R2 discusses using 24-bit L2 ID as the UE ID to be included in SRAP header.

Proposal 6	For U2U Relay, the IDs on the 2 hops are same, i.e., there is no ID replacement operation by relay UE.

Proposal 7	If short ID is agreed, include both source UE ID and destination UE ID in SRAP subheader.

Proposal 8	Rely on specified SRAP/RLC/MAC configuration for E2E SL-SRB in L2 U2U Relay.

Proposal 9	For QoS split in L2 U2U Relay, the QoS split is performed per-QoS flow as in L3 U2U Relay.

Proposal 10	For QoS split in L2 U2U relay, it is up to relay UE implementation to decide the split-QoS for the 2 hops as for L3 U2U Relay.

Proposal 11	For QoS split in L2 U2U relay, RAN2 to define the PC5-RRC signalling between Source remote UE and Relay UE to support Source UE provides the E2E QoS (negotiated with Target UE) to Relay UE and Relay UE to reject, or accept by sending the hop-1 split-QoS result to Source remote UE.

Proposal 12	For QoS split in L2 U2U relay, RAN2 to define the PC5-RRC signalling between Relay UE and Target remote UE to support Relay UE to send the hop-2 split-QoS to Target remote UE and Target remote UE to feedback accept or reject to Relay UE.

Proposal 13	For QoS split in L2 U2U relay, if Target remote UE reject the split-QoS configuration, RAN2 to further discuss relay UE up to its implementation to decide whether to re-initiate split-QoS negotiation with the target remote UE or reject Source remote UE, i.e., feedback the failure message.

Proposal 14	For QoS split in L2 U2U relay, if split-QoS failure feedback is received at Source remote UE, the AS layer at Source remote UE indicate the failure to its upper layer.

Proposal 15	Define a new PC5-RRC signal for the QoS split procedure in L2 U2U Relay.

Proposal 16	Include PDB into the PC5-RRC message for QoS split in L2 U2U Relay.

Proposal 17	Not include PER into the PC5-RRC message for QoS split in L2 U2U Relay, by assuming that PER is split equally between the two hops.

Proposal 18	For L2 U2U relay UE as Tx-UE of the second hop, R2 discusses whether rely on relay UE itself (or the serving gNB) or the Tx end-UE (or the serving gNB) to decide on the Tx side related PC5 RLC channel parameters.

Proposal 19	For OOC/IDLE/INACTIVE L2 U2U End-UE, PDCP/SDAP setting is obtained via Pre-configuration/SIB by referring to end-to-end QoS as in legacy.

Proposal 20	For OOC/IDLE/INACTIVE L2 U2U End-UE, SRAP/RLC/MAC setting is obtained via Pre-configuration/SIB by referring to end-to-end QoS as in legacy.

Proposal 21	RAN2 confirms L3 U2U remote/end-UE can derive SLRB configuration as non-relay UEs.

Proposal 22	Define BEARER used for PDCP ciphering/deciphering via: 1) specified values for SL-SRBs, and 2) slrb-PC5-ConfigIndex for DRB (excluding the specified values for SL-SRBs).

Proposal 23	Introduce indication(s) in SIB message for the network capability on U2U service.

Proposal 24	L2 U2U relay and remote UE reports the UE role to network.

Proposal 25	R2 discuss the need for L3 U2U relay and remote UE to report the UE role to network, by considering the need for dedicated configuration of relay (re)selection and discovery.

Proposal 26	Besides the AS layer criterion for U2U relay reselection, R2 not pursue further AS-layer spec impact for relay reselection, but just rely on the higher layer procedure defined by SA2.

Reference

[1] TS 38.331 Radio Resource Control (RRC) protocol specification

[2] TS 23.304 Proximity based Services (ProSe) in the 5G System

[3] Report from session on positioning and sidelink relay RAN2 #119

[4] Report from session on positioning and sidelink relay RAN2 #119bis

[5] Report from session on positioning and sidelink relay RAN2 #120

[6] Report from session on positioning and sidelink relay RAN2 #121

[7] Report from session on positioning and sidelink relay RAN2 #121bis

[8] Report from session on positioning and sidelink relay RAN2 #122

[9] TS 24.554 Proximity-services (ProSe) in 5G System (5GS) protocol