3GPP TSG-RAN WG2 #123	R2-2307242

Toulouse, France, August 2023

Agenda Item:	7.24.2

Source:	OPPO

Title:	Discussion on inter-frequency measurements

Document for:	Discussion, Decision

Introduction

This paper will discuss the following inter-frequency measurement issue.

The issue is about latency from MO configuration to Measurement Report for the most interesting frequency(ies) e.g. for the service of the UE. More specifically at the time of measurement report, the network cannot know which frequencies the UEs has already measured, so there is no way for the network to decide to wait or not for another potential measurement report (expect the network to act immediately on the first measurement report). There is support to attempt a solution for Rel-18. Solution discussion next meeting.

Discussion

As indicated in the conclusion from RAN2 #122, this inter-frequency measurement issue is about how for the network to make the best decision regarding whether to act immediately on the first measurement report or to wait for a while for the most interesting frequency(ies),

- To act immediately, the reported frequency may be not wanted mostly by the network by considering service/load/coverage aspects;

- To wait for a while, the network doesn’t know how long should wait since the network has no information on when the most interesting frequency will be (has been) measured;

To address the above issue, companies have discussed the solutions and the scope of 3GPP work. The common understanding summarized by the offline discussion rapporteur is that to address the issue in TEI18, the scope of work needs to be minimized and manageable. And there is also a strong view on limit the impact to RAN2 only.

To address the issue in TEI18, it is better to limit the impact to RAN2 only.

For the candidate solutions, if we try to avoid the other WG (RAN4) involvement, and design a RAN2-centric solution, the UE measurement behaviours should not be changed since that are mainly defined by RAN4.

UE measurement behaviour is mainly defined by RAN4 thus should not be impacted.

By not impacting UE measurement behaviour (RAN4), the other direction is to consider the UE measurement report behaviour, and the candidates include:

- Either to find a way to let the UE reports frequencies in an order that align with network’s expectation;

- Or the UE can include more information in the measurement report (e.g., indicate the frequencies have been measured (with available measurement results) but doesn’t trigger the report event).

For the solution of controlling UE measurement report order, it is to prioritize some of the frequencies according to network expectation considering the service type/network load/network deployment. One way to achieve the prioritization is to use different trigger condition configuration, i.e., high priority measurement object can be configured to associate to a report configuration with lower/easier event-trigger threshold and low priority measurement object can be configured to associate a report configuration with higher/difficult event-trigger threshold.

Figure 1. Example of prioritization of frequency in the measurement report

The prioritization of frequency in the measurement report can be achieved by different event trigger threshold configuration.

For the solution of include more information in the measurement report, currently, UE can include the available measurement result of serving cells in the measResultServingMOList which includes the RSRP, RSRQ and SINR of each serving cell.

1> for each serving cell configured with servingCellMO: 2> if the reportConfig associated with the measId that triggered the measurement reporting includes rsType: 3> if the serving cell measurements based on the rsType included in the reportConfig that triggered the measurement report are available: 4> set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfig that triggered the measurement report; 2> else: 3> if SSB based serving cell measurements are available: 4> set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on SSB; 3> else if CSI-RS based serving cell measurements are available: 4> set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on CSI-RS;

Currently, UE can include the available measurement result of serving cells in the measResultServingMOList which includes the RSRP, RSRQ and SINR of each serving cell.

And the similar approach can be used to also include the measurement results of non-serving cells.

In summary, the following 2 options can be considered to solve the issue and limit the work in RAN2:

- Option-1: using different event trigger threshold configuration to prioritize different frequencies and thus let the UE to report the frequencies in an order that align with network expectation;

- Option-2: add a new field in the measurement result to include the available measurement results of non-serving frequencies with configured MO which is similar to the serving cell mechanism.

For the issue on “latency from MO configuration to Measurement Report for the most interesting frequency(ies)”, RAN2 to discuss the following 2 Options:

- Option-1: to prioritize different frequencies by setting different event trigger threshold configurations.

- Option-2: add a new field in the measurement result to include the available measurement results of non-serving frequencies with configured MO.

Conclusion

We have the following observations:

Observation 1	To address the issue in TEI18, it is better to limit the impact to RAN2 only.

Observation 2	UE measurement behaviour is mainly defined by RAN4 thus should not be impacted.

Observation 3	The prioritization of frequency in the measurement report can be achieved by different event trigger threshold configuration.

Observation 4	Currently, UE can include the available measurement result of serving cells in the measResultServingMOList which includes the RSRP, RSRQ and SINR of each serving cell.

We have the following proposals:

Proposal 1	For the issue on “latency from MO configuration to Measurement Report for the most interesting frequency(ies)”, RAN2 to discuss the following 2 Options:

Option-1: to prioritize different frequencies by setting different event trigger threshold configurations.

Option-2: add a new field in the measurement result to include the available measurement results of non-serving frequencies with configured MO.