3GPP TSG-RAN WG2 Meeting #123	R2-2307245

# Toulouse, France, 21 – 25 August 2023

Agenda item:		7.5.4.3

Source:	Xiaomi

Title:	Configured Grant enhancements for XR

Document for:		Discussion and Decision

Introduction

WID of Rel-18 XR [1] has the following scope related to CG enhancements:

- Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2);

In this contribution, we discuss CG enhancements for XR.

Discussion

Configuration of multiple PUSCH in one CG

The main motivation of introducing multiple PUSCH transmission occasions in one CG is to handle the dynamic size of XR packets. While the XR traffic can be periodic, the size of XP packet in each periodicity can vary, e.g. due to different frame types (I/P/B frames). Such XR traffic characteristic makes the configuration of TBS for CG a dilemma, i.e. too small TBS results in UE requests additional grants via SR/BSR, while too large TBS results in resource waste. With the introduction of multiple PUSCH in one CG, gNB can over provision the resource in granularity of PUSCH transmission occasions, and UE can dynamically indicate the unused CG PUSCH transmission occasions via “UTO-UCI”. With such information, gNB can allocate the unused CG occasions for other purposes, e.g. to other UEs.

RAN1 is still discussing the configuration aspects of multiple PUSCH in one CG, e.g. there is some progress regarding time domain allocation aspects in RAN1#113 meeting, as below:

Agreement For time domain resource allocation for multi-PUSCH CGs, support - For TDRA determination (based on NR-U framework) - For Type-1, follow the rules for DCI format 0_0 on UE specific search space, as defined in Clause 6.1.2.1.1 of TS 38.214. - Note: To determine the configuration of TDRA, PUSCH repetition type A is assumed according to description in 6.1.2.3 in 38.214 for Type-1. - It is still an open issue whether repetition is supported. If it is decided repetition is not supported, it implies the corresponding repetition factor for is one. - For Type-2, the TDRA table is determined by the TDRA table associated with activation DCI, as defined in Clause 6.1.2.1 of TS 38.214. - Note: The DCI format for activation DCI with pusch-RepTypeA is applicable. - It is still an open issue whether repetition is supported. If it is decided repetition is not supported, it implies the corresponding repetition factor for is one. - N is configured by higher layers - A single SLIV is determined from TDRA. - The SLIV used for 1st PUSCH per CG period. - PUSCH is used in each of N consecutive slots per CG period - Note: N is configured independently from cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively. N configuration is independent from cgRetransmissionTimer configuration. - To determine corresponding slots for CG PUSCHs in a period of a multi-PUSCH CG configuration: - For the first PUSCH in the period, follow the legacy procedures. - For remaining PUSCHs in the period - ForType-1 and Type-2, reuse the corresponding procedures for NR-U by applying the RRC parameters N and M, instead of cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively.

Given that RAN1 is still discussing this topic and it is in RAN1’s expertise on which configuration is optimal, it is proposed that RAN2 should wait for RAN1’s progress regarding the configuration of multiple PUSCH in one CG.

Proposal 1: RAN2 should wait for RAN1’s progress regarding the configuration of multiple PUSCH in one CG.

HARQ process ID determination

In RAN1 LS R1-2306233 [2], RAN1 indicates the following agreements regarding HARQ process ID determination for multi-PUSCH in one CG configuration:

Agreement From RAN1 perspective, for determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH: - The HARQ process ID for the first configured PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying the following formula, whichever is applicable - HARQ Process ID = [X*floor( (CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes - HARQ Process ID = [X*floor((CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2 - X= the number of configured PUSCHs in the CG period - The HARQ process ID of the remaining configured and valid CG PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by one with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2), whichever applicable. Note: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB.

HARQ process ID determination for configured grants has been discussed by RAN2 in previous releases and is specified in TS 38.321 clause 5.4.1, as below:

For configured uplink grants neither configured with harq-ProcID-Offset2 nor with cg-RetransmissionTimer, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation: HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes For configured uplink grants with harq-ProcID-Offset2, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation: HARQ Process ID = [floor(CURRENT_symbol / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2 where CURRENT_symbol = (SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8]. For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation selects an HARQ Process ID among the HARQ process IDs available for the configured grant configuration.

For Rel-18 HARQ process ID determination for multi-PUSCH in one CG configuration, our understanding is that it is RAN2 that should make the final decision, by taking into account the input from RAN1. Regarding RAN1 LS, RAN2 can reply RAN1 along this line.

Proposal 2: RAN2 should make the final decision for Rel-18 HARQ process ID determination for multi-PUSCH in one CG configuration, taking into account the input from RAN1.

As for the HARQ process ID determination method proposed by RAN1, after analysis, our view is that the method handles the multi-PUSCH CG well and is backward compatible with Rel-16 IIoT approach.

Observation 1: HARQ process ID determination method proposed by RAN1 is technically OK.

Proposal 3: RAN2 to reply RAN1 LS with the following:

- HARQ process ID determination method proposed by RAN1 is technically OK.

- RAN2 will make the final decision for Rel-18 HARQ process ID determination for multi-PUSCH in one CG configuration, taking into account the input from RAN1.

Handling of UTO-UCI

With the introduction of multiple PUSCH CG, UE can select suitable number of CG occasion(s) to carry XR traffic and indicates whether CG occasions can be used or not in UTO-UCI. With such information, gNB can allocate the unused CG occasions for other purposes.

RAN1 has made the following agreements regarding the definition and content of UTO-UCI:

Agreement - Option 1: For a CG PUSCH configuration, the UTO-UCI is included in every CG PUSCH that is transmitted (that is Option 1 in corresponding agreement in RAN1#112) - FFS details - Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience. Agreement The UCI that provides information about unused CG PUSCH transmission occasions is defined as a “new UCI” (i.e. Alt. 1 of previous agreement). Agreement The UTO-UCI provides a bitmap where a bit corresponds to a TO within a time duration/range. The bit indicates whether the TO is “unused”. FFS: Details including time duration/range

RAN1 agrees that “The UTO-UCI provides a bitmap where a bit corresponds to a TO within a time duration/range. The bit indicates whether the TO is ‘unused’.” Whether to indicate a CG occasion as “unused” or “not unused” depends on UE’s knowledge of traffic arrival information and UL grant size of CG occasions. LCP procedure in MAC layer determines that for a given UL grant, the number of bytes that can be carried for each LCH. With the help of proper configuration of LCH to CG mapping restriction (allowedCG-List-r16 IE), UE could run such LCP prediction well in advance in case of XR traffic (compared with traditional PUSCH preparation time as defined in clause 7.7 of TS 38.213). All such information and procedure are only known and performed in MAC layer, not in PHY layer. Therefore it is obvious that the content of UTO-UCI (“unused” or “not unused”) should be determined by MAC layer.

Proposal 4: The MAC layer makes the decision about the content of UTO-UCI (“unused” or “not unused” of each CG PUSCH occasion).

In RAN1#113 meeting, RAN1 has made the following agreements regarding the semantics of “unused” and “not unused”:

- A CG PUSCH occasion indicated as “unused” earlier, is not allowed to be indicated as “NOT unused later”. - A CG PUSCH occasion indicated as “NOT unused” earlier, can be indicated as “unused” later. - When a CG PUSCH occasion is indicated as “unused”, the UE is not allowed to transmit CG PUSCH on that CG PUSCH occasion. - For any other CG PUSCH occasion that is NOT indicated as “unused”, the UE is allowed to transmit or not to transmit CG PUSCH on that CG PUSCH occasion as per legacy specification.

The above agreements imply that the indication of “unused” is final (since gNB can allocate CG occasions indicated as “unused” to other UEs), while indication of “not unused” does not affect gNB scheduling and UE behavior. Therefore, from RAN2 perspective, only condition of setting UTO-UCI to “unused” and related UE behavior should be specified.

Proposal 5: RAN2 to specify the condition of setting UTO-UCI to “unused” and related UE behavior.

Necessity to align CG periodicity to XR traffic

In contributions submitted to previous RAN2 meeting, there was proposal to align CG periodicity to XR traffic. Although rational number DRX cycle was introduced to support non-integer periodicity, it is questionable to introduce non-integer periodicities for CG. The reason is that it is more flexible to configure CG compared with DRX. Existing mechanisms like multiple CG configurations, slot granularity of CG periodicity, and dynamic scheduling can be used to mitigate the misalignment between CG periodicity and XR traffic. In Rel-16, the potential enhancement for non-integer periodicity of CG was also discussed, with related discussion captured in TR 38.825 Table 6.5.2-1, as copied below:

Table 6.5.2-1:	TSN traffic characteristics with potential issues and enhancements

| No. | TSN traffic characteristic | Description | Potential solutions and enhancements |
| --- | --- | --- | --- |
| … |  |  |  |
| 4 | TSN message periodicities with non-integer multiple of NR supported periodicities | In TSN use cases, the periodicity of data packets which are sent depends on application and in majority of the cases, it is not possible to modify it. There may be use cases where periodicity values are not multiple of NR slot or symbol period, which are used to configure CG/SPS periodicity. The severity of this issue depends on the traffic's latency requirement and the frequency of misalignment occurrences. | The following solutions could potentially help in resolving or mitigating the issue: - Adjustment of SPS/CG resource by RRC reconfiguration (as per current specification) - Usage of short SPS/CG periodicities and/or multiple SPS/CG configurations and/or combination thereof (for SPS, support for shorter periodicities than those available in Rel-15 may be required) - More efficient adjustment of SPS/CG resource timing in the UE as compared to RRC reconfiguration, e.g. based on network configuration or dynamic network signalling and which could be based on knowledge of TSN traffic pattern - Applying de-jittering buffer at the edges of 5G system |

RAN2#105bis meeting agreed the following: “R2 assumes short SPS/CG periodicities and/or multiple SPS/CG configurations and/or combination thereof could be used to mitigate the periodicity misalignment between the TSN periodicity and CG/SPS periodicity. Other solutions not precluded, e.g. to address resource consumption.” In RAN2#108 meeting, RAN2 eventually agreed that “We don’t introduce additional mechanism to align CG/SPS to TSC traffic pattern period”.

In Rel-18, with the introduction of multiple PUSCH CG, UE can select suitable CG occasion(s) to carry XR traffic and indicate whether CG occasions can be used or not in UTO-UCI. Therefore there is even less motivation in Rel-18 to introduce non-integer periodicity of CG.

Proposal 6: No additional mechanisms are introduced to align CG periodicity to XR traffic.

Conclusion

In this contribution, we discuss CG enhancements for XR, and have the following observation:

Observation 1: HARQ process ID determination method proposed by RAN1 is technically OK.

We propose the following:

Proposal 1: RAN2 should wait for RAN1’s progress regarding the configuration of multiple PUSCH in one CG.

Proposal 2: RAN2 should make the final decision for Rel-18 HARQ process ID determination for multi-PUSCH in one CG configuration, taking into account the input from RAN1.

Proposal 3: RAN2 to reply RAN1 LS with the following:

- HARQ process ID determination method proposed by RAN1 is technically OK.

- RAN2 will make the final decision for Rel-18 HARQ process ID determination for multi-PUSCH in one CG configuration, taking into account the input from RAN1.

Proposal 4: The MAC layer makes the decision about the content of UTO-UCI (“unused” or “not unused” of each CG PUSCH occasion).

Proposal 5: RAN2 to specify the condition of setting UTO-UCI to “unused” and related UE behavior.

Proposal 6: No additional mechanisms are introduced to align CG periodicity to XR traffic.

- References

[1] RP-230786, Nokia et al, "Updated WID on XR Enhancements for NR"

[2] RAN1 LS R1-2306233, "LS on XR capacity enhancements"