3GPP TSG-RAN WG2 Meeting #123	R2-2307247

# Toulouse, France, 21 – 25 August 2023

Agenda item:		7.16.2.3

Source:	Xiaomi

Title:	AI/ML model delivery

Document for:		Discussion and Decision

Introduction

In RAN2#121 meeting, following was agreed regarding AI/ML model transfer/delivery:

- Aim to at least analyze the feasibility and benefits of model/transfer solutions based on the following: - Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4: Server (e.g. OAM, OTT) can transfer/delivery AI/ML model(s) to UE (e.g. transparent to 3GPP).

Table: relations between solutions and use cases

| Solutions | Applicable use cases |
| --- | --- |
| Solution 1a, 1b | CSI feedback enhancement Beam management Note: No specific considerations for Positioning accuracy enhancement for Solution 1a and 1b. |
| Solution 2a, 2b | CSI feedback enhancement Beam management Note: No specific considerations for Positioning accuracy enhancement for Solution 2a and 2b. |
| Solution 3a, 3b | Positioning accuracy enhancement |
| Solution 4 | CSI feedback enhancement Beam management Positioning accuracy enhancement |

The pros and cons of each option were further analyzed in R2-2302268 [1], and RAN2 agreed that “The table can serve as starting point for continued discussion (but contains some parts that seems non consensus, e.g. delta configuration).”

In this contribution, we discuss AI/ML model delivery methods.

Discussion

Solution 2a/2b and 3b

For the solutions involving CN entities except LMF (solution 2a and 2b), AI model is carried by NAS signalling or UP data. Since AI/ML use cases under study are mainly about physical layer, it is not clear whether CN nodes can determine the applicable AI model for physical layer use case. In addition, it is difficult for RAN2 to evaluate the feasibility and pros/cons of such solutions. Therefore, we suggest RAN2 to focus on the solutions without the involvement of CN entities other than LMF. The evaluation and study of solutions involving CN entities except LMF (solution 2a and 2b) can be done by other groups, e.g. SA2. RAN2 can send LS to corresponding groups to trigger the evaluation and study if needed.

Proposal 1: Leave the study and evaluation of solutions involving CN entities other than LMF (i.e. solution 2a/2b) to other groups.

In solution 3b, AI/ML model is transferred between LMF and UE via UP data. This is related to solution #1 (Positioning protocol transport over User Plane) in SA2 TR 23.700-71. However, whether to have normative work on positioning UP solution depends on SA/SA2 decision. In addition, the UP solution has little impact in RAN2, since the model is carried as UP data. It is therefore proposed that RAN2 to down-prioritize the discussion on solution 3b until there is decision to progress the normative work in SA2.

Proposal 2: RAN2 to down-prioritize the discussion on solution 3b (AI/ML model is transferred between LMF and UE via UP data) until there is decision to progress the normative work in SA2.

Model delivery between UE and gNB (solution 1a and 1b)

Current description for solution 1a and 1b in RAN2 agreements are as follows:

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

- Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

It seems that the above description implies that the model delivery is for downlink only. However, in email discussion “[Post122][060][AIML] Mapping of functions to physical entities (CMCC)” [2], for training Type 1 of CSI compression with two-sided model, model delivery can be from OTT server to gNB&UE, or from UE to gNB. Therefore model delivery solution shall support both downlink and uplink.

Proposal 3: Model delivery solution shall support both downlink and uplink.

CP solution (1a)

Email discussion report R2-2302268 [1] lists the following pros and cons for CP solution (1a).

|  | Pros | Cons |
| --- | --- | --- |
| Solution 1a | 6. The existing RRC signaling solutions can be reused as baseline, at least including delta signaling and segmentation 9. Additional security and verification may not be necessary as the UE already established security before the transfer is initiated 11. gNB can take the control of the AIML model transfer itself, which can not be achieved by traditional UP based solution | 1. Face challenges to convey large size or “no upper limit size” AI model by RRC message (e.g. >45kBytes) 2. Maybe high control plane overhead, as a large model size may need segmentation/transmission/acknowledgment. This consumes critical configuration time for model transfer/delivery 3. An incomplete control plane model transfer has to be restarted upon mobility, as there are no current procedures to resume transmission across gNBs. Some companies wonder whether it is critical or not as it depends on how frequent the gNB to send new/updated AI/ML to the UE |

In solution 1a, RRC message is used to carry AI model between UE and gNB. In downlink, RRCReconfiguration or DLInformationTransfer message can be reused, with limited specification impacts. A new IE or container can be defined to carry the AI model. If the AI model is large, the RRC message may be segmented into multiple parts. As in TS 38.300 clause 7.10 shown below, it is required that “all segments of an RRC message are transmitted before sending another RRC message”. This means that when transmitting a large AI model, other RRC messages, which could be delay sensitive, might be delayed.

An RRC message may be segmented in case the size of the encoded RRC message PDU exceeds the maximum PDCP SDU size. Segmentation is performed in the RRC layer using a separate RRC PDU to carry each segment. The receiver reassembles the segments to form the complete RRC message. All segments of an RRC message are transmitted before sending another RRC message. Segmentation is supported in both uplink and downlink as specified in TS 38.331 [12].

Observation 1: Due to RRC segmentation, additional delay may be introduced to legacy radio reconfiguration if RRC message is used to deliver AI model in downlink.

In uplink, there is no appropriate existing UL RRC message for the model delivery. One may argue to reuse existing RRC message UEAssistanceInformation (UAI) to deliver AI model. However the characteristics of the content in UAI and AI model are different. UAI is mainly used to deliver dynamic UE information / preference, which may change from time to time. Therefore for UAI, the triggering condition is specified so that UE can send UAI if its preference changes. There is also triggering for UE to send UAI to target gNB if it sends UAI to source gNB within 1 second before handover. On the other hand, AI model delivery is a “one-shot” action, e.g. upon gNB request. Considering above differences, UAI is not suitable to deliver AI model. It is necessary to define new RRC message and procedure to support AI model delivery in uplink using solution 1a.

Observation 2: New RRC message and procedure are required to support AI model delivery in uplink using solution 1a.

Above observation 1 and 2 are not considered in email discussion report R2-2302268 [1], and it is therefore proposed to consider them when comparing AI/ML delivery options.

Proposal 4: Following is considered as cons of solution 1a when comparing AI/ML delivery options:

- Due to RRC segmentation, additional delay may be introduced to legacy radio reconfiguration if RRC message is used to deliver AI model in downlink.

- New RRC message and procedure are required to support AI model delivery in uplink.

UP solution (1b)

In the UP model delivery solution (1b) between UE and gNB, existing user plane is not applicable since it terminates at UE and UPF. A new layer can be introduced to handle the AI model transfer functionality. Furthermore, other AI related functionalities, e.g. data collection, model registration, may also be supported in the new layer, if existing protocol layers cannot support the functionalities.

The protocol stack of new layer is depicted in Figure 1 below. The new layer is temporarily called AI for simplicity.

Figure 1: Protocol stack for UP based model delivery

The AI layer provides following services to upper layer:

- AI Model delivery;

- …

The AI layer expects following services from lower layer:

- Integrity protection, ciphering and loss-less in-sequence delivery of information without duplication;

To support model delivery, AI layer delivers the AI model to the lower layer, i.e. PDCP. New bearer or existing bearer can be used to carry AI model. Existing user plane transmission procedure can be reused and it is expected that there are no impacts to lower layers. Both uplink and downlink delivery can be supported. The AI model can be delivered by separate radio bearers. Since the new layer terminates at UE and gNB, the UP solution is in RAN2 scope without involvement of other WGs.

Proposal 5: New layer on top of PDCP can be introduced to support UP AI model delivery between UE and gNB. The work can be done within RAN2.

Model delivery between UE and LMF (solution 3a)

In the legacy positioning procedures, LMF is responsible for managing positioning. It is straightforward that LMF is also responsible for managing AI based positioning. For model delivery between UE and LMF, if inference is performed by UE, LMF could provide AI model to UE and the existing LPP message can be reused, for instance, the LMF sends the AI model to UE by LPP ProvideAssistanceData or LPP RequestLocationInformation message.

Proposal 6: When inference is performed by UE, the existing LPP message can be used for LMF to provide AI model to UE, for example, the LPP ProvideAssistanceData or LPP RequestLocationInformation message can be considered.

When inference is performed by UE, the AI model may come from the OTT server of the UE. In this case, UE may need to provide the AI model information/function to LMF. The existing LPP message could be considered, for example, UE provides the AI model information/function to LMF by LPP ProvideCapabilities.

Proposal 7: When inference is performed by UE and the AI model comes from UE OTT server, UE may provide the AI model information/function to LMF for LMF to manage AI models. The existing LPP message, for example, the LPP ProvideCapabilities can be considered.

When inference is performed by LMF, the AI model may come from UE, and the different AI models may be used for different UEs. If the UE delivers the UE specific AI model to LMF, LMF applies the AI model when locating the UE. As a result, LMF should manage large number of AI models, which increases LMF complexity significantly. In addition, in email discussion “[Post122][060][AIML] Mapping of functions to physical entities (CMCC)” [2], most companies prefer that for LMF-side model, model training and inference are both at LMF side. In this case, model transfer/delivery is not needed.

Proposal 8: When inference is performed by LMF, model transfer/delivery from UE to LMF is not needed.

Conclusion

In this contribution, we discuss AI/ML model delivery methods, and have the following observations:

Observation 1: Due to RRC segmentation, additional delay may be introduced to legacy radio reconfiguration if RRC message is used to deliver AI model in downlink.

Observation 2: New RRC message and procedure are required to support AI model delivery in uplink using solution 1a.

We propose the following:

Proposal 1: Leave the study and evaluation of solutions involving CN entities other than LMF (i.e. solution 2a/2b) to other groups.

Proposal 2: RAN2 to down-prioritize the discussion on solution 3b (AI/ML model is transferred between LMF and UE via UP data) until there is decision to progress the normative work in SA2.

Proposal 3: Model delivery solution shall support both downlink and uplink.

Proposal 4: Following is considered as cons of solution 1a when comparing AI/ML delivery options:

- Due to RRC segmentation, additional delay may be introduced to legacy radio reconfiguration if RRC message is used to deliver AI model in downlink.

- New RRC message and procedure are required to support AI model delivery in uplink.

Proposal 5: New layer on top of PDCP can be introduced to support UP AI model delivery between UE and gNB. The work can be done within RAN2.

Proposal 6: When inference is performed by UE, the existing LPP message can be used for LMF to provide AI model to UE, for example, the LPP ProvideAssistanceData or LPP RequestLocationInformation message can be considered.

Proposal 7: When inference is performed by UE and the AI model comes from UE OTT server, UE may provide the AI model information/function to LMF for LMF to manage AI models. The existing LPP message, for example, the LPP ProvideCapabilities can be considered.

Proposal 8: When inference is performed by LMF, model transfer/delivery from UE to LMF is not needed.

- References

[1] R2-2302268, Huawei (email rapporteur), "Report of Offline 027 model transfer delivery (Huawei)"

[2] R2-2308286, CMCC, "Report of [Post122][060][AIML] Mapping of functions to physical entities (CMCC)"