3GPP TSG-RAN WG2 Meeting #123	R2-2307251

Toulouse, France, 21st – 25th August, 2023

Title:	Draft reply LS on NPDCCH monitoring restriction for NB-IoT NTN

Response to:	LS on NPDCCH monitoring restriction for NB-IoT NTN (R1-2306245)

Release:	Release 18

Work Item:	       IoT_NTN_enh-Core

Source:	OPPO (to be RAN2)

To:	RAN1

Cc:	-

Contact Person:

Name:	Haitao Li

E-mail Address:	lihaitao@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 would like to thank RAN1 for their LS on NPDCCH monitoring restriction for NB-IoT NTN in R1-2306245.RAN2 has discussed Working assumption 2 provided in the LS. In RAN2’s understanding, for a HARQ process configured as HARQ feedback disabled by RRC and further reversed to HARQ feedback enabled by DCI, since UE will transmit HARQ feedback, which is helpful for network to decide whether to schedule the retransmission or a new transmission for the given HARQ process, it seems more reasonable that the NB-IoT UE’s behaviour on NPDCCH monitoring is the same as that in the case of HARQ feedback enabled, i.e. UE is not required to monitor NPDCCH for the same HARQ process until an RTT+3ms has elapsed since the HARQ feedback transmission.

2. Actions:

To RAN1

ACTION: 	RAN2 respectfully asks RAN1 to take above information into account in the future work.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #123bis                 9th - 13th October 2023		Xiamen, China

TSG-RAN WG2 Meeting #124	13th - 17th November 2023 		Chicago, USA