3GPP TSG-RAN WG2 Meeting #123                                                                                   R2-2307263

Toulouse, France, August 21-25, 2023

Source:	CATT, CBN

Title:	  Discussion on Control Plane for Multicast Reception in RRC_INACTIVE

Agenda Item:	7.11.2.1

Document for:	Discussion and Decision

- 1. Introduction

This paper further discusses the remaining issues on Control Plane for multicast reception in RRC_INACTIVE. In section 2, the following aspects are covered,

- Multicast MCCH handling

- Details on frequency prioritization

- Service continuity

- Notification for session activation

- Coexistence between inactive multicast and SDT

- UE capabilities

The proposals are summarized in Section 3.

- 2. Discussions

2.1 Multicast MCCH

The multicast MCCH is introduced to support the multicast reception for RRC_INACTIVE UEs. In the current 38.331 running CR [1], it remains an FFS whether the multicast MCCH is optional,

Editor’s Note: FFS if MCCH is optional and whether “if SIBx is not scheduled in SIB1” needs some rewording.

Some companies think the multicast MCCH is optional as it is only used to update the PTM configuration, so the multicast MCCH can be provided by the gNB when the PTM configuration needs a change.

However, MCCH is used not just for PTM configuration update but also for mobility,according to the agreements on mixed approach,

We will have a mixed approach and we start with the following: - When NW configures UE to continue the multicast reception in INACTIVE state, NW provides the PTM configuration for the activated multicast session via the RRC dedicated signalling, at least for the serving cell (FFS other cases). - MCCH is used in case there is a need to indicate a PTM configuration in case there is a need for change in PTM config or during mobility beyond serving cell / gNB. FFS session status change and other indications.

If multicast reception in INACTIVE is provided but the multicast MCCH is not provided in one cell, UE reselected to this cell needs to resume the RRC connection to get the PTM configuration,which is not intended.

Therefore, it is proposed that,

Proposal 1: Multicast MCCH should be present for a cell providing multicast reception in RRC_INACTIVE.

2.2 Details on frequency prioritization

We have agreed to support frequency prioritization during cell reselection for multicast reception in RRC_INACTIVE,

Frequency prioritization may be provided to the UE for cell reselection for multicast reception in RRC_INACTIVE, detailed mechanism on how to identify the frequency info (e.g., SAI, USD, or frequency info directly provided by network) is FFS.

For the details there are mainly two candidate solutions on the table, i.e., SIB+USD (similar as Rel-17 broadcast) and frequency priority provided by dedicated signalling. For the latter one, it is workable if the multicast service is deployed only on a single frequency in the multicast service area. If the multicast service is deployed on different frequency in different area, the frequency provided by dedicated signalling will be invalid after UE reselects to a new cell on another frequency, and thus UE may need to enter RRC_CONNECTED to get the latest frequency priority. So it is more flexible to take the Rel-17 broadcast mechanism (i.e. frequency prioritization based on SIB21+USD) is taken as baseline.

Therefore, it is proposed that,

Proposal 2: For frequency prioritization of multicast reception in RRC_INACTIVE, Rel-17 broadcast mechanism is taken as baseline, i.e., SIB21+USD based frequency prioritization.

To use similar frequency prioritization mechanism as R17 MBS broadcast, USD needs to contain the mapping between MBS FSA ID and multicast sessions in INACTIVE needs to be included in USD. We can make an assumption in RAN2 and further check with SA2.

Proposal 3: RAN2 assumes USD can contain the mapping between MBS FSA ID and multicast session provided to UEs in INACTIVE. Send LS to SA2 to confirm this assumption.

Similar as R17 MBS broadcast, the mapping between FSAI and frequencies for multicast reception in INACTIVE should also be included in a SIB (e.g., SIB21).The question is whether to introduce a new SIB or reuse SIB21. Since multicast reception in INACTIVE and MBS broadcast can be two separate capabilities supported by gNB, and SIB21 is used to control MII reporting for MBS broadcast, it’s better to also introduce a new SIB for multicast reception in INACTIVE to avoid the impact to R17 MBS broadcast.

Proposal 4: The mapping between frequency and FSAI for multicast reception in INACTIVE is provided in a new SIB.

2.3 Service continuity

Details of threshold/reception quality

Previously we have agreed that the UE can resume the RRC connection due to bad quality, with the details needing further clarification,

UE may trigger RRC connection resumption if the reception quality of the multicast data is below a configured threshold, FFS how to specify the threshold/reception quality.

Firstly, it needs to clarify which parameter to use as the threshold to trigger the RRC resume procedure. The candidate solutions may be RSRP, RSRQ, SINR or BLER. In the current L3 measurement, the RSRP/RSRQ has been well defined and used in the NR specs, e.g., cell selection or cell reselection during mobility. The RSRP is used for evaluation of the received power level, and the RSRQ is used for evaluation of the received quality of the signal. Also a better RSRP/RSRQ leads to a lower BLER. Thus it is enough to use the RSRP/RSRQ as the threshold, since this can implicitly ensure the QoS requirement for the multicast session. To reduce the spec impact, it is suggested to follow the legacy evaluation of received signal quality.

The second remaining issue is whether to include the threshold in the RRC Release message or multicast MCCH. Note that it should be flexible for the NW to manage a per-cell threshold as different gNBs may have its own strategy for quality control. From this perspective, it’s better to include the threshold parameter in the multicast MCCH, otherwise the RRC_INACTIVE UE that moves from other cells needs to trigger the RRC connection resume to update the quality threshold. Regarding the granularity of the threshold, it’s better to be configured on the level of per service, as different multicast services may have different QoS requirement.

Proposal 5a: RSRP/RSRQ is used as the quality threshold.

Proposal 5b: The quality threshold is configured in multicast MCCH.

Proposal 5c: The granularity of quality threshold is per multicast session.

Access control for connection resume due to MBS

Another issue is how to set the cause value when performing RRC resume due to multicast reception, e.g.,

Scenario 1: Paging by the NW

Scenario 2: PTM configuration is not available for UE in RRC_INACTIVE in the reselected cell

Scenario 3: The reception quality of multicast in RRC_INACTIVE is below the configured threshold

- Paging by the NW

We have agreed that which UEs to receive the multicast service in RRC_INACTIVE or in RRC_CONNECTED is under the NW control, and group paging or individual paging can be used by the NW for this purpose. In this scenario, the UE behavior is just to response the NW paging, thus the cause value of mt-Access can be re-used so that no UAC is performed from the UE side.

- PTM configuration is not available for UE in RRC_INACTIVE in the reselected cell

There may be three cases when UE in RRC_INACTIVE moves to a cell without available PTM configuration:

Case 1: UE moves out of the service area for a multicast service, i.e., the target cell does not support this multicast session;

Case 2: UE does not move out of the service area for a multicast service, but the target cell does not have the capability to provide the function of inactive multicast reception;

Case 3: UE does not move out of the service area for this multicast service, and the target cell has the capability to provide the function of inactive multicast reception, but the target cell does not transmit the multicast MCCH.

For above three cases, the UE behavior is same, i.e., to trigger RRC resume procedure. Then the current gNB will request the UE context from the anchor gNB in order to, if allowed, re-locate the UE context and obtain the corresponding multicast context information for further keeping UE in RRC_CONNECTED or releasing UE to RRC_INACTIVE based on its capability.

Currently there are following cause values that have been introduced:

ResumeCause ::=             ENUMERATED {emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, rna-Update, mps-PriorityAccess, mcs-PriorityAccess, spare1, spare2, spare3, spare4, spare5 }

Besides the cause value of rna-Update, other causes are mainly used to indicate the NW of unicast data access or signaling transmission for different use cases, and the NW prefers to change UE to RRC_CONNECTED based on the load condition. In this situation, the current gNB needs to check from the UE context that whether the UE has joined a certain multicast session. If so, the gNB may further find no unicast service for the UE after waiting a time period, then the gNB can provide the PTM configuration to UE either by RRCRelease or multicast MCCH in order to let UE continue receiving the multicast service in RRC_INACTIVE or keep it in RRC_CONNECTED. In summary, this causes the complex NW implementation, and thus a new cause value is more intuitive to let the gNB know the purpose of resume is due to no available PTM configuration for inactive multicast reception, without waiting whether to have unicast service for UE.

Then we have a look at whether the cause rna-Update can be reused.

If the reselected cell is out of the RNA, the legacy cause rna-Update can be reused. When the gNB receives the cause value for RNA update, it retrieves the UE context from the anchor gNB. The anchor gNB further checks that the resumption cell is not in the previously configured RNA, and then provides the UE context to the current gNB. By this way, the current gNB knows which multicast session the UE is receiving and can further let UE continue receiving the multicast session in RRC_CONNECTED or RRC_INACTIVE based on the gNB capability.

If the reselected cell is still in the same RNA,it is problematic to use cause rna-Update. According to the current 38.300 (Figure 9.2.2.5-2: Periodic RNA update procedure without UE context relocation), when UE triggers the resume procedure (e.g., due to Periodic RNA update) in the RNA, the anchor gNB may reject the request of UE context from the current gNB, as the anchor gNB may decide to not re-locate the UE context due to the fact that the reselected cell is still in the current RNA. Consequently, the current gNB cannot successfully acquire the UE multicast context, and thus cannot further check whether the UE has joined a certain multicast session, i.e., the gNB does not know to provide which multicast service configuration to UE.

Observation 1: The legacy resume cause is not suitable for RRC resume due to that PTM configuration is no available for UE in INACTIVE in the reselected cell which is in the RNA.

- The reception quality of multicast in RRC_INACTIVE is below the configured threshold

For the RRC resume procedure triggered by the bad reception quality, the UE intention is to enter RRC_CONNECTED and continue the multicast reception in RRC_CONNECTED. If we reuse the legacy cause value for unicast service, it also introduces a complex NW implementation. In detail, the current gNB knows from the UE context that the UE has joined a certain multicast session, but cannot verify whether the reception quality is good enough, since the gNB needs to wait for the measurement results to determine whether to keep UE in RRC_CONNECTED. For the cause value of rna-Update, as mentioned above, it cannot ensure the gNB to get the UE context from the anchor gNB, so it is also not suitable for this scenario. From this perspective, it’s better to use a new resume cause.

Observation 2: The legacy resume cause is not suitable for RRC resume due to bad reception quality.

Therefore, it is proposed,

Proposal 6: Discuss whether to introduce new resume cause for RRC resume due to the following,

- PTM configuration is no available for UE in INACTIVE in the reselected cell which is in the RNA

- bad reception quality

State transition from RRC_CONNECTED state to RRC_INACTIVE state

Another FFS captured in the current 38.331 running CR [1] was related to the UE behavior upon reception of the RRCRelease by the UE,

2> if the multicastConfigInactive is configured: 3> apply the configuration and perform MBS multicast reception in RRC_INACTIVE. Editor’s note: FFS on UE’s behaviour if one MBS session is not activated.

The above UE behavior aligns with the one for the active multicast session, as it has confirmed that the RRCRelease will provide the PTM configuration for UE to continue multicast reception in RRC_INACTIVE,

We will have a mixed approach and we start with the following: - When NW configures UE to continue the multicast reception in INACTIVE state, NW provides the PTM configuration for the activated multicast session via the RRC dedicated signalling, at least for the serving cell (FFS other cases). - MCCH is used in case there is a need to indicate a PTM configuration in case there is a need for change in PTM config or during mobility beyond serving cell / gNB. FFS session status change and other indications. - We assume that the UE can only receive multicast service after it joined the session. - FFS whether MCCH configuration is initially provided to the UE via dedicated signalling.

For the multicast session that is not active, the NW also “can” provide the PTM configuration for UE to start multicast reception upon session activation. According to the RAN2#121 agreements as below,

If network finds it useful, the PTM configuration for the (single) serving cell can be configured to UE before the session activation, and UE stores the configuration. When session is activated, UE can receive multicast in INACTIVE state by applying the configuration without going back to RRC_CONNECTED, if not updated by MCCH after being configured.

But we need to clarify how UE knows whether the session is active upon receiving the PTM configuration by RRCRelease. The simplest way is that the UE verifies the multicast session is active if it has received this multicast session for a time period in RRC_CONNECTED. Companies may concern there may be the scenario where UE enters RRC_CONNECTED for joining the session which is in active state, and the NW directly releases UE to RRC_INACTIVE with providing the PTM configuration by RRCRelease. We do not need to spend much time on this scenario as it is indeed a corner case.

Therefore, if UE determines that the multicast session is not active upon receiving the PTM configuration in RRCRelease, UE does not need to start multicast reception (e.g., start monitoring G-RNTI) for power savings, as it shall be indicated by group paging upon session activation.

Proposal 7a: UE determines the MBS session as active upon receiving the PTM configuration in RRCRelease if it has been configured to receive multicast in RRC_CONNECTED state.

Proposal 7b: If the MBS session is not activated when receives PTM configuration in RRCRelease, UE does not perform MBS multicast reception in RRC_INACTIVE immediately.

In the current 38.331 running CR [1], a FFS related to UE behavior on multicast MCCH information acquisition upon session activation are captured as the following,

Editor’s note: FFS whether a UE shall apply the multicast MCCH information acquisition procedure upon receiving paging with inactiveReceptionAllowed included for the concerned TMGI(s).

The FFS can be considered for two cases: UE have received PTM configuration in RRCRelease, or UE have not received PTM configuration in RRCRelease.

Regarding the case that UE have received PTM configuration in RRCRelease, UE can receive multicast in INACTIVE state by applying the stored PTM configuration if it is up to date according to the agreements as below. However, one thing to clarify is how UE can determines whether the PTM configuration is updated or not by MCCH after being configured.

If network finds it useful, the PTM configuration for the (single) serving cell can be configured to UE before the session activation, and UE stores the configuration. When session is activated, UE can receive multicast in INACTIVE state by applying the configuration without going back to RRC_CONNECTED, if not updated by MCCH after being configured.

Potentially there are two options as following,

- Option 1: PTM configuration in RRCRelease cannot be updated by MCCH before session activation;

Option 1 is simple and makes sense. It seems there is no reason for NW to change the PTM configuration until session activation. With option 1, UE can use the stored PTM configuration directly for multicast reception upon session activation and starts to monitor MCCH DCI for change notification at the same time.

- Option 2: UE starts to regularly monitor MCCH DCI in INACTIVE upon receiving PTM configuration in RRCRelease to check whether the PTM configuration is changed before session activation;

Option 2 requires UE to monitor MCCH-RNTI regularly for potential MCCH change notification. The UE power consumption is increased unnecessarily

Proposal 7c: If the MBS session is not activated when UE receives PTM configuration in RRCRelease, it is assumed the PTM configuration does not change until session activation.

Proposal 7d: If the MBS session is not activated when UE receives PTM configuration in RRCRelease, UE directly uses the stored PTM configuration for multicast reception and starts to monitor MCCH DCI for change notification upon receiving group paging that indicates to allow the inactive multicast reception.

Regarding the case that UE does not receive PTM configuration in RRCRelease, it is a FFS in the current 38.331 running CR [1],

Editor’s note: FFS what is the UE behaviour when the session is activated, if the configuration was not configured in RRCRelease due to session deactivation.

In this case, it is reasonable that UE starts to read MCCH to acquire the PTM configuration upon receiving group paging that indicates to allow the inactive multicast reception.

Proposal 7e: If UE did not receive PTM configuration in RRCRelease due to session deactivation, UE reads MCCH to acquire the PTM configuration upon receiving group paging that indicates to allow the inactive multicast reception.

2.4 Notification for session deactivation

Multicast MCCH shall be used to indicate UE of session deactivation,

Working assumption (to be confirmed by RAN1 via pending reply LS): One bit in the MCCH DCI is used to notify the change of the multicast MCCH. We reuse the bit used for MCCH change indication from Rel-17 MBS broadcast. This does not cover session deactivation which is FFS. MCCH is used for notifying MC session deactivation for multicast reception in RRC_INACTIVE to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI. This is assumed to have no/minor impact on RAN1/PHY

The details need further discussion,

Editor’s note: FFS on the details of notifying session deactivation.

For simplicity, it is straightforward to introduce 1-bit indication per multicast session for notification of session deactivation in the content of multicast MCCH. By this way we can reuse legacy mechanism as much as possible, as we have agreed to reuse the bit in DCI format 4_0 to notify the change of MCCH content with no/minor impact on RAN1. In legacy, the LSB of the two bits in DCI format 4_0 is used to indicate the modification of MCCH content. When considering session deactivation notification for inactive multicast, this bit can be reused to indicate the change of multicast MCCH content caused by the session state change.

Companies may raise that whether we can directly use MCCH DCI to indicate the session deactivation. In our view this is overkill and will cause much RAN1 impact as per session indication will occupy a large number of DCI bits, which is indeed not the RAN1 expectation.

Therefore, it is proposed that,

Proposal 8: The session deactivation is indicated in the content of multicast MCCH.

2.5 Coexistence between inactive multicast and SDT

Whether the inactive multicast can be configured together with SDT is mentioned in the running CR [1],

Editor’s Note: FFS whether SDT and MBS multicast reception in RRC_INACTIVE can be configured together. And if yes, whether UE configured for MBS multicast reception in RRC_INACTIVE monitors group paging during SDT.

In Rel-17, the multicast feature can be configured together with SDT with the following restriction,

- If configured by upper layers for MBS multicast reception, while SDT procedure is not ongoing, monitors a Paging channel for paging using TMGI;

This means after UE is released to RRC_INACTIVE, UE does not monitor group paging for session activation notification during SDT, which means UE may lose the session activation notification for continuing the multicast reception in RRC_CONNECTED.

When considering Rel-18 inactive multicast, we think the inactive multicast reception can be configured together with SDT, as the SDT procedure only allows UE to perform small data transmission while keeping UE in RRC_INACTIVE. Regarding whether to monitor group paging during SDT, we see Rel-17 multicast may also lose the group paging for session activation notification, so it is not such urgent to make further optimization for Rel-18 inactive multicast. Also we have no much time for such optimization due to the frozen deadline, therefore it is suggested that,

Proposal 9: The MBS multicast reception in RRC_INACTIVE can be configured together with SDT, but UE does not monitor group paging during SDT.

2.6 UE capabilities

On the UE capability, RAN2 had given an initial consideration on this issue in the reply LS of [2], but without further confirmation,

RAN2 answer to Q3 SA2 would like to ask if the UE radio capability provided directly from UE to NG-RAN will contain the information whether the UE supports Rel-18 MBS capability to receive multicast data in RRC_INACTIVE state: - Yes, the UE radio capability indicating support of multicast reception in RRC_INACTIVE state can be reported to RAN, which is subject to the discussion of UE radio capability.

From RAN perspective, the capability to indicate whether UE supports inactive multicast reception helps gNB to determine which UEs to be released to RRC_INACTIVE for congestion relief. Therefore, explicit signaling should be supported to report this capability to the gNB. Besides, even UE does not support the feature of inactive multicast reception, the gNB can allow it to receive the multicast service by Rel-17 manner, i.e., it should not be mandatory for UE to support inactive multicast reception.

Proposal 10: The MBS multicast reception in RRC_INACTIVE is defined as an optional UE capability with signaling.

- 3. Conclusion

Based on the discussions, we have the following observations and proposals for CP for Multicast reception in RRC_INACTIVE,

Observation 1: The legacy resume cause is not suitable for RRC resume due to that PTM configuration is no available for UE in INACTIVE in the reselected cell which is in the RNA.

Observation 2: The legacy resume cause is not suitable for RRC resume due to bad reception quality.

Multicast MCCH

Proposal 1: Multicast MCCH should be present for a cell providing multicast reception in RRC_INACTIVE.

Details on frequency prioritization

Proposal 2: For frequency prioritization of multicast reception in RRC_INACTIVE, Rel-17 broadcast mechanism is taken as baseline, i.e., SIB21+USD based frequency prioritization.

Proposal 3: RAN2 assumes USD can contain the mapping between MBS FSA ID and multicast session provided to UEs in INACTIVE. Send LS to SA2 to confirm this assumption.

Proposal 4: The mapping between frequency and FSAI for multicast reception in INACTIVE is provided in a new SIB.

Service continuity

Proposal 5a: RSRP/RSRQ is used as the quality threshold.

Proposal 5b: The quality threshold is configured in multicast MCCH.

Proposal 5c: The granularity of quality threshold is per multicast session.

Proposal 6: Discuss whether to introduce new resume cause for RRC resume due to the following,

- PTM configuration is no available for UE in INACTIVE in the reselected cell which is in the RNA

- bad reception quality

Proposal 7a: UE determines the MBS session as active upon receiving the PTM configuration in RRCRelease if it has been configured to receive multicast in RRC_CONNECTED state.

Proposal 7b: If the MBS session is not activated when receives PTM configuration in RRCRelease, UE does not perform MBS multicast reception in RRC_INACTIVE immediately.

Proposal 7c: If the MBS session is not activated when UE receives PTM configuration in RRCRelease, it is assumed the PTM configuration does not change until session activation.

Proposal 7d: If the MBS session is not activated when UE receives PTM configuration in RRCRelease, UE directly uses the stored PTM configuration for multicast reception and starts to monitor MCCH DCI for change notification upon receiving group paging that indicates to allow the inactive multicast reception.

Proposal 7e: If UE did not receive PTM configuration in RRCRelease due to session deactivation, UE reads MCCH to acquire the PTM configuration upon receiving group paging that indicates to allow the inactive multicast reception.

Notification for session deactivation

Proposal 8: The session deactivation is indicated in the content of multicast MCCH.

Coexistence between inactive multicast and SDT

Proposal 9: The MBS multicast reception in RRC_INACTIVE can be configured together with SDT, but UE does not monitor group paging during SDT.

UE capabilities

Proposal 10: The MBS multicast reception in RRC_INACTIVE is defined as an optional UE capability with signaling.

- 4. Reference

[1] R2-2306855 RRC running CR for eMBS

[2] R2-2210882	Reply LS on FS_5MBS_Ph2 progress RAN2 LS out Rel-18 NR_MBS_enh-Core, FS_5MBS_Ph2 To:SA2, RAN3 Cc:RAN1