3GPP TSG-RAN WG2 Meeting #123                                                                                          R2-2307264

Toulouse, France, August 21-25, 2023

Source:	CATT, CBN

Title:	Discussion on User Plane for Multicast reception in RRC_INACTIVE

Agenda Item:	7.11.2.2

Document for:	Discussion and Decision

1	Introduction

In RAN2#121bis-e meeting, the following agreements have been reached on user plane for multicast reception in RRC_INACTIVE as in [1].

From the location&bandwidth and SCS configuration perspective, follow R17 MBS broadcast CFR principle (i.e. case A,C,E) to provide multicast CFR configuration in RRC_INACTIVE. Multicast CFR in RRC_INACTIVE and broadcast CFR can be configured differently. FFS whether we need to restrict that one CFR is completely contained within the other in this case (we should understand what the issue is otherwise). Case B and case D are not supported for multicast CFR in RRC_INACTIVE; Whether multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different is up to NW implementation. FFS whether this causes some issues which need to be addressed. Change the working agreement to the agreement below: - Agreement: The same CFR is used for multicast MCCH and MTCH. It can be revisited if there is any issue found, e.g. for RedCap UEs. HARQ feedback related information in the DCI is not needed or can be ignored for multicast transmission to RRC_INACTIVE UE. ( The HARQ operation for multicast reception in RRC_INACTIVE is same as the operation without HARQ feedback in RRC_CONNECTED state. The multicast transmission RRC_INACTIVE is performed via beam sweeping based on SSB index like broadcast MBS (i.e. beam information is not need in DCI. For MTCH, RAN2 assumes to reuse the same DCI format of R17 multicast (i.e. DCI format 4-1/4-2) for dynamic scheduling of multicast in RRC INACTIVE. RAN2 assumes for MCCH scheduling, DCI format 4-0 is used. We will ask RAN1 to confirm whether it is feasible and whether both 4-1 and 4-2 are needed. We will also indicate other relevant agreements to RAN1 (e.g. on beam sweeping etc.) On support of multicast SPS in RRC_INACTIVE, postpone RAN2 discussion to next meeting. On DRX operation for multicast in RRC_INACTIVE, take the multicast DRX as baseline. FFS handling on PTM related HARQ RTT Timer and DRX Retransmission Timer. The common LCID space is used for multicast MRB and unicast DRB regardless of UE RRC state (i.e. no change on the LCID table for MTCH). Postpone the UP discussion on L2 operation during RRC state transition until the signaling design of PTM configuration in RRCRelease message is concluded. Postpone the discussion on L2 operation during mobility to next RAN2 meeting. Including the following two issues in LS to RAN1: - Issue 1: RAN1 to confirm RAN2 understanding that PDSCH aggregation is supported for multicast MTCH in RRC_INACTIVE (as that is supported in Rel-17 multicast MTCH in RRC_CONNECTED as well as broadcast MTCH). - Issue 2: RAN1 to check the feasibility of following Rel-17 CSS design for multicast MTCH and MCCH: 1) reusing the same CSS for multicast MTCH in RRC_INACTIVE (same as multicast MTCH in RRC_CONNECTED); 2) separate CSS for MCCH and MTCH.

In this contribution, we discuss the remaining issues. The contribution is organized as the following. In section 2, we discussed remaining issues on the following aspects,

- Multicast CFR in RRC_INACTIVE

- L2 operation during state transitions

- L2 operation during mobility

- MAC operation

- SPS for multicast in RRC_INACTIVE

Our proposals are summarized in Section 3.

2	Discussion

## 2.1 Multicast CFR in RRC_INACTIVE

Relation between Multicast CFR in RRC_INACTIVE and broadcast CFR

For the relation between Multicast CFR in RRC_INACTIVE and broadcast CFR (i.e., whether one CFR is completely contained within the other), it was discussed previously in RAN2#121bies-e meeting but not concluded. A FFS is left in the agreements as in [1],

Multicast CFR in RRC_INACTIVE and broadcast CFR can be configured differently. FFS whether we need to restrict that one CFR is completely contained within the other in this case (we should understand what the issue is otherwise).

Some one company think one of the two CFRs need to be fully contained (or overlapping) with the other CFR and it enables UE to just monitor the larger CFR and to receive both services without BWP switch.

However, there is no issue if one CFR is not completely contained within the other. First of all, BWP switching is not needed as anyway the two CFRs are partial overlapped due to that both the CFRs need to contain CORSET#0. UE only need to monitor a large region that covers both the two CFRs without BWP/CFR switching.

Then, even in case one CFR is completely contained within the other, UE cannot work on one CFR (e.g., the larger CFR). UE needs to monitor PDCCH DCIs in separate CFRs for multicast and broadcast and to decode PDSCH in different CFRs according to the content of DCI. The reason is that downlink resource allocation for multicast/broadcast is calculated according to the related CFR, According to TS 38.214, as below,

A downlink type 1 resource block assignment field in the DCI format 4_0 or DCI format 4_1 consists of a RIV corresponding to a starting resource block in reference to the lowest RB of the CFR and a length in terms of virtually contiguously allocated resource blocks LRBs, where is given by - the size of CORESET 0 if CORESET 0 is configured for the cell; - the size of initial DL bandwidth part if CORESET 0 is not configured for the cell. The resource indication value is defined by: if then else where, and where shall not exceed . If NCFR>NBWPinitial, K is the maximum value from set {1, 2, 4, 6, 8, 10, 12} which satisfies K≤NCFR/NBWPinitial; otherwise K = 1.

Therefore, it is proposed that,

Proposal 1: If multicast CFR in RRC_INACTIVE and broadcast CFR are configured differently, there is no restriction that one CFR is completely contained within the other.

Relation between multicast CFR in RRC_CONNECTED and in RRC_INACTIVE

In RAN2#121bis-e, it was agreed that multicast CFR in RRC_CONNECTED and in RRC_INACTIVE can be configured different and a FFS is left on whether it may causes some issues, as in [1],

Whether multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different is up to NW implementation. FFS whether this causes some issues which need to be addressed.

If multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different, one thing to clarify is whether to use one DCI to schedule both UEs in RRC_INACTIVE and RRC_CONNECTED; or we have 1 DCI to schedule UEs in RRC_INACTIVE and 1 DCI to schedule UEs in RRC_CONNECTED.

According to RAN1 spec, the frequency resource allocation in DCI format for multicast is related to the CFR used. So it is not feasible to use only one DCI to schedule both UEs in RRC_INACTIVE and RRC_CONNECTED.

Proposal 2: If multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different, two DCIs are used to schedule UEs in RRC_INACTIVE and UEs in RRC_CONNECTED separately.

In RAN2 TEI18, redcap CFR for MBS broadcast service was introduced in recent RAN2 meetings. i.e., in one cell there can be one normal CFR for normal UE and one redcap CFR for redcap UEs. Later a DCI scheduling issue on redcap CFR was discussed in RAN2#122 meeting, i.e., UE may receive an unexpected DCI if two separate DCIs are scheduled in the overlapping frequency area and UE use a wrong DCI to decode PDSCH. To avoid the issue, it is necessary to specify that UE does not expect to receive two DCIs (i.e. target for normal UE and redcap UE, respectively) scheduling the PDSCH(s) for the same broadcast service. Finally an agreement was reached on this as in [2],

Network shall ensure that the UE doesn’t receive DCIs targeting different CFR for same GRNTI.

If multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different and they are overlapped, the above issue for redcap CFR also exist in this case as the situation are the same, i.e, UE may receive an unexpected DCI if two separate DCIs are scheduled in the overlapping frequency area of the two CFRs and UE uses a wrong DCI to decode PDSCH.

Therefore, NW should ensure that the UE doesn’t receive DCIs targeting different CFRs for same G-RNTI if multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different and they are overlapped.

Proposal 3: If multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different and they are overlapped, NW should ensure that the UE doesn’t receive DCIs targeting different CFRs for same G-RNTI.

## 2.2 L2 operation during state transitions

L2 operation during state transitions was discussed in RAN2#121bis-e meeting via an offline discussion [3], there were some proposals generated by the offline discussion as below,

Proposal 12 (for agreement, 13/17): When entering RRC_INACTIVE state, UE does not suspend multicast MRB(s) configured for the multicast reception in RRC_INACTIVE. Proposal 13 (for agreement, 13/17): When entering RRC_INACTIVE state, UE doesnot stop the MAC DRX timers (i.e. drx-onDurationTimerPTM or drx-InactivityTimerPTM) configured for the multicast reception in RRC_INACTIVE. Proposal 14 (for discussion, 12/17):RAN2 to discuss whether supports the HARQ continuation for MBS multicast reception (i.e. not flushing the soft buffer used for MBS multicast) during the RRC state transition.

But finally the related discussion was postponed to wait for the conclusion on the signaling design of PTM configuration in RRCRelease message as there is a dependency on it.

Postpone the UP discussion on L2 operation during RRC state transition until the signaling design of PTM configuration in RRCRelease message is concluded.

Then in RAN2#122 meeting, the signaling design of PTM configuration in RRCRelease message is concluded but handling of the existing MRBs used in CONNECTED is still kept as FFS,

As a baseline, The PTM configuration in the RRCRelease message with suspendconfig has the same structure as the PTM configuration in multicast MCCH. FFS how existing MRBs are handled.

Since the PTM configuration for inactive multicast reception (i.e., the one in RRC release or Multicast MCCH) is take the broadcast configuration as baseline and in R17 MBS the multicast configuration and broadcast configuration are in completely different structure, UE cannot continue to use the existing MRBs in CONNECTED when UE moves to INACTIVE state. Then the next question is whether UE should keep the existing MRBs for later use (i.e., when UE enter CONNECTED state later). It seems straightforward to support it as it is already supported in R17 MBS.

Therefore, it is proposed that,

Proposal 4: When entering RRC_INACTIVE state, UE suspends the existing MRBs used in CONNECTED state.

## 2.3 L2 operation during mobility

L2 operation during state transitions was also discussed in RAN2#121bis-e via an offline discussion [3], there were some proposals generated by the offline discussion as below. The basic idea is UE does not need to release and re-setup the PDCP entity (i.e. re-initiate the PDCP variables) of the multicast MRB if PDCP COUNT are synchronized between source and target cell during cell reselection in INACTIVE state.

Proposal 15.1 (for agreement, 12/17): UE does not need to re-establish PDCP entity (i.e. re-initiate the PDCP variables) of the multicast MRB if PDCP COUNT can be sync between source and target cell during INACTIVE mobility. Proposal 15.2 (for discussion): If PDCP count is sync between source and target cells, following open issues on PDCP handling during INACTIVE mobility need further discussion: - Open issue 1: How does UE know PDCP count is sync between cells? - Open issue 2: Whether to standardize the UE PDCP operation during INACTIVE mobility?

But the discussion on this issue was postponed,

Postpone the discussion on L2 operation during mobility to next RAN2 meeting.

In R17 MBS broadcast, there is no mechanism specified to ensure PDCP continuity during mobility (i.e., in IDLE/INACTIVE). For inactive multicast reception, it has similar reliability mechanism (i.e., no HARQ feedback based retransmission, no L2 feedback based retransmission), QoS requirement for inactive multicast reception should be similar as MBS broadcast (i.e., PDCP continuity during mobility in INACTIVE is not needed as data lossless is not required). It seems not essential to specify a mechanism to support PDCP continuity during mobility in INACTIVE.

Observation 1: It is not essential to support PDCP continuity during mobility in INACTIVE.

If PDCP continuity during mobility in INACTIVE is needed, we should only consider a simple solution considering the TU and timeline. e.g., we should avoid RAN3 impacts at this stage due to that it is an optimization and we are approaching feature frozen timeline.

Observation 2: If PDCP continuity during mobility in INACTIVE is to be supported, RAN3 impacts should be avoided.

In R17 MBS, the same QoS flow of one broadcast session can be mapped to different MRB ID in source cell and target cell. So this case should be considered if we consider mechanism for PDCP continuity during mobility in INACTIVE.

Observation 3: If PDCP continuity during mobility in INACTIVE is to be supported, we need to consider the case that MRB ID allocation for a multicast session in source cell and target cell are different.

If PDCP continuity during mobility in INACTIVE is to be supported, potentially there is one simple solution can be used. In R17 MBS, derivation of the PDCP COUNT values by means of a DL MBS QFI Sequence Number is optionally supported. Based on this, The gNB can inform the UE that which QoS flow(s) of the multicast session maps to which MRB If PDCP COUNT values are derived from a DL MBS QFI Sequence Number provided on NG-U. Then, based on the mapping relationship, the UE determines whether the MRB is mapped from same QoS flows on the source and target cell. If it is, UE can skip the step of releasing old PDCP entity for this MRB and instead update the PDCP configuration directly based on the old PDCP configuration, while retaining the value of state variables of the PDCP entity.

Proposal 5: If PDCP continuity during mobility in INACTIVE is to be supported, UE is indicated the mapping between QoS flow(s) to the MRB.

## 2.4 MAC operation

The handling of PTM related HARQ RTT Timer and DRX Retransmission Timer for multicast in RRC_INACTIVE is FFS according to the agreement of RAN2#121bis-e meeting [1],

On DRX operation for multicast in RRC_INACTIVE, take the multicast DRX as baseline. FFS handling on PTM related HARQ RTT Timer and DRX Retransmission Timer.

PTM related HARQ RTT Timer and DRX Retransmission Timer are only used when HARQ feedback is enabled. However, it is agreed that HARQ feedback is not support for multicast reception in RRC_INACTIVE, according to RAN2 agreements [1] as below,

HARQ feedback related information in the DCI is not needed or can be ignored for multicast transmission to RRC_INACTIVE UE. ( The HARQ operation for multicast reception in RRC_INACTIVE is same as the operation without HARQ feedback in RRC_CONNECTED state.

So PTM related HARQ RTT Timer and DRX Retransmission Timer should not be configured for inactive multicast reception.

Proposal 6: For the DRX operation for multicast in RRC_INACTIVE, PTM related HARQ RTT Timer and DRX Retransmission Timer are not configured.

Another issue related to HARQ is whether UE can receive the HARQ retransmission triggered by UE receiving the same multicast service in CONNECTED state. One may argue that it is beneficial to improve the reliability of inactive multicast reception. However, the UE in INACTIVE is not aware whether there are UEs receiving the same multicast service in the RRC_CONNECTED state and whether the HARQ feedback is enabled in case there are UEs receiving the same multicast service in the RRC_CONNECTED state. So it is not reasonable for UE in INACTIVE state to blindly wait for the HARQ retransmission and it increase the UE power consumption unnecessarily.

Proposal 7: UE receiving multicast in INACTIVE state does not expect to receive HARQ retransmission.

## 2.5 SPS for multicast in RRC_INACTIVE

Whether to support SPS for multicast in RRC_INACTIVE was discussed in RAN2#121bis-e via an offline discussion [3] but it was postponed finally in that meeting,

On support of multicast SPS in RRC_INACTIVE, postpone RAN2 discussion to next meeting.

During the offline discussion [3], issues on missing L1 Activation/deactivation indication in different cases was raised by companies as below,

- How to ensure the UEs receive the L1 Activation/deactivation indications. The repetition way doesn’t work, as the SPS occasion is related to the timing of receiving L1 indication. If using repetition, different UE may receive the L1 indication at different times and interprets different SPS occasions.

- How can a UE moving to this cell receive a L1 indication sent already to the other UEs previously?

- How to handle INACTIVE UEs if NW uses unicast way for the CONNECTED UEs?

Due to that the identified issues above is hard to resolve and SPS is not essential for multicast reception in INACTIVE and we are approaching the feature frozen timeline, it is preferred to not support SPS for inactive multicast reception, and if common resources/CFR are used for UEs in INACTIVE and UEs in CONNECTED, SPS is also not configured for UE in CONNECTED.

Proposal 8: SPS is not supported for multicast reception in RRC_INACTIVE.

Proposal 9: If common resources/CFR are used for UEs in INACTIVE and UEs in CONNECTED, SPS is also not configured for UEs in CONNECTED.

3	Conclusion

Based on the discussions, we have the following observations and proposals for UP for Multicast reception in RRC_INACTIVE,

Observation 1: It is not essential to support PDCP continuity during mobility in INACTIVE.

Observation 2: If PDCP continuity during mobility in INACTIVE is to be supported, RAN3 impacts should be avoided.

Observation 3: If PDCP continuity during mobility in INACTIVE is to be supported, we need to consider the case that MRB ID allocation for a multicast session in source cell and target cell are different.

Multicast CFR in RRC_INACTIVE

Proposal 1: If multicast CFR in RRC_INACTIVE and broadcast CFR are configured differently, there is no restriction that one CFR is completely contained within the other.

Proposal 2: If multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different, two DCIs are used to schedule UEs in RRC_INACTIVE and UEs in RRC_CONNECTED separately.

Proposal 3: If multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different and they are overlapped, NW should ensure that the UE doesn’t receive DCIs targeting different CFRs for same G-RNTI.

L2 operation during state transitions

Proposal 4: When entering RRC_INACTIVE state, UE suspends the existing MRBs used in CONNECTED state.

L2 operation during mobility

Proposal 5: If PDCP continuity during mobility in INACTIVE is to be supported, UE is indicated the mapping between QoS flow(s) to the MRB.

MAC operation

Proposal 6: For the DRX operation for multicast in RRC_INACTIVE, PTM related HARQ RTT Timer and DRX Retransmission Timer are not configured.

Proposal 7: UE receiving multicast in INACTIVE state does not expect to receive HARQ retransmission.

SPS for multicast in RRC_INACTIVE

Proposal 8: SPS is not supported for multicast reception in RRC_INACTIVE.

Proposal 9: If common resources/CFR are used for UEs in INACTIVE and UEs in CONNECTED, SPS is also not configured for UEs in CONNECTED.

4	Reference

[1] R2-2306553, Report of 3GPP TSG RAN WG2 meeting #121bis-e

[2] Draft_R2_122_meeting_report_v2, https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_122/Report

[3] R2-2303420, Report of [Post121][607][eMBS] UP issues for Multicast in RRC Inactive (Apple)