3GPP TSG-RAN WG2 Meeting #123 	R2-2307270

Toulouse, France, Aug 21st – Aug 25th, 2023

Title:	Connected mode enhancement for mobile IAB

Source:	Huawei, HiSilicon

Agenda item:	7.12.2.1

Document for:	Discussion

- Introduction

During the RAN2#122 meeting, following agreement was achieved for the connected mode UE mobility enhancement for mobile IAB [1].

- RAN2 think that to have a fast handover from UE point of view for legacy UEs it is important that the target cell is known to the UE (detected and measured). - For RACH-less, if supported, there would need to be a beam indication (in RRC HO command), which seems feasible in this release from R2 perspective. R2 assumes that the network can know/select the beam, either from network impl specific knowledge or from UE measurement report (legacy report). - for the UL grant and HO completion in RACH-less HO: 1. Both type-1 configured grant and dynamic grant are supported 2. FFS handling of supervision timer and when HO is considered successfully complete (expect to align with other WI). - Send LS to RAN3 to check whether there are issues / feasibility concerns

This contribution mainly focuses on the remaining issues of connected mode UE mobility enhancement for mobile IAB.

- Discussion

2.1 Connected mode on-board UE identification by NW

As for on-board UE in RRC_CONNECTED state, once identified by the NW, some possible enhancements may be performed for this UE, e.g., CHO configuration, RACH-less, and RRM measurement relaxation.

Observation 1: NW may consider to configure on-board UE with CHO, based on its knowledge of the on-board UE status in addition to the legacy RSRP measurement report.

Another usage for NW to identify on-board UE can be configuring the measurement configuration. During DU migration, on-board UE is supposed to be handed over from source DU to target DU. However, typically, the RSRP of this source cell and target cell are quite similar. There may be quite a chance that on-board UE cannot trigger measurement report, due to the similar RSRP between source cell and target cell. So, NW implementation may want to configure some different trigger event parameters for on-board UE, so that at least on-board UE can report the measurement result to initiate the full migration. In this case, it is beneficial for NW to identify the connected on-board UE.

Observation 2: NW may configure specific measurement report configurations to on-board UEs (could be different with other UEs), so that those UE can timely report the measurement result of the target DU’s cell during DU migration.

Besides, currently UE can include its latest mobility state in RRCSetupComplete/RRCResumeComplete and NW may provide proper mobility configuration to UE according to this state. So if on-board UE is moving fast together with the mobile IAB-node, it will detect itself in Normal-mobility state since it is relatively stationary with the serving cell i.e. the mobile-IAB cell, and report this state to NW. As to surrounding UE, it will also report Normal-mobility state to NW because of its low mobility in the surrounding cells before it camps on the mobile-IAB cell. Therefore, it is necessary for served UE to indicate its on-board status to NW during the establishment or resume of RRC connection, so that NW can identify the type of a specific UE and consider the suitable handling for later mobility configuration (since on-board UE will move together but surrounding UE will leave the mobile IAB cell).

Observation 3: NW may consider different handling between on-boarding UEs and surrounding UEs, even when both kinds of UEs report the same value of mobilityState as “Normal”.

Hence, it is proposed,

Proposal 1: RAN2 to support the on-board UE identification in RRC_CONNECTED for below motivations:

- used for NW to determine whether UE is suitable to be configured with CHO, in addition to the RSRP measurement reporting;

- used for NW to configure on-board UE with specific measurement report configuration, so that it can timely report the measurement result of target DU’s cell during DU migration;

- used for NW to consider different handling between on-boarding UEs and surrounding UEs, even when both kinds of UEs report the same value of mobilityState as “Normal”.

2.2 RACH-less HO

RACH-less HO happens in DU migration scenario, where the target and source cells locate at the same mobile IAB-node so that the served UE’s TA remains unchanged. The same TA indication should be supported in mobile IAB, while some other kinds of TA indications may be supported in other WIs, e.g., the 0 TA indication in NTN. Therefore, for the mobile IAB scenario, RAN2 should focus on the TA related issues, e.g., how the NW know the RACH-less with same TA can be used. As for other issues like the configured/dynamic grant, we can wait for other WIs discussion (NTN and LTM), and to see whether the conclusion of other WIs can be reused or need to be revised. For the mobile IAB running CR, we suggest not to capture the RACH-less related part at this stage, and wait to see if we can reuse the text from running CR from other WIs. It is proposed:

Proposal 2: RAN2 consider the following way forward for RACH-less in mobile IAB:

- In mobile IAB, RAN2 focuses on the TA related issues in RACH-less, while for other issues like the details of CG/DG, RAN2 waits for the conclusion from other WIs (NTN, LTM).

- In mobile IAB running CR, RAN2 postpone capturing the RACH-less related impacts and wait to see if the text from running CRs from other WIs (NTN, LTM) can be reused.

- Conclusion

This paper mainly discusses the remaining issues of mobility enhancement for mobile IAB, and the following observations and proposals are provided.

Observation 1: NW may consider to configure on-board UE with CHO, based on its knowledge of the on-board UE status in addition to the legacy RSRP measurement report.

Observation 2: NW may configure specific measurement report configurations to on-board UEs (could be different with other UEs), so that those UE can timely report the measurement result of the target DU’s cell during DU migration.

Observation 3: NW may consider different handling between on-boarding UEs and surrounding UEs, even when both kinds of UEs report the same value of mobilityState as “Normal”.

Connected mode on-board UE identification by NW

Proposal 1: RAN2 to support the on-board UE identification in RRC_CONNECTED for below motivations:

- used for NW to determine whether UE is suitable to be configured with CHO, in addition to the RSRP measurement reporting;

- used for NW to configure on-board UE with specific measurement report configuration, so that it can timely report the measurement result of target DU’s cell during DU migration;

- used for NW to consider different handling between on-boarding UEs and surrounding UEs, even when both kinds of UEs report the same value of mobilityState as “Normal”.

RACH-less

Proposal 2: RAN2 consider the following way forward for RACH-less in mobile IAB:

- In mobile IAB, RAN2 focuses on the TA related issues in RACH-less, while for other issues like the details of CG/DG, RAN2 waits for the conclusion from other WIs (NTN, LTM).

- In mobile IAB running CR, RAN2 postpone capturing the RACH-less related impacts and wait to see if the text from running CRs from other WIs (NTN, LTM) can be reused.

- References

- Chairman notes of 3GPP TSG-RAN WG2 meeting #122.