3GPP TSG-RAN WG2 Meeting #123	R2-2307283

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.13.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Reply LS proposal to R2-2307030/R3-233380

WID/SID:	NR_ENDC_SON_MDT_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN2 has received LS from RAN3 on the topic of the information retrieval (“LS on SHR and SPR”) in R2-2307030 (R3-233380.zip). We think that a similar issue is also discussed within the Reply LS on LBT-FailureRecoveryConfig (“Response LS on Possibility on LBT-FailureRecoveryConfig” in R2-2300031.zip).

This paper discusses this issue and proposes a reply LS to RAN3.

# 2	Discussion

## 2.1	UE identification issue in SON/MDT reports

According to the RAN3 LS there is a problem how to retrieve mobility information related to a UE in different mobility scenarios. RAN3 has found some issues with the solution of reporting the source C-RNTI and time since receiving HO/PSCell change/addition command:

But since the possibility to retrieve information related to a specific UE depends on gNB's implementation. e.g. whether it can store the UE related configuration of interest during the wanted time (up to 48 hours) after a successful HO or PSCell change/addition, RAN3 discussed an alternative solution to enable the retrieval of UE related information without requiring this to be stored on a per UE basis.

In previous RAN3 reply LS on Possibility on LBT-FailureRecoveryConfig (R2-2300031.zip) a similar issue was brought up that using the C-RNTI to identify the UE has some limitations:

RAN3 has discussed the issue and observed that there is an existing network-based mechanism that can be reused for the NR-U case, based on the information provided from the UE (last serving PCell ID and C-RNTI), that enables the RAN to retrieve the UE context or the configuration used for the UE in the last serving node and possibly the configuration used for the UE in the source node (if the Mobility Information defined in TS 38.423 is used).

RAN3 observes though, that the above mechanism was designed with intention to retrieve UE context or the configuration information if the UE attempts reconnection and reports the failure right after connection failure; if the failure information is fetched from the UE hours after the failure, then the likelihood that the source and the last serving node can retrieve the needed information depends on RAN implementation and is practically minimal (this depends on RAN implementation, e.g., how long the gNB stores the UE context or how long that allocated C-RNTI is not reused by the RAN).

Observation 1.1: Using the C-RNTI for UE identification in SON/MDT reports is not safe solution, especially if the report is provided to the network a significant time after an event happened.

As RAN3 also recognized the problem of the above observation, further studied the issue, and concluded that a potential solution is to use a “reference index” to the “configuration information”:

- The node involved in mobility procedure (via implementation) create references to a configuration or a set of parameters used by one or a group of UEs and can send this “Configuration Information” to the UE in order to assist in the analysis of SHR or SPR or other SON reports (if needed).

- this “Configuration Information” can be optionally sent to the UE in dedicated signaling (e.g., together with the SHR/SPR configuration or in any other RRCReconfiguration). If received, UE should then store this “Configuration Information” together with the SON report(s) and UE should report it back to the gNB along with the SON report(s) (e.g., SHR/SPR).

- How to encode this “Configuration Information” is up to RAN2, taking into account RAN3 guidance related to the size of this information. One example to do this would be to encode this as an OCTET STRING (e.g., 32 bits) as is done for Mobility Information in XnAP (i.e., TS 38.423).

- This solution shall cover all existing mobility scenarios, including dual-connectivity scenarios.

We think that this type of solution is a future proof solution and efficient solution that can be applied in other cases as well, e.g., in the issue described above related to RAN3 reply LS on Possibility on LBT-FailureRecoveryConfig. The addition of the new parameter (reference index) into the RRC messages is simple, especially that it is a relatively short transparent parameter (e.g., 32 bits), which requires no additional processing in the UE beyond its storage.

Proposal 1: RAN2 should reply to Q1 and Q2 of the RAN3 LS on SHR and SPR that the proposed solution is feasible and the required RRC changes can be specified.

The proposed solution approach of the RAN3 LS on SHR and SPR could also be applied in the case of RLF reports:

- A new configuration index parameter is provided by the network with the configuration.

- The UE stores the configuration index and includes it within the RLF reports.

The configuration index is associated in the network node with specific configuration parameters that are important for the network node to analyse the reported failure. This mechanism could also be applied or extended to the RLF reports which are related to LBT or RA issues. The information associated with the index is up-to network node implementation and the UE is not aware of it. The configuration index does not need to identify UE’s configuration as a block: it may be implemented so that e.g., first few bits refer to one of UE types (for example, for fast UEs, for modem devices etc), then few next bits may identify one of the used mobility configurations for the given UE type, and then few next bits one of the used NR-U configurations for this UE. Or, alternatively, an implementation may use all the 32 bits to identify a UE context with an own ID that is kept for longer than the C-RNTI.

This configuration index can be a relative short (e.g., 32-bit, to match RAN3’s Mobility Information) optional parameter, and thus it will not generate significant signalling overhead, and storage requirement. It will automatically solve the issue that the network node may change its configuration after the event happened or removed the UE context, as in this case the network can also change the relevant configuration index value. The solution is also future proof, as a network node implementation may add new associated parameters to a configuration index without additional standardization work.

The configuration index for the RLF Report may be delivered to the UE as an optional IE in the RRCRecomfiguration message. Once delivered, the UE stores it and replaces old index (if any stored previously). It is up to the implementation of the serving node to decide when a new configuration index is to be delivered to the UE – if particular reconfiguration concerns parameters that are not relevant for possible future MRO analysis, the network may skip updating the index. Once a failure occurs and the UE compiles the RLF Report, it includes the last received configuration index. When the report is finally fetched and delivered to the last serving node, the node knows how to read it: either uses it to identify the configuration used for the UE, or in any other way the index was formulated (possibly even to identify the UE context, if the implementation used the index this way).

One point to observe is that if the configuration index defined for the UE matches RAN3’s Mobility Information, the implementation of the feature on the network side will be trivial – the same logic that is used already now for the Mobility Information can be used to define the UE’s configuration index.

Proposal 2.1: Introduce a new optional configuration index parameter that is provided by the network with the configuration in the RRCReconfiguration message. The UE stores only the most recently received configuration index and provides it within the RLF reports.

Proposal 2.2: Inform RAN3 in the reply LS about the introduction of the configuration index for RLF reports.

## 2.2	Other questions of LS on SHR and SPR” (R3-233380)

The RAN3 LS contains an additional related question:

Q3: In cases when this “Configuration Information” is not configured by the network to the UE, RAN3 discussed whether UE can include the source cell C-RNTI and the time between the event that triggered the report and the sending of the report to the network. RAN3 wants to check with RAN2 if it’s feasible in the above scenario?

Our view is that this alternative option is feasible, but not desired. It would be less complex from RAN2 specification and UE/network implementation perspective if only a single solution is specified.

Proposal 3: RAN2 should reply to Q3 of R3-233380 that it is feasible, but having two solution options would increase specification and implementation complexity.

The last question in the RAN3 LS is the following:

Further, RAN3 discussed the following information to be reported in the SPR to assist in the forwarding of SPR over network interfaces:

- CGI of the PCell which sent the SPR configuration (already agreed in RAN3);

- Indication whether the PSCell change was MN-initiated or SN-initiated (WA but not agreed in RAN3). Explicit or implicit indicator can be decided by RAN2.

Q4: RAN3 kindly asks RAN2 to confirm RAN3’s understanding on CGI of the PCell and update their specifications if feasible, and also take the above information for indication on whether the PSCell change was MN-initiated or SN-initiated into account and provide feedback.

The Rel-17 RAN2 specification already mandates to store the CGI of the source PCell if it is available:

4> set the sourceCellID in sourceCellInfo to the global cell identity and tracking area code, if available, of the source PCell;

We think this approach is also applicable for SPR in Rel-18. Therefore, we propose that RAN2 confirms RAN3’s understanding. We also think that RAN2 could confirm that an indication on whether the PSCell change was MN-initiated or SN-initiated is feasible.

Proposal 4: RAN2 confirms RAN3’s understanding on CGI of the PCell reporting and RAN2 sees no problem of indicating whether the PSCell change was MN-initiated or SN-initiated.

A draft reply LS is provided in the Annex based on the above proposals.

Proposal 5: Consider the draft reply LS provided in the Annex as a baseline.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: Using the C-RNTI for UE identification in SON/MDT reports is not safe solution, especially if the report is provided to the network a significant time after an event happened.

Proposal 1: RAN2 should reply to Q1 and Q2 of the RAN3 LS on SHR and SPR that the proposed solution is feasible and the required RRC changes can be specified.

Proposal 2.1: Introduce a new optional configuration index parameter that is provided by the network with the configuration in the RRCReconfiguration message. The UE stores only the most recently received configuration index and provides it within the RLF reports.

Proposal 2.2: Inform RAN3 in the reply LS about the introduction of the configuration index for RLF reports.

Proposal 3: RAN2 should reply to Q3 of R3-233380 that it is feasible, but having two solution options would increase specification and implementation complexity.

Proposal 4: RAN2 confirms RAN3’s understanding on CGI of the PCell reporting and RAN2 sees no problem of indicating whether the PSCell change was MN-initiated or SN-initiated.

Proposal 5: Consider the draft reply LS provided in the Annex as a baseline.

# Annex A: Draft Reply LS to R2-2307030

Title:	[DRAFT] Response LS on SHR and SPR

Response to:	R3-233380/ R2-2307030

Release:	Release 18

Work Item:	NR_ENDC_SON_MDT_enh2-Core

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN WG3

Cc:

Contact Person:

#### Name:	György Wolfner

E-mail Address:	gyorgy<dot>Wolfner<at>nokia<dot>com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 would like to thank RAN3 for their LS on SHR and SPR. RAN2 discussed the issue, and provide the following answers to the questions:

Q1: Whether RAN2 sees any issues in defining a solution for “Configuration Information” as described above?

Q2: For SHR/SPR, is there any issue to include this “Configuration Information” in the RRC Reconfiguration message with sync containing Handover Command or PSCell change command?

Answer to Q1 and Q2:

RAN2 concluded that it is feasible to specify this type of solution from RAN2 perspective and to include this “Configuration Information” in the RRC Reconfiguration message with sync containing Handover Command or PSCell change command.

RAN2 also noticed that a similar type of issue was discovered in RAN3 Response LS on Possibility on LBT-FailureRecoveryConfig (R3-226809/R2-2300031):

RAN3 has discussed the issue and observed that there is an existing network-based mechanism that can be reused for the NR-U case, based on the information provided from the UE (last serving PCell ID and C-RNTI), that enables the RAN to retrieve the UE context or the configuration used for the UE in the last serving node and possibly the configuration used for the UE in the source node (if the Mobility Information defined in TS 38.423 is used).

RAN3 observes though, that the above mechanism was designed with intention to retrieve UE context or the configuration information if the UE attempts reconnection and reports the failure right after connection failure; if the failure information is fetched from the UE hours after the failure, then the likelihood that the source and the last serving node can retrieve the needed information depends on RAN implementation and is practically minimal (this depends on RAN implementation, e.g., how long the gNB stores the UE context or how long that allocated C-RNTI is not reused by the RAN).

RAN2 further discussed this issue and concluded to introduce a new optional configuration index parameter that is provided by the network with the configuration in the RRCReconfiguration message. The UE stores only the most recently received configuration index and provides it within the RLF reports if it is sent by the network to enable the identification of the network configuration when RLF happened.

Q3: In cases when this “Configuration Information” is not configured by the network to the UE, RAN3 discussed whether UE can include the source cell C-RNTI and the time between the event that triggered the report and the sending of the report to the network. RAN3 wants to check with RAN2 if it’s feasible in the above scenario?

Answer to Q3:

RAN2 concluded that the UE can include the source cell C-RNTI and the time between the event that triggered the report and the sending of the report to the network. However, RAN2 thinks, if the solution based on the description of the LS is specified (see answers to Q1 and Q2) then this solution is not necessary, and it is better to avoid implementation options in the specifications.

Q4: RAN3 kindly asks RAN2 to confirm RAN3’s understanding on CGI of the PCell and update their specifications if feasible, and also take the above information for indication on whether the PSCell change was MN-initiated or SN-initiated into account and provide feedback.

Answer to Q4:

RAN2 confirms RAN3’s understanding on CGI of the PCell reporting. RAN2 sees no problem of indicating whether the PSCell change was MN-initiated or SN-initiated.

2. Actions:

To RAN WG3

ACTION: 	RAN2 respectfully asks RAN3 to take the above into account.

3. Date of Next TSG-RAN WG2 Meeting:

RAN2#123bis	from 2023-10-09	to 2023-10-13		Xiamen, CN

RAN2#124	from 2023-11-13	to 2023-11-17		Chicago, US