3GPP TSG-RAN WG2 Meeting #123		   R2-2307293

Toulouse, France, 21st – 25th Aug. 2023

Source:	vivo

Title:	Remaining issues for NR-DC with selective activation cell of groups

Agenda item:	7.4.3

Document for:		Discussion & Decision

# 1. Introduction

In this contribution, we will further discuss some remaining issues which are related to FFS points in previous meetings and also discuss some open issues which may brought from running CR discussion:

- How should the subsequent CPAC configuration be like if we allow candidate SN to generates execution conditions;

- How to configure different triggering condition when CPA conditional configuration can be used for CPC;

- When should the subsequent CPAC configuration be release besides explicitly by RRC reconfiguration procedure;

- Which node to generate the reference configuration in preparation phase in MN and SN initiated subsequent CPAC.

# 2. Discussion

## 2.1 Candidate SN generating execution conditions

For SN-initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPC.

In RAN2 #122 meeting [1], we agreed that candidate SN can generate the execution conditions for subsequent CPC, but it is still not clear what the configuration looks like especially considering that different candidate SN can configure different execution conditions to a same PSCell.

For example, one UE performs PSCell changes three times via subsequent CPC. The serving PSCell becomes PSCell1, PSCell2, PSCell3, and at last back to PSCell2 in turn. In our understanding, the agreement means that the execution conditions in this case are generated by the node indicated as in the following table.

| PSCell change(Source/Target) | SN generates the execution conditions |
| --- | --- |
| PSCell1/PSCell2 | SN serving PSCell1(SN-A) |
| PSCell2/PSCell3 | SN serving PSCell2(SN-B) |
| PSCell3/PSCell2 | SN serving PSCell3(SN-C) |

The motivation is the candidate SN has better information of the NEXT CPAC procedure than the very first source SN. As a result, we understand it means that the next CPAC configuration should now be associated with the measurement configuration of UE’s current serving SN/PSCell. Specifically, it means the next CPAC configuration is only valid after the UE’s serving PSCell is determined.

Based on that, as an example, a subsequent CPAC configuration should contain the following information:

- condReconfigId

- condRRCReconfig

- source PSCell identifier

- execution conditions, where the measurement ID should refer to measconfig of the source PSCell

Where condRRCReconfig is the target PSCell configuration and the source PSCell identifier is provided by the candidate SN who generates execution conditions for the next CPC. By this way, when UE stays in PSCell1 under SN-A and when the UE stays in PSCell-3 under SN-C, it may use two different execution conditions (which are provided by SN-A and SN-C) within CPAC configurations towards a same PSCell2. We would like to further confirm if this is the right understanding to allow candidate SN to generate execution conditions for the next CPC. Therefore, we propose:

Proposal 1: RAN2 to confirm that for SN-initiated SCG selective activation, a candidate PSCell, as the source PSCell, generates execution conditions for next CPC to a certain target PSCell, i.e. the execution conditions are applied to a pair of source and target PSCell.

FFS if it shall be possible to do something like MN-initiated CPA/CPC where Candidate SN generate execution conditions for subsequent CPC

Another FFS[1] is about whether we should support something like MN-initiated CPA/CPC where candidate SN generates execution conditions for subsequent CPC. We understand this scenario will bring more complexity and also some more specification impacts. Considering the limited time to complete this WI, it is simple and straightforward to make just MN responsible for generating execution conditions for subsequent CPC. Candidate SN can anyway be relied on in SN-initiated subsequent CPAC procedure. Therefore, we propose:

Proposal 2: For MN-initiated subsequent CPAC procedure, the candidate SN is not involved in generating execution conditions for subsequent CPC.

## 2.2 Differentiation for CPA and CPC Configuration

Another agreement in #121 meeting:

R2 assumes that a CPA conditional configuration can be used for CPC (but with different triggering conditions)

And also there are some related editor notes in running CR discussion[2]:

Editor’s Note: FFS on whether A3/A5 event are supported for MN-initiated case. If not, whether different triggering conditions (i.e. two A4 events) are needed for a candidate for initial and subsequent CPC.

Editor’s Note: FFS on how to differentiate the execution conditions for CPA and CPC if two trigger conditions of a candidate are provided to UE.

In our understanding, to support configuration for both CPA and CPC and with different triggering conditions, it seems two execution condition IE would be needed. E.g.:

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...,

[[

condExecutionCondSCG-r17         OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL     -- Need M

]],

[[

condExecutionCondCPA-r18            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

condExecutionCondCPC-r18            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

]]

}

When there are condExecutionCondCPA and condExecutionCondCPC IEs, the UE can easily know which is for CPA and which is for CPC, while if a single condExecutionCond IE is used (e.g. condExecutionCond2-r18), it would bring more complexity to define new rules to differentiate the execution conditions for CPA and CPC if there can be two trigger conditions.

Therefore, we propose that two condExecutionCond IEs are used which would be simple and clear.

Proposal 3: Two new IEs, i.e. condExecutionCondCPA and condExecutionCondCPC are intruduced separately for CPA and CPC in a single conditional reconfiguration for subsequent CPAC.

As for another editor note:

Editor’s Note: FFS on whether CPA configuration can be used for CPC by default. If not, whether to introduce an additional indication to indicate that the CPA candidate configuration can be used for subsequent CPC or not.

Our understanding is that if Proposal 3 is confirmed, then it is clear that when both condExecutionCondCPA and condExecutionCondCPC are configured, a conditional reconfiguration can be used for both CPA and CPC by default. Otherwise, it may only be used for CPA or CPC if only one condExecutionCond is configured.

Proposal 4: For a conditional reconfiguration with both condExecutionCondCPA and condExecutionCondCPC configured, the condRRCReconfig is used for both CPA and CPC.

## 2.3 Release for subsequent CPAC configuration

The agreements in RAN2 #121 and RAN2 #121bis-e are as follows:

- RAN2 #121 agreement In SCG selective activation, the CPC/CPA configurations of the UE should be released after Pcell change, at least for inter MN (by explicit indication from network, FFS other case). - RAN2 #121bis-e agreement - UE will keep R18 CPC configurations after CPC execution. It should be possible to release a CPC candidate explicitly by RRC reconfiguration procedure.

And also there is a related editor note in running CR discussion[2]:

Editor’s Note: FFS on other cases to release Subsequent CPAC config, e.g. upon SCG release, upon going to RRC_IDLE, upon reception of RRC release, upon RRC reestablishment, upon intra-MN pcell change. FFS on whether to rely on explicit indication to release the Subsequent CPAC config.

We think the general principle could be that, when the explicit RRC reconfiguration procedure cannot be used to release a CPC candidate, the UE can choose to release R18 CPC configurations autonomously. So, for the possible cases, our preferences are as follows:

| Scenario | Whether to release subsequent CPAC config autonomously by UE |
| --- | --- |
| Upon SCG release | No, can rely on RRC reconfiguration from MN |
| Going to IDLE | YesThe network doesn’t keep idle UE’s context, a UE can not perform CPAC after network releases its context. Hence, UE should release the CPAC config upon going to RRC_IDLE. |
| Upon reception of RRC release | Yes for releasing to idle The network doesn’t keep idle UE’s context, a UE can not perform CPAC after network releases its context. Hence, UE should release the CPAC config upon going to RRC_IDLE. Yes for releasing to inactive In theory, an UE can keep the CPAC configuration upon going to inactive state. The kept CPAC configuration can be used after RRC connection resume. But we think it is just an optimization to reduce signalling overhead, it is not essential for Rel-18 and can be considered in the following release. |
| Upon RRC reestablishment | Yes When UE performs RRC reestablishment, it is likely that the UE would reestablish to a new cell which may even do not have UE context, and the reestablishment may fail and UE goes to idle. When the reestablishment is successful, there may be possibility that the stored subsequent CPAC configuration can still be used, but we think it is an optimization with limited benefit. Hence, we prefer the UE to just release the subsequent CPAC configuration upon RRC reestablishment for simplicity. |
| Upon intra-MN PCell change | Yes, if no indication for keeping CPAC configuration is included in the RRC handover command In our understanding, when a handover command is received, the UE has no idea whether it is an intra-MN PCell change or inter-MN PCell change. Hence, NW needs to indicate UE whether to keep the CPAC configuration. If the handover is an inter-MN PCell change and the target cell is a Rel-17 cell, the handover command obviously will not include the indication. Hence, UE should release subsequent CPAC config upon PCell change (no matter inter-/intra-MN) when no indication for keeping CPAC configuration is included in the handover command. |

Proposal 5: Subsequent CPAC configuration is released by UE in the following cases:

- Going to IDLE

- Upon reception of RRC release

- Upon RRC reestablishment

- Upon intra-MN PCell change if no indication for keeping CPAC configuration

As for the explicit indication, whether we should support it could also be discussed. With this indication, even if a CPC candidate is not released explicitly by RRC reconfiguration procedure, it can be released autonomously after CHO/CPAC execution, which can further reduce the signalling between network and the UE in subsequent CPAC. However, there is another discussion about an indication to differentiate SCPAC and R16/17 CPA/CPC candidates:

Editor’s Note: FFS on whether to introduce an indication to differentiate SCPAC and R16/17 CPA/CPC candidates. FFS on the granularity of the indication, i.e., per candidate or per conditional reconfiguration.

The relationship between these two kinds of indication is not clear, because:

- On one hand, we are introducing an indication to indicate a R18 CPC configuration should be released after CPC by avoiding always relying on explicit RRC reconfiguration procedure to release R18 CPC configuration;

- On the other hand, we are also introducing an indication to indicate a R18 CPC configuration should NOT be released after CPC because it is R18 and is different with R16/17 CPA/CPC candidates.

Based on that, the explicit indications could be further discussed. In our understanding, as we already agreed to use explicit RRC reconfiguration procedure to release the subsequent CPAC configuration as well as some other possible solutions (as discussed in above table), it seems not much motivation to have an extra explicit indication to release the subsequent CPAC configuration. All we need is an indication to differentiate SCPAC and R16/17 CPA/CPC candidates if we reuse R16 IE for SCPAC, and that is all.

Therefore, we propose

Proposal 6: Explicit indication to release the Subsequent CPAC config is not needed.

Proposal 7: Explicit indication to differentiate SCPAC and R16/17 CPA/CPC candidates can be introduced.

## 2.4 Reference Configuration

The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures.

For the reference configuration topic[1], we agreed to aim at following similar design as LTM. But there is one left issue can be discussed as in above FFS.

We understand that for MN-initiated and SN-initiated inter-SN procedure, we can just rely on MN to initially generates the reference configuration and send it to all candidates. There was actually an agreement related to this when discussing delta configuration:

RAN2 #121 agreement

- For inter-SN CPC, MN should provide the reference configuration to all candidate T-SNs (in order to generate the T-SN candidate configuration).

The only unsolved question seems to be whether a UE may receive more than one reference configuration from more than one node (e.g. by SRB3 from SN). In our understanding, no matter whether this is possible, we can anyway leave it to coordination between MN and SN to ensure there would not be multiple reference configurations and cause ambiguity at the UE side.

Therefore, we propose:

Proposal 8: RAN2 to confirm the reference configuration is initially generated by MN and is provided to all candidates involved in the preparation phase, and it is left to network implementation to make sure there is only one latest reference configuration for all candidate cells.

# 3. Conclusion

In this contribution, we discussed about the remaining issues for NR-DC with selective activation cell of groups, mainly based on the RAN2 #122 agreements and the editor’s notes in running CR discussion..

We have the following observations and proposals:

For Candidate SN generating execution conditions

Proposal 1: RAN2 to confirm that for SN-initiated SCG selective activation, a candidate PSCell, as the source PSCell, generates execution conditions for next CPC to a certain target PSCell, i.e. the execution conditions are applied to a pair of source and target PSCell.

Proposal 2: For MN-initiated subsequent CPAC procedure, the candidate SN is not involved in generating execution conditions for subsequent CPC.

Differentiation for CPA and CPC Configuration

Proposal 3: Two new IEs, i.e. condExecutionCondCPA and condExecutionCondCPC are intruduced separately for CPA and CPC in a single conditional reconfiguration for subsequent CPAC.

Proposal 4: For a conditional reconfiguration with both condExecutionCondCPA and condExecutionCondCPC configured, the condRRCReconfig is used for both CPA and CPC.

Release for subsequent CPAC configuration

Proposal 5: Subsequent CPAC configuration is released by UE in the following cases:

- Going to IDLE

- Upon reception of RRC release

- Upon RRC reestablishment

- Upon intra-MN PCell change if no indication for keeping CPAC configuration

Proposal 6: Explicit indication to release the Subsequent CPAC config is not needed.

Proposal 7: Explicit indication to differentiate SCPAC and R16/17 CPA/CPC candidates can be introduced.

For Reference configuration

Proposal 8: RAN2 to confirm the reference configuration is initially generated by MN and is provided to all candidates involved in the preparation phase, and it is left to network implementation to make sure there is only one latest reference configuration for all candidate cells.

# 4. References

RAN2 #122 chairman notes;

R2-230xxxx, 38.331 Running CR for selective activation of SCGs for NR-DC