3GPP TSG-RAN WG2 Meeting #123	R2-2307296

Toulouse, France, 21st - 25th Aug. 2023

Source:	vivo

Title:	Analysis on SFN wrap around issue

Agenda Item:	7.5.3

Document for:	Discussion and Decision

- Introduction

In the last meeting, the following agreement was made for SFN wrap-around issue:

Not use broadcast signalling for counter and reference SFN in XR.

In this contribution, we discuss the remaining issues for SFN wrap-around issue, including whether need to define a maximum value for the counter and how to use the reference SFN.

- Discussion

In RAN2#121bis, it was agreed to introduce a counter in the DRX formula and a DRX reference SFN signaled by the network.

To address SFN wrap around, it is proposed to adopt option with a counter in DRX formula that increments at every SFN wrap around and an DRX reference SFN signalled by network. FFS if this is based on H-SFN, E-SFN or a generic counter.

Issue#1: Maximum value of the counter

The counter (e.g., E-SFN) in the DRX formula is used to solve the SFN wrap-around issue, and the new DRX formula can be presented as follows if the non-integer DRX cycle is supported:

- For long DRX cycle:

floor([(SFN + 1024* E-SFN) × 10+ subframe number] mod (drx-LongCycle)) = drx-StartOffset

- For short DRX cycle:

floor([(SFN + 1024* E-SFN) × 10+ subframe number] mod (drx-ShortCycle)) = floor[(drx-StartOffset) mod (drx-ShortCycle)]

Regarding the new counter, one remaining issue is whether the counter has a maximum value and whether this value needs to be provided by the network. In our view, the maximum value of the counter does not need to be provided by the network. The UE will initialize the counter to zero when receiving the RRC message for DRX configuration, and it will increment at every SFN wrap-around. It’s up to UE implementation to ensure that the storage for the counter is large enough and that no overflow issues occur. For example, if the counter occupies 16 bits, it will overflow after 65536 SFN wrap-arounds, which means overflow will happen if the network does not provide a new DRX configuration for more than 186 hours and this is unlikely to happen.

Proposal 1: UE will initialize the counter to zero when receiving the RRC message for DRX configuration. No maximum value is provided by the network and it’s up to NW/UE implementation to make sure that no overflow issues will happen.

Issue#2: How to use the reference SFN

If the RRC message for DRX reconfiguration is sent just before the SFN wrap-around and is received by the UE after the SFN wrap-around due to HARQ/ARQ retransmissions, there may be ambiguity between UE and the network about the effective time of the DRX. The reference SFN is signalled by the network with a value of 0 or 512 to indicate the time (before or after SFN wrap-around) when the DRX configuration is sent to UE.

Regarding how to use the reference SFN, there are mainly two options:

- Option 1: Add the DRX reference SFN in the DRX formula. The initial value of E-SFN = 0

- Option 2: DRX reference SFN is not in the DRX formula, and the initial value of E-SFN is set according to reference SFN:

  - If current SFN < 512 and reference SFN = 512, UE initializes the counter to 1

  - Otherwise, UE initializes the counter to 0

In our view, both options can achieve the same effect to avoid RRC configuration ambiguity. Since the motivation and mechanism of reference SFN in DRX are the same as Rel-16 IIoT type1 CG, we prefer to use option 1, adding the reference SFN in the DRX formula, just as in Rel-16 IIoT type1 CG. Compared to option 2, option 1 also has less specification impact in TS 38.321.

Proposal 2: Reference SFN is added in the DRX formula as follows:

  - For long DRX cycle: floor[(SFN + 1024* E-SFN) × 10+ subframe number] mod (drx-LongCycle) =floor[(drx-ReferenceSFN*10 + drx-StartOffset) mod (drx-LongCycle)]

  - For short DRX cycle: floor[(SFN + 1024* E-SFN) × 10+ subframe number] mod (drx-ShortCycle) =floor[(drx-ReferenceSFN*10 + drx-StartOffset) mod (drx-ShortCycle)]

- Conclusion

In this contribution, we discuss the solutions to address SFN wrap-around issue. Based on the discussion, we have the following proposals:

Proposal 1: UE will initialize the counter to zero when receiving the RRC message for DRX configuration. No maximum value is provided by the network and it’s up to NW/UE implementation to make sure that no overflow issues will happen.

Proposal 2: Reference SFN is added in the DRX formula as follows:

  - For long DRX cycle: floor[(SFN + 1024* E-SFN) × 10+ subframe number] mod (drx-LongCycle) =floor[(drx-ReferenceSFN*10 + drx-StartOffset) mod (drx-LongCycle)]

  - For short DRX cycle: floor[(SFN + 1024* E-SFN) × 10+ subframe number] mod (drx-ShortCycle) =floor[(drx-ReferenceSFN*10 + drx-StartOffset) mod (drx-ShortCycle)]

- References