3GPP TSG-RAN WG2 Meeting #123	R2-2307314

Toulouse, France, August 21-25, 2023

Agenda item:	7.7.4.1

Source:	Samsung

Title:	Discussion on Cell Reselection Enhancements

WID/SID:	NR_NTN_enh-Core - Release 18

Document for:	Discussion and Decision

# Introduction

In the past RAN2 meetings, enhancements on cell reselection enhancement have been discussed. In this document, we provide our views on the remaining issues for TN coverage information and earth-moving cell.

# Discussion

## TN coverage area information

In RAN2#122 meeting, the following agreements have been reached for TN coverage area information.

Agreements:

- An RRC_IDLE/RRC_INACTIVE UE is not required to perform neighbour cell measurements for cell reselection for a TN frequency in the area, if configured, where there is no coverage of that frequency, regardless of the frequency priority

- Reuse the same format of Rel-17 referenceLocation and distanceThresh for signaling the TN coverage area centre and radius

- TN coverage info is NOT included in SIB19. FFS if we use an existing SIB or a new one

- We don’t introduce RRC dedicated signalling to provide more accurate TN coverage information

Agreements:

- We no longer consider option 3 alone for signaling the frequency information for TN coverage area (in case option 3 should be combined with option 1). Come back in the next meeting to decide between option 2 (plus possible fixes if needed) and option 1+3.

Regarding the signaling of TN coverage information, Option 1 combined with Option 3 is that a new SIB includes TN coverage area list and each TN coverage area contains a frequency list or a bitmap of the frequencies in SIB4/5. By this signalling approach, the existing frequencies in SIB4/5 are indicated by a bitmap and new frequencies associated to a TN area are contained in the new SIB. If frequencies in SIB4/5 needs to be updated, the new SIB has to be updated together.

Option 2 is that TN coverage area ID list is configured in SIB4/5 under the frequency list, and a new SIB provides TN coverage area information. By option 2, all frequencies for neighbour cell measurement are provided in SIB4/5 and existing signalling structure in SIB4/5 can be utilized. If TN coverage area is configured for a frequency, UE does not need to measure that TN frequency if it is not within the corresponding TN coverage area. Option 2 is preferred.

Proposal 1: TN coverage area ID(s) is configured in SIB4/5 under the frequency list and a new SIB provides corresponding TN coverage area information. As already agreed, if TN coverage area is configured for a frequency, UE is not required to measure the frequency if it is not within the corresponding TN coverage area, regardless of the frequency priority.

## Earth-moving cell information

RAN2 has agreed that UE can estimate the real-time reference location based on the ephemeris and reference location at the epoch time with rough accuracy. The open issue is whether additional information is needed to allow more accurate estimation.

Agreements RAN2#121bis:

- RAN2 understands that for earth-moving cell reselection, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time (FFS whether a new epochTime IE is needed). RAN2 understanding is that both PVT and orbital parameters can be used for this. FFS if additional information is needed to allow more accurate measurements.

To derive the real-time reference location based on ephemeris and reference location at the epoch time, there are two underlying assumptions.

- The first assumption is that UE derives the trajectory of sub-satellite point based on the ephemeris,

- The second assumption is that the non-steerable beam makes the serving cell reference location relatively static with respect to the sub-satellite point. Once the UE determines the distance between the sub-satellite point and the reference location at the epoch time, UE assumes the distance and relative positions between the points are fixed. Then, in any future time, when the sub-satellite point is derived based on the ephemeris, UE can use the fixed distance and relative position to determine where the reference location is at that time.

For the first assumption, the concern is that from the UE perspective, additional computational effort is required to derive the real-time location of sub-satellite point based on the ephemeris, which is not required for any feature in Release-17 NTN. For the second assumption, the concern is that how accurately the reference location can be derived when assuming the reference location is relatively static with respect to the sub-satellite point if in reality the relative movement between the reference location and sub-satellite point exists. If the additional computation is affordable and rough estimation of serving cell reference location is sufficient for cell reselection, additional information is not needed.

In Figure 1, we illustrate an example to quantify how the earth-moving cell reference location moves with respect to the sub-satellite point. Consider a LEO satellite in a circular orbit with altitude 600 km [1] radiating a non-steering beam. At an epoch time, the serving cell reference location is the beam center which is at point A on the ground, and the sub-satellite point is at B. Assume for non-steerable beam, the angle beta is fixed. Then, the distance between the reference location and the sub-satellite point can be derived given the earth radius R, the satellite altitude h, and the fixed beam angle beta. Even for circular orbits, the altitude above ground can vary by as much as 30 km (19 mi) (especially for polar orbits) [2]. The varying altitude here can also mimic the scenario of elliptical orbits.

Figure 1. Illustration of the distance between reference location and sub-satellite point (AB)

In Figure 2, we show how the distance between the reference location and the sub-satellite point is varying with the altitude for different beam angle. Consider the epoch time is the time when the altitude equals 585 km, and the reference location coordinates is provided by the network, which is point A. As the satellite moves on the orbit, the altitude changes which leads to the change of distance AB. As shown in the right plot, the maximum change of distance AB can vary from 0 (for beta=0 deg) to 80 km (for beta=60 deg) compared to the initial AB at the epoch time. This means when the beam angle is large and the altitude change is large, the distance between the reference location and the sub-satellite point varies significantly compared to the initial distance at the epoch time. Compared with the typical beam footprint size 100-1000 km of LEO satellite, the deviation of 80 km is relatively large for small beam footprint size. Hence, for small beam angle (equivalently, large elevation angle) and small change of altitude, the assumption 2 above can provide a rough estimation of the reference location with a relatively small deviation to the true reference location; the larger the beam angle and the larger the altitude changes, the assumption 2 leads to more deviation in estimating the reference location.

Figure 2. Distance change between reference location and sub-satellite point (compared to the initial distance AB at the epoch time where altitude = 585 km)

Observation 1: The real-time reference location of earth-moving cell derived based on the ephemeris and the reference location at the epoch time can provide rough accuracy for small beam angle (equivalently, larger elevation angle) and small altitude change, but becomes inaccurate (i.e., with tens of km deviation to the true value) in case of large beam angle (equivalently, small elevation angle) and large altitude change.

As shown above, due to the change of satellite altitude, the distance between the reference location and the sub-satellite varies. Thus, a fixed distance between the two point does not provide any additional information for estimating the real-time reference location. Instead, the straightforward way is to provide the velocity of the reference point, so that the UE can derive the real-time coordinates easily. This is beneficial especially for estimating earth-moving cell reference location for CHO in connected mode.

In the example of NTN implementation in TS 38.300 (B.4), for earth moving beams, the information about the beam direction and motion of the beam's footprint are expected to be available at the gNB.

At least the following NTN related parameters are expected to be provided by O&M to the gNB for its operation: a) Earth-fixed beams: for each beam provided by a given NTN-payload: - The Cell identifier (NG and Uu) mapped to the beam; - The Cell's reference location (e.g. cell's center and range). b) Quasi-Earth-fixed: for each beam provided by a given NTN payload: - The Cell identifier (NG and Uu) and time window mapped to a beam; - The Cell's/beam's reference location (e.g. cell's center and range); - The time window of the successive switch overs (feeder link, service link); - The identifier and time window of all serving satellites (resp. HAPS) and NTN Gateways. c) Earth moving beams: for each beam provided by a given NTN payload: - The Uu Cell identifier mapped to a beam and mapping information to fixed geographical areas reported on NG, including information about the beams direction and motion of the beam's foot print on Earth; - Its elevation wrt NTN payload; - Schedule of successive serving NTN Gateways/gNBs; - Schedule of successive switch overs (feeder link, service link).

As the motion information of earth moving beam is available at the gNB, the gNB can easily provide the velocity of reference location for an earth moving cell/beam, and does not need to provide other information. The velocity information can also implicitly indicate the cell is earth-moving.

Proposal 2: For an earth-moving cell, NW provides the reference location velocity for UE to estimate the real-time reference location.

## Neighbour cell measurement initiation

Due to the fixed TN cell constellation and predictable NTN cell constellation based on satellite information, the NW has knowledge of the incoming cell(s) that will replace the current serving cell to serve the area. To save UE power consumption on neighbor cell search and measurement in cell reselection, the NW can provide the incoming cell information associated with the current serving cell service stop time, i.e., t-Service. In other words, the NW can provide the incoming cell ID and frequency band associated with the t-Service, and the UE can prioritize the incoming cell in measurement and/or ranking.

Observation 2: NW has knowledge of the incoming cell(s) that will replace the current serving cell approaching t-Service and the incoming cell information can bring UE power saving in measurement.

Proposal 3: RAN2 to discuss providing the incoming cell information associated with the t-Service.

In case that the serving cell or neighbor cells are earth-moving cells, due to the relative movement of cells, the information of when a neighbor cell is close to the UE or neighboring to the current serving cell is helpful so that the UE only need to measure the neighbor cell when it is nearby. When the serving cell or a neighbor cell in SIB3/SIB4 is an earth-moving cell, one option is to directly provide the timing information, e.g., neighboring time for the neighbor cell; an alternative is to provide the neighbor cell reference location moving information as for the serving cell, so that UE can estimate autonomously the trajectory of the neighbor cell.

Proposal 4: In the case the serving/neighbor cell is an earth-moving cell, for neighbor cell measurement the network provides:

Option 1: the neighboring time information;

Option 2: the neighbor cell reference location moving information.

# Conclusion

Based on the discussion, we have the following observations and proposals.

Proposal 1: A new SIB provides TN coverage area information and TN coverage area ID(s) is configured in SIB4/5 under the frequency list. As already agreed, if TN coverage area is configured for a frequency, UE is not required to measure the frequency if it is not within the corresponding TN coverage area, regardless of the frequency priority.

Observation 1: The real-time reference location of earth-moving cell derived based on the ephemeris and the reference location at the epoch time can provide rough accuracy for small beam angle (equivalently, larger elevation angle) and small altitude change, but becomes inaccurate (i.e., with tens of km deviation to the true value) in case of large beam angle (equivalently, small elevation angle) and large altitude change.

Proposal 2: For an earth-moving cell, NW provides the reference location velocity for UE to estimate the real-time reference location.

Observation 2: NW has knowledge of the incoming cell(s) that will replace the current serving cell approaching t-Service and the incoming cell information can bring UE power saving in measurement.

Proposal 3: RAN2 to discuss providing the incoming cell information associated with the t-Service.

Proposal 4: In the case the serving/neighbor cell is an earth-moving cell, for neighbor cell measurement the network provides:

Option 1: the neighboring time information;

Option 2: the neighbor cell reference location moving information.

# Reference

[1] 3GPP TR 38.821 V16.0.0 Solutions for NR to support non-terrestrial networks (NTN)

[2] https://en.wikipedia.org/wiki/Low_Earth_orbit