3GPP TSG-RAN WG2 Meeting #123	R2-2307315

Toulouse, France, August 21-25, 2023

Agenda item:	7.7.4.2

Source:	Samsung

Title:	Discussion on Handover Enhancements

WID/SID:	NR_NTN_enh -Core

Document for:	Discussion and Decision

# Introduction

In RAN2 meetings, Rel-18 NR NTN handover enhancements were discussed including aspects on CHO enhancements, RACH-less HO, and satellite switch with PCI unchanged. In this document, we continue to discuss the HO enhancements on these aspects.

# Discussion

## RACH-less Handover

NTN RACH-less HO has been discussed in the last two meetings. The relevant agreements are listed in the appendix. Here we continue to discuss remaining open issues.

### RAN1 LS reply

RAN1 has replied the LS on RACH-less HO in [2]. Their responses are discussed here.

Q1. Regarding the pre-allocated grant for initial UL transmission, considering the similarity to Msg1 in RACH and the similarity to the initial UL transmission in CG-SDT, where PRACH/PUSCH resource is mapped to SSBs, whether the pre-allocated grant is provided with association to SSB(s)? If yes, whether a RSRP threshold is needed for SSB selection for initial UL transmission?

RAN1 response: One company thinks that when the network knows the suitable DL beam for RACH-less handover, the pre-allocated grant can be associated with a SSB index of the target cell, and when the network does not know the suitable DL beam, RACH-based HO can be used instead of introducing beam-sweeped pre-allocated grants associated with multiple SSB indexes. Other companies think that the association between the pre-allocated grant for initial transmission and SSB index should be supported without any condition(s), and think that RSRP threshold may be helpful.

Based on the above RAN1 response, the association between the pre-allocated grant and the SSB(s) should be configured.

Proposal 1: The association between pre-allocated grant and SSB(s) is configured for initial UL transmission in RACH-less HO.

Companies have different views on whether to associate with one SSB index or multiple SSB indexes. We think a pre-allocated grant with association to multiple SSB(s) is more robust than that with association to a single SSB for the first UL transmission. Currently, for a type-1 configured grant, the UL grant occasion sequentially occurs in time as specified in TS 38.321 section 5.8.2.

After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider sequentially that the Nth (N >= 0) uplink grant occurs in the symbol for which: [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] = (timeReferenceSFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

For CG-SDT, the association between the CG and SSBs are defined by sdt-SSB-Subset and sdt-SSB-PerCG-PUSCH-r17 as specified in TS 38.331, where each CG occasion is mapped to one or multiple SSB indexes.

CG-SDT-Configuration-r17 ::= SEQUENCE {

cg-SDT-RetransmissionTimer   INTEGER (1..64)                                                 OPTIONAL,   -- Need R

sdt-SSB-Subset-r17       CHOICE {

shortBitmap-r17          BIT STRING (SIZE (4)),

mediumBitmap-r17         BIT STRING (SIZE (8)),

longBitmap-r17           BIT STRING (SIZE (64))

}                                                                                            OPTIONAL,   -- Need S

sdt-SSB-PerCG-PUSCH-r17   ENUMERATED {oneEighth, oneFourth, half, one, two, four, eight, sixteen}  OPTIONAL,   -- Need M

sdt-P0-PUSCH-r17         INTEGER (-16..15)                                                   OPTIONAL, -- Need M

sdt-Alpha-r17            ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} OPTIONAL, -- Need M

sdt-DMRS-Ports-r17       CHOICE {

dmrsType1-r17            BIT STRING (SIZE (8)),

dmrsType2-r17            BIT STRING (SIZE (12))

}                                                                                            OPTIONAL,  -- Need M

sdt-NrofDMRS-Sequences-r17  INTEGER (1..2)                                                   OPTIONAL   -- Need M

}

We think for the pre-allocated grant, which is agreed to be provided as type-1 CG, the association to SSBs can be configured same as the CG-SDT that CG occasions are mapped to SSB indexes in a pre-allocated grant, configured by SSB-Subset and SSB-PerCG-PUSCH.

Proposal 2: In a pre-allocated grant, CG occasions are mapped to SSB indexes, configured by SSB-Subset and SSB-PerCG-PUSCH.

RAN1 has also confirmed that RSRP threshold can be helpful for SSB selection. If a RSRP threshold is configured, UE selects a SSB associated to a pre-allocated grant with RSRP above the threshold, and use that pre-allocated grant and the selected SSB for initial UL transmission.

Proposal 3: A RSRP threshold is configured for SSB selection for initial UL transmission with pre-allocated grant. UE selects an SSB associated to a pre-allocated grant with RSRP above the threshold, and use that pre-allocated grant and the selected SSB for initial UL transmission.

Q2. To monitor target cell PDCCH for dynamic grant for initial UL transmission, whether beam selection is needed (e.g., performed by NW with selected beam(s) indicated, or performed by UE)?

RAN1 response: If single beam is indicated, UE will monitor the target cell PDCCH scheduling the first PUSCH based on the indicated beam. RAN1 will further discuss the case where multiple beams are indicated.

Based on RAN1’s response, beam indication is provided to monitor target cell PDCCH for dynamic grant for initial UL transmission. We think SSB index(es) should be provided as the beam indication.

Proposal 4: Beam indication (i.e., SSB index(es)) is provided to monitor target cell PDCCH for dynamic grant for initial UL transmission. Wait for RAN1 further response for the case where multiple beams are indicated.

3. Regarding the power control for initial UL transmission, whether it follows the rules specified for PUSCH scheduled by Random Access grant or by configured grant or others?

RAN1 response:  RAN1 will further discuss the issue.

Proposal 5: Wait for RAN1 further response on power control for initial UL transmission.

### UL synchronization

As specified in TS 38.211 and TS 38.213, UE applies TA, i.e., TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETc, for the UL transmission. In RAN2#122, the following agreement has been made for N_TA.

Agreement RAN2#122:

- In NTN RACH-less handover, NW either indicates NTA in the target cell is identical to the source cell, or the NTA explicitly provided by the NW is 0. RAN2 will not discuss the case where NTA does not equal to 0

- Remove “FFS how to perform RACH-less UL synchronization to NTN target cell”, RAN2 assumes the UL sync handling in the target cell is the same in RACH-based HO and RACH-less HO, except how to acquire NTA (FFS on the spec impact , if any)

Based on the above RAN2 agreements, for the first UL transmission in RACH-less HO,

- NTA,offset is configured by parameter n-TimingAdvanceOffset or a default value is used if not configured, same as RACH-based HO;

- NTA,adjcommon is configured by common TA parameters of the target cell, same as RACH-based HO;

- NTA,adjUE is computed based on UE location and ephemeris of the target cell, same as RACH-based HO;

- NTA is indicated by the NW in RACH-less HO configuration for reconfiguration with sync.

Different from RACH-based HO where NTA is given in RAR, for RACH-less HO, NTA is provided in the RACH-less HO configuration included in the RRCReconfiguration message for reconfiguration with sync. The impact for the MAC entity is that the NTA in the RACH-less HO configuration shall be applied for UL time alignment. This is same as LTE RACH-less HO, as shown below in TS36.321 clause 5.2.

Proposal 6: If configured with RACH-less configuration, the MAC entity applies the configured N_TA for the TAG of the serving cell, and starts the timerAlignmentTimer associated with this TAG.

### UL carrier and BWP selection

In RACH-based HO, if supplementary UL carrier is configured, UE first selects UL carrier, as specified in TS 38.321 5.1.1 for Random Access procedure.

1> if the carrier to use for the Random Access procedure is explicitly signalled: 2> select the signalled carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the signalled carrier. 1> else if the carrier to use for the Random Access procedure is not explicitly signalled; and 1> if the Serving Cell for the Random Access procedure is configured with supplementary uplink as specified in TS 38.331 [5]; and 1> if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL: 2> select the SUL carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the SUL carrier. 1> else: 2> select the NUL carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the NUL carrier.

Similarly, for the initial UL transmission in RACH-less HO, if supplementary UL carrier is configured, UE has to first select UL carrier. For pre-allocated grant, if the pre-allocated grant is configured for the normal UL (NUL) carrier, UE selects NUL carrier to send the initial UL transmission; if the pre-allocated grant is configured for supplementary UL (SUL) carrier, UE selects SUL carrier to send the initial UL transmission. For dynamic grant provided in PDCCH, NW indicates the NUL or SUL carrier (e.g., in DCI) and UE selects the indicated carrier to send the initial UL transmission. If UL carrier is not explicitly signaled either for pre-allocated grant or for DG, and if SUL carrier is configured, UE selects UL carrier based on a RSRP threshold of the downlink pathloss reference as in Random Access procedure.

Proposal 7: The pre-allocated grant is configured for NUL or SUL, and UE select the corresponding UL carrier for the initial UL transmission.

Proposal 8: For dynamic grant provided in PDCCH, NW indicates NUL or SUL carrier for initial UL transmission.

Proposal 9: If SUL is configured and if the UL carrier for initial UL transmission is not explicitly signaled, UE selects NUL or SUL carrier based on a RSRP threshold of the downlink pathloss reference.

For the initial UL transmission in RACH-less HO, UE needs to identify the UL BWP to be used. For RACH-based HO, the UE performs CFRA on the BWP indicated by firstActiveUplinkBWP. Similarly, the BWP indicated by firstActiveUplinkBWP should be selected for the initial UL transmission in RACH-less HO. If the firstActiveUplinkBWP is not signaled, initialUplinkBWP is selected.

Proposal 10: The BWP indicated by firstActiveUplinkBWP is selected for the initial UL transmission. If the firstActiveUplinkBWP is not signaled, initialUplinkBWP is selected.

### TA report MAC CE

RAN2 has agreed that the initial UL transmission includes the RRCReconfigurationComplete message. The question is what else should be included in the initial UL transmission, whether TA report MAC CE should be included in the initial UL transmission in RACH-less HO. For NTN RACH-based HO, if ta-Report enabled is included in ServingCellConfigCommon for reconfiguration with sync, TA report is triggered and the TAR MAC CE is included in Msg3/MsgA (or Msg5 if no enough UL grant for Msg3/MsgA). Similar principle can be followed for RACH-less HO.

Proposal 11: If ta-Report enabled is included in ServingCellConfigCommon for RACH-less HO, a TA report shall be triggered.

### HARQ process for initial UL transmission

Based on the reception of the first UL transmission by pre-allocated grant, NW can schedule retransmission by PDCCH. However, it is possible that NW missed the first transmission and no feedback to UE. To achieve robust and reliable RACH-less HO, UE autonomous retransmission should be supported if pre-allocated grant is configured. UE can send retransmission using the sequential PUSCH occasions in the pre-allocated grant. Similar to CG-SDT, a retransmission timer can be defined and UE autonomously retransmits when the timer expires.

Proposal 12: For the pre-allocated grant, support autonomous retransmission of the initial UL transmission with a retransmission timer configured. UE retransmits by pre-allocated grant when the timer expires.

In Rel-17, HARQ mode A and B are introduced for UL. HARQ mode A is intended to support reception of UL retransmission grant based on UL decoding result. HARQ mode B is intended to support no UL retransmission and/or blind UL retransmission. For the initial UL transmission in RACH-less HO, by HARQ mode A, retransmissions can be scheduled by PDCCH, and HARQ mode B is suitable when the pre-allocated grant is used for autonomous retransmission. Thus, both HARQ mode A and HARQ mode B should be supported.

Proposal 13: Both HARQ mode A and HARQ mode B are supported for the HARQ process of RACH-less initial UL transmission.

For the HARQ process for the initial UL transmission, a specific HARQ process ID can be specified, regardless of using pre-allocated grant or DG. HARQ process ID 0 can be used, same as Msg3/MsgA transmission in Random Access procedure. This simplifies the NW scheduling and UE does not need to derive HARQ process ID based on parameters.

Proposal 14: HARQ process ID 0 is used for initial UL transmission.

Another issue to be considered is whether UE is allowed to transmit user data in the HARQ process of initial UL transmission before the successful completion of the RACH-less HO. We think before UE receiving NW confirmation of RACH-less HO completion, UE cannot confirm UL transmission can be reliably received by NW so it should not send any user data (i.e., not select the logical channel(s) corresponding to DRB(s) for the UL grant of initial UL transmission) before RACH-less HO completion.

Proposal 15: Before the successful completion of the RACH-less HO, the MAC entity shall not select the logical channel(s) corresponding to DRB(s) for the uplink grant for the HARQ process of initial UL transmission.

If DRX is configured, UE behavior for the HARQ process of initial UL transmission needs to be considered. The Active Time and UE behavior in Active time are specified as follows.

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: - drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or - drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or - ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or - a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a). ---omitted--- 1> if a DRX group is in Active Time: 2> monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6]; 2> if the PDCCH indicates a DL transmission; or 2> if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or 2> if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]: 3> if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled: 4> if the corresponding HARQ process is configured with HARQ feedback enabled: 5> set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value; 5> start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback. 3> else: 4> start or restart the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback. NOTE 3: When HARQ feedback is postponed by PDSCH-to-HARQ_feedback timing indicating an inapplicable k1 value, as specified in TS 38.213 [6], the corresponding transmission opportunity to send the DL HARQ feedback is indicated in a later PDCCH requesting the HARQ-ACK feedback. 3> stop the drx-RetransmissionTimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported; 3> stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process; 3> if the PDSCH-to-HARQ_feedback timing indicate an inapplicable k1 value as specified in TS 38.213 [6]: 4> start the drx-RetransmissionTimerDL in the first symbol after the (end of the last) PDSCH transmission (within a bundle) for the corresponding HARQ process. 2> if the PDCCH indicates a UL transmission: 3> if this Serving Cell is configured with uplinkHARQ-Mode: 4> if the corresponding HARQ process is configured as HARQModeA: 5> set HARQ-RTT-TimerUL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerUL plus the latest available UE-gNB RTT value; 5> if drx-LastTransmissionUL is configured: 6> start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission. 5> else: 6> start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission. 3> else: 4> if drx-LastTransmissionUL is configured: 5> start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission. 4> else: 5> start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission. 3> stop the drx-RetransmissionTimerUL for the corresponding HARQ process. ---omitted---

As it has been agreed that if pre-allocate grant is not configured, UE monitors PDCCH for DG for initial UL transmission. Then, if DRX is configured, the Active time for Serving Cells in a DRX group should include the time while UE is monitoring the PDCCH that provides dynamic grant for the initial UL transmission.

Proposal 16: If DRX is configured, the Active time for Serving Cells in a DRX group includes the time while UE is monitoring the PDCCH that provides dynamic grant for the initial UL transmission.

### Confirmation of RACH-less HO completion

RAN2 has agreed reuse LTE solution for RACH-less HO completion confirmation for both pre-allocated grant and dynamic grant. That is, for RACH-less HO, if a UE Contention Resolution Identity MAC CE is received on the PDSCH indicated by the PDCCH addressed to the C-RNTI, MAC entity ignores the contents of this field UE Contention Resolution Identity and indicates to upper layer the successful reception of a PDCCH addressed to C-RNTI, then consider the RACH-less HO is completed. As agreed in the last meeting, enhancements can be further studied.

Since there is no contention when sending the initial UL transmission including the RRCReconfigurationComplete message, the information in the UE Contention Resolution Identity MAC CE is redundant. Instead, if NW has received the RRCReconfigurationComplete message, and if DL or UL transmission needs to be scheduled for UE, NW can directly schedule DL or UL transmission, which can be considered as a confirmation. Thus, a simpler and more efficient way is that if a PDCCH addressed to C-RNTI scheduling DL/UL is received after the initial UL transmission, consider RACH-less HO completed.

Proposal 17: If a PDCCH addressed to C-RNTI scheduling DL/UL is received after the initial UL transmission, consider RACH-less HO completed.

## CHO for earth-moving cell

In the previous RAN2 meetings, the following agreement have been made regarding the mobility for earth-moving cell.

Agreements in RAN2#121:

- For location-based CHO for earth-moving cells we follow the solution being investigated for cell reselection to allow the UE to derive the serving cell’s reference locations as the cells move. FFS whether the same mechanism can also be used for the candidate cell’s reference location

Agreements in RAN2#121bis-e:

- RAN2 understands that for earth-moving cell reselection, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time (FFS whether a new epochTime IE is needed). RAN2 understanding is that both PVT and orbital parameters can be used for this. FFS if additional information is needed to allow more accurate measurements.

- For earth-moving cell, new IE is introduced to indicate the reference location of serving cell.

- For cell (re)selection in earth-moving system, a distance threshold is introduced for location-based measurement initiation, which reuses distanceThresh in SIB19.

If we consider the solution for cell reselection for earth-moving cells to be reused for location-based CHO for earth-moving cells, rough estimation of real-time reference location of an earth-moving cell can be done by UE. That is, for rough estimation of real-time cell movement, if the serving cell is an earth-moving cell, the serving cell’s reference location at the epoch time has to be provided; if a candidate cell is an earth-moving cell, the candidate cell’s reference location at the epoch time has to be provided.

However, as analyzed in our paper R2-2307314 [5], the real-time reference location of an earth-moving cell that is derived based on the ephemeris and the reference location coordinates at the epoch time becomes inaccurate (i.e., with tens of km deviation to the true value) in case of large beam angle (equivalently, small elevation angle) and large altitude change. For mobility in RRC_CONNECTED, different from cell reselection in RRC_IDLE, a more accurate estimation of real-time reference location is needed. Thus, reference location velocity information has to be provided for UE to derive the real-time reference location of the serving cell or a CHO candidate cell.

Proposal 18: For location-based CHO for earth-moving cells, more accurate estimation of real-time reference location is required.

Proposal 19: For location-based CHO for earth-moving cells, if the serving cell is moving, the serving cell’s reference location coordinates and velocity at the epoch time of the serving cell ephemeris are provided by the NW; if a candidate cell is moving, the candidate cell’s reference location coordinates and velocity at the epoch time of the candidate cell ephemeris are provided by the NW.

# Conclusion

Based on the discussion, we have the following proposals.

For RACH-less HO:

Proposal 1: The association between pre-allocated grant and SSB(s) is configured for initial UL transmission in RACH-less HO.

Proposal 2: In a pre-allocated grant, CG occasions are mapped to SSB indexes, configured by SSB-Subset and SSB-PerCG-PUSCH.

Proposal 3: A RSRP threshold is configured for SSB selection for initial UL transmission with pre-allocated grant. UE selects an SSB associated to a pre-allocated grant with RSRP above the threshold, and use that pre-allocated grant and the selected SSB for initial UL transmission.

Proposal 4: Beam indication (i.e., SSB index(es)) is provided to monitor target cell PDCCH for dynamic grant for initial UL transmission. Wait for RAN1 further response for the case where multiple beams are indicated.

Proposal 5: Wait for RAN1 further response on power control for initial UL transmission.

Proposal 6: If configured with RACH-less configuration, the MAC entity applies the configured N_TA for the TAG of the serving cell, and starts the timerAlignmentTimer associated with this TAG.

Proposal 7: The pre-allocated grant is configured for NUL or SUL, and UE selects the corresponding UL carrier for the initial UL transmission.

Proposal 8: For dynamic grant provided in PDCCH, NW indicates NUL or SUL carrier for initial UL transmission.

Proposal 9: If SUL is configured and if the UL carrier for initial UL transmission is not explicitly signaled, UE selects NUL or SUL carrier based on a RSRP threshold of the downlink pathloss reference.

Proposal 10: The BWP indicated by firstActiveUplinkBWP is selected for the initial UL transmission. If the firstActiveUplinkBWP is not signaled, initialUplinkBWP is selected.

Proposal 11: If ta-Report enabled is included in ServingCellConfigCommon for RACH-less HO, a TA report shall be triggered.

Proposal 12: For the pre-allocated grant, support autonomous retransmission of the initial UL transmission with a retransmission timer configured. UE retransmits by pre-allocated grant when the timer expires.

Proposal 13: Both HARQ mode A and HARQ mode B are supported for the HARQ process of RACH-less initial UL transmission.

Proposal 14: HARQ process ID 0 is used for initial UL transmission.

Proposal 15: Before the successful completion of the RACH-less HO, the MAC entity shall not select the logical channel(s) corresponding to DRB(s) for the uplink grant for the HARQ process of initial UL transmission.

Proposal 16: If DRX is configured, the Active time for Serving Cells in a DRX group includes the time while UE is monitoring the PDCCH that provides dynamic grant for the initial UL transmission.

Proposal 17: If a PDCCH addressed to C-RNTI scheduling DL/UL is received after the initial UL transmission, consider RACH-less HO completed.

For CHO enhancement:

Proposal 18: For location-based CHO for earth-moving cells, more accurate estimation of real-time reference location is required.

Proposal 19: For location-based CHO for earth-moving cells, if the serving cell is moving, the serving cell’s reference location coordinates and velocity at the epoch time of the serving cell ephemeris are provided by the NW; if a candidate cell is moving, the candidate cell’s reference location coordinates and velocity at the epoch time of the candidate cell ephemeris are provided by the NW.

# Reference

[1] RP-223534 WID: Revised WID: NR NTN (Non-Terrestrial Networks) enhancements

[2] R1-2306217 Reply LS on RACH-less Handover

[3] R1-2306105 LS on higher layer signaling in Msg3 PUSCH for PUCCH repetition for Msg4 HARQ-ACK

[4] R1-2306271 Consolidated higher layers parameter list for Rel-18

[5] R2-2307314 Discussion on Cell Reselection Enhancements

# Appendix

Agreement RAN2#121bis-e: RAN2 confirms the general UE procedure for NTN RACH-less HO 1. receive a RACH-less HO command which can include pre-allocated grant optionally. FFS N_TA is optional. (RRC) 2. start timer T304 for the target cell (RRC) 3. perform DL and UL synchronization, and start timer T430. FFS how to perform RACH-less UL synchronization to NTN target cell. (RRC, MAC) 4. start time alignment timer (MAC) 5. monitor target cell PDCCH for dynamic grant if pre-allocated grant is not configured in RACH-less HO command (MAC, PHY) 6. send initial UL transmission including RRCReconfigurationComplete message using the available UL grant (RRC, MAC, PHY) 7. consider RACH-less HO is completed upon receiving NW confirmation. FFS how to confirm RACH-less HO is successfully completed. (RRC, MAC) 8. stop timer T304 for the target cell. (RRC) FFS whether to release UL grant if pre-allocated after RACH-less HO completion FFS RACH-less HO failure handling, e.g. whether UE fallback to RACH-based HO to the target cell FFS procedure for RACH-less HO combined with PCI unchanged or CHO if supported Agreement RAN2#122: 1. In NTN RACH-less handover, NW either indicates NTA in the target cell is identical to the source cell, or the NTA explicitly provided by the NW is 0. RAN2 will not discuss the case where NTA does not equal to 0 Agreement RAN2#122: 1. From RAN2 perspective synchronization among source and target cells is not an issue in NTN RACH-less HO 2. Release pre-allocated UL grant after RACH-less HO completion 3. LTE approach (of confirming the HO completion) is reused for both pre-allocated grant and dynamic grant. FFS any enhancement to the confirmation of RACH-less HO completion, e.g. the NW does not send the UE Contention Resolution Identity MAC CE, and sends PDCCH/PDSCH addressed to C-RNTI 4. Remove “FFS how to perform RACH-less UL synchronization to NTN target cell”, RAN2 assumes the UL sync handling in the target cell is the same in RACH-based HO and RACH-less HO, except how to acquire NTA (FFS on the spec impact , if any)