3GPP TSG-RAN WG2 Meeting #123	R2-2307316

Toulouse, France, August 21-25, 2023

Agenda item:	7.20.2

Source:	Samsung

Title:	Discussion on two TAs for multi-DCI multi-TRP

WID/SID:	NR_MIMO_evo_DL_UL-Core

Document for:	Discussion and Decision

# Introduction

In Rel-18 discussions on MIMO Evolution for Downlink and Uplink, RAN1 has agreed to support multi-DCI multi-TRP operation with two TAs. In this document, we continue discussion on open issues of two TAs for multi-DCI multi-TRP.

# Discussion

## TAG

In legacy TAG operation, the TAG that the SpCell belongs to is PTAG. For multi-DCI multi-TRP operation with 2 TAs, 2 TAGs are supported for SpCell. The issue is whether to define only one PTAG or 2 PTAGs.

For 1-PTAG model, only one TAG of the SpCell is defined as PTAG and the other TAG of SpCell is STAG. For 2-PTAG model, both TAGs of SpCell are PTAG. The limitation of 1-PTAG model is that when the TAT of PTAG is expired the legacy operation is applied, even though the TAT of the other TAG of SpCell is running and allowing UL transmission. Regarding specification impact, we need to specify: 1) which TAG is considered as PTAG; 2) the operation when a STAG is expired and the other TAG is running for a serving cell.

With 2 PTAGs in SpCell, only when both PTAGs are expired, legacy operation applies. If one PTAG is expired and the other PTAG is still valid, the DL and UL operation associated to the valid PTAG is not impacted. Thus, operation on serving cells are less impacted by the interruption due to TAT expiry. This is the advantage to be taken by supporting 2 TAs in multi-TRP operation. Regarding impacts to specification, we think 2-PTAG model does not cause any complication. When both PTAGs are expired, legacy operation of PTAG expiry applies. We only need to specify the operation when a TAG expired and the other TAG is running for a serving cell, with no difference to PTAG expiry or STAG expiry because the TAT for at least one PTAG is running.

Proposal 1: When 2 TAGs are configured for SpCell, both TAGs are PTAGs with no difference, i.e., 2-PTAG model.

Proposal 2: For 2-PTAG model, when TATs of both PTAGs are expired, legacy operation of PTAG expiry applies.

Proposal 3: For 2-PTAG model, specify the operation when TAT for one TAG is expired and TAT for the other TAG is running for a serving cell, with no difference whether the expired TAT is for a PTAG or a STAG as the TAT for at least one PTAG is running.

## TAT expiry

As specified in TS 38.321, the actions to be applied at TAT expiry are listed as follows in legacy operation.

- not perform any uplink transmission except the Random Access Preamble and MSGA transmission;

- flush all HARQ buffers;

- notify RRC to release PUCCH, if configured;

- notify RRC to release SRS, if configured;

- clear any configured downlink assignments and configured uplink grants;

- clear any PUSCH resource for semi-persistent CSI reporting;

- maintain NTA (defined in TS 38.211 [8]) of this TAG;

- consider all running timeAlignmentTimers as expired.

When two TATs are maintained for two TAGs respectively for a serving cell, the operation needs to be discussed is for the case the TAT of one TAG expires while the TAT for the other TAG is still running. In this case, UL transmissions using TCI states of the TRP with TAT expired should be suspended, while DL receptions on the two TRPs and UL transmissions on the TRP with TAT running are not impacted. HARQ retransmissions can be scheduled using TCI states of the TRP with TAT running, thus, the HARQ buffer should not be flushed.

Proposal 4: For a serving cell with 2 TAGs, when the TAT for a TAG expired (PTAG in 2-PTAG model or STAG), while the TAT for the other TAG running, HARQ buffers are not flushed, i.e., action 2 does not apply.

For actions 3-6, in legacy operation, DL/UL resource is configured per BWP. For multi-DCI multi-TRP operation, the DL/UL resource for different channels and signals is used by applying the associated TCI state(s). The TCI state to be applied for different channels and signals for multi-DCI multi-TRP operation are shown in the table below.

Figure 1. TCI selection scheme for mDCI mTRP operation

As RAN1 agreed, each joint/UL TCI state is configured with a TAG, thus, UL resource is associated to TAG via TCI state selection. When the TAT of a TAG is expired while the other TAT is running, DL reception is not impacted for either TAG, so configured downlink assignment should not be released; the UL resource scheduled with TCI state(s) that are associated to the TAG with TAT running should not be released. The remaining issue is whether the UL resource that is only associated to TCI states with TAT expired should be released or not. We think there is no need to release the configured UL resources for a serving cell if one TAT for the serving cell is still running. As the TCI state(s) assigned for a configured grant is dynamically indicated by MAC CE and DCI, it is possible that TCI state(s) associated to a TAG with the running TAT are reassigned to a configured UL resource. UE can suspend UL transmission on the configured UL resource that is only associated to TCI states with TAT expired and resume UL transmission on the configured UL resource once TCI states with TAT running are indicated or once the expired TAT is recovered.

Proposal 5: For a serving cell with 2 TAGs, when one TAT for a TAG expired (PTAG in 2-PTAG model or STAG), while the other TAT for the other TAG running, action 3-6 do not apply.

## CFRA by PDCCH order

RAN1 has agreed to support CFRA by PDCCH order to obtain TAs for multi-DCI multi-TRP operation for both intra-cell and inter-cell cases [2].

Agreement RAN1-111 For multi-DCI based Multi-TRP operation with two TA enhancement, support CFRA triggered by PDCCH order for both intra-cell and inter-cell cases.

For inter-cell case, RACH configuration for RACH towards additional PCIs is needed. Based on RAN1 agreement, one additional PRACH configuration for each additional PCI is supported for RACH triggered by PDCCH order.

Agreement RAN1-112 Confirm the following working assumption: For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI - the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI

Proposal 6: For mDCI multi-TRP, one PRACH configuration is configured for each additionalPCI for inter-cell PDCCH order CFRA, in addition to the serving cell PRACH configuration.

# Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: When 2 TAGs are configured for SpCell, both TAGs are PTAGs with no difference, i.e., 2-PTAG model.

Proposal 2: For 2-PTAG model, when TATs of both PTAGs are expired, legacy operation of PTAG expiry applies.

Proposal 3: For 2-PTAG model, specify the operation when TAT for one TAG is expired and TAT for the other TAG is running for a serving cell, with no difference whether the expired TAT is for a PTAG or a STAG as the TAT for at least one PTAG is running.

Proposal 4: For a serving cell with 2 TAGs, when the TAT for a TAG expired (PTAG in 2-PTAG model or STAG), while the TAT for the other TAG running, HARQ buffers are not flushed, i.e., action 2 does not apply.

Proposal 5: For a serving cell with 2 TAGs, when one TAT for a TAG expired (PTAG in 2-PTAG model or STAG), while the other TAT for the other TAG running, action 3-6 do not apply.

Proposal 6: For mDCI multi-TRP, one PRACH configuration is configured for each additionalPCI for inter-cell PDCCH order CFRA, in addition to the serving cell PRACH configuration.

# Reference

[1] RP-213598 WID: MIMO Evolution for Downlink and Uplink

[2] R1-2306270 LS on Rel-18 higher-layers parameter list

[3] R1-2306271 Consolidated higher layers parameter list for Rel-18