3GPP TSG RAN WG2 #123                                                                                R2-2307319

Toulouse, France, August 21 – August 25, 2023

Agenda Item:	7.6.4

Source:	THALES, Telit

Title:	Discontinuous coverage handling enhancement for IoT NTN

Document for:	Discussion and Decision

# Introduction

In the latest IoT NTN WID RP-223519 [1], the objective of further enhancement to discontinuous coverage is listed below:

-	Study and specify, if needed, mobility management enhancements and power saving enhancements for discontinuous coverage, taking into account the conclusions from the SA2 study FS_5GSAT_Ph2.  [RAN2, RAN3].

For enhancements on discontinuous coverage, SA2 has finished their study on Integration of satellite components in the 5G architecture (Phase 2) and 23 solutions have been adopted in the TR 23.700-28 V18.0.0 [2]. Under the new WID proposed in SA2#154AH meeting [3], SA2 has already started the normative work from SA2#154AH meeting on January, 2023. Meanwhile, RAN3 has had initial discussion on the RAN3 impacts of some solutions.

In RAN2#121 meeting, the following agreement has been achieved:

RAN2 can continue to check whether dedicated RRC signalling can be used for providing satellite information corresponding to discontinuous coverage.

In RAN2#121bis meeting, the following agreement has been achieved:

RAN2 to introduce enhancement to RRC Release using one of the following options (FFS which one):

- Explicit RRC Release using a new RRC Release cause

- UE Autonomous release (e.g. timer based or upon detection of coverage gap)

In this document, we propose to introduce a new capability for the UE to inform network of the real satellite visibility window to enhance the discontinuous coverage handling.

# Discussion

IoT devices are likely to be served by LEO satellites in complement to GEO satellites and Terrestrial Networks. LEO satellites are moving regarding to a fixed point on the ground. Typically, a given satellite covers one area on earth every ~4 hours with a visibility of 1 to 10 minutes. Hence, an IoT device cannot be connected to the satellite network for a long period time, which should be taken into account to save its battery. Especially, PSM (and eDRX) for IoT NTN i.e. UE activity time should be adapted to the period when a satellite becomes visible. The network should provide some information enabling the UE to determine the planned satellite visibility window based on its position (as well as potential motion) and the satellite ephemeris.

However, this approach covers only the “nominal” case where the UE is on a “flat” Earth and has no environment elements (e.g. mountains, buildings, etc.) on the Line-of-Sight (LOS). A typical scenario for IoT deployment is a static UE deployed in an area where it has no free LOS to the entire sky, e.g. mountain area, where road-side units are considered to be one use case in such scenarios for NTN connectivity. As a consequence, the actual environment of the UE may disturb the computed visibility window with the consequence of a Radio Link Failure (RLF) at the transmission of user data.

Figure 1: the planned satellite visibility window based on UE position vs real visibility window.

Observation 1: An early disappearance of the satellite according to the planned visibility window leads to a Radio Link Failure (RLF) and interrupted user data session, which could be avoided if the network would be aware on early disappearance of the satellite.

Another issue of a late appearance or an early disappearance of the satellite compared to planned window of visibility may lead to missed paging information by the device. A missed paging information leads to a paging escalation that increases the signalling load and the risk that network remove device from the network, i.e. if network pages according to nominal window, especially at the beginning of the window some UEs may not see the satellite yet. Leading to unnecessary paging escalation and sparring said window time that is in general not an option given the limited visibility time resources.

Observation 2: A late appearance of the satellite according to the planned visibility window may lead to unnecessary paging escalation, which could be avoided when network would be aware on late appearance for said UE.

The current solution to support discontinuous coverage is based on eDRX mechanism to address this intermittent window of visibility based on geometric considerations. However, this is insufficient to address the impact of UE actual environment on the planned window of visibility.

The proposed further enhancement aims at particularly address static UE in their specific environment constraints.

The idea is to adjust the computed planned visibility window taking into account indication reported by the UE. When connection breaks due to an early disappearance of the satellite or when the UE cannot see the satellite while being wake-up, the UE can send a message to the network through the next satellite appearance to inform about the mismatch between the computed visibility window and the actual visibility window.

The result will be higher rate of successful traffic and paging reception.

Proposal 1: For quasi earth fixed beam scenario, after an anticipated satellite end-of-coverage or a late satellite appearance compared to the calculated one, the UE should inform the network about the erroneous planned satellite visibility window.

At an early end-of-coverage event, the UE can send a message to the network OAM containing necessary information for the network to adapt the visibility window. The necessary information includes the time of RLF (i.e. the end of coverage), the serving satellite ephemeris and the UE position.

Proposal 2: Define a new RRC signalling message for the UE to report an early end-of-coverage event containing the end time of coverage, the UE position and the satellite ephemeris to the OAM.

When the satellite appears to late, the UE can send a message to the network OAM containing the necessary information for the network to adapt the visibility window. The necessary information includes the start of the service, the serving satellite ephemeris and the UE position.

Proposal 3: Define a new RRC signalling message for the UE to report a late satellite appearance event containing the start time of coverage, the UE position and the satellite ephemeris to the OAM.

The messages are optional for the UE and should be taken into account by the network at reception to adjust the planned satellite visibility window.

Proposal 4: Enable the network to adjust the planned satellite visibility window (i.e. T3324 and T3412 timers) according to the actual start of the visibility window indicated by the UE.

Especially for fixed location IoT devices or at least momentary fixed location IoT devices, there is an effect of accumulation of data over several satellite observations to determine a precise map of the UE environment and optimize the expected visibility window for the next satellites. Finally, the network can adapt PSM, eDRX and paging activity according to the optimized expected visibility window.

# Conclusion

Observation 1: An early disappearance of the satellite according to the planned visibility window leads to a Radio Link Failure (RLF) and interrupted user data session, which could be avoided if the network would be aware on early disappearance of the satellite.

Observation 2: A late appearance of the satellite according to the planned visibility window may lead to unnecessary paging escalation, which could be avoided when network would be aware on late appearance for said UE.

Proposal 1: For quasi earth fixed beam scenario, after an anticipated satellite end-of-coverage or a late satellite appearance compared to the calculated one, the UE should inform the network about the erroneous planned satellite visibility window.

Proposal 2: Define a new RRC signalling message for the UE to report an early end-of-coverage event containing the end time of coverage, the UE position and the satellite ephemeris to the OAM.

Proposal 3: Define a new RRC signalling message for the UE to report a late satellite appearance event containing the start time of coverage, the UE position and the satellite ephemeris to the OAM.

Proposal 4: Enable the network to adjust the planned satellite visibility window (i.e. T3324 and T3412 timers) according to the actual start of the visibility window indicated by the UE.

# References

- RP-223519, Revised WID: IoT (Internet of Things) NTN (non-terrestrial network) enhancements, MediaTek Inc., December 2022

- TR 23.700-28 V18.0.0 Study on Integration of satellite components in the 5G architecture; Phase 2 (Release 18), 2022-12.