3GPP TSG-RAN WG2 Meeting #123                                                        R2-2307346

Toulouse, France, 21st - 25th August, 2023

Source:              CATT

Title:	             Leftover issues on XR awareness

Agenda Item:	7.5.2

Document for:	Discussion and Decision

Introduction

In RAN2#122 meeting [1], some good progress was achieved on XR awareness, as captured below:

- Jitter signaling:

- 1: Do not use PIN delay budget request for jitter reporting for XR services. - 1. UE reports to RAN the range of jitter in its UL traffic, defined in the similar way as the one for N6 jitter. - 2: Reference time is defined in similar way as BAT (Burst Arrival Time) at UE side. - 2. UL assistance information (burst arrival time, UL jitter, FFS on periodicity) is reported per QoS flow. Network can configure for which QoS flow UE should report assistance information. - 3 RRC UAI framework is updated for Rel-18 to support signalling UL assistance information agreed so far for XR (Jitter, burst arrival time, FFS on periodicity).

- EoDB determination:

- 3: Reuse existing mechanisms (e.g. (Padding) BSR with BS value equal to zero) as implicit End of Data Burst (EoDB) indicator for the RAN. - 1: On the UL, the identification of PDU sets, data bursts and PSI is left to UE implementation. This doesn’t mean UE cannot use information provided by upper layers, but RAN2 does not intend to specify how.

This contribution discusses the leftover issues on this topic:

- Contents of UAI for XR traffic assistance information from UE to network (e.g. jitter signalling details, whether periodicity is needed, PSI signalling, etc.).

- How/what network configures for the UE on XR awareness (e.g. PSI/PSIHI, UAI framework, etc.) and how network uses the UE information (e.g. padding BSR to detect EoDB etc.)

Discussion

  - Contents of UAI for XR traffic assistance information

The need for reporting the periodicity

In RAN2#122 meeting, some companies challenged the need for the UE to measure and report the UL XR traffic periodicity, considering that it is already provided to RAN by CN (TS23.501 clause 5.37.8.1 [2]):

The following traffic assistance information may be provided by the CN to NG RAN in order to configure UE power saving management scheme for connected mode DRX: - UL and/or DL Periodicity; - N6 Jitter Information associated with the DL Periodicity; - Indication of End of Data Burst.

Note that, it is possible that AF does not provide the UL periodicity to NEF/PCF, but in that case, SMF can still request UPF to measure it [2]. And in such case, it is assumed that SMF would provide it to RAN. However in the case where neither AF nor UPF provide this UL periodicity, considering it is an important component of the XR traffic pattern impacting both DRX and CG configurations, RAN could request the UE to measure and report it.

Proposal 1: In the case where CN does not provide the UL periodicity information to RAN for a QoS flow, RAN should be able to request the UE to measure it and report this information to RAN.

Signaling details of the reported UL traffic characteristics

Assuming P1 is agreed, the UL traffic parameters to be reported by the UE are:

- Burst arrival time

- Burst periodicity

- Jitter

Although RAN2 advises to define the UL jitter in the similar way as the one for N6 jitter, SA2/RAN3 have not yet progressed the encoding details in TSCAI. On the other hand, the UL traffic characteristics should be “RAN friendly” meaning appropriately defined to match the gNB needs. We suggest below a definition for each parameter along these lines.

Burst arrival time

This parameter defines the average burst arrival time, i.e. the expected time occurrences of the bursts assuming the average periodicity. Practically, the BAT report is intended to be used by gNB to configure the time offset of the CG configuration to be mapped on this UL QoS flow. Therefore, a convenient approach is to reuse the start time of a CG Type 1 configuration, namely: timeReferenceSFN, timeDomainOffset, startSymbol.

Proposal 2: The reported burst arrival time is the average arrival time of the traffic bursts and is defined in UAI by reusing the start time of a CG Type 1 configuration, namely: timeReferenceSFN, timeDomainOffset, startSymbol.

Burst periodicity

This parameter defines the average periodicity of the traffic bursts and, similar to the enhancements agreed on DRX periodicity, it should be defined in rational number to properly match the XR frame rates.

Proposal 3: The reported burst periodicity is the average periodicity of the traffic bursts and is defined in rational number.

Jitter

This parameter defines the range of the variation of the bursts periodicity. Since this variation depends on the encoding delay of different types of video frames, the distribution of this variation should not be expected to be Gaussian, but rather a bounded peak-to-peak value as also characterized in Figure 3b of [3]. As a result, the jitter should be defined by the lower and upper bounds of the observed traffic burst periodicity variation around its average value, namely [-tmin, +tmax].

Proposal 4: The reported jitter parameter is defined by the lower and upper bounds of the variation around its average value of the traffic burst periodicity, [-tmin, +tmax].

  - RRC configuration for XR traffic

In RAN2#122, it was agreed:

2: PDU-set discard indication for UL is configured using RRC to handle the PDU Set based discard functionality (i.e. whether UE discards all packets in PDU set when one PDU is discarded). The configuration is per PDCP entity. Network indicates UE to apply PSI-based XR discard mechanism via dedicated signalling.

RRC configuration indicating UE to perform PDU Set based discard

We use pduSetDiscarding for this parameter name, although a different name may be eventually chosen by RAN2. We show in [4] that the current PDCP discard timer needs some enhancements to properly handle the PDU Set discard mechanism. Therefore the RRC parameter pduSetDiscarding also controls which of the legacy or the enhanced PDCP discard timer the UE should use. In the below proposal, the parenthesis addresses two different model options discussed in [4]. Given the impact of this RRC parameter is reduced to PDCP, it should be part of the IE PDCP-Config.

Proposal 5: Introduce a new field pduSetDiscarding in the IE PDCP-Config and, when configured:

- UE implements the enhanced PDCP timer for PDU Set discard

- UE discards all PDUs of a PDU Set when (any of) the PDCP discard timer(s) of the PDU Set expires

PSI-based discard

Per the above RAN2 agreement, the primary purpose of this new RRC parameter is to activate PSI-based discarding in the UE. In addition, as discussed in [5], we think that regardless of the method being used for implementing the PSI-based XR discard mechanism, a threshold should be provided to distinguish high and low importance PSIs. And similar to pduSetDiscarding, the impact of this RRC parameter is reduced to PDCP, so it should be part of the IE PDCP-Config.

Proposal 6: Introduce a new field psi-basedDiscard in the IE PDCP-Config to activate PSI-based discarding in the UE and provide the PSI threshold to distinguish high and low importance PSIs.

  - How network uses the UE information for XR

    - End of Data Burst (EoDB) detection in UL

Detecting the EoDB in UL is useful for the gNB to e.g. send a UE to sleep before the end of the DRX on-duration, to optimize UE power saving. In RAN2#122, it was discussed the use of legacy padding BSR as a way for gNB to detect when a data burst has been fully received. Clearly, when the padding BSR reports zero data in the buffer, and some data burst has already been received in this burst period, it can be safely derived that this burst is completed. However, the contrary is not necessarily true: a non-zero padding BSR does not necessarily mean that there is pending data to transmit for the current burst, because it can belong to the following burst. In such case, gNB may, instead of scheduling that UE for this pending data, prefer to schedule another UE.

Observation 1: Padding BSR is not 100% reliable to detect EoDB

Another UE indication exists that addresses the limitations of the padding BSR: the multi-PUSCH UTO-UCI defined by RAN1:

Indeed, RAN1 agreed enhancements of the legacy CG configuration to better match XR traffic by extending the number of CGOs from one to multiple PUSCHs per CG period. The mechanism reuses the NR-U’s framework for TDRA determination but where the number N of PUSCHs (aka transmissions opportunities, TOs) per period is configured independently from cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16 and the HPID determination is not left to UE’s choice. Clearly, such enhanced CGs aim at matching XR bursts where each bursts is mapped on a multi-PUSCH CG period. Given, it is expected that a UE does not always make use of all the TOs in a CG period, for example when the number of TBs is small, or if the burst arrives late within the jitter span, RAN1 agreed that the unused resources can be reported to gNB via “UTO-UCI”. Thus, when UE reports that the leftover TOs are unused for that period, it is an unambiguous indication that the EoDB has been reached.

Observation 2: UTO-UCI provides an unambiguous indication that the EoDB has been reached for the current burst.

Proposal 7: Capture in stage-2 that UTO-UCI of multi-PUSCH can be used as an implicit End of Data Burst indication in UL.

    - PSER related

The latest normative text from SA2 in TS23.501 on PSER was agreed in SA2#156-e and is not captured in [2] as follows:

#### 5.7.7.3 PDU Set Error Rate The PDU Set Error Rate (PSER) defines an upper bound for the rate of PDU Sets that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access). Thus, the PSER defines an upper bound for a rate of non-congestion related PDU Set losses.The purpose of the PSER is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in RAN of a 3GPP access). NOTE 1: In this Release, a PDU Set is considered as successfully delivered only when all PDUs of a PDU Set are delivered successfully. NOTE 2: How RAN enforces PSER is up to RAN implementation. A QoS Flow is associated with only one PDU Set Error Rate. PSER is an optional parameter. If the PSER is available, the usage of PSER supersedes the usage of PER. The value of the PDU Set Error Rate is the same in UL and DL.

From the above, it is clear that the PSER applies to both DL and UL QoS flows. Moreover, it was already agreed that PSER enforcement is fully left to gNB implementation, e.g. by adapting the DL and UL BLER accordingly (or any other action) based on the measured DL and UL PSER. We note though that in DL, gNB has all information to measure/monitor the PSER and set/adapt the DL BLER accordingly. But in UL, RAN2 agreed no in-band marking in support of PDU Set identification over Uu. In other words, the gNB has no clue about PDU Sets it receives in UL, it only sees PDUs. Therefore, in the current state of the design, gNB cannot track the PSER in UL. On the contrary, UE has full knowledge of which PDUs belong to each UL PDU Set and can easily monitor the relative amount of transmitted PDU Sets vs those that are discarded by PDCP discard timer. Therefore, one solution is that UE informs the gNB every time it discards a PDU Set, thus allowing gNB to count those. This information can be transferred via PDCP control PDU.

Proposal 8: When pduSetDiscarding is set/configured, UE informs gNB via PDCP control PDU every time it discards a PDU Set.

Conclusion

According to the analysis in section 2, it is observed and proposed:

Proposal 1: In the case where CN does not provide the UL periodicity information to RAN for a QoS flow, RAN should be able to request the UE to measure it and report this information to RAN.

Proposal 2: The reported burst arrival time is the average arrival time of the traffic bursts and is defined in UAI by reusing the start time of a CG Type 1 configuration, namely: timeReferenceSFN, timeDomainOffset, startSymbol.

Proposal 3: The reported burst periodicity is the average periodicity of the traffic bursts and is defined in rational number.

Proposal 4: The reported jitter parameter is defined by the lower and upper bounds of the variation around its average value of the traffic burst periodicity, [-tmin, +tmax].

Proposal 5: Introduce a new field pduSetDiscarding in the IE PDCP-Config and, when configured:

- UE implements the enhanced PDCP timer for PDU Set discard

- UE discards all PDUs of a PDU Set when (any of) the PDCP discard timer(s) of the PDU Set expires

Proposal 6: Introduce a new field psi-basedDiscard in the IE PDCP-Config to activate PSI-based discarding in the UE and provide the PSI threshold to distinguish high and low importance PSIs.

Observation 1: Padding BSR is not 100% reliable to detect EoDB

Observation 2: UTO-UCI provides an unambiguous indication that the EoDB has been reached for the current burst.

Proposal 7: Capture in stage-2 that UTO-UCI of multi-PUSCH can be used as an implicit End of Data Burst indication in UL.

Proposal 8: When pduSetDiscarding is set/configured, UE informs gNB via PDCP control PDU every time it discards a PDU Set.

Reference

- Draft Report of 3GPP TSG RAN WG2 meeting #122

- TS23.501 System architecture for the 5G System (5GS); Stage 2 (Release 18); 3GPP

- M. Lecci, M. Drago, A. Zanella, M. Zorzi, An Open Framework for Analyzing and Modeling XR Network Traffic, IEEE Access, Sept 2021

- R2-2307349, PDCP discard timer model for PDU Set discard, CATT, China Unicom, DELL, Ericsson, Intel Corporation, OPPO

- R2-2307350, Discard Operation for XR, CATT