3GPP TSG-RAN WG2 Meeting #123                                                     R2-2307348

Toulouse, France, August 21st-25th, 2023

Source:              CATT

Title:	             Consideration on DSR and BSR

Agenda Item:	7.5.4.1

Document for:	Discussion and Decision

- Introduction

Some key points of DSR and BSR enhancements have been agreed in last meetings. This contribution discusses details of the two technologies based on previous agreements.

- Discussion

2.1 DSR

RAN2 reached below agreements in last meeting. We address each FFS and two remaining issues: format and how to report.

1: UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this. When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this.

2.1.1 Issue 1: FFS if UE reports one or multiple values.

Referring to [1], the latency of video could be of the order of 10 ms or in a range from 7-15 milliseconds. And the latency of pose information is in the range of 10-20ms. Some companies propose to extend the DSR to all kinds of services, including the service with large delay budget.

However, the delay reporting for the service with long remaining time is not necessary, even harmful. First, gNB should allocate UL grants for UEs according to the priority and QoS requirements when received BSR reports from different UEs. Only when the remaining time is short enough, gNB should prioritize the associated UE over other UEs within the same priority. In another aspect, reporting multiple values will occupy more UL resources and then decrease the resource efficiency. The worst case is that oversize DSR may delay data transmission. Then one value for the shortest remaining time is enough.

Since DSR is useful when the remaining time is short enough, trigger condition needs to be considered. The DSR trigger should be configured by gNB. It can be a time threshold. UE will trigger DSR when the remaining time is equal to or lower than the time threshold and no recent UL grant can be used.

Observation 1: The delay report is useful only when the remaining time is short enough.

Observation 2: Oversize DSR due to multiple delay values will decrease resource efficiency and may delay data transmission.

Proposal 1: DSR should be applied to the time urgent service only.

Proposal 2: UE can report only one delay value in one DSR.

Proposal 3: DSR should be triggered when the remaining time is equal to or lower than a configured time threshold and no recent UL grant can be used.

DSR should be configured per LCH. But when DSR is reported with BSR, the associated LCG may contain several LCHs. We need to specify the relationship between DS value and LCH within the associated LCG. Two options could be considered: minimum delay value of the buffered data, or delay information of the oldest buffered data. Considering DSR is for delay-aware scheduling and the data with lowest remaining time should be scheduled first, minimum delay value of the buffered data should be considered for DSR.

Proposal 4: DSR should be configured per LCH.

Proposal 5: The reported DS value is associated with the LCH with minimal remaining time in the LCG.

2.1.2 Issue 2: FFS how this is modeled in PDCP specification.

Considering the agreements, we have two models: UE reports the running discardTimer directly; or UE reports the time from now to the expiry of discardTimer.

Model 1: remaining time = the running discardTimer in PDCP

Model 1 is simple for UEs but gNB cannot get the actual remaining time unless the delay information is for each LCH, that is, the LCG includes one LCH only.

And, even if gNB can deduce the actual remaining time for each LCH of each UE, the complexity of gNB is considerable. The procedure for gNB to perform the delay-aware scheduling should be:

- Receive the delay information (the running discardTimer) for each LCH of each UE;

- Invoke the configured values of discardTimer for each LCH of each UE;

- Calculate the actual remaining time for each UE;

- Perform UL scheduling for all UEs.

Model 2: remaining time = max value of discardTimer (configured by RRC) – the running discardTimer in PDCP

Model 2 requires UE to perform a simple subtraction. And gNB can apply the DSRs from UEs to perform delay-aware UL scheduling directly. It is simple and feasible in both gNB side and UE side. Hence it is preferred.

Proposal 6: The remaining time = max value of discardTimer (configured by RRC) – the running discardTimer in PDCP.

2.1.3 Issue 3: FFS which UEs support DSR

DSR is introduced for delay aware scheduling. Besides XR UEs, some other UEs, such as UEs with IIOT services require low latency and high reliability. So for the Rel-18 and later UE, only if it has the capability to report DSR, gNB can configure DSR for it.

Proposal 7: XR UEs and non-XR UEs (such as IIOT UE) may support DSR, which depends on gNB configuration and UE capability.

2.1.4 Issue 4: FFS if intra-UE prioritization can impact DSR

RAN2 agreed “When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information.”

And in MAC specification, MAC entity obtains a MAC PDU and triggers a new transmission as below. The MAC PDU can be obtained before the transmission occasion.

3> else if this uplink grant is a configured grant configured with autonomousTx; and 3> if the previous configured uplink grant, in the BWP, for this HARQ process was not prioritized; and 3> if a MAC PDU had already been obtained for this HARQ process; and 3> if the uplink grant size matches with size of the obtained MAC PDU; and 3> if none of PUSCH transmission(s) of the obtained MAC PDU has been completely performed: 4> consider the MAC PDU has been obtained. 3> else if the MAC entity is not configured with lch-basedPrioritization; or 3> if this uplink grant is a prioritized uplink grant: 4> obtain the MAC PDU to transmit from the Multiplexing and assembly entity, if any; 3> if a MAC PDU to transmit has been obtained: 4> if the uplink grant is not a configured grant configured with autonomousTx; or 4> if the uplink grant is a prioritized uplink grant: 5> deliver the MAC PDU and the uplink grant and the HARQ information of the TB to the identified HARQ process; 5> instruct the identified HARQ process to trigger a new transmission;

Combining the agreement of DSR and current MAC specification, DSR should be updated if the DSR has been included in a MAC PDU before the initial transmission. However, based on the agreement, we should specify the reference time of DSR explicitly, so no extra treatment is needed for intra-UE prioritization.

Observation 3: MAC PDU is (re)generated when uplink grant is a prioritized uplink grant.

Proposal 8: No extra treatment is needed for intra-UE prioritization.

2.1.5 Format

During the SI stage, it was argued that the delay status report (DSR) should be in BSR MAC CE or a separate MAC CE. As stated in [1], DSR is associated with specific data buffered in UE. If it is reported in a separate MAC CE, we need to set the rules of how to associate it with a BSR. The benefit of separate MAC CE is that it can be multiplexed in MAC PDU easily considering its small size. However, containing DSR in BSR MAC CE owns the benefit that the relationship between buffered data and remaining time can be acquired by gNB directly. Majority companies considered DSR in BSR MAC CE in previous meetings. If RAN2 adopts this option, a compacted MAC CE structure should be designed to avoid a too large MAC CE size to be multiplexed in a MAC PDU. For example, the new BSR MAC CE can contain BSR+DSR for one LCG as shown in Figure 1. A new LCID should be used to indicate the BSR MAC CE containing DSR to differentiate from other kinds of BSR MAC CEs.

Figure 1 An example of DSR in BSR MAC CE

In the BSR MAC CE containing DSR, the granularity of DSR should be decided. Considering the latency time range of XR services, 4 bits are enough for the DS (Delay Status) field. 4 bits can indicate 16ms at most and we don’t think remaining time larger than 16ms needs to be reported. Hence the DS field should report a remaining time in absolute value, and the unit can be ms.

Proposal 9: DSR can be included in a new compacted BSR MAC CE as Figure 1.

Proposal 10: DS field in the new BSR MAC CE can be 4-bit length. The absolute value with the unit ms should be reported.

2.1.6 How to report

While DSR is included in BSR MAC CE, we need to confirm if DSR is reported when legacy BSR is triggered. As Proposal 9, we consider BSR MAC CE including both BSR and DSR as an independent BSR MAC CE format. So the simplest way is UE reports BSR MAC CE containing BSR only for legacy BSR trigger, and reports BSR MAC CE containing both BSR and DSR for DSR trigger.

Proposal 11: UE reports BSR MAC CE containing BSR only for legacy BSR triggers, and reports BSR MAC CE containing both BSR and DSR for the DSR trigger.

  - BSR

The new BSR table has been simplified as a fixed static table. We discuss how to build the BSR table and some other details below.

Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases). We do not support additional piecewise linear BSR table in Rel-18. Can consider piecewise linearity when discussing how the BSR table values are defined.

2.2.1 New BSR table

Some considerations on value ranges of BSR table have been quoted in Table 1 [2][3][4][5]. The values of Bmax were deduced by different assumptions of XR services. We can see huge gaps among the values of Bmax.

Table 1 Bmax/Bmin in some contributions

|  | ZTE5018 | FW4864 | Honor5388 | QC4711 |
| --- | --- | --- | --- | --- |
| Bmin | 117409 | 0 |  | 44,289 |
| Bmax | 16,777,216(Maximum PDU Set size is encoded using a 24 bit value. This translates to a maximum value of 16,777,216 bytes.) | 204,000(60 Hz and a data rate of 10Mbps (baseline) or 20Mbps (optional) | 1,562,500 (bitrate= 200Mbps, frame rate=24 fps) | 781,250 (R/8/F =150Mbps/8/24) |

Note neither SA2 nor SA4 provides the potential data size range of XR services. And, the packet sizes in TR38.838 are examples for evaluation. Because of divergent characteristics of XR services, there is no authorized value of data size range can be anticipated. Accordingly, it is impossible to get a convergent Bmax based on XR service characteristics.

Observation 4: It is impossible to get a convergent Bmax based on XR service characteristics.

We can consider Bmax from another aspect: based on UL transmission capabilities as legacy BSR table. Reviewing the BSR table design in Rel-15, the Formula B has been used to decide Bmax.

Bmax    = Maximum Transport Block Size * 2 * RTT * Nmimo * Ncarrier  (Formula B)

For Bmax in legacy BSR table (81,338,368 bytes), RTT=8, Nmimo=2, Ncarrier=16 are applied.

For XR services, the Bmax could be resized based on below considerations:

- (2*RTT) can be replaced by (1*RTT) because UE could report another BSR quickly if gNB does not allocate enough UL grant;

- RTT=1ms can be considered for XR services;

- Ncarrier can be 4 or 2 considering the progress of commercial products.

Hence,

Bmax = 158864 Bytes * 1 * 1 * 2 * 4=1,270,912, or

Bmax = 158864 Bytes * 1 * 1 * 2 * 2= 635,456

For Bmin selection, the principle is that the quantization error should be no worse than that of the legacy table. And the quantization error is same as the step = (Bmax-Bmin)/255.

Then with linear distribution, 8-bit BS field, we have two options:

- Option 1: Ncarrier=4, Bmin= 75598 (index=142 in legacy table), Bmax= 1,270,912, step= 4687.5

- Option 2: Ncarrier=2, Bmin= 37850 (index=131 in legacy table), Bmax=635,456, step= 2343.5

Proposal 12: Bmin, Bmax are decoupled from the XR services characteristics.

Proposal 13: For the BS value in the new table, the quantization error should be no worse than that of the legacy table.

Proposal 14: The Bmax is calculated via legacy R15 formula with smaller inputs, e.g., Bmax = 1,270,912 or 635,456 could be considered.

2.2.2 How to report

The application of the new BSR table should be configured by gNB. To simplify the BSR configuration and report, when the new BSR table is configured, it can be used for any LCG. That is, UE does not need to identify which LCG(s) can use the new BSR table and which LCG(s) cannot.

Proposal 15: When gNB configures the new BSR table to the UE, the new BSR table can be used for any LCG.

When the buffered data volume is out of the scope of new BSR table, legacy BSR table should be used. The BSR MAC CE using new BSR table should be distinguished from the BSR MAC CE using legacy BSR table by (e)LCID value.

When the LCGs in the UE buffer need to use different BSR tables, two BSR MAC CEs should be reported and the two BSR MAC CEs can be in one MAC PDU.

Note the design for new BSR table is applicable to the BSR MAC CE containing DSR described in section 2.1 too.

Proposal 16: New (e)LCID(s) should be used to indicate the BSR MAC CE(s) using the new BSR table.

Proposal 17: UE selects BSR table based on the BS values of LCGs.

- If the BS values of all LCGs are in the value range of the new BSR table, one BSR MAC CE with the new BSR table is reported.

- If the BS values of all LCGs are out of the value range of the new BSR table, one BSR MAC CE with the legacy BSR table is reported.

- If part BS values of all LCGs are out of the value range of the new BSR table, two BSR MAC CEs can be reported in one MAC PDU.

2.2.3 Additional trigger

The XR traffic has the characteristics of periodicity, low latency, large packets and the variable packet size, especially for the video services. Legacy BSR triggers cannot satisfy the low latency requirements in some cases. For example, as shown in Figure 2, when PDU set 1 arrives at the UE buffer,  if there is another PDU0 belonging to an LCH with same or higher priority, regular BSR is not triggerred. Even CG resource has been prepared for the XR services, the remaining data of PDU set 1 is stuck in the UE buffer after UL transmission in CG resources due to the variable size of packets. Then the transmission of PDU set1 will be delayed.

Figure 2 An example of PDU set transmission

The PDU Set definition is as follows [1]:

PDU Set: A PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. a frame or video slice for XRM Services, as used in TR 26.926 [6]). In some implementations all PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information. In other implementations, the application layer can still recover parts or all of the information unit, when some PDUs are missing.

From the above, it is clear that packets of one PDU set need to be jointly processed for XR traffics. Therefore, once a PDU Set transmission has started, its completion should not be delayed.

Observation 5: From UL scheduler perspective, packets of one PDU set should be jointly processed as much as possible, i.e. once a PDU Set transmission has started, its completion should not be delayed.

As shown in Figure 3, after UE transmits part of data of PDU set 1 (e.g. in CG), if the UE can report the remaining data from PDU set 1 to the gNB, gNB will allocate UL resource to the UE immediately. Therefore, new BSR trigger due to PDU set should be considered. Note that, in order to limit the amount of such triggered PDU Set BSRs, this trigger can be complemented with a remaining time condition, so that the PDU Set BSR would only be triggered for those PDU Sets getting close to their PSDB budget.

Figure 3 An example of BSR enhancement for XR services

Proposal 18: RAN2 introduce BSR trigger due to PDU set data leftover.

One question is whether the BSR triggered by PDU set data leftover should report the remaining data volume of the PDU set, or the data volume of LCG as in legacy. We think for this information to be useful, it should be limited to the current on-going PDU Set (see Figure 3), which calls for a new BSR MAC CE.

To decrease the impact on data transmission, the BSR for PDU set should be compact. For example, it can include one fixed MAC subheader and one byte BS value. The (e)LCID in MAC subheader is used to indicate the PDU set level BSR. And the LCG field is not needed, because it would implicitly indicate the BS of the PDU Set carried in the MAC PDU together with the PDU Set BSR. In case the MAC PDU carries multiple PDU Sets, the BSR can indicate the remaining data for the most urgent PDU Set.

When DSR is triggered simultaneously, the BSR MAC CE format in Figure 1 can be used.

Proposal 19: RAN2 introduce a new and compact BSR MAC CE, “PDU Set BSR”, reporting the remaining data volume of the PDU set contained in current MAC PDU.

- Conclusion

According to the analysis in section 2, we provide below observations and proposals.

Observations:

Observation 1: The delay report is useful only when the remaining time is short enough.

Observation 2: Oversize DSR due to multiple delay values will decrease resource efficiency and may delay data transmission.

Observation 3: MAC PDU is (re)generated when uplink grant is a prioritized uplink grant.

Observation 4: It is impossible to get a convergent Bmax based on XR service characteristics.

Observation 5: From UL scheduler perspective, packets of one PDU set should be jointly processed as much as possible, i.e. once a PDU Set transmission has started, its completion should not be delayed.

Proposals:

Proposal 1: DSR should be applied to the time urgent service only.

Proposal 2: UE can report only one delay value in one DSR.

Proposal 3: DSR should be triggered when the remaining time is equal to or lower than a configured time threshold and no recent UL grant can be used.

Proposal 4: DSR should be configured per LCH.

Proposal 5: The reported DS value is associated with the LCH with minimal remaining time in the LCG.

Proposal 6: The remaining time = max value of discardTimer (configured by RRC) – the running discardTimer in PDCP.

Proposal 7: XR UEs and non-XR UEs (such as IIOT UE) may support DSR, which depends on gNB configuration and UE capability.

Proposal 8: No extra treatment is needed for intra-UE prioritization.

Proposal 9: DSR can be included in a new compacted BSR MAC CE as Figure 1.

Proposal 10: DS field in the new BSR MAC CE can be 4-bit length. The absolute value with the unit ms should be reported.

Proposal 11: UE reports BSR MAC CE containing BSR only for legacy BSR triggers, and reports BSR MAC CE containing both BSR and DSR for the DSR trigger.

Proposal 12: Bmin, Bmax are decoupled from the XR services characteristics.

Proposal 13: For the BS value in the new table, the quantization error should be no worse than that of the legacy table.

Proposal 14: The Bmax is calculated via legacy R15 formula with smaller inputs, e.g., Bmax = 1,270,912 or 635,456 could be considered.

Proposal 15: When gNB configures the new BSR table to the UE, the new BSR table can be used for any LCG.

Proposal 16: New (e)LCID(s) should be used to indicate the BSR MAC CE(s) using the new BSR table.

Proposal 17: UE selects BSR table based on the BS values of LCGs.

- If the BS values of all LCGs are in the value range of the new BSR table, one BSR MAC CE with the new BSR table is reported.

- If the BS values of all LCGs are out of the value range of the new BSR table, one BSR MAC CE with the legacy BSR table is reported.

- If part BS values of all LCGs are out of the value range of the new BSR table, two BSR MAC CEs can be reported in one MAC PDU.

Proposal 18: RAN2 introduce BSR trigger due to PDU set data leftover.

Proposal 19: RAN2 introduce a new and compact BSR MAC CE, “PDU Set BSR”, reporting the remaining data volume of the PDU set contained in current MAC PDU.

- References

- TR 38.835  NR; Study on XR enhancements for NR vi01

- R2-2305018	BSR enhancements for XR	ZTE

- R2-2304864	Further discussions on BSR enhancements for XR	Futurewei

- R2-2305388	Discussion on BSR enhancements for XR	Honor

- R2-2304711	BSR and delay status report for XR	Qualcomm