3GPP TSG-RAN WG2 #123	R2-2307363

Toulouse, France, August  2023

Agenda Item:	7.9.4

Source:	OPPO, Samsung, China Telecom, Huawei, HiSilicon, Ericsson, vivo, CMCC

Title:	Discussion on non-split SRB

Document for:	Discussion, Decision

Introduction

This paper will discuss the transmission of non-split SRB1/2 in Multi-path relay

Discussion

In RAN2 #119bis, it was agreed that

For scenario 1, SRB1 and SRB2 can be configured on either the direct or the indirect path, or on both at least with duplication.  FFS if they can be configured on different paths from one another.

In RAN2 #120, it was agreed that

Support PCell on the direct path only when the UE is in multi-path operation, for both scenario 1 and scenario 2.

There seems some collisions between the two conclusions since it makes no sense to configure SRB on indirect-path only (not a PCell).

In case in-direct path quality is worse and thus may trigger a failure before PCell (direct path), having non-split SRB1 on indirect path would lead to a case when PCell has no radio quality problem but UE needs to perform RRC re-establishment due to indirect-path RLF, which would cause unnecessary service interruption. So in this case, it is straightforward to have SRB on the direct path (or duplicate for reliability), in which case the SRB delivery will not be interrupted during the failure period.

Configuring non-split SRB1 on indirect path would lead to a case where PCell has no radio quality problem but UE needs to perform RRC re-establishment due to indirect path failure, which would cause unnecessary service interruption.

Then for the case when the PCell (direct path) quality is worse and thus may trigger a failure before indirect-path, which is the main target scenario of supporting non-split SRB1 on indirect-path from the proponent perspective, it is not a reasonable deployment scenario that the network configures multipath relay even the direct path is unreliable which leads to an unreliable PCell deployment.

Configuring multipath even if the direct path is not reliable is an unreasonable network deployment that should not be seen as a typical case.

In RAN2 #122, it is agreed that SRB1 and SRB2 should be handled in the same way in terms of non-split SRB on indirect-path

For scenario 1, SRB1 and SRB2 are not decoupled in terms of support of non-split SRB on indirect path; i.e., if SRB1 can be supported on indirect path, so can SRB2.

There are configuration parameters carried in DL SRB1/2 that must be transmitted through the direct path, e.g., the referenceTimeInfo, which means the SRB1/2 transmission on the direct path can not be avoided, i.e, either non-split SRB1/2 on the direct path or duplicated SRB1/2.

There are configuration parameters carried in DL SRB1/2 (e.g., referenceTimeInfo) that must be transmitted through the direct path.

And we have agreed not to support non-split SRB1/2 on indirect path for scenario-2.

Non-split SRB1 and 2 over indirect path is not supported in Scenario 2.

Non-split SRB1 and 2 over indirect path is concluded as not supported in Scenario 2.

In summary, supporting non-split SRB1/2 on indirect path in scenario-1 has no clear target scenario/benefit but requires further discussions, therefore RAN2 should align with scenario-2 and doesn’t support non-split SRB1/2 on indirect path in scenario-1 as well.

Non-split SRB1 and 2 over indirect path is not supported in Scenario 1.

Conclusion

We have the following observations:

Observation 1	Configuring non-split SRB1 on indirect path would lead to a case where PCell has no radio quality problem but UE needs to perform RRC re-establishment due to indirect path failure, which would cause unnecessary service interruption.

Observation 2	Configuring multipath even if the direct path is not reliable is an unreasonable network deployment that should not be seen as a typical case.

Observation 3	There are configuration parameters carried in DL SRB1/2 (e.g., referenceTimeInfo) that must be transmitted through the direct path.

Observation 4	Non-split SRB1 and 2 over indirect path is concluded as not supported in Scenario 2.

We have the following proposals:

Proposal 1	Non-split SRB1 and 2 over indirect path is not supported in Scenario 1.