3GPP TSG-RAN WG2 Meeting #123                                              	R2-2307364

Toulouse, France, 21st– 25th Aug, 2023

Source:	CATT, Turkcell

Title:	Further discussions on architecture general aspects of AI/ML for NR air-interface

Agenda Item:	7.16.2.1

Document for:	Discussion and Decision

Introduction

In last RAN2 meeting, for functional arch part the following agreements were achieved:

Intention is to cover functional arch in general, e.g. covering both be model based and/or functionality based LCM “Model Storage” in the figure is only intended as a reference point (if any) for protocol terminations etc for model transfer/delivery etc. It is not intended to limit where models are actually stored. Add a note for this. Remove “Model” in Model Managemt and Model Inference and for the actions/the arrow form Management to Inference (to reduce the risk for misunderstanding). Management may be model based management, or functionality based management. Add a mote for this. With the modifications above Figure 2 from R2-2305327 is agreed

For Physical Network Arch, a post email of [Post122][060][AIML] Mapping of functions to physical entities was ongoing and other issues were not discussed yet. In this contribution, we mainly focus on the general parts of AI/ML, based on the existing RAN1 and RAN2 agreements.

Discussion

  - AIML algorithm dependency

The AI/ML algorithm dependency is highlighted in RAN2 #123 meeting for the locality (e.g. cell-specific), UE-side AI/ML dependency on gNodeB configuration, etc, dependency on other aspects such as UE speed, Network-side AI/ML dependency to be UE specific, etc, and the related procedure impacts. We discuss the expected impacts of network-side algorithms.

AI/ML models deployed by the UE and the network can depend on various aspects of UE and network configuration. The network side AI/ML can be chosen based on the location or cell-specific for example a university campus or factory warehouse. The UE side AI/ML algorithm can be deployed based on gNodeB configuration or UE speeds like the UEs on a highway or high-speed train.

Observation 1: AI/ML models deployed by the UE and the network can depend on various aspects of UE and network configuration.

It is essential to support specific and generic dependency and locality for both UE and network sides. Specific data collection may be needed for different locality e.g., cell specific, gNodeB specific, gNodeB group specific. Different additional conditions may impact:

- Different training methods for different conditions

- Identification of different models/functionalities

- Model selection/switch for inference of different conditions, e.g., UE speed-specific models

Proposal 1: It is essential to support specific and generic dependency and locality for both UE and network sides.

- Different training methods for different conditions

- Identification of different models/functionalities

- Model selection/switch for inference of different conditions, e.g., UE speed-specific models

  - Metadata of model/functionality

For model-ID based LCM, metadata (i.e. model description information) is the core part of model identification and shall be provided during model identification procedure. It will be impossible to perform LCM within 3GPP without sufficient metadata of the identified model.

RAN2 has agreed that applicable conditions and model performance indicators should be part of meta information as a starting point. Vendor info is FFS. At the same time, in RAN1 agreement from RAN1#112bis-e, the following bullets are emphasized for model-ID-based LCM:

- model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda.

Considering both the agreements in RAN2 and RAN1, it can be deduced that ‘applicable condition’ in RAN2 may include both ‘association with specific configurations/conditions of AI/ML-enabled Feature/FG’ and ‘additional conditions (e.g., scenarios, sites, and datasets)’.

In our understanding, as a starting point, the following information can be considered for metadata of an AI/ML model, taking UE-sided model and UE part of two-sided model as example:

- Applicable conditions

  - Associated AI/ML-enabled Feature/FG.

  - Association with configurations/conditions, including e.g., configuration for nominal input and/or nominal output (e.g. payload size for CSI compression), configuration for RS measurement, configuration for quantization, etc.

  - Additional conditions, e.g. scenarios, sites, datasets, paring information (for two-sided model), concurrent use with other AI/ML models and/or non-AI/ML features.

- Model performance indicators

  - Performance of inference accuracy or system performance, which may be hypothetic or predicted one.

Note that:

- Nominal input is useful at least for model inference and model monitoring, e.g. whether the input is raw channel or eigenvector, for intermediate KPI calculation in CSI compression. It is also useful if UE asks network to provide some dataset collected in local site for model fine-tuning.

- Nominal output is needed for NW to understand the role the model can play. This is also critical when model monitoring is at NW side.

- Information on concurrent use with other AI/ML models and/or non-AI/ML features is useful for network to plan the LCM of AI/ML model, when multiple AI/ML models (e.g. with different functionality) cannot be activated simultaneously. It is foreseen that multiple AI/ML models share the limited storage and computation power.

- Dedicated paring information for two-sided model may or may not exist. For example, if the model ID or dataset ID serve as paring information, then dedicated paring information is not needed.

Besides, for vendor information, it seems not related to LCM procedure. But it may be useful to achieve the purpose of unique global ID.  Hence we are not against including vendor information in metadata, but the applicability/usefulness shall be discussed carefully.

Proposal 2: The following information can be considered for metadata of an AI/ML model:

- Applicable conditions

  - Associated AI/ML-enabled Feature/FG.

  - Association with configurations/conditions, including, e.g. configuration for nominal input and/or nominal output (e.g. payload size for CSI compression), configuration for RS measurement, configuration for quantization, etc.

  - Additional conditions, e.g. scenarios, sites, datasets, paring information (for two-sided model, if applicable), concurrent use with other AI/ML models and/or non-AI/ML features.

- Model performance indicators

  - Performance of inference accuracy or system performance, which may be hypothetic or predicted one.

Metadata is not only critical in model identification, but also in model transfer via 3GPP signaling. We think metadata for model identification can be reused for model transfer directly.

Proposal 3: Metadata for model identification can be reused for model transfer.

For functionality based LCM, metadata is also necessary to be provided during functionality identification procedure. Thus, NW can perform the functionality based LCM based on the metadata of functionality (i.e. applicable conditions for supported AI/ML functionality). The content of metadata (e.g., scenarios, sites, and datasets) can be further discussed based on RAN1 progress.

Proposal 4: For functionality identification, the information for metadata of functionality (i.e., applicable conditions for supported AI/ML functionality) is necessary to be provided.

  - UE capability

It is aware that AI/ML-based approaches are power consuming and computation consuming. The required buffer size may not be negligible if multiple AI/ML models are implemented. However, assuming UE can manage well on the storage and computation, it does not need to report its capability on storage and computation. Instead, it may just report/update the applicable model/functionalities.

Observation 2: Assuming the UE is able to report/update the applicable models/functionalities, there is no strong need to report its hardware capability, e.g. storage, computation power.

It is expected that the UE shall support some LCM procedures which are more ‘essential’, e.g. model activation/deactivation. However, some other LCM operations may be more ‘optional’ and require higher capabilities from UE, including:

- Capability of online training

Online training requires frequent update of deployed AI/ML model in real-time or near-real-time. This brings non-negligible burden to the UE in regard of computation and power consumption. It is more realistic to consider online training as an ‘optional capability’ for a UE supporting AI/ML-based approach.

- Capability of implementing transferred AI/ML model

Due to the higher requirement storage and computation, AI/ML-based approach may need more hardware optimization than usual. For example, if the transferred/downloaded model is based in form of MRF (model representation format) rather than binary runtime image, it needs to be compiled before loaded into hardware. The difficulty is identified at least for Case z5 (model transfer in open format of an unknown model structure at UE) defined by RAN1, i.e. the model structure is unknown. It is possible that a UE can only supports a proprietary model, but not the one downloaded from the network, even if the sizes and computation power between them are similar.

- Dynamic capability of supported model/functionality type

The UE may not have all the AI/ML models/ functionalities in the storage. Since some UE could have chance to get new model/functionality based on e.g. remote download from a third-party server, it may be reasonable to be defined and indicated to the network whether the UE could support some kind of model/functionality type(s), e.g. whether some specific AI/ML feature(s) can be enabled. Due to the legacy UE capability is stable and cannot be triggered for change from UE side, a new mechanism of UE capability update initiated by UE can be considered. But in our understanding, the storage/UE computation resources which may change frequently should not be indicated by the UE capability procedure.

Although it may be a little too early to consider UE capability for now, we should keep in mind that a practical AI/ML framework is strive to.

Proposal 5: For support of AI/ML-based approach at UE, the following UE capabilities can be considered as optional:

- Capability of online training;

- Capability of implementing transferred AI/ML model;

- Dynamic capability of supported model/functionality type.

Proposal 6: The storage/UE computation resources which may change frequently should not be indicated by the UE capability procedure.

Conclusion

In this contribution, we have some discussions on the following leftover issues of the Artificial Intelligence Machine Learning Architecture for NR Air Interface and the general aspects, and the following observations and proposals are made:

For AIML algorithm dependency:

Observation 1: AI/ML models deployed by the UE and the network can depend on various aspects of UE and network configuration.

Proposal 1: It is essential to support specific and generic dependency and locality for both UE and network sides.

- Different training methods for different conditions

- Identification of different models/functionalities

- Model selection/switch for inference of different conditions, e.g., UE speed-specific models

For metadata of model/functionality:

Proposal 2: The following information shall be considered for metadata of an AI/ML model:

- Applicable conditions

  - Associated AI/ML-enabled Feature/FG.

  - Association with configurations/conditions, including, e.g. configuration for nominal input and/or nominal output (e.g. payload size for CSI compression), configuration for RS measurement, configuration for quantization, etc.

  - Additional conditions, e.g. scenarios, sites, datasets, paring information (for two-sided model, if applicable), concurrent use with other AI/ML models and/or non-AI/ML features.

- Model performance indicators

  - Performance of inference accuracy or system performance, which may be hypothetic or predicted one.

Proposal 3: Metadata for model identification can be reused for model transfer.

Proposal 4: For functionality identification, the information for metadata of functionality (i.e., applicable conditions for supported AI/ML functionality) is necessary to be provided.

For UE capability:

Observation 2: Assuming the UE is able to report/update the applicable models/functionalities, there is no strong need to report its hardware capability, e.g. storage, computation power.

Proposal 5: For support of AI/ML-based approach at UE, the following UE capabilities can be considered as optional:

- Capability of online training;

- Capability of implementing transferred AI/ML model;

- Dynamic capability of supported model/functionality type.

Proposal 6: The storage/UE computation resources which may change frequently should not be indicated by the UE capability procedure.