3GPP TSG-RAN WG2 Meeting #123                                              	R2-2307365

Toulouse, France, 21st– 25th Aug, 2023

Source:	CATT, Turkcell

Title:	Considerations on data collection of AI/ML for NR air-interface

Agenda Item:	7.16.2.2

Document for:	Discussion and Decision

Introduction

For data collection of AI/ML for NR air-interface, to coordinate the LS to RAN1, a lot of agreements were achieved in last RAN2 meeting. And then the LS out was approved in [1]. The LS has 2 parts of contents:

Part A: RAN2 Assumptions on data collection that require RAN1 confirmation

Part B: Aspects of data collection that require RAN1 feedback/inputs

In part A, 4 assumptions are listed and need RAN1 to confirm:

- Assumption 1: RAN2 assumes that for the data collection in some scenarios (e.g., internal data up to implementation or the existing data are enough), possibly no RAN2 specification effort is needed in some scenarios;

- Assumption 2: For the latency requirement of data collection, RAN2 assumes for all types of offline model training, there is no latency requirement for data collection. But for model inference or (real-time) model monitoring, there is a latency requirement for data collection;

- Assumption 3: RAN2 assumes that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). Analysis and potential enhancement of the non-connected state can be revisited when needed;

- Assumption 4: For the data generation entity and termination entity deployed at different entities, RAN2 made the assumptions on the generated / terminated entities for model training/inference/monitoring per use case.

In part B, RAN2 would like RAN1 to provide feedback/inputs on the following essential aspects per LCM purpose (i.e., model training, inference and monitoring) for each (sub)use case, and the LCM sidedness should also be considered:

- Data content

- Typical data size (value or value range) of the identified data content

- Reporting type (e.g., periodic, event triggered, other) of the identified data content

- Typical latency requirement (value or value range) to transfer the identified data content

And then based on RAN1 feedback, RAN2 could facilitate the discussion on data collection for further progress.

Since RAN1's relay LS is expected to be generated at the earliest in this meeting period, in this contribution, first we intend to further analyse the data collection requirement based on previous agreements of RAN1 and the feasibility of reusing the existing frameworks, and then to discuss whether and how to apply or enhance the existing frameworks to achieve the AI/ML model LCM purposes.

Discussion

  - Data collection per use case

In the offline discussion#025 [2] during RAN2#121 meeting, a table on existing data collection framework analysis was agreed as a starting point for subsequent discussion, and more columns can be added in the table if needed later. The table covers the metrics analyses of Involved Network entity, RRC state to generate data, Max payload size per reporting, Contents to be collected, End-to-End report latency, Report type and Security/Privacy for the existing frameworks of both L1 and L3 measurements, including:

- Logged MDT;

- Immediate MDT;

- L3 measurements;

- L1 measurement (CSI reporting);

- UAI;

- Early measurements;

- LPP.

In RAN2 121bis meeting, it was agreed to modify the previously endorsed table by adding 3 additional columns. But the endorsed table is indexed by existing frameworks as the highest level. This table can be utilized in RAN2 to discuss the pros and cons of the listed existing frameworks. But in our understanding, to choose which framework for the AI/ML data collection to be used or to make clear which framework can be improved should base on the requirement from RAN1. It is:

- First to identify which LCM purposes need data collection: model training, model inference, model monitoring (this has already identified by RAN2);

- Second to make clear the requirements of data collection for each LCM purpose based on RAN1 agreements/relay LS, e.g. data content, size, latency .etc.

- Third to compare the requirements with existing framework metrics, to determine:

  - Whether the existing framework can be used directly;

  - Whether the existing framework needs enhancement to adapt to the AI/ML data collection;

  - Whether new framework is needed.

Observation 1: To modify the endorsed table by adding 3 additional columns cannot effectively corresponds the RAN1 requirements to the frameworks.

Proposal 1: Introduce new tables involving LCM purposes, data collection requirements and possible frameworks per use case for data collection analysis.

Since RAN1's relay LS about the data collection requirements is expected to be generated at earliest in this meeting period, we provide a general analysis on the data collection requirements, and the feasibility of the possible framework based on the RAN2 assumptions and our comprehension as below. If the requirements in the RAN1 reply LS do not match our analysis as described below, we can easily update them according to the requirements from RAN1 and evaluate whether the potential framework we have currently chosen is suitable.

- 2.2.1	CSI feedback enhancement

For CSI feedback enhancement, two sub use cases i.e. spatial-frequency domain CSI compression using two-sided AI model and time domain CSI prediction using UE sided model are selected as representative sub-use case in RAN1. RAN1 has agreed to defer potential specification impact discussion for time domain CSI prediction using UE sided model sub use case. Therefore, we will discuss data collection for CSI feedback enhancement focusing on spatial-frequency domain CSI compression using two-sided AI model sub use case.

In RAN1#112 meeting, RAN1 had agreed that for CSI compression using two-sided model use case, RAN1 will further study the necessity, feasibility, and potential specification impact of UE side data collection enhancement as below.

- Enhancement of CSI-RS configuration to enable higher accuracy measurement. - Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc. - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side. - Signaling for triggering the data collection

RAN1 had agreed also that for CSI compression using two-sided model use case, RAN1 will further discuss the necessity, feasibility, and potential specification impact for NW side data collection including at least the follow.

- Enhancement of SRS and/or CSI-RS measurement and/or CSI reporting to enable higher accuracy measurement. - Contents of the ground-truth CSI including: - Data sample type, e.g., precoding matrix, channel matrix etc. - Data sample format: scaler quantization and/or codebook-based quantization (e.g., e-type II like). - Assistance information (e.g., time stamps, and/or cell ID, Assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator) - Latency requirement for data collection - Signaling for triggering the data collection

Based on RAN1’s agreement above, for UE side data collection, maybe RAN2 needs further consider the delivery of enhanced CSI-RS configuration and the assistance information (e.g. for categorizing the data). For NW side data collection, the delivery of enhanced SRS/CSI-RS configuration and the ground–truth CSI needs to be considered.  Actually, more discussion is needed for UE side data collection and NW side data collection in RAN1. RAN2 needs to wait for RAN1’s more progress on this issue.

Proposal 2: For data collection in CSI compression, RAN2 further considers the enhancement of CSI-RS configuration and the assistance information for UE side data collection, and the enhancement of SRS/CSI-RS configuration and the ground-truth CSI for network side data collection.

Based on the proposal 1 above, it is more suitable to introduce new tables to analyse the requirement of data collection for different use case for different LCM purposes, then by comparing the new introduced requirement tables and the existing framework analysis table [2], we can choose the appropriate framework(s).

At the current stage of RAN2, based on RAN1’s agreements for CSI compression using two-sided AI model sub use case, we give an initial analysis of the requirement of data collection for different LCM purposes and the applicability of existing framework in table 1 as below.

Table 1 New Table to Map LCM Purposes to Data Collection Frameworks for CSI use case

| LCM purpose | Data collection requirements | Framework | Feasibility analysis |
| --- | --- | --- | --- |
| Offline-training | Data content: ground truth CSI Size: large Latency: low | UAI Immediate MDT/RRM measurement CSI reporting | If model training at network side, network can collect data from multiple UEs, and perform offline-training based on collected dataset. UAI, RRM measurements, immediate MDT and CSI reporting can be considered as data collection framework with enhancement to support AI/ML based data collection. If model training at UE side, UE can collect data by itself, which is up to UE implementation. It also can receive dataset from network which will involve the data collection of network and dataset transfer from network to UE. UAI, RRM measurements, immediate MDT and CSI reporting can be considered as data collection framework with enhancement to support AI/ML based data collection. |
|  |  | Other framework | If model training at UE, UE receives dataset from network for offline-training, signalling for dataset transfer also needs to be considered. |
| Monitoring | Data content: 1) the target CSI with realistic channel estimation associated to the CSI report (NW-side monitoring) 2) the output of the CSI reconstruction model (UE-side monitoring) Size: small Latency: medium | UAI Immediate MDT/RRM measurement CSI reporting | If model monitoring at network side, the network needs to acquire the target CSI with realistic channel estimation associated to the CSI report from UE. UAI, RRM measurements, immediate MDT and CSI reporting framework with enhancement can be used for the data collection. E.g., enhance to support sending the target CSI with realistic channel estimation associated to the CSI report from UE to network. |
|  |  | Other framework | If model monitoring at UE side, the UE needs to acquire the output of the CSI reconstruction model from network. Signalling for sending the output of the CSI reconstruction model from network to UE needs to be considered. If model monitoring at UE side, the output of the CSI reconstruction model at the UE-side, which is up to UE implementation. |
| Inference | Data content: 1) ground truth CSI (UE sided model) 2) compressed CSI (NW sided model) Size: small Latency: High | CSI reporting | UE side collects the ground truth CSI as the input data of CSI generation model. It is up to UE implementation. Network side collects the compressed CSI as the input data of CSI reconstruction model. The compressed CSI is reported by UE to network through air interface. The existing L1 framework can be used with enhancement, such as the format of compressed CSI etc. |

Proposal 3: For data collection of offline-training as well model monitoring for CSI compression, RRC framework (immediate MDT/RRM measurement/UAI) and L1 framework (CSI reporting) can be considered as high priority.

Proposal 4: For data collection of model inference for CSI compression, L1 framework (CSI reporting) can be considered as high priority.

- 2.2.2	Positioning accuracy enhancement

For positioning accuracy enhancement, 5 cases have been identified for both sub-use cases of direct AI/ML positioning and AI/ML assisted positioning:

- Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

- Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

- Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

Cases 1, 2a and 2b are involving UE reporting, and cases 3a and 3b are the positioning only involving LMF and gNB. So to consider the specification impact of air-interface, RAN2 should firstly focus on the first three cases.

Proposal 5: RAN2 takes cases 1/2a/2b as high priority for positioning accuracy enhancement use case.

The data collection of positioning accuracy enhancement has been discussed from RAN1#110 meeting until now. The main content needs to be collected agreed by RAN1is listed as below for summary, which including:

- for AI/ML model training:

  - Ground truth label (e.g., based on UE/PRU/TRP measurement/report), report from the label data generation entity

    - Location for direct AI/ML positioning, including:

      - PRU with known location;

      - UE/LMF generates location based on any positioning methods;

      - LMF with known PRU location;

    - One or more of the intermediate parameter(s) for AI/ML assisted positioning, including:

      - UE/PRU/Network entity generates label directly or calculates based on measurement/location

    - Quality indicator, report from the label and/or the measurement data generation entity and/or as request from a different (e.g., data collection, etc.) entity

      - To indicate the quality of label and/or measurement at least for model training

  - Model input (other training data), report from the measurement data generation entity

    - Measurement corresponding to model input, generated by UE/PRU for cases 1/2a/2b and generated by TRP for cases 3a/3b

  - Other information

    - Information related training dataset/samples, information related to scenario, resource configuration & mapping, timing for training data, information on implementation imperfections, etc.

  - Assistance information

    - at least RS (e.g., PRS/SRS) configuration(s) and configuration identifier, at least for deriving measurement

    - Request from data generation entity (UE/PRU/TRP) to LMF and/or as LMF assistance signaling to UE/PRU/TRP

  - Time stamp

    - At least for and/or associated with training data for model training

      - Separate time stamp for measurement and ground truth label, when measurement and ground truth label are generated by different entities

    - Report from data generation entity together with training data and/or as LMF assistance signaling

- For AI/ML model inference:

  - For direct AI/ML positioning

    - Potential new measurement: CIR/PDP

    - Existing measurement: e.g., RSRP/RSRPP/RSTD

  - For AI/ML assisted positioning

    - New measurement report: e.g., ToA, path phase

    - Existing measurement report: e.g., RSTD, LOS/NLOS indicator, RSRPP

    - Enhancement of existing measurement report: e.g., soft information/high resolution of RSTD

  - Assistance signaling and procedure for both UE-side and Network-side model

    - RS configurations

    - Other assistance information

- for AI/ML model monitoring:

  - monitoring based on model output (ground truth label)

    - UE location, ground truth label for direct AI/ML positioning;

      - The usefulness involving ground truth label

        - Compare the difference between ground truth label and model output

        - Supply ground truth label and associated label quality

    - intermediate parameter(s), ground truth label for AI/ML assisted positioning

  - monitoring based on model input

    - statistics of measurement(s) compared to the statistics associated with training data

  - Assistance signaling from LMF to UE/PRU/gNB for UE/gNB-side model monitoring, or assistance signaling from UE/PRU for network-side model monitoring

Based on all the content listed, we can deduce the data size and the required latency for corresponding data collection of different LCM purposes. We list them in the table 2 as below:

Table 2 New Table to Map LCM Purposes to Data Collection Frameworks for Positioning use case

| LCM purpose | Data collection requirements | Framework | Feasibility analysis |
| --- | --- | --- | --- |
| Offline-training | Data content: 1) Ground truth label (UE/PRU location, label of intermediate parameter(s) for AI/ML assisted positioning, Quality indicator); 2) Model input (CIR, PDP); 3) RS (e.g., PRS/SRS) configuration(s); 4) Time stamp, separate time stamp for measurement and ground truth label; 5) Other information related to scenario, resource configuration. etc; 6) Request message for Quality indicator and/or RS configuration Size: 1), 2) large; 3), 4), 5), 6) medium/small Latency: low | LPP for case 1/2a/2b; LPPa for 3a/3b | LPP and LPPa are the simplest way to transmit the command or data for AI/ML based positioning enhancement. And some extended content such as Ground truth label, time stamp, CIR/PDP may need to be enhanced. Case 1 and case 2a are UE side model; Case 2b and case 3b are LMF side model; Case 3a is gNB side model; So, for training entity: Case 1/2a can be in UE/LMF; Case 2b and case 3b can be in LMF; Case 3a can be in gNB/LMF; |
| Monitoring | Data content: 1) Compare with ground truth label: UE location/ intermediate parameter(s); 2) Without ground truth label: statistics of measurement(s) and training data Size: small Latency: medium/low | LPP for case 1/2a/2b; LPPa for 3a/3b | LPP and LPPa are the simplest way to transmit the command or data for AI/ML based positioning enhancement. And some extended content such as Ground truth label, CIR/PDP may need to be enhanced for different monitoring entity decision. For monitoring entity: Case 1 can be in UE; Case 2a can be in UE/LMF; Case 2b can be in LMF; Case 3a can be in gNB/LMF; Case 3b can be in LMF; |
| Inference | Data content: 1) For direct: CIR/PDP, RSRP/RSRPP/RSTD; 2) For AI/ML assisted: ToA, path phase, RSTD, LOS/NLOS indicator, RSRPP; 3) RS configuration, Other assistance information Size: 1) large; 2), 3) small Latency: medium/low | LPP for case 1/2a/2b; LPPa for 3a/3b | LPP and LPPa are the simplest way to transmit the command or data for AI/ML based positioning enhancement. And some extended content such as CIR/PDP may need to be enhanced for different inference entity selection. Case 1 and case 2a are UE side model; Case 2b and case 3b are LMF side model; Case 3a is gNB side model; |

From the table we can see that, some data are used in the entity which generates the parameters; some needs to be delivered to other entity, e.g. for model training/inference/monitoring; and some needs configured by the network, e.g. RS. Therefore for general, the potential specification impact in RAN2 should also be identified for different LCM purposes:

- For AI/ML model training, request/report of ground truth label and/or other training data (at least measurement) should be considered when the training entity is not the same entity to generate training data, and the time stamp and the assistance information facilitate generating training data such as RS (e.g., PRS/SRS) configuration is also needed;

- For AI/ML model inference, if UE perform the direct AI/ML positioning (case 1), the CIR/PDP generate in the UE can be used directly, no information needs to be exchanged in the air-interface; to perform UE-assisted/LMF-based positioning (case a2/ab), the intermediate parameter (e.g. ToA/AoA/AoD) of new and existing measurement should be transmitted to the LMF side;

- For AI/ML model monitoring, the inferred UE location, intermediate parameter(s), the ground truth label and corresponding measurement can be delivery, for model output based monitoring and model input based monitoring, respectively.

Proposal 6: For data collection of model training for positioning accuracy enhancement, the request and report for ground truth label and corresponding measurement, the configuration of RS and the report of time stamp should be considered.

Proposal 7: For data collection of model inference for positioning accuracy enhancement, the report of intermediate parameter(s) (e.g. ToA/AoA/AoD) based on new or existing measurement for UE-assisted/LMF-based positioning should be considered.

Proposal 8: For data collection of model monitoring for positioning accuracy enhancement, the delivery of the inferred UE location, intermediate parameter(s), the ground truth label and corresponding measurement should be considered.

And for framework choose, since the five positioning cases can involve UE and LMF, or involve gNB and LMF, the data collection of either model training/inference/monitoring can be uniformed to use the LPP/LPPa specification.

Proposal 9: RAN2 take LPP specification for case 1/2a/2b, and take LPPa specification for case 3a/3b, for the data collection of model training, model inference and model monitoring of positioning accuracy enhancement use case.

- 2.2.3	Beam management

For BM, the following use cases are supported based on agreements made in RAN1:

- BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams

- BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams

And both UE-sided model and network sided model has been agreed to be further studied.

Regarding configuration for data collection, RAN1 has made the following agreements:

RAN1#110 Agreement For the data collection for AI/ML model training (if supported), study the following aspects as a starting point for potential necessary specification impact: - Signaling/configuration/measurement/report for data collection, e.g., signaling aspects related to assistance information (if supported), Reference signals - Content/type of the collected data - Other aspect(s) is not precluded

RAN1#111 Agreement Regarding the data collection for AI/ML model training at UE side, study the potential specification impact considering the following additional aspects. - Whether and how to initiate data collection - Configurations, e.g., configuration related to set A and/or Set B, information on association/mapping of Set A and Set B - Assistance information from Network to UE (If supported) - Other aspect(s) is not precluded

RAN1#112bis Agreement Regarding the data collection at UE side for UE-side AI/ML model, study the potential specification impact of UE reporting to network from the following aspect - Supported/preferred configurations of DL RS transmission - Other aspect(s) is not precluded Agreement Regarding the data collection at UE side for UE-side AI/ML model, study the potential specification impact (if any) to initiate/trigger data collection from RAN1 point of view by considering the following options as a starting point - Option 1: data collection initiated/triggered by configuration from NW - Option 2: request from UE for data collection - FFS: details

Based on the agreements in RAN1, it can be seen that the enhancements are related to:

  - The configuration on beam set(s), e.g. set A and/or set B;

  - The specification impacts UE reports supported/preferred configuration of DL RS transmission;

  - The mechanism to initiate/trigger data collection.

Hence, it is proposed that:

Proposal 10: RAN2 to study the potential specification impacts on data collection for BM including:

  - The configuration on beam set(s), e.g. set A and/or set B;

  - The specification impacts UE reports supported/preferred configuration of DL RS transmission;

  - The mechanism to initiate/trigger data collection.

In RAN2#121 meeting, it was agreed that:

Proposal 6 Consider the following existing frameworks as starting points to be considered for data collection: SON & MDT, UE assistance information, RRM measurement reports, CSI reporting framework, LPP Provide location information. FFS whether other frameworks should be discussed. - P1-P8 are loosely endorsed with the understanding that we can also go beyond, e.g. analyse other methods.

Based on the RAN1 progress as well as the agreement made in RAN2, the data collection for BM is summarized in the following table.

Table 3 New Table to Map LCM Purposes to Data Collection Frameworks for BM use case

| LCM purpose | Data collection requirements | Framework | Feasibility analysis |
| --- | --- | --- | --- |
| Offline-Training | Data content: L1-RSRPs and/or the indication of beams (or beam pairs) Size: Medium /Small Latency: Low | Immediate MDT/RRM measurement CSI reporting | For Network-sided model, if SON & MDT or UE assistance information is applied for data collection, some enhancements on how to calculate to the RSRP and how to report the beam ID may be involved. This brings some duplicated procedure as RRM measurement reports and CSI reporting frame work. To simple the specification work, existing framework, i.e. Immediate MDT/RRM measurement report/CSI report framework can be studied as the first step. SON & MDT and UE assistance information procedures should be considered with low priority. For UE-sided model, it can be left to UE implementation. |
| Monitoring | Data content: L1-RSRPs and the Top-N genie-aided beam indices Size: Small Latency: High | CSI reporting | For Network-sided model, considering high latency requirement is needed for model monitoring, SON & MDT, UE assistance information and Immediate MDT/RRM measurement reports may be not suitable for model monitoring. And based on the progress in RAN1, this may have no spec impacts. Hence, current CSI reporting may be reused. For UE-sided model, the performance of monitoring may be reported to the network. |
| Inference | Data content For UE-side AI/ML model: 1-1) For BM-Case1: Beam(s)based on the output of AI/ML model inference; 1-2) For BM-Case2: Beam(s) of N future time instance(s) based on the output of AI/ML model inference; 2) Predicted L1-RSRP(s) corresponding to the DL Tx beam(s) or beam pair(s); 3) Confidence/probability information related to the output of AI/ML model inference (e.g., predicted beams); 4) For BM-Case2: Information about the timestamp corresponding the reported beam(s) For Network sided model: L1-RSRPs of more than 4 beams in one reporting instance Size: Small Latency: High | CSI reporting | For inference procedure, it may have high requirement on latency. Hence, RRC signalling, e.g. SON & MDT, UE assistance information and Immediate MDT/RRM measurement reports may be not suitable. CSI reporting framework can be considered with high priority. |

Based on the summary in Table 3, it can be seen that:

For offline-training, due to the loose requirement on latency, RRC-signalling based solution can be treated as high priority. And since SON & MDT and UE assistance information may require duplicate work on how to calculate the RSRP result. Hence, RRM measurement can be considered as high priority.

But for model monitoring and model inference, due to its high requirement on latency, CSI framework is recommended with high priority. Therefore, it is proposed that:

Proposal 11: For data collection of offline-training for BM, immediate MDT/RRM measurement/CSI reporting can be considered as high priority.

Proposal 12: For data collection of model monitoring and model inference for BM, CSI reporting can be considered as high priority.

  - Applicability and enhancement of existing frameworks

In above section we analyze the data requirement per LCM purpose per use case, and propose that the existing frameworks can be reused for these cases as much as possible. In this section, we intend to discuss whether and how these existing frameworks should be enhanced for AI/ML usage.

Logged MDT

Logged MDT is a non-real time report based on UE recording of the cell/beam measurement results for a long time, e.g. 2 hours. It can be an appropriate mechanism to collect large information in the UE and then send to the network side. Logged MDT is introduced based on the Trace mechanism in LTE R10, so the trace relate index such as traceReference and traceRecordingSessionRef is mandatory configured in the logged MDT configuration. Even the measurement result collected by the logged MDT can be utilized by the gNB node, the results should still be sent to the OAM collection entity of TCE. Moreover, the logged MDT can only be performed in IDLE/INACTIVE mode, and the logged MDT in CONNECTED mode is not support since in LTE.

Observation 2: Logged MDT cannot be performed in CONNECTED mode now, and the trace related ID is mandatory in the configuration and reporting.

Proposal 13: Logged MDT can be used for offline model training based data collection if the conditions as below can be satisfied:

- Support of CONNECTED mode logged MDT;

- Allowed by OAM to use MDT to collect AI related data besides the existing MDT results.

Immediate MDT and L3 measurements

The differences between these two frameworks are:

- Immediate MDT needs UE to best effort report the location information besides the L3 RRM measurement result;

- Immediate MDT also needs to be performed under the trace framework, i.e. the collected data from UE needs to be sent to TCE by the gNB.

Immediate MDT and L3 measurements can be performed in CONNECTED mode which seems more appropriate to real-time or low latency data collection. So these two frameworks maybe used for transmitting the data for offline model training or model monitoring. Legacy L3 RRM measurement report is used for HO decision which carries RSRP/RSRQ/SINR result for event triggered measurement or periodically reported measurement, so if the size of the AI related data transmitted in the air-interface is large, to transmit the data with RRM measurement report is unacceptable.

Proposal 14: Immediate MDT and L3 RRM measurements can be used for offline model training and model monitoring based data collection if satisfying the conditions as below, respectively:

- For L3 RRM measurements, the transmitted data size should be limited;

- For Immediate MDT, allowed by OAM to use MDT to collect AI related data besides the existing MDT results.

L1 measurement (CSI reporting)

L1 measurement result is based on UCI with explicit format. In our understanding, due to the resource limitation, the UCI may not carry large amount of raw data for model training input e.g. Precoding matrix, CIR, PDP. So for offline training which may needs large size data transmission but is not time-sensitive, L1 measurement seems not appropriate, and the L3 measurement report can cover the requirement. But using CSI reporting for ground-truth CSI transmission or for extended beam ID number transmission is not precluded by RAN1, so it is also possible to use this framework for data collection of model training. And to perform model inference or model monitoring, the information may need to be sent to the network side immediately, the L1 measurement can be used.

Proposal 15: L1 measurement can be used for data collection of time-sensitive purpose, e.g. model inference or model monitoring, or can be enhanced by carrying e.g. ground-truth CSI, for model training.

UAI

The UAI message may be sent to the network if it was configured to do so, or upon some criterion for a UE is met based on the network configuration. The message itself can be considered to send anything the network requests. Since the configuration and report method are flexible, it can be used for the data collection of offline model training. But in order to use this framework, enhanced data for AI/ML in the network configuration e.g. other-Config in RRCReconfiguration is needed.

Proposal 16: UAI can be used for model training based data collection with enhanced data for AI/ML by network configuration.

Early measurements

Early measurements can only be performed in IDLE or INACTIVE mode and the result can be reported in CONNECTED mode. The UE should send available indicator to the gNB and wait for the network request. The mechanism in air-interface is similar as logged MDT. But this mechanism does not need OAM configuration. But since the early measurement should be performed in IDLE/INACTIVE mode which does not match the current 3 use cases, this framework can be de-prioritized.

Proposal 17: De-prioritize early measurements for AIML based data collection.

LPP

For the use cases of CSI feedback enhancement and for beam management which are terminated at gNB node at network side, other frameworks involving both UE and gNB can be utilized. But for the case 1/2a/2b which involves UE reporting for positioning accuracy enhancement use case, the gNB can be transparent for the data transmission between UE and LMF, so to extend current LPP specification is a simpler and easier way to transmit the AI related data. Furthermore based on the same reasons, for the case 3a/3b, to extend current LPPa specification could be considered.

Proposal 18: LPP specification can be extended for positioning accuracy enhancement case 1/2a/2b, and LPPa specification can be extended for case 3a/3b, for model training model inference and model monitoring.

Conclusion

According to the analysis in section 2, we propose:

For data collection per use case

Observation 1: To modify the endorsed table by adding 3 additional columns cannot effectively corresponds the RAN1 requirements to the frameworks.

Proposal 1: Introduce new tables involving LCM purposes, data collection requirements and possible frameworks per use case for data collection analysis.

- For CSI feedback enhancement

Proposal 2: For data collection in CSI compression, RAN2 further considers the enhancement of CSI-RS configuration and the assistance information for UE side data collection, and the enhancement of SRS/CSI-RS configuration and the ground-truth CSI for network side data collection.

Proposal 3: For data collection of offline-training as well model monitoring for CSI compression, RRC framework (immediate MDT/RRM measurement/UAI) and L1 framework (CSI reporting) can be considered as high priority.

Proposal 4: For data collection of model inference for CSI compression, L1 framework (CSI reporting) can be considered as high priority.

- For positioning accuracy enhancement

Proposal 5: RAN2 takes cases 1/2a/2b as high priority for positioning accuracy enhancement use case.

Proposal 6: For data collection of model training for positioning accuracy enhancement, the request and report for ground truth label and corresponding measurement, the configuration of RS and the report of time stamp should be considered.

Proposal 7: For data collection of model inference for positioning accuracy enhancement, the report of intermediate parameter(s) (e.g. ToA/AoA/AoD) based on new or existing measurement for UE-assisted/LMF-based positioning should be considered.

Proposal 8: For data collection of model monitoring for positioning accuracy enhancement, the delivery of the inferred UE location, intermediate parameter(s), the ground truth label and corresponding measurement should be considered.

Proposal 9: RAN2 take LPP specification for case 1/2a/2b, and take LPPa specification for case 3a/3b, for the data collection of model training, model inference and model monitoring of positioning accuracy enhancement use case.

- For beam management

Proposal 10: RAN2 to study the potential specification impacts on data collection for BM including:

  - The configuration on beam set(s), e.g. set A and/or set B;

  - The specification impacts UE reports supported/preferred configuration of DL RS transmission;

  - The mechanism to initiate/trigger data collection.

Proposal 11: For data collection of offline-training for BM, immediate MDT/RRM measurement/CSI reporting can be considered as high priority.

Proposal 12: For data collection of model monitoring and model inference for BM, CSI reporting can be considered as high priority.

For existing frameworks use or update

Observation 2: Logged MDT cannot be performed in CONNECTED mode now, and the trace related ID is mandatory in the configuration and reporting.

Proposal 13: Logged MDT can be used for offline model training based data collection if the conditions as below can be satisfied:

- Support of CONNECTED mode logged MDT;

- Allowed by OAM to use MDT to collect AI related data besides the existing MDT results.

Proposal 14: Immediate MDT and L3 RRM measurements can be used for offline model training and model monitoring based data collection if satisfying the conditions as below, respectively:

- For L3 RRM measurements, the transmitted data size should be limited;

- For Immediate MDT, allowed by OAM to use MDT to collect AI related data besides the existing MDT results.

Proposal 15: L1 measurement can be used for data collection of time-sensitive purpose, e.g. model inference or model monitoring, or can be enhanced by carrying e.g. ground-truth CSI, for model training.

Proposal 16: UAI can be used for model training based data collection with enhanced data for AI/ML by network configuration.

Proposal 17: De-prioritize early measurements for AIML based data collection.

Proposal 18: LPP specification can be extended for positioning accuracy enhancement case 1/2a/2b, and LPPa specification can be extended for case 3a/3b, for model training model inference and model monitoring.

Reference

- R2-2306906, LS out on Data Collection Requirements and Assumptions RAN2

- R2-2302286, Summary of [AT121][025]: Progress table of analyzing data collection framework (Apple)	Apple