3GPP TSG-RAN WG2 Meeting #123                                             	R2-2307366

Toulouse, France, August 21-25, 2023

Source:	CATT, Turkcell

Title:	Further discussions on AIML model transfer

Agenda Item:	7.16.2.3

Document for:	Discussion and Decision

Introduction

In previous RAN2 meeting, based on the Email#053 of RAN2#120 meeting [1], RAN2 agreed to aim to at least analyze the feasibility and benefits of model/transfer solutions based on the following solutions:

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

- Solution 2a: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling.

- Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling.

- Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 2b: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 4: Server can transfer/delivery AI/ML model(s) to UE (transparent to 3GPP).

The possible relations between solutions and use cases are given in the table:

Table: relations between solutions and use cases

| Solutions | Applicable use cases |
| --- | --- |
| Solution 1a, 1b | CSI feedback enhancement Beam management Note: No specific considerations for Positioning accuracy enhancement for Solution 1a and 1b. |
| Solution 2a, 2b | CSI feedback enhancement Beam management Note: No specific considerations for Positioning accuracy enhancement for Solution 2a and 2b. |
| Solution 3a, 3b | Positioning accuracy enhancement |
| Solution 4 | CSI feedback enhancement Beam management Positioning accuracy enhancement |

- Note: the solutions use case relation is preliminary (work in progress), and the purpose is to have better understanding on what to further analyse

And then based on the offline#027 of RAN2#121 meeting [2], the expected pros and cons are captured for the listed solutions, the agreeable pros/cons by most companies are achieved and can be considered as starting point for further discussion:

The table can serve as starting point for continued discussion (but contains some parts that seems non consensus, e.g. delta configuration).

In this contribution, we further discuss whether some solutions can be considered as high priority for different use cases.

Discussion

For the solutions summarized from the email discussion, the pros and cons of each solution were summarized [1]. Thus, in this paper, we further discuss the solutions for each use case in the aspects of:

- Terminated entities of UE/gNB/LMF/CN;

- Impact on other WGs.

Since Solution 4 is transparent to 3GPP, we mainly focus on Solution 1a/1b, 2a/2b and 3a/3b in this paper.

For the CSI feedback enhancement and the beam management use cases

At least in current RAN1 discussion and assumption, the use cases of CSI feedback enhancement and Beam management only involve UE and gNB, so these two use cases can be discussed together. We list the possible model transfer solutions of these two use cases in the table below:

Table1: Model transfer solutions of CSI and BM use cases

| Solutions | The sender and receiver of model transfer |
| --- | --- |
| Solution 1a, 1b | gNB to UE, UE to gNB |
| Solution 2a, 2b | CN to UE, UE to CN |

In table 1 Solution 1a/1b, the model is transferred between UE and gNB. For CSI compression sub use case, if model training is performed at gNB with type 1 training collaboration, the model transfer/deliver from gNB to UE is needed (i.e., Solution 1a/1b). If model training is performed at UE with type 1 training collaboration, the model transfer/deliver from UE to gNB is needed. It is not covered in agreed potential solutions as all agreed potential solutions are related to DL direction, not in UL direction. Here, we include it in solution 1a/1b for better understand and discussion. For BM use cases, if model training at gNB, the gNB needs to transfer/deliver the model to UE for UE-sided model.

To compare between the solution 1a and 1b, the pros and cons are captured in the Email discussion [1]. Whether a first-best solution can be decided depends on the further discussion on the pros and cons. Generally speaking, these two use cases can work without any CN participation. Therefore the Solution 1a/1b between UE and gNB can take higher priority for these two cases.

In our point of view, it is suggested RAN2 further discuss Solution 1a first, since it is the solution which can be more controlled and totally decided by RAN2, and has minimum impact on other WGs.

Observation 1: For the model transfer/delivery of CSI feedback and the beam management use cases, Solution 1a has minimum impact on other WGs and can be totally controlled and decided by RAN2.

Proposal 1: For the CSI feedback and the beam management use cases, prioritize the Solution 1a for model transfer/delivery.

In table 1 , the AI model is transferred between UE and CN node for Solution 2a/2b. For CSI compression sub use case, if model training is performed at gNB with type 1 training collaboration, the model transfer/deliver from gNB to CN node and CN node to UE (i.e., Soulution 2a/2b) are needed. If model training is performed at UE with type 1 training collaboration, the model transfer/deliver from UE to CN node and CN node to gNB are needed. Regarding the case “from UE to CN node”, it is not covered in agreed potential solutions as all agreed potential solutions are related to DL direction. Here, we include it in Soulution 2a/2b for better understand and discussion. For BM use cases, if model training at gNB, the gNB needs to transfer/deliver the model to CN node and CN node to UE for UE-sided model. Therefore to use Solution 2a, 2b, the model transfer from gNB to the CN node or from CN node to the gNB is necessary in the flow.

Proposal 2: For Solution 2a/2b, the model transfer/delivery between gNB and the CN node is necessary in the flow.

For the positioning accuracy enhancement use case

For AI/ML based positioning accuracy enhancement, direct AI/ML positioning and AI/ML assisted positioning are selected as two representative sub-use cases in RAN1#111 meeting and 5 detailed positioning cases are identified:

- Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

- Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

- Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

The positioning Case 1/2a/2b is involving UE or involving UE and LMF, therefore at least the LCM purpose of model transfer can be transparent to the gNB entity; and the positioning Case 3a/3b is involving gNB and LMF which has no relationship with UE. So to consider the model transfer/delivery, it is suggested RAN2 firstly focus on the first two cases as other cases have no model transfer/deliver by air interface involved.

Proposal 3: RAN2 takes positioning cases 1/2a as high priority for the model transfer/delivery of the positioning accuracy enhancement use case.

For positioning accuracy enhancement use case, we think the LMF is an appropriate node to perform model training for network side model training and then to start DL model transfer to the UE, or OAM/OTT server can perform the model training and then transmit the model via LMF node to the UE. But this is only applicable for the case 1 and case 2a which performs the model inference in UE side. For gNB side model, it is appropriate to transfer the model from LMF/OAM/OTT server to gNB. And for LMF side model, it is simpler to perform both model training and model inference inside LMF, or to transfer the model for the training node to LMF. For different positioning cases, we list the possible model transfer solutions in the table below (omit the case of training and inference in the same node which have no spec impact):

Table2: Model transfer solutions of Positioning use cases

| Model side | Solutions | The sender and receiver of model transfer |
| --- | --- | --- |
| UE-sided model (Case 1/2a) | Solution 2a, 2b | OAM/TCE to UE, via CN |
|  | Solution 3a, 3b (Preferable) | LMF to UE |
|  | Solution 4 | OAM or OTT server to UE |
| LMF-sided model (Case 2b/3b) | No related | OAM or OTT server to LMF |
| gNB-sided model (Case 3a) | No related | LMF, or OAM, or OTT server to gNB |

Solution 3a/3b can only be used for positioning accuracy enhancement use case, and these two solutions can be applicable to the first 2 positioning cases listed above. Therefore, compared with solution 1a/1b or solution 2a/2b for which the model transfer should be terminated in gNB or CN, it is more convenient to transfer the AI model between UE and LMF directly e.g. using LPP specification.

By using solution 3a/3b, the model can be trained in LMF for network side training, and it can be used in LMF or transferred to the UE directly. But if using other solutions, additional model transfer between LMF and gNB (for solution 1a/1b) or between LMF and other CN node (for solution 2a/2b) is necessary to be introduced, if model training is performed in the network side of LMF.

Based on the analysis above, we propose to prioritize the Solution 3a/3b for model transfer/delivery for positioning accuracy enhancement use case, at least for positioning enhancement case 1/2a, as long as the Solution 3a/3b can work well.

Observation 2: Additional model transfer between LMF and gNB (for solution 1a/1b) or between LMF and other CN node (for solution 2a/2b) is necessary to be introduced, for network side model training in LMF.

Proposal 4: For the positioning accuracy enhancement use case, prioritize the Solution 3a/3b for model transfer/delivery, at least for positioning enhancement case 1/2a.

Conclusion

According to the analysis in section 2, we propose:

For CSI feedback enhancement and beam management

Proposal 1: For the CSI feedback and the beam management use cases, prioritize the Solution 1a for model transfer/delivery.

Observation 1: For the model transfer/delivery of CSI feedback and the beam management use cases, Solution 1a has minimum impact on other WGs and can be totally controlled and decided by RAN2.

Proposal 2: For Solution 2a/2b, the model transfer/delivery between gNB and the CN node is necessary in the flow.

For positioning accuracy enhancement

Proposal 3: RAN2 takes positioning cases 1/2a as high priority for the model transfer/delivery of the positioning accuracy enhancement use case.

Observation 2: Additional model transfer between LMF and gNB (for solution 1a/1b) or between LMF and other CN node (for solution 2a/2b) is necessary to be introduced, for network side model training in LMF.

Proposal 4: For the positioning accuracy enhancement use case, prioritize the Solution 3a/3b for model transfer/delivery, at least for positioning enhancement case 1/2a.

Reference

- R2-2301576, Report of [Post120][053][AIML18] model transfer delivery (Huawei), Huawei

- R2-2302268, Report of Offline 027 model transfer delivery (Huawei), Huawei