3GPP TSG-RAN WG2 Meeting #123		R2-2307367

Toulouse, France, August 21-25, 2023

Source:	CATT, Turkcell

Title:	Considerations on other model control procedures

Agenda Item:	7.16.2.4

Document for:	Discussion and Decision

Introduction

In RAN1#110 meeting [1], RAN1 agreed to an initial procedure of Life Cycle Management (LCM) as follows, to control and manage the whole life of the AI/ML based function enhancement. But the steps and the necessity of some components are not clearly defined yet.

Agreement Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management - Data collection - Note: This also includes associated assistance information, if applicable. - Model training - [Model registration] - Model deployment - Note: Terminology is to be defined. This includes process of compiling a trained AI/ML model and packaging it into an executable format and delivering to a target device. - [Model configuration] - Model inference operation - Model selection, activation, deactivation, switching, and fallback operation - Note: some of them to be refined - Model monitoring - Model update - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training. - Model transfer - UE capability Note: Some aspects in the list may not have specification impact. Note: Aspects with square brackets are tentative and pending terminology definition. Note: More aspects may be added as study progresses.

In the LCM procedure, some steps are tentative and some steps may not have been well defined. Therefore generally speaking, RAN2 could first identify which steps can be discussed in RAN2 from specification impact aspect, and then the detailed content can be discussed based on RAN1 progress made at each meeting.

Since data collection and model transfer/delivery are discussed in separate sections, this contribution aims at some other model control purposes.

- Model training and inference;

- Subsequent action(s) after Model monitoring;

- Model update.

Discussion

  - Model training and model inference

The spec impacts on model training and model inference are mainly related with data collection from RAN2 point of view. In many cases the model training and the model inference mainly involve internal operation of the UE and/or the network, and the protocol/signaling impact may not be as much as some of the other LCM steps.

The terminology of One-sided (AI/ML) model and Two-sided (AI/ML) model are defined to indicate whether a joint inference is performed by both the network and the UE, e.g. for CSI compression, a two-sided model is used. And besides the inference type, whether a joint training is performed by both sides is also discussed for specific use case.

Basically, the model training can be studied based on specific use case. By deciding the model training/inference side(s), main structure and the necessary assistance information exchanged between UE and gNB can be identified for different LCM steps for different sub-use cases.

Observation 1: The deployed side(s) of model training/inference may largely impact the RAN2 specification for different sub-use cases.

Based on the email discussion [2], it can be seen that for different use cases, the mapped entity can be different. Generally, for CSI compression and BM which mainly involve physical layer operation, the mapped entities can be UE and/or network. While for Positioning, it mainly impacts UE, LMF and etc.

Therefore, it is proposed that:

Proposal 1: Detailed RAN2 protocol/signaling impact between UE and network for model training/inference should be studied on the model training/inference side(s) per sub-use case.

  - Subsequent action(s) after Model monitoring

The involved RAN1 agreements in previous RAN1 meetings are as below:

RAN1#110bis Agreement Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations when network needs to be aware of UE AI/ML models FFS: Detailed discussion of model ID with associated information and/or model functionality. FFS: usage of model ID with associated information and/or model functionality based LCM procedure FFS: whether support of model ID FFS: the detailed applicable AI/ML operations Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network FFS: for network sided models FFS: other mechanisms Agreement Study the specification impact to support multiple AI models for the same functionality, at least including the following aspects: - Procedure and assistance signaling for the AI model switching and/or selection FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) Agreement Study AI/ML model monitoring for at least the following purposes: model activation, deactivation, selection, switching, fallback, and update (including re-training). FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

RAN1#111 Agreement For UE-part/UE-side models, study the following mechanisms for LCM procedures: - For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality. - FFS: Whether or how to indicate functionality - For model-ID-based LCM procedure, indication of model selection/activation/deactivation/switching/fallback based on individual model IDs - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID.

RAN1#113 Agreement For functionality/model-ID based LCM, - Once functionalities/models are identified, the same or similar procedures may be used for their activation, deactivation, switching, fallback, and monitoring.

It is RAN2 agreement that a model is identified by a model ID, and RAN1 agreed to support multiple AI models for the same functionality. Therefore, the RAN1 agreements can be concluded as two parts:

- Multiple AI models for the same functionality based on different model IDs is supported, and the model selection and switching may need procedure and assistance signaling transfer;

- Policy management function can be stayed in the UE or in the NW after monitoring. On one hand, the conditions to trigger different actions can be different, and the input parameters for Policy management function e.g. trigger criteria may need to be configured; one the other hand, the output of the Policy management function e.g. model switching/fallback/update may be informed to the other side. In a word, the configuration of monitoring parameters and the monitoring result/triggered action may need to be indicated to the other side.

For example, if model training is performed in the network side, and if the model inference and monitoring are performed in the UE side, the general procedures of network or UE decision can be as the figures below (for functionality based LCM, the similar procedure can be applied):

Figure 1(a) Decision after monitoring by UE                Figure 1(b) Decision after monitoring by Network

Therefore, the procedure for model monitoring and the subsequent action(s) depends on the model monitoring side and in which side to decide the action(s) based on the evaluation of model monitoring.

Observation 2: The model monitoring/decision side(s) may largely impact the RAN2 specification for model monitoring and the subsequent action(s).

Proposal 2: Detailed RAN2 protocol/signaling impact between UE and gNB for model monitoring and the subsequent action(s) e.g. model selection, activation, deactivation, switching, fallback and update should be studied on the model monitoring side.

Furthermore, based on the agreements made in RAN1, it can be seen that indication of activation/deactivation/switching/fallback is based on individual AI/ML functionality or individual model IDs respectively according to the corresponding procedure, i.e. functionality-based LCM or model-ID-based LCM. It is still FFS on whether the same procedure can be used to indicate activation/deactivation/fallback/switching of AI/ML functionality or model and whether the same signaling except the content, i.e. functionality or model based, is used to indicate activation/deactivation/fallback/switching of AI/ML functionality or model.

For CSI compression and BM, the latency may be one sensitive KPI. Hence, MAC CE/DCI based signalling is more applicable to indicate activation/deactivation/switching/fallback for the functionality or model, which is in RAN1 scope. However, for positioning enhancement which does not have high requirement on latency, RRC signaling is more suitable.

Therefore, whether the same procedure/signalling is used to indicate activation/deactivation/fallback/switching of AI/ML functionality or model highly depends on the sub-use case. Therefore it is proposed that:

Proposal 3: For CSI/BM sub-use cases, RAN2 waits for RAN1 progress to decide the siganlling /procedure used to indicate activation/deactivation/switching/fallback for both functionality and model-ID based LCM.

Proposal 4: For positioning sub-use case, RRC signalling is used to indicate activation/deactivation/switching/fallback for both functionality and model.

  - Model update

RAN1#111 meeting made the definition of the model update and model parameter update:

| Terminology | Description |
| --- | --- |
| Model update | Process of updating the model parameters and/or model structure of a model |
| Model parameter update | Process of updating the model parameters of a model |

For the description of above definitions, the procedure of model update and model parameter update is a kind of model re-training or fine-tuning based on the updated data set. Thus, the spec impacts on model update and model parameter update are mainly related with data collection for model training, and the request signalling to trigger the model update or model parameter update.

Proposal 5: From RAN2 point of view, the spec impacts on the model update and model parameter update are mainly related with data collection for model training, and the request signalling to trigger the model update or model parameter update.

Conclusion

According to the analysis in section 2, we propose:

Observation 1: The deployed side(s) of model training/inference may largely impact the RAN2 specification for different sub-use cases.

Proposal 1: Detailed RAN2 protocol/signaling impact between UE and network for model training/inference should be studied on the model training/inference side(s) per sub-use case.

Observation 2: The model monitoring/decision side(s) may largely impact the RAN2 specification for model monitoring and the subsequent action(s).

Proposal 2: Detailed RAN2 protocol/signaling impact between UE and gNB for model monitoring and the subsequent action(s) e.g. model selection, activation, deactivation, switching, fallback and update should be studied on the model monitoring side.

Proposal 3: For CSI/BM sub-use cases, RAN2 waits for RAN1 progress to decide the siganlling /procedure used to indicate activation/deactivation/switching/fallback for both functionality and model-ID based LCM.

Proposal 4: For positioning sub-use case, RRC signalling is used to indicate activation/deactivation/switching/fallback for both functionality and model.

Proposal 5: From RAN2 point of view, the spec impacts on the model update and model parameter update are mainly related with data collection for model training, and the request signalling to trigger the model update or model parameter update.

Reference

- Chair's notes RAN1_110 v20, Chair, RAN1#110, Toulouse, France, August 22nd – 26th, 2022.

- R2-2308286, [Post122][060][AIML] Mapping of functions to physical entities (CMCC)