3GPP TSG-RAN WG2 Meeting #123		         R2-2307382

Toulouse, France, August 21-25, 2023

Agenda Item:	7.4.2.2

Source: 	MediaTek Inc.

Title:  	Open Issues for LTM RRC

Document for:	Discussion and decision

# Introduction

After last RAN2 meeting, we started a post-meeting email discussion [1], covering various open issues for LTM RRC. In this contribution, we provide our views on several LTM RRC open issues that may require further investigation.

# Discussion

## NR-DC support in LTM

### LTM for SCG?

In RAN#100 meeting (June 2023), the WID was revised [2], and one of the changes for LTM (Objective 1) was to “prioritize” MCG.

Note 4: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG, prioritizing MCG

While the main reason of such prioritization is about RAN4 workload, from RAN2 RRC perspective, it is desired to have earlier decision on whether to support LTM for SCG in this Release. Otherwise, we need to keep the structure (procedure and ASN.1) for SCG in RRC running CR (with lots of FFSs) until clear RAN4 decision. Considering RAN4 progress, we suggest that RAN2 decide not to support LTM for SCG in Release 18. Notice that this does not mean SCG must be released upon MCG LTM. It means that LTM is not triggered based L1 measurements on SCG, and not by SCG MAC CE.

Proposal 1: Do not support LTM for SCG (i.e., LTM is not triggered based on L1 measurements on SCG, and not by SCG MAC CE) in Release 18.

### Handling SCG upon MCG LTM execution

If RAN2 decides not to support LTM for SCG, we need to discuss how to handle SCG upon MCG LTM execution, since UE may be in NR-DC mode. We see two options:

- Specify UE behavior upon receiving MCG LTM command, e.g., releasing SCG

- Let network handle SCG. The network may

  - Release SCG before LTM command

  - Release SCG upon LTM execution (using the RRC configuration to be applied)

  - Keep the SCG after MCG LTM (e.g., by providing the same SCG configuration in the candidate configuration)

  - Add or modify the SCG configuration after MCG LTM

In the last two options, LTM is triggered by MCG MAC CE based on L1 measurements on MCG, and we assume that network is allowed to (blindly) configure SCG. This is similar to ‘conditional handover with target SCG’ introduced in Rel-17 (before that, CHO configuration cannot include SCG). RAN2 may discuss the feasibility of having SCG configured in MCG LTM. If too complicated, we may not support it in this Release, and SCG is always released before or upon LTM.

Whether SCG needs to be We believe that network should be responsible for SCG handling upon MCG LTM, and thus no need to specify related new UE behavior.

Proposal 2: RAN2 decides whether SCG can be included in MCG LTM configurations.

Proposal 3: Network is responsible for handling SCG upon MCG LTM execution. Do not specify related new UE behavior.

## Reference, candidate, and complete configurations

### Generation of complete configuration

According to RAN2 agreements, upon LTM cell switch, a complete configuration replaces UE’s current RRC configuration. The complete configuration can be obtained in the following ways:

- LTM candidate is modelled as a delta configuration, and applied on top of a separate reference configuration to generate the complete configuration

- LTM candidate itself is a complete configuration (then reference configuration can be empty)

Unless LTM candidate configuration is complete, RAN2 agreed that candidate is modelled as a delta configuration on top of reference. In our understanding, this implies that the generation of complete configuration is done by considering reference as ‘current’ configuration and applying candidate configuration using normal delta configuration procedure. However, there seems to be different understandings about how the complete configuration is generated. In RRC running CR discussion [3], rapporteur introduces a new subclause (5.3.5.x.5) about handling of fields in reference and candidate configurations.

We now discuss the feasibility of using normal delta configuration, considering different kinds of field presence and different Need codes.

Table 1. Handling fields in reference and candidate configurations

|  | Field present in both reference and candidate | Field present reference, absent in candidate | Field present candidate, absent in reference |
| --- | --- | --- | --- |
| Need M | Adopt the value from candidate | Keep the value in reference | Add the field from candidate |
| Need N | (1) | Keep the value in reference (no action) | Add the field from candidate |
| Need R | Adopt the value from candidate | (2) | Add the field from candidate |
| Need S | Adopt the value from candidate | (3) | Add the field from candidate |

While UE behaviours in most cases are straightforward, there are some cases requiring further clarifications, as shown in the table.

Case (1): Need N field present in both reference and candidate configurations.

- For ToReleaseList in candidate configuration: Release corresponding elements from ‘current’ configuration.

- For ToAddModList in candidate configuration: Add elements not in current configuration and modify elements in current configuration

- For other fields (not add/mod/release): Adopt the value from candidate. This may be different from legacy behaviour, where a Need N field causes a ‘one-time action’ and is not stored. For example, a reestablishRLC (Need N) field in candidate configuration should not trigger RLC reestablishment. Instead, RLC should be re-established upon LTM execution. To make delta configuration work in this case, we may clarify in Table 6.1.2-1 that for Need N fields that do not release, add, or modify elements in the reference, the ‘one-time action’ is taken upon LTM execution. Example text proposal is given below.

| Abbreviation | Meaning |
| --- | --- |
| Need N | No action (one-shot configuration that is not maintained) Used for (configuration) fields that are not stored and whose presence causes a one-time action by the UE. Upon receiving message with the field absent, the UE takes no action. In LTM, for fields in ltm-CandidateConfig that do not release, add, or modify elements in reference configuration, the one-time actions do not take place when generating the complete configuration (by applying candidate configuration on top of reference configuration); UE performs corresponding actions upon LTM execution instead. |

With such operation, there should be no ‘ToReleaseList’ in reference configuration; we can add a NOTE in RRC to clarify this.

Case (2): Need R fields present reference, but absent in candidate configuration

- If candidate is applied on top of reference configuration using normal delta configuration procedure, according to the rules in Clause 6.1.2, TS 38.331, such fields should be removed from the complete configuration.

- This implies that the reference configuration should not include Need R fields, and network must ensure that Need R fields that should present in the complete configurations are always included in candidate configurations. We can add a NOTE to clarify this.

Case (3): Need S fields present reference, but absent in candidate configuration

- Following delta configuration rules, the field description or procedure specifies UE behaviour performed upon receiving a message with the field absent (e.g., default values to be adopted). We believe that this rule can be applied to fields absent in candidate configurations, as long as network ensures proper UE behaviour. We can clarify this with a NOTE.

Based on the above discussion, we suggest that when generating complete configuration from reference and candidate configurations, normal delta configuration procedure is applied, and thus we do not need a specific clause in RRC to describe this procedure. The meaning of Need N field can be amended to clarify when the one-time action should take place. Several NOTEs can be added to clarify expected network behaviors.

We have the following proposals:

Proposal 4: When generating complete configuration from reference and candidate configurations, normal delta configuration procedure is applied. Thus, we do not need a specific clause in RRC to describe this procedure.

Proposal 5: In Table 6.1.2-1, TS 38.331, modify the meaning of Need N field to clarify that in LTM, for fields in ltm-CandidateConfig that do not release, add, or modify elements in reference configuration, the ‘one-time actions’ do not take place when generating the complete configuration (by applying candidate configuration on top of reference configuration). UE performs corresponding actions upon LTM execution instead.

Proposal 6: Consider the following NOTEs in RRC Clause 6.1.2 to clarify expected network behaviors:

- When generating complete configuration from reference and candidate configurations, ‘current’ refers to the reference configuration, and ‘message’ refers to the candidate configuration.

- There should not be ‘ToReleaseList’ in reference configuration.

- For Need R fields: Reference configuration should not include Need R fields, and network includes Need R fields that are expected to present in complete configuration in the candidate configuration.

- For Need S fields: Network must ensure correct UE behaviour when a Need S field is absent in the candidate configuration.

### Necessity of reference configuration

In RAN#100 meeting (June 2023), companies discussed the need of reference configuration. It was mentioned in [3] that the ‘reference plus delta’ signaling design is an optimization and brings potential complexity in the specifications and in the implementation. The contribution [4] suggested that it should be possible to configure candidate that can be applied on top of current RRC configuration even if UE has changed cells (i.e., subsequent LTM).

Our understanding is that RAN2 agreed to the ‘reference plus delta’ signaling design with the mindset that a big portion of RRC configurations is the same among candidate cells. However, this is an optimization not essential for the LTM feature to work, and network may provide candidates as complete configurations for simplicity and to avoid the risk of invalid configuration. Moreover, the signaling overhead reduction brought by reference configuration is questionable: If reference configuration covers more cells, the delta configurations may be bigger since there are fewer common fields that can be carried by reference. In contrast, if reference configuration covers less cells, the delta configurations may be smaller, but network needs to update the reference configuration more frequently.

Observation 1:	LTM can work without reference configuration, and the signaling overhead reduction brought by reference configuration is questionable.

We suggest that candidates are provided as complete configurations by default, and the support of reference configuration (and generation of complete configuration using reference plus delta method) is optional and is indicated by UE capability.

Proposal 7: Candidates are provided as complete configurations by default, and the support of reference configuration is optional and is indicated by UE capability.

## ‘ReconfigurationWithSync’ for LTM

In RRC-based handover, the ReconfigurationWithSync IE indicates UE to perform synchronization towards target cell, and carries necessary information such as new UE ID, SpCell common configurations, SMTC, etc. In LTM, we expect to see an IE with similar purposes in RRC configuration. In current RRC running CR implementation [5], an IE LTM-CellSwtichInfo is introduced, which has a structure like ReconfigurationWithSync. One may argue that we can simply reuse ReconfigurationWithSync in LTM, however, the behavior UE performs upon LTM may still be (slightly) different from that upon RRC-based handover. For example,

- The presence of the ReconfigurationWithSync within an RRCReconfiguration message always leads to random access procedure. However, RACH-less LTM is supported and thus we need to introduce extra if-else structure to the related procedural text.

- In RRC-based handover, UE starts timer T304 with the timer value set to t304 in ReconfigurationWithSync. In LTM, we agreed to have a T304-like timer for failure handling. However, the value range may be different and thus we may not reuse T304 directly. In this case, the new timer (with its conditional presence information) needs to be added to ReconfigurationWithSync.

To avoid introducing complex procedural text and ASN.1 structure, we suggest not reusing legacy reconfiguration with sync procedure and the IE in LTM. A new IE (e.g., LTM-CellSwitchInfo) and corresponding procedural text should be introduced in RRC.

Proposal 8: Do not reuse legacy reconfiguration with sync procedure and the IE in LTM. A new IE (e.g., LTM-CellSwitchInfo) and corresponding procedural text should be introduced in RRC.

As mentioned by RRC running CR rapporteur in [1], there are currently about 125 sentences mentioning “reconfigurationWithSync” in RRC. If the proposal is agreed, we need to check which parts of reconfiguration with sync procedure should be executed in LTM.

Proposal 9: From the reconfiguration with sync procedure, identify which parts should be executed in LTM.

## Handling RadioBearerConfig upon LTM

Since LTM only supports intra-CU scenario, in most cases we can assume that PDCP recovery or reestablishment is not needed upon LTM execution. As RAN2 agreed that RRC configurations are ‘replaced’ with the complete configuration, and thus current radio bearer configurations shall be cleared and then the new configurations are applied. However, the PDCP entity shall be kept unless the corresponding radio bearer is to be released, and thus the state variables and data stored in buffers are maintained. This is different from the full configuration procedures, where DRBs in current configurations are released and the PDCP entities are released.

Proposal 10: Upon LTM execution, the radio bearer configurations are replaced with that in the complete configuration. The PDCP entities are maintained unless network configures to release corresponding radio bearer.

# Conclusion

We have the following observation:

Observation 1:	LTM can work without reference configuration, and the signaling overhead reduction brought by reference configuration is questionable.

It is proposed to discuss and decide on the following proposals:

NR-DC support in LTM

Proposal 1: Do not support LTM for SCG (i.e., LTM is not triggered based on L1 measurements on SCG, and not by SCG MAC CE) in Release 18.

Proposal 2: RAN2 decides whether SCG can be included in MCG LTM configurations.

Proposal 3: Network is responsible for handling SCG upon MCG LTM execution. Do not specify related new UE behavior.

Reference, candidate, and complete configurations

Proposal 4: When generating complete configuration from reference and candidate configurations, normal delta configuration procedure is applied. Thus, we do not need a specific clause in RRC to describe this procedure.

Proposal 5: In Table 6.1.2-1, TS 38.331, modify the meaning of Need N field to clarify that in LTM, for fields in ltm-CandidateConfig that do not release, add, or modify elements in reference configuration, the ‘one-time actions’ do not take place when generating the complete configuration (by applying candidate configuration on top of reference configuration). UE performs corresponding actions upon LTM execution instead.

Proposal 6: Consider the following NOTEs in RRC Clause 6.1.2 to clarify expected network behaviors:

- When generating complete configuration from reference and candidate configurations, ‘current’ refers to the reference configuration, and ‘message’ refers to the candidate configuration.

- There should not be ‘ToReleaseList’ in reference configuration.

- For Need R fields: Reference configuration should not include Need R fields, and network includes Need R fields that are expected to present in complete configuration in the candidate configuration.

- For Need S fields: Network must ensure correct UE behaviour when a Need S field is absent in the candidate configuration.

Proposal 7: Candidates are provided as complete configurations by default, and the support of reference configuration is optional and indicated by UE capability.

‘ReconfigurationWithSync’ for LTM

Proposal 8: Do not reuse legacy reconfiguration with sync procedure (and ASN.1 IE) in LTM. A new IE (e.g., LTM-CellSwitchInfo) and corresponding procedural text should be introduced in RRC.

Proposal 9: From the reconfiguration with sync procedure, identify which parts should be executed in LTM.

Handling RadioBearerConfig upon LTM

Proposal 10: Upon LTM execution, the radio bearer configurations are replaced with that in the complete configuration. The PDCP entities are maintained unless network configures to release corresponding radio bearer.

# Reference

- R2-2308434, [Post122][055][Mob18] Discussion on RRC open issues list for LTM, Ericsson, August 2023

- RP-2303475, Revised WID on Further NR mobility enhancements, RAN#100, June 2023

- RP-231110, Down-scoping of Rel-18 NR Mobility enhancements, Ericsson, RAN#100, June 2023

- RP-231312, Discussion on Rel-18 Further NR Mobility Enhancements WID Revision, MediaTek Inc., RAN#100, June 2023

- R2-2308435, RRC running CR for LTM, Ericsson, August 2023