3GPP TSG-RAN WG2 #123	R2-2307383

Toulouse, France, August 2023

Agenda Item:	7.15.2

Source:	NEC

Title:	Discussion on remaining issue of SL-U

Document for:	Discussion, Decision

Introduction

During the previous two meetings, RAN2 has made the following agreement regarding to FR1 unlicensed spectrum for sidelink:

NR Sidelink operation in FR1 unlicensed spectrum - Progress made in RAN2#121bis-e Agreements on C-LBT failure granularity: 1: SL C-LBT failure is declared per RB-set. Agreements on C-LBT Failure handling/recovery: 1: UE uses the MAC CE to report consistent LBT failure to the gNB. 2: Uu MAC CE indicates RB set(s) where C-LBT failure happens. 3: UE triggers SL RLF for all UC connections when UE has triggered consistent SL LBT failure in all RB sets. 4: RAN2 will wait for more conclusion from RAN1 on the assistance information for COT sharing. Agreements on COT sharing and LCP enhancement: 1: If the resource to be used is within a shared COT, and if PDU not generated before COT arrival, and there is data in buffer satisfying COT requirement, at least enhanced LCP should be allowed. FFS on the condition for UE to use enhanced LCP. FFS on spec impact. 2: If a UE decides to use the resource in a shared COT, and when enhanced LCP is decided to be used, for destination selection step in enhanced LCP, at least further restrict the destinations to be the candidates allowed by the COT (as defined by RAN1). Agreements on CG retransmission timer: 1: Confirm the working assumption: Not to support CG retransmission timer in SL-U. Agreements on SL Best-match decision for per-flow CAPC: 1: For ‘best-match’ issue, UE may determine it based on closest PDB, and capture it in stage-2 spec only. Detailed wording can be discussed in running CR phase. FFS on whether to consider default priority as well. - Progress made in RAN2#122 Agreements on SL C-LBT failure recovery (mode 2, RRC ide/inactive UE): 1: Exclusion of RB set(s) that SL C-LBT failure was detected in candidate resource selection + resource pool (re)selection 2: The UE performs resource pool (re)selection - When SL C-LBT failure was detected for all RB-sets within a selected resource pool or; - Up to UE implementation although the above condition is not met 3a: MAC informs L1 of the RB set information where SL C-LBT failure was detected. 3b: L1 performs the resource exclusion for the RB set that SL C-LBT failure was detected. 3c: RAN2 will send a LS to RAN1 to ask to take it into consideration in their job. 4: It is up to UE implementation to select a resource pool out of resource pools that has at least one RB-set that SL C-LBT failure was not detected. Agreements on SL C-LBT failure recovery (mode 1): 1: Leave it to gNB implementation after UE reporting SL C-LBT failure indication. No spec change. Agreements on SL C-LBT failure recovery (mode 2, RRC connected UE): 1: RAN2 confirms that SL C-LBT failure indication is reported to the gNB also for mode 2, RRC connected UE. Agreements on SL C-LBT failure and S-SSB: 1: Counting LBT failure indication regardless of whether LBT failure was provided because of S-SSB transmission or data transmission when RB set for S-SSB transmission belongs to the selected TX resource pool. Agreements on SL C-LBT failure and PSFCH: 1: Counting LBT failure indication regardless of whether LBT failure was provided because of PSFCH transmission or not when RB set for PSFCH transmission belongs to the selected TX resource pool. FFS when multiple PSFCH occasions are configured. Agreements on SL C-LBT cancellation 1: For mode 1, SL C-LBT is cancelled upon SL C-LBT failure MAC CE transmission. Agreements on SL enhanced LCP 1: Working assumption: For shared COT, CAPC restriction is applicable to enhanced LCP according RAN1 agreement on CAPC requirement. Agreements on MCSt 1: For Qustion-1 from RAN1 (Q1 in R1-2304257), R2 replies that it is feasible to select the resource for a single TB in MAC layer and concatenate across separate resource selection triggers across TBs in a best-effort manner. 2: For Qustion-2 from RAN1 (Q2 in R1-2304257), R2 replies that the approach 3 is not compatible with the current specification and it may bring big specification impacts. 3: For Question-3 from RAN1 (Q3 in R1-2304257), R2 replies that it is feasible to provide a new parameter “number of slots for MCSt” to L1 when triggering resource (re-)selection for MCSt. Agreements on best-matched rule for non-standardized PQI 1: Priority is not considered in best-matched rule. Agreements on multiple PSFCH occasions 1: Working assumption “In case of multiple PSFCH occasion per PSCCH/PSSCH, if HARQ A/N is successfully transmitted in one PSFCH occasion, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback.” is agreed at least for UC. 2: Working assumption “In case of multiple PSFCH occasion per PSCCH/PSSCH, if LBT failure happens in all PSFCH occasions, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the last PSFCH occasion for the SL HARQ feedback.” is agreed at least for UC. Agreements on SL DRX active time 1: Working assumption “Not define shared COT as SL DRX active time” is agreed. If RAN1 introduces additional ID, we can revisit it. Agreements on SL CAPC when CAPC of the default SLRB is not configured 1: UE selects the lowest CAPC priority level (highest CAPC value) among the associated QoS flows.

It can be observed that most of the work has been finished. However, there are still some leftover issues where we will address them in this contribution.

Discussion

LBT failure for SSB/PSFCH transmission

During last meeting, RAN2 has made the following two agreements:

Agreements on SL C-LBT failure and S-SSB: 1: Counting LBT failure indication regardless of whether LBT failure was provided because of S-SSB transmission or data transmission when RB set for S-SSB transmission belongs to the selected TX resource pool. Agreements on SL C-LBT failure and PSFCH: 1: Counting LBT failure indication regardless of whether LBT failure was provided because of PSFCH transmission or not when RB set for PSFCH transmission belongs to the selected TX resource pool. FFS when multiple PSFCH occasions are configured.

In details, since LBT failure is detected as per RB set level, so RB set may include sidelink resources used for PSCCH/PSSCH transmission, which is within the resource pool configuration; as well as sidelink resources used for SSB/PSFCH transmission, which is out of the resource pool configuration. Therefore, if UE detecting LBT failure for one specific RB set, it does not need to distinguish whether it is caused by SSB transmission, PSFCH transmission or PSSCH/PSCCH transmission, as long as the RB set belongs to the selected Tx resource pool, which is shown in the below figure, which is marked as case A.

Figure 1: RB set for SSB/PSFCH belongs to selected resource pool

In that case, UE just counts LBT failure indication within the selected Tx resource pool, which is exact the same as LBT failure happens for PSCCH/PSSCH transmission within the selected resource pool.

However, there are still are cases which have not been covered by the above-mentioned agreements. In that case when LBT failure happens for SSB/PSFCH transmission of which the RB set does not belong to the selected Tx resource pool, but belongs to other candidate Tx resource pool, of which shown in the below figure, marked as case B.

Figure 2: RB set for SSB/PSFCH belongs to candidate resource pool

In this case, when the C-LBT failure happens on the RB set belongs to other candidate resource pools, UE should also count LBT failure indication for that RB set, since UE may perform resource pool selection in the following, where that candidate resource pool may be selected. If all RB sets of that candidate resource pool encounters C-LBT failure, then UE should not select that candidate resource pool. Furthermore, when UE performs resource pool reselection, it should consider RB set belongs to the resource pool encounters C-LBT failure as one additional condition

Counting LBT failure indication of SSB transmission or PSFCH transmission when the RB set belongs to other candidate Tx resource pool

UE should not (re)select the candidate resource pool when all of the RB sets of that resource pool encounter C-LBT failure.

When UE performing resource pool reselection, it should consider RB sets belong to each resource pool encounters C-LBT failure as one additional factor.

In addition, there is also another case, when LBT failure happens for SSB/PSFCH transmission of which the RB set does not belong to any resource pool, of which shown in the below figure, marked as case-C:

Figure 3: RB set for SSB/PSFCH does not belong to any resource pool

In this case, UE does not need to count the LBT failure indication, since the counting result will not affect the resource (pool) (re)selection anymore. However, there are still benefit for network to know such LBT failure as soon as possible, since network may provide reconfiguration for UE to get successful SSB/PSFCH transmission.

When UE is in RRC_CONNECTED, it should report LBT failure information towards network when LBT failure happens on the RB set which does not belong to any resource pool.

Different from case-A and case-B, case-C will only happen when UE perform SSB/PSFCH transmission. However, MAC layer would be agnostic to SSB transmission and corresponding SSB transmission resource, which is totally taken charged by physical layer. So, if MAC layer gets the LBT failure report for the RB set which does not belong to any configured resource pool, then MAC layer would be confused.

MAC layer is agnostic to SSB transmission, if the LBT failure report is for the RB set which does not belong to any configured resource pool due to SSB transmission, MAC layer would be confused.

In order to allow MAC layer to get correct understanding of the case, to avoid MAC layer consider case-B and case-C as error cases, there are two alternatives:

- Alt1: Through a deduction procedure in MAC layer, in details, if the RB set does not belong to the selected resource pool, then MAC layer will consider it as LBT failure due to SSB transmission or PSFCH transmission

- Alt2: Through physical layer indication, in details, when physical layer indicates the LBT failure, it will also indicate the failure is caused by SSB transmission or PSFCH transmission.

RAN2 should consider the following two alternatives to allow MAC layer get correct understanding when RB set encounters LBT failure does not belong to the selected resource pool:

- Alt1: If the RB set does not belong to the selected resource pool, then MAC layer will consider it is due to SSB transmission or PSFCH transmission. Such deduction procedure will cause spec impact in MAC

- Alt2: Physical layer will indicate the LBT failure is due to SSB transmission or PSFCH transmission

If alt-2 is preferred in RAN2, some spec impact would be caused to RAN1, then RAN2 should send the LS[1] towards RAN1 to check if they have any concern.

If Alt2 in proposal 5 is preferred, RAN2 should send LS towards RAN1 to check if they have any concern.

LBT cancellation

During last meeting, RAN2 has discussed how to perform LBT cancellation when the UE is in RRC_CONNECTED state. In details, UE should report the LBT failure with associated RB set towards network.

However, when the UE is in mode 2 RRC_IDLE/INACTIVE state, it seems UE cannot rely on network in this case. In details, some different granularity alternatives can be considered for LBT failure cancellation:

- Per UE granularity: If UE keeps performing LBT, after some time, UE would detect LBT failure in more and more RB sets. Then, at one time being, UE would be lack of RB set to use. So, in order to keep the UE always have resource to perform LBT, one ratio value should be maintained at the UE side, where the ratio value is the division between the amount of RB sets encounters C-LBT failure and the total amount of configured RB sets. Once the ratio value is greater than a configured threshold, the UE should autonomously cancel the C-LBT failure for the corresponding RB sets.

- Per resource pool granularity: As it is agreed in the previous meeting, UE would perform resource pool reselection if C-LBT failure is detected in the current resource pool. It means, UE will not use the current resource pool for certain duration after it performs resource pool reselection. Therefore, there would be no harm for the UE to cancel the C-LBT failure for the corresponding in the current resource pool. Furthermore, if one concerns the duration is not longer enough, a timer can also be configured. So, the overall procedure is when UE detecting LBT failure in the current resource pool 1, it would perform resource pool reselection towards resource pool 2, and once the resource pool reselection procedure is done, UE would trigger the timer to start, once the timer expired, UE would cancel the LBT failure for the corresponding RB set(s) in resource pool1.

- Per RB set granularity: Once UE detect C-LBT failure for the specific RB set, surly UE cannot cancel the C-LBT failure immediately since the condition will not change in such a short duration. So the most ensured way is once C-LBT failure is detected on the certain RB set, UE will trigger the timer to start, and once the timer is expired, UE can cancel the C-LBT failure for the corresponding RB set.

From our view, all the mechanisms for the three different granularities can work properly. So RAN2 should check companies’ preference and decide which granularity can be applied as the C-LBT failure cancellation principle.

The C-LBT failure cancellation from per-UE, per-resource pool or per RB set granularity can all work properly.

RAN2 should decide which granularity (per UE, per resource pool, per RB set) can be applied as the C-LBT failure cancellation principle.

Conclusion and Proposal

In this paper, we have discussed the remaining issue of LBT failure a brunch of observations and proposals are listed below:

Observation 1	MAC layer is agnostic to SSB transmission, if the LBT failure report is for the RB set which does not belong to any configured resource pool due to SSB transmission, MAC layer would be confused.

Observation 2	The C-LBT failure cancellation from per-UE, per-resource pool or per RB set granularity can all work properly.

Proposal 1	Counting LBT failure indication of SSB transmission or PSFCH transmission when the RB set belongs to other candidate Tx resource pool

Proposal 2	UE should not (re)select the candidate resource pool when all of the RB sets of that resource pool encounter C-LBT failure.

Proposal 3	When UE performing resource pool reselection, it should consider RB sets belong to each resource pool encounters C-LBT failure as one additional factor.

Proposal 4	When UE is in RRC_CONNECTED, it should report LBT failure information towards network when LBT failure happens on the RB set which does not belong to any resource pool.

Proposal 5	RAN2 should consider the following two alternatives to allow MAC layer get correct understanding when RB set encounters LBT failure does not belong to the selected resource pool:

Alt1: If the RB set does not belong to the selected resource pool, then MAC layer will consider it is due to SSB transmission or PSFCH transmission. Such deduction procedure will cause spec impact in MAC

Alt2: Physical layer will indicate the LBT failure is due to SSB transmission or PSFCH transmission

Proposal 6	If Alt2 in proposal 5 is preferred, RAN2 should send LS towards RAN1 to check if they have any concern.

Proposal 7	RAN2 should decide which granularity (per UE, per resource pool, per RB set) can be applied as the C-LBT failure cancellation principle.

Reference

[1]R2-2307385	(draft)LS towards RAN1 on LBT failure indication		NEC