3GPP TSG-RAN WG2 Meeting #123	R2-2307388

Toulouse, France, 21st-25th August, 2023

Agenda Item:	7.4.1

Source: 	Fujitsu, CATT, MediaTek

Title:	Discussion on replying to the RAN1 LS on beam application time for LTM

Document for:	Discussion and Decision

- Introduction

In the RAN1#113 meeting, RAN1 sent an LS to RAN2, RAN3 and RAN4 on beam application time, contents of cell switch command, TCI state activation and UE based TA measurement for LTM [1]. In the LS, RAN1 provides their agreements on beam application time and asks RAN2 to provide our feedback on their agreement.

- Beam application time RAN1 has made the following agreement in RAN1#113: Agreement - For the beam application time for Rel-18 LTM, - Beam application time is supported, and starts after the last symbol of the PUCCH or PUSCH carrying the HARQ-ACK for the PDSCH which carries MAC-CE containing cell switch command with the beam indication for the target cell(s) - FFS: reference SCS, i.e. serving cell and/or target cell - At least the following components are further studied to define the beam application time - Whether TCI state activation is received before/together with cell switch command - Legacy values, i.e. and BeamAppTime-r17 - RF retuning time when inter-frequency switch is performed, which is up to RAN4 - Whether the target cell is one of the current serving cells - Cell switching time, which is defined by RAN2 and RAN4, may or may not include the potential components of beam application time above. RAN1 respectfully asks RAN2 and RAN4 to provide their feedback to complete the RAN1 work on beam switching time for LTM. … 2. Actions: To RAN2 group. ACTION: RAN1 respectfully asks RAN2 to take the RAN1 agreements into consideration for their work and kindly asks to provide their feedback on the RAN1 requests described in section A.

This document discusses how RAN2 should reply to RAN1 on beam application time for LTM.

- Discussions

2.1	beam application time and cell switching time

In the agreement on beam application time for LTM, the following is included:

- Cell switching time, which is defined by RAN2 and RAN4, may or may not include the potential components of beam application time above.

This means that RAN1 concerns whether RAN2 and RAN4 have already discussed or decided certain target value for cell switching time which may include beam application time. However, it is unclear what the cell switching time refers to.

From RAN2 perspective, it can be assumed the cell switching time refers to some components of Mobility latency model captured in the running 38.300 CR or LTM supervisor timer.

Observation 1: “Cell switching time” described in the agreement on beam application time may refer to some components of Mobility latency model captured in the running 38.300 CR or LTM supervisor timer.

For the former case, the beam application time seems to be a part of or includes Tprocessing,2 , i.e., “This may include L2/3 reconfiguration, RF retuning, baseband retuning, security update if needed, etc.” Although RAN2 set a tentative value for each component, we think RAN2 will not intend to decide the exact value. Therefore, the value of beam application time should be decided in RAN1 and RAN4.

For the latter case, the beam application time is obviously a part of LTM superviser timer. However, RAN2 only agreed that the usage of LTM superviser timer and has not discussed exact value yet. Actually it would be better that RAN1 and RAN4 decide the value for the beam application time first and inform RAN2 of the value.

Figure X.1-1: Components of Mobility Latency

Proposal 1: Regardless of what the cell switching time refers to, the value of the beam application time should be discussed in RAN1 and RAN4, and the value should be input to RAN2.

2.2	RAN3 related potential impact

Although RAN3 is not requested any action regarding beam application time, the network interface delay for inter-DU case may need to be taken into consideration when RAN1 and RAN4 decide the value for beam processing time. This consideration may be necessary to reduce PDCCH monitoring time until receiving the first dynamic UL grant scheduling at the target cell after beam application completion.

In the RAN2#122 meeting, the following agreement was made:

Þ    Dynamic grant can be used for RACH-less LTM, for the first UL data transmission to the target cell:

- the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch.

- upon cell switch decision, R2 assumes that the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant.

If beam application time is much shorter compared with network interface delay, e.g., beam application time is 5ms and network interface delay is 10ms, the UE will start monitoring PDCCH on the selected beam of target cell 5ms later the inter-DU LTM command, but the UE will not receive the PDCCH scheduling UL grant for at least 5ms.

Observation 2: When RAN1 and RAN4 decide the value of beam application time, the interface delay for inter-DU LTM case may need to be considered so that the UE can reduce PDCCH monitoring time until receiving the first dynamic UL grant scheduling at the target cell after the beam application completion.

We propose to inform RAN1 and RAN4 that interface delay may need to be considered and suggest them that RAN3 should be involved if they find it will be necessary.

Proposal 2: RAN2 to inform RAN1 and RAN4 that interface delay may need to be considered and suggest them that RAN3 should be involved if they find it is necessary.

Proposal 3: RAN2 to prepare a reply LS to RAN1 and RAN4 based on Proposal 1 and Proposal 2.

- Conclusion

Observation 1: “Cell switching time” described in the agreement on beam application time may refer to some components of Mobility latency model captured in the running 38.300 CR or LTM supervisor timer.

Proposal 1: Regardless of what the cell switching time refers to, the value of the beam application time should be discussed in RAN1 and RAN4, and the value should be input to RAN2.

Observation 2: When RAN1 and RAN4 decide the value of beam application time, the interface delay for inter-DU LTM case may need to be considered so that the UE can reduce PDCCH monitoring time until receiving the first dynamic UL grant scheduling at the target cell after the beam application completion.

Proposal 2: RAN2 to inform RAN1 and RAN4 that interface delay may need to be considered and suggest them that RAN3 should be involved if they find it is necessary.

Proposal 3: RAN2 to prepare a reply LS to RAN1 and RAN4 based on Proposal 1 and Proposal 2.

# References

- R1-2306259, "LS on beam application time, contents of cell switch command, TCI state activation and UE based TA measurement for LTM "