3GPP TSG-RAN WG2 Meeting #123	R2-2307393

# Toulouse, FR, Aug 21 - 25, 2023

Source:	CATT

Title:	Discussion on RAT-Dependent integrity

Agenda item:	7.2.3

Document for:	Discussion and Decision

Introduction

In this contribution, we continue to discuss the remaining issues for RAT-dependent positioning integrity.

Discussion

On provision of the AL/TTA:

For GNSS, UE calculates the PL based on integrity assistance data provided by LMF. The assistance data for GNSS includes the error sources, and the IR. And in that case, the AL/TTA is not provided, with the assumption that the AL can be any random value. As for RAT-dependent integrity, same principle like GNSS can be reused. So it is proposed that.

Proposal 1: For UE-based integrity, no need to provide the AL/TTA to UE.

On error causes for RAT-D integrity:

In legacy request/provide assistance procedure, in case the required assistance data cannot be provided by NW, the corresponding error cause value shall be indicated within the provide assistance data message. Similarly, for UE-based positioning, in case UE cannot provide the positioning calculation to LMF, UE will include the corresponding reasons via the error cause to NW. Such indication is helpful, which can help the UE/NW to decide what happened, and can also be served as assistance data to help the UE and NW to decides what assistance is required/provided, or what positioning method can be used. As for the R18 UE-based integrity, we think similar principles also applies, e.g., in case UE cannot calculate the PL based on integrity assistance data, some error causes as in legacy should be provided to LMF, so that LMF can aware the reason why the integrity fails.

Proposal 2: For UE-based integrity, RAN2 to agree to introduce new error cause dedicated for integrity within the LPP Provide Location message and the LPP Provide assistance message.

On signalling of the RAT-D integrity capability:

For LMF-based integrity, it is LMF to calculate the positioning integrity based on TRP related error sources and/or the measurement error sources. However, as discussed in P1/P2, since the measurement error sources and the TRP related error sources are already known to LMF due to implementation, thus from RAN perspective, there is not any additional spec impacts to support the LMF-based integrity except the integrity capability coordination between UE and LMF.

Proposal 3: For UE-based and LMF-based integrity, UE need to provide the integrity capability to LMF per positioning method.

Conclusions

In this contribution, we discussed the solutions for the integrity of RAT dependent positioning techniques. Based on the discussion, we have the following proposals:

On provision of the AL/TTA:

Proposal 1: For UE-based integrity, no need to provide the AL/TTA to UE.

On error causes for RAT-D integrity:

Proposal 2: For UE-based integrity, RAN2 to agree to introduce new error cause dedicated for integrity within the LPP Provide Location message and the LPP Provide assistance message.

On signalling of the RAT-D integrity capability:

Proposal 3: For UE-based and LMF-based integrity, UE need to provide the integrity capability to LMF per positioning method.

References

[1] TR 38.305