3GPP TSG-RAN WG2 Meeting #123	R2-2307396

Toulouse, France, August 21-25, 2023

Agenda item:	7.4.2.1

Source: 	Fujitsu

Title: 	RAN2 aspects of RACH-based early TA acquisition

Document for:	Discussion and decision

1. Introduction

This contribution will discuss RAN2 details of RACH for early TA acquisition.

2. Discussion

Usually, one MAC entity is maintained per cell group. In case of RACH based early TA acquisition of candidate cell, PDCCH order is received from a serving cell to trigger a RACH on candidate cell. From PDCCH order point of view, the RA procedure can be considered as the UE behavior on the serving cell where PDCCH order is received, i.e. the MAC entity which the serving cell belongs to. One MAC entity is used for serving cell(s) and candidate cell(s) when RACH is initiated. If dual-connectivity is configured, i.e. there are MCG MAC entity and SCG MAC entity for the UE, MAC entity associated with received PDCCH order is used for RACH over candidate cell(s).

Proposal 1: One MAC entity is maintained for RACH over serving cell and candidate cell. In case of DC, MAC entity associated with received PDCCH order is used for RACH procedure over candidate cell(s).

According to MAC specification [1], there is only one Random Access procedure ongoing at any point in time in a MAC entity. If Proposal 1 is agreed, i.e. one MAC entity is used for RACH over serving cell(s) and candidate cell(s), prioritization of RA procedure between serving cell and candidate cell should be discussed. For example, if a new RACH procedure over a candidate cell is triggered by gNB and there is an ongoing RACH procedure over serving cell, RAN2 should determine whether the new RACH procedure is initiated, the ongoing RACH procedure continues, or it is up to UE implementation.

Proposal 1bis: If Proposal 1 is agreed, RAN2 to discuss the prioritization of RA procedure on serving cell and RA procedure on candidate cell.

In addition, according to MAC specification [1], if a new Random Access procedure is triggered while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure. We think that it is applicable for RA procedure on the candidate cell. If the UE determines to continue the current RACH procedure, the network can send PDCCH order again to trigger TA acquisition on another candidate cell. So, it is up to UE implementation whether to start with a new RACH procedure if the new RACH procedure for early TA acquisition is triggered while there is ongoing RACH procedure for early TA acquisition.

Proposal 2: It is up to UE implementation whether to start with a new RACH procedure if the new RACH for early TA acquisition is triggered while there is RACH for early TA acquisition ongoing.

Then the next question is how to identify whether the PDCCH order triggers a new RA procedure or not. According to MAC spec [1], if there was an ongoing Random Access procedure that is triggered by a PDCCH order while the UE receives another PDCCH order indicating the same Random Access Preamble, PRACH mask index and uplink carrier, the Random Access procedure is considered as the same Random Access procedure as the ongoing one. It means that Random Access Preamble, PRACH mask index and uplink carrier in PDCCH order are used for the UE to identify whether a new or same RA procedure is triggered. If the legacy mechanism is the baseline, cell information needs to be considered as well since RAN1 agreed that the PDCCH order contains the indication of candidate cell [2]. Alternatively, the UE can determine the RA procedure according to explicit indication about initial transmission or retransmission of PRACH. The explicit indication is agreed by RAN1 [3] to be included in the PDCCH order for the UE determination of the PRACH transmission power when reception of RAR is not configured. If initial transmission of PRACH is indicated, the UE considers that a new RA procedure is triggered. The RA procedure is considered as the same procedure if retransmission of PRACH is indicated in the PDCCH order.

Proposal 2bis: RAN2 to discuss how the UE identifies whether a new or same RA procedure on candidate cell is triggered.

When a Random Access procedure is initiated, the MAC entity shall:

- flush the Msg3 buffer and MSGA buffer and initialize variables (not specific to Random Access type);

- select the carrier for performing Random Access procedure;

- perform the BWP operation;

- select the set of Random Access resources applicable to the current Random Access procedure;

- set Random Access type and perform initialization of variables specific to Random Access type.

In existing RA procedure, the UE selects the signalled carrier if the carrier to use for the Random Access procedure is explicitly signalled. If the carrier to use for the Random Access procedure is not explicitly signalled, the carrier is selected based on the RSRP of the downlink pathloss reference. Considering available L1 measurement result at gNB and the RA procedure is triggered by gNB, the carrier to use for the RA procedure can be indicated in the PDCCH order to trigger the RA, as legacy.

Proposal 3: The carrier to use for the RA procedure can be indicated in PDCCH order to trigger early TA acquisition.

The RA procedure is performed on the active BWP configured with PRACH occasion while the BWP of candidate cell may not be active BWP. So, BWP switching is performed when the RA for early TA acquisition of candidate cell is initiated. The further question is which BWP is selected. BWP information can be included in the PDCCH order to trigger the RA. Or, the first activate BWP of candidate cell is used for the RA procedure initiated for early TA acquisition. Alternatively, the BWP can be selected by the UE among the BWPs configured with PRACH occasion.

Proposal 4: RAN2 to discuss the selection of UL BWP for early TA acquisition of candidate cell.

3. Conclusion

In this contribution, we have some discussions on RAN2 details of RACH for early TA acquisition and the following proposals are made:

Proposal 1: One MAC entity is maintained for RACH over serving cell and candidate cell. In case of DC, MAC entity associated with received PDCCH order is used for RACH procedure over candidate cell(s).

Proposal 1bis: If Proposal 1 is agreed, RAN2 to discuss the prioritization of RA procedure on serving cell and RA procedure on candidate cell.

Proposal 2: It is up to UE implementation whether to start with a new RACH procedure if the new RACH for early TA acquisition is triggered while there is RACH for early TA acquisition ongoing.

Proposal 2bis: RAN2 to discuss how the UE identifies whether a new or same RA procedure on candidate cell is triggered.

Proposal 3: The carrier to use for the RA procedure can be indicated in PDCCH order to trigger early TA acquisition.

Proposal 4: RAN2 to discuss the selection of UL BWP for early TA acquisition of candidate cell.

References

- TS 38.321: ‘NR; Medium Access Control (MAC) protocol specification’, V17.3.0

- R2-2302412, ‘LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM’, RAN1 LS, RAN2#121bis

- R2-2307020, "LS on beam application time, contents of cell switch command, TCI state activation and UE based TA measurement for LTM ", RAN1 LS, RAN2#123