3GPP TSG-RAN WG2 Meeting #123	R2-2307411

# Toulouse, France, 21 - 25 Aug, 2023

Agenda Item:    7.13.3

Source:         Xiaomi

Title:         	Considerations on MDT override enhancement for E-UTRAN

Document for:	Discussion and Decision

Introduction

For the MDT override enhancement in inter-RAT scenario, RAN2 achieved the following agreements in last meeting [1].

1	In NR, considering UE capability, UE reports availability of signalling based logged MDT configuration without checking the RAT information.

2	RAN2 confirms that sigLogMeasConfigAvailable can be re-used for to indicate the availability of the LTE signalling based logged MDT in NR.

In this contribution, we provide our considerations on the details of cross-RAT signaling for the MDT override enhancement in the inter-RAT scenario.

Discussion

In previous meeting, for the signalling based logged MDT override protection mechanism in the inter-RAT scenario where UE configured with the signalling based logged MDT in LTE reselects to a NR cell, RAN2 agreed to adopt a similar mechanism as that for R17 intra-NR case and agreed to reuse R17 NR signalling by the UE to inform gNB whether the signalling based logged MDT is configured even when it is configured by E-UTRA.

However, if the sigLogMeasConfigAvailable-r17 used for signalling based logged MDT in NR is also used to indicate the availability of the signalling based logged MDT configured in LTE, gNB cannot distinguish whether the signalling based MDT configured to UE is from NR or E-UTRA without any other information. Consequently, gNB may initiate the cross-RAT reporting based on the sigLogMeasConfigAvailable-r17, but UE will send an empty report as only the intra-RAT reporting is allowed in existing mechanism (i.e. only the logged measurement entries in VarLogMeasReport which is specific to NR is allowed to be included in the UEInformationResponse message) as quoted below [2]:

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from the VarLogMeasReport starting from the entries logged first, and for each entry of the logMeasInfoList that is included, include all information stored in the corresponding logMeasInfoList entry in VarLogMeasReport;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

4>	if bt-LocationInfo is included in locationInfo of one or more of the additional logged measurement entries in VarLogMeasReport that are not included in the logMeasInfoList within the UEInformationResponse message:

5>	include the logMeasAvailableBT;

4>	if wlan-LocationInfo is included in locationInfo of one or more of the additional logged measurement entries in VarLogMeasReport that are not included in the logMeasInfoList within the UEInformationResponse message:

5>	include the logMeasAvailableWLAN;

From the network perspective, if there is an availability indicator but no report is retrieved from the UE, the gNB may execute the report request multiple times or send a new signaling based logged MDT configuration to cover the current configuration. This may cause extra signaling overhead or result in the loss of the configuration/report.

To avoid the above issue, gNB needs to be aware of the configured signalling based logged MDT in UE is for LTE or NR, and the following options can be considered:

Option1: OAM provides RAT type information for the configured signalling based logged MDT in UE.

In details, the RAT type information can be configured with the signalling based logged MDT configuration and propagated as UE context during the mobility from source node to the target node. And as an alternative, the RAT type information can be sent to the gNB as the response to the gNB request upon reception of the signalling based logged MDT availability indictor or the empty report. As this option has impacts on other WGs such as RAN3 and SA5, if it is agreed, an LS is needed to inform other WGs and ask for the confirmation on the flexibility.

Option2: UE indicates the RAT type along with the existing sigLogMeasConfigAvailable-r17.

Observation: gNB may initiate the cross-RAT reporting without being awareness of the RAT type of signalling based logged MDT configured in UE, but only intra-RAT reporting is allowed currently.

Proposal 1: gNB needs to identify the RAT type of the signalling based logged MDT configured in UE so that to decide whether to retrieve the report.

Proposal 2：To enable gNB to identify the RAT type of the available signalling based logged MDT in UE, the following options can be considered:

- Option1: OAM provides RAT type information for the configured signalling based logged MDT in UE.

- Option2: UE indicates the RAT type along with the existing sigLogMeasConfigAvailable-r17.

Conclusions

During the discussion above, we have the following observations and proposals:

Observation: gNB may initiate the cross-RAT reporting without being awareness of the RAT type of signalling based logged MDT configured in UE, but only intra-RAT reporting is allowed currently.

Proposal 1: gNB needs to identify the RAT type of the signalling based logged MDT configured in UE so that to decide whether to retrieve the report.

Proposal 2：To enable gNB to identify the RAT type of the available signalling based logged MDT in UE, the following options can be considered:

- Option1: OAM provides RAT type information for the configured signalling based logged MDT in UE.

- Option2: UE indicates the RAT type along with the existing sigLogMeasConfigAvailable-r17.

References

- R2-230xxxx Report of 3GPP TSG RAN WG2 meeting #122, Incheon, Korea.

- 3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".