3GPP TSG-RAN WG2 Meeting #123		R2-2307422

Toulouse, France, August 21st–25th 2023

Source:	CATT

Title:	Discussion on Remaining UP issues for MSG1 repetition

Agenda Item:	7.21.3

Document for:	Discussion and Decision

Introduction

In RAN2#122 meeting, preliminary agreements on MSG1 repetition were agreed [1]. In this document, we continue to discuss the UP issues for MSG1 repetition.

Discussion

2.1	SUL/NUL

In RAN2#121bis-e meeting, it was agreed that:

Agreements - RAN2 assumes that MSG1 repetition can be applicable to NUL - RAN2 assumes that MSG1 repetition can be applicable to SUL

Based on the current specification, if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL, the UE selects the SUL carrier for performing Random Access procedure. Otherwise, the UE selects the NUL carrier. If MSG1 repetition is supported to both NUL and SUL, whether to define one separate RSRP threshold for MSG1 repetition needs to be studied.

If we define one new rsrp-ThresholdSSB-SUL for MSG1 repetition (the rsrp-ThresholdSSB-SUL-Msg1Repetition in the following figure), the value is probably lower than the legacy rsrp-ThresholdSSB-SUL. On the one hand, the UEs served by SUL (the UEs in the grey circle) are offloaded to NUL, which definitely aggravates the issue of resource congestion of NUL. On the other hand, since MSG1 repetition is performed on NUL, it indicates that the channel condition is poor. Hence, it is high probability that repetition for PUSCH/PUCCH is needed to avoid incorrect decoding. So this also brings repetition for PUSCH/PUCCH. Otherwise, imbalance for repetition number may happen for MSG1 and PUSCH/PUCCH. This should be avoided for MSG1 repetition.

Figure 1: SUL/NUL selection for MSG1 repetition

Furthermore, the RACH procedure defined for RACH partitioning in Rel-17 is that the UE firstly selects carrier before feature selection. Hence, if one separate RSRP threshold is defined for MSG1 repetition will change the common procedure for RACH partitioning. In order to simplify the specification work, the procedure should not be modified.

Based on the reason analysed above, it is proposed that:

Proposal 1: No separate RSRP threshold for NUL/SUL selection is defined for MSG1 repetition.

2.2	RA-RNTI

In RAN2#121bis-e meeting, it was agreed that:

Agreements - RAN2 waits for further inputs from RAN1 for how to associate RA-RNTI to the PRACH occasion for multiple PRACH transmissions and also for ra-ResponseWindow start point

And in RAN1#112bis-e meeting [2], it was agreed that:

Agreement The starting point of RAR window is after the last symbol of the last valid RO in the RO group corresponding to the multiple PRACH transmissions. Note: Valid RO(s) refers to what is defined in existing specification, i.e., Section 8.1 in TS 38.213. Note: The last valid RO is irrespective of whether the PRACH transmission on the last valid RO in the RO group is dropped or not

Based on the agreements made in RAN1, RAR window is started after the last symbol of the last valid RO in the RO group corresponding to the MSG1 repetition.

For RA-RNTI, it is associated with the PRACH occasion in which the Random Access Preamble is transmitted. Per RAN1 agreements, we think the RA-RNTI can be computed using the last valid RO in the RO group corresponding to the MSG1 repetition which is similar to the mechanism in LTE.

Hence, it is proposed that:

Proposal 2: RA-RNTI is associated with the last valid RO in the RO group corresponding to the MSG1 repetition.

2.3	Association with Msg3 repetition

In Rel-17, Msg3 repetition was agreed. And one new RSRP threshold, i.e. rsrp-ThresholdMsg3 was introduced to determine whether Msg3 repetition can be initiated by the UE. Besides, in order to notify the network that the UE supports Msg3 repetition, PRACH resources for Msg3 repetition can be reserved in SIB. When the RSRP is below rsrp-ThresholdMsg3 and PRACH resources are configured for Msg3 repetition, the UE may select PRACH resources corresponding to Msg3 repetition and initiate corresponding RACH procedure.

Normally, the UE notifies the network that it supports Msg3 repetition when the UE selects the PRACH resource configured for Msg3 repetition. However, in Rel-18, the UE may support both Msg3 and Msg1. And how to associate the Msg1 and Msg3 resource need to be studied.

One solution is that the network configures only the PRACH resources for MSG1 repetition. And when the UE performs MSG1 repetition, it implicitly indicates that the UE also supports Msg3 repetition. But in this solution, there is ambiguity that the network can’t figure out the separate capability/request for MSG1 repetition and Msg3 repetition. And it makes mandatory for the UE to support MSG3 repetition when it supports MSG1 repetition.

On other hand, if separate PRACH resources are configured for each combination, i.e. Msg3 repetition, multiple PRACH transmissions, and Msg3 together multiple PRACH transmissions, this brings PRACH resource fractions. Besides, considering the Msg3 which transmits on PUSCH has higher requirements on channel condition, it is likely the condition of Msg3 repetition has been satisfied when MSG1 repetition is triggered. This means the Msg3 repetition should be applied when MSG1 repetition condition has been satisfied.

Hence, we slightly prefer that UE implicitly indicates it supports/requests Msg3 repetition when it selects PRACH resources for multiple PRACH transmissions.

Proposal 3: The UE implicitly indicates it supports/requests Msg3 repetition when it selects PRACH resources for multiple PRACH transmissions in Rel-18.

Another issue is whether repetition number for Msg3 is associated with the repetition number for PRACH. For Msg3 repetition, the repetition is signalled in RAR UL grant, which has been specified in Rel-17. And multiple PRACH transmissions number can be determined by UE based on RSRP. It is not flexible to associate repetition number for Msg3 to the repetition number for PRACH.

Considering analysis above, it is proposed that:

Proposal 4: Msg3 repetition number is not associated with the repetition number of MSG1, i.e., Msg3 repetition number is indicated in RAR like Rel-17 when both MSG1 repetition and Msg3 repetition are triggered.

Conclusion

In this document, we analyse issues for PRACH coverage enhancement, and we find the observations and proposls as following:

Proposal 1: No separate RSRP threshold for NUL/SUL selection is defined for MSG1 repetition.

Proposal 2: RA-RNTI is associated with the last valid RO in the RO group corresponding to the MSG1 repetition.

Proposal 3: The UE implicitly indicates it supports/requests Msg3 repetition when it selects PRACH resources for multiple PRACH transmissions.

Proposal 4: Msg3 repetition number is not associated with the repetition number of MSG1, i.e., Msg3 repetition number is indicated in RAR like Rel-17 when both MSG1 repetition and Msg3 repetition are triggered.

Reference

- RAN2#121bis-e meeting report;

- RAN2#122 meeting report;