3GPP TSG-RAN WG2 Meeting #123		R2-2307429

Toulouse, France, August 21-25, 2023	was R2-2305334

Source:	vivo

Title:	RAN2-related issues about bandwidth aggregation

Agenda Item:	7.2.5

Document for:	Discussion and Decision

- Introduction

The following objectives are captured in the WID[1] for bandwidth aggregation.

- Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference). - Specify RRM requirements with measurement gaps in connected mode, and in inactive mode, including PRS measurement period/reporting [RAN4].

The specification of bandwidth aggregation is led by RAN1 and some issues need RAN2 to further discuss. This contribution provides an initial analysis of the issues about PRS and SRS bandwidth aggregation from RAN2’s perspective.

- Discussion

- 2.1	PRS bandwidth aggregation

At the last RAN1 meetings, the following agreements were made related to the procedure of PRS bandwidth aggregation[2].

Agreement For PRS bandwidth aggregation across PFLs, support enhancement of PRS configuration to inform UE by LMF (or inform LMF by NG-RAN) PRS resources from which two or three PFLs are linked. Agreement Support joint measurement and report for the PRS resources aggregated across the PFLs for DL-TDOA and multi-RTT positioning methods - In a measurement report element, single RSTD or single UE Rx-Tx time difference is reported for the PRS resources across aggregated PFLs Agreement For PRS bandwidth aggregation, with regards to the signaling in the location information request message, introduce the following: - A request to indicate UE which two or three PFLs to be used for performing joint measurement - A new ReportingGranularityfactor smaller than 0 which can be applicable at least when the LMF requests aggregated measurements Agreement For PRS bandwidth aggregation across PFLs, in a measurement report element, support - Single RSRP or single RSRPP - FFS: the single RSRP/RSRPP is based on aggregated PRS resources across aggregated PFLs - The aggregated reference RSTD - The used PRS resource set IDs for the aggregated measurement which are shared for RSRP/RSRPP and/or timing measurement results Agreement Introduce new UE capability(-ies) to support PRS bandwidth aggregation measurement

Based on the above agreements, the general procedures of PRS bandwidth aggregation are:

- The LMF may obtain the TRP information and be aware of PRS bandwidth aggregation.

- The target UE may indicate the capability of supporting PRS bandwidth aggregation measurement.

- The LMF may inform UE that the PRS in two or three different PFLs are aggregated in assistance data.

- The LMF may indicate UE to perform joint measurement across aggregated PFLs in request location information.

- The target UE may indicate whether/which measurement is aggregated in provide location information.

RAN2 may capture the procedure in the stage 2 specification. However, in the table of high-layer parameters, the explicit format of the indications are not still unstable[3]. RAN2 should wait for RAN1's further progress on the high-layer parameters before the stage 3 specification.

Proposal 1: Capture the overall procedures of PRS bandwidth aggregation in the stage 2 specification:

- The LMF may obtain the TRP information and be aware of PRS bandwidth aggregation.

- The target UE may indicate the capability of supporting PRS bandwidth aggregation measurement.

- The LMF may inform UE that the PRS in different PFLs are aggregated in assistance data.

- The LMF may indicate UE to perform joint measurement across aggregated PFLs in request location information.

- The target UE may indicate whether/which measurement is aggregated in the report.

Besides, RAN1 identified the solution of on-demand PRS for PRS bandwidth aggregation is up to RAN2 and RAN3.

Conclusion (RAN1#112bis-e) The details for on-demand PRS on PRS bandwidth aggregation are up to RAN2 and RAN3. Agreement For PRS bandwidth aggregation across PFLs, support - Option 2: Per TRP basis and per PRS resource set basis. - For each TRP, support new signaling to indicate which PRS resource sets across PFLs are linked. It is assumed that the PRS resources across the linked PRS resource sets are linked if the conditions are satisfied. For the non-linked PRS resource sets, no aggregation is assumed even if the conditions are satisfied.

For UE-initiated On-Demand PRS, the LMF may configure the UE with pre-defined PRS configurations via LPP Provide Assistance Data message. The configurations include a list of available On-Demand-DL-PRS-Configuration and each is associated with an ID, the On-Demand-DL-PRS-Configuration is per frequency layer.

-- ASN1START NR-On-Demand-DL-PRS-Configurations-r17 ::= SEQUENCE { on-demand-dl-prs-configuration-list-r17 SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF On-Demand-DL-PRS-Configuration-r17, ... } On-Demand-DL-PRS-Configuration-r17 ::= SEQUENCE { dl-prs-configuration-id-r17 DL-PRS-Configuration-ID-r17, nr-DL-PRS-PositioningFrequencyLayer-r17 NR-DL-PRS-PositioningFrequencyLayer-r16, nr-DL-PRS-Info-r17 NR-DL-PRS-Info-r16, ... } DL-PRS-Configuration-ID-r17 ::= SEQUENCE { nr-dl-prs-configuration-id-r17 INTEGER (1..maxOD-DL-PRS-Configs-r17), ... } -- ASN1STOP

When UE would initiate On-Demand PRS, the UE sends an On-Demand PRS request to the LMF via LPP Request Assistance Data message. The On-Demand PRS request can be a request for a pre-defined PRS configuration indicated with pre-defined PRS configuration ID(s) or explicit parameter for PRS configuration and may be a request for PRS transmission or change to the PRS transmission characteristics for positioning measurements.

-- ASN1START NR-On-Demand-DL-PRS-Request-r17 ::= SEQUENCE { dl-prs-StartTime-and-Duration-r17 DL-PRS-StartTime-and-Duration-r17 OPTIONAL, nr-on-demand-DL-PRS-Information-r17 NR-On-Demand-DL-PRS-Information-r17 OPTIONAL, dl-prs-configuration-id-PrefList-r17 SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF DL-PRS-Configuration-ID-r17 OPTIONAL, ... } DL-PRS-StartTime-and-Duration-r17 ::= SEQUENCE { dl-prs-start-time-r17 INTEGER (1..1024) OPTIONAL, dl-prs-duration-r17 SEQUENCE { seconds-r17 INTEGER (0..59) OPTIONAL, minutes-r17 INTEGER (0..59) OPTIONAL, hours-r17 INTEGER (0..23) OPTIONAL, ... } OPTIONAL, ... } -- ASN1STOP

To support the on-demand PRS request for PRS bandwidth aggregation, the following solutions can be considered:

Solution 1：Introducing multiple PFLs in the On-Demand-DL-PRS-Configuration to indicate that multiple PFLs associated with the dl-prs-configuration-id can be aggregated.

For this solution, the UE can request PRS aggregation with the dl-prs-configuration-id explicitly.

As some PRS resource parameters (e.g., Periodicity, Bandwidth) across the aggregated PFLs may be different, multiple values of these parameters for different PFLs should be included in one single On-Demand-DL-PRS-Configuration.

Solution 2: Indicating in the On-Demand-DL-PRS-Configuration that the PFL associated with this dl-prs-configuration-id can be aggregated with other PFLs.

For this solution, the UE may send the PRS aggregation request implicitly with a dl-prs-configuration-id and a bandwidth value greater than the associated PFL bandwidth. Or UE may request lists of PFL combinations in the NR-On-Demand-DL-PRS-Request to indicate the requested PFLs to be aggregated explicitly.

Note that the configuration and request of on-demand PRS aggregation may have more combinations. Therefore, we propose:

Proposal 2: To support the on-demand PRS configuration for PRS bandwidth aggregation, the following solutions can be considered:

- Solution 1: Introducing multiple PFLs in the On-Demand-DL-PRS-Configuration to indicate that multiple PFLs associated with the dl-prs-configuration-id can be aggregated.

- Solution 2: Introducing an indication in the On-Demand-DL-PRS-Configuration to indicate that the PFL associated with the dl-prs-configuration-id can be aggregated with other PFLs.

Proposal 3: To support the on-demand PRS request for PRS bandwidth aggregation, the following alternatives can be considered:

- Alt 1: UE may request PRS aggregation using explicit dl-prs-configuration-id.

- Alt 2: Introducing one or multiple lists of PFL combinations in the NR-On-Demand-DL-PRS-Request to indicate the requested PFLs to be aggregated.

- Alt 3: UE may request a bandwidth value that exceeds the PFL bandwidth to request PRS aggregation implicitly.

- 2.2	SRS bandwidth aggregation

At the last RAN1 meetings, the following agreements were made related to the procedure of SRS bandwidth aggregation[2].

Agreement At least from UE capability perspective, the UE support of positioning SRS bandwidth aggregation in RRC_CONNECTED state is decoupled from the UE support of communication CA. Agreement For SRS bandwidth aggregation across two or three carriers, support enhancement of SRS configuration to indicate the SRS resources from which two or three carriers are linked Agreement For SRS bandwidth aggregation across two or three carriers, support - Option 2: Per SRS resource set basis. - Support new signaling to indicate which SRS resource sets across carriers are linked. - It is assumed that the SRS resources across the linked SRS resource sets are linked if the conditions are satisfied. For the non-linked SRS resource sets, no aggregation is assumed even if the conditions are satisfied. Agreement With regard to support of aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state, at least the existing Rel-17 DCI framework (i.e. use multiple DCIs schedule SRSs in multiple carriers) can be reused Agreement Support joint measurement and report for the SRS resources across the aggregated carriers for UL-TDOA and Multi-RTT positioning methods - Single UL RTOA or gNB Rx-Tx time difference is reported for the SRS resources across aggregated carriers - FFS: RSRP or RSRPP - FFS: SRS carrier aggregation indication is reported along with the measurement results to indicate whether/which carriers are aggregated for the joint SRS measurement - Support LMF to request gNB for the UL positioning measurement from aggregated SRS resources across multiple CCs Agreement For SRS bandwidth aggregation across carriers, support - Single RSRP or RSRPP is reported - FFS: the single RSRP/RSRPP is based on aggregated SRS resources across aggregated carriers The used SRS resource IDs for the aggregated measurement are shared for RSRP/RSRPP and/or timing measurement results

Based on the above agreements, the general procedures of SRS bandwidth aggregation are:

- The target UE may indicate the capability of supporting SRS bandwidth aggregation measurement.

- The LMF may request the serving gNB to configure SRS bandwidth aggregation for the target UE.

- The gNB configures the target device with the SRS resource sets with SRS bandwidth aggregation indication.

- The serving gNB provides the SRS configuration to the LMF with SRS bandwidth aggregation indication.

- In the case of semi-persistent, the LMF may request activation of UE SRS transmission to the serving gNB with bandwidth aggregation indication. The gNB then activates the SRS transmission.

- The LMF provides the SRS configuration to the selected gNBs with SRS bandwidth aggregation indication.

- Each gNB reports the SRS measurements to the LMF and indicates whether/which carriers are aggregated for the joint SRS measurement.

RAN2 may capture the procedure in the stage 2 specification.

Similar to the status of PRS bandwidth aggregation, RAN2 should wait for RAN1 further progress on the high-layer parameters before the stage 3 specification.

Proposal 4: Capture the overall procedures of SRS bandwidth aggregation in the stage 2 specification:

- The target UE may indicate the capability of supporting SRS bandwidth aggregation measurement.

- The LMF may request the serving gNB to configure SRS bandwidth aggregation for the target UE.

- The gNB configures the target device with the SRS resource sets with SRS bandwidth aggregation indication and provides the SRS configuration to the LMF.

- In the case of semi-persistent, the LMF may request activation of UE SRS transmission to the serving gNB with bandwidth aggregation indication. The gNB then activates the SRS transmission.

- The LMF provides the SRS configuration to the selected gNBs with SRS bandwidth aggregation indication in the measurement request.

- Each gNB reports the SRS measurements to the LMF and indicates whether/which carriers are aggregated for the joint SRS measurement.

With regard to SP-SRS, RAN1 made the following working assumption and would like RAN2 to check the feasibility. [4]

Working assumption For semi-persistent positioning SRS for bandwidth aggregation, a single MAC CE can activate or deactivate: - SRS resource set(s) in one or two or three of three aggregated carriers - SRS resource set(s) in one or two of two aggregated carriers. Note: the single spatial relation is indicated by the MAC CE for each of two or three aggregated SRS resources. Send an LS to RAN2 to confirm the feasibility.

Figure 1 shows the format of SP Positioning SRS Activation/Deactivation MAC CE. It can be observed that only one reserved bit is available. It’s not feasible to reuse the reserved bit to indicate the activation/deactivation of one or two or three of three aggregated carriers.

Figure 1: SP Positioning SRS Activation/Deactivation MAC CE

To avoid introducing a new MAC CE, a compromise way is to activate/deactivate the aggregated carriers simultaneously.

Proposal 5: Reply LS to RAN1 on SP-SRS bandwidth aggregation:

- Inform RAN1 that the current SP Positioning SRS Activation/Deactivation MAC CE does not support activating/deactivating SRS resource set(s) in one or two or three of three aggregated carriers as only one reserved bit is available.

- To avoid introducing a new MAC CE, ask RAN1 whether it is possible to only support activating/deactivating the aggregated carriers simultaneously.

- 2.3	LS to RAN3

As the above procedures may have RAN3 impact while RAN3 is not involved in the WID, RAN2 may LS to RAN3 to trigger the discussion when RAN2 reaches the consensus on the overall procedure.

Proposal 6: LS to RAN3 on the overall procedures of SRS and PRS bandwidth aggregation, when available.

- Conclusion

PRS bandwidth aggregation

Proposal 1: Capture the overall procedures of PRS bandwidth aggregation in the stage 2 specification:

- The LMF may obtain the TRP information and be aware of PRS bandwidth aggregation.

- The target UE may indicate the capability of supporting PRS bandwidth aggregation measurement.

- The LMF may inform UE that the PRS in different PFLs are aggregated in assistance data.

- The LMF may indicate UE to perform joint measurement across aggregated PFLs in request location information.

- The target UE may indicate whether/which measurement is aggregated in the report.

Proposal 2: To support the on-demand PRS configuration for PRS bandwidth aggregation, the following solutions can be considered:

- Solution 1: Introducing multiple PFLs in the On-Demand-DL-PRS-Configuration to indicate that multiple PFLs associated with the dl-prs-configuration-id can be aggregated.

- Solution 2: Introducing an indication in the On-Demand-DL-PRS-Configuration to indicate that the PFL associated with the dl-prs-configuration-id can be aggregated with other PFLs.

Proposal 3: To support the on-demand PRS request for PRS bandwidth aggregation, the following alternatives can be considered:

- Alt 1: UE may request PRS aggregation using explicit dl-prs-configuration-id.

- Alt 2: Introducing one or multiple lists of PFL combinations in the NR-On-Demand-DL-PRS-Request to indicate the requested PFLs to be aggregated.

- Alt 3: UE may request a bandwidth value that exceeds the PFL bandwidth to request PRS aggregation implicitly.

SRS bandwidth aggregation

Proposal 4: Capture the overall procedures of SRS bandwidth aggregation in the stage 2 specification:

- The target UE may indicate the capability of supporting SRS bandwidth aggregation measurement.

- The LMF may request the serving gNB to configure SRS bandwidth aggregation for the target UE.

- The gNB configures the target device with the SRS resource sets with SRS bandwidth aggregation indication and provides the SRS configuration to the LMF.

- In the case of semi-persistent, the LMF may request activation of UE SRS transmission to the serving gNB with bandwidth aggregation indication. The gNB then activates the SRS transmission.

- The LMF provides the SRS configuration to the selected gNBs with SRS bandwidth aggregation indication in the measurement request.

- Each gNB reports the SRS measurements to the LMF and indicates whether/which carriers are aggregated for the joint SRS measurement.

Proposal 5: Reply LS to RAN1 on SP-SRS bandwidth aggregation:

- Inform RAN1 that the current SP Positioning SRS Activation/Deactivation MAC CE does not support activating/deactivating SRS resource set(s) in one or two or three of three aggregated carriers as only one reserved bit is available.

- To avoid introducing a new MAC CE, ask RAN1 whether it is possible to only support activating/deactivating the aggregated carriers simultaneously.

LS to RAN3

Proposal 6: LS to RAN3 on the overall procedures of SRS and PRS bandwidth aggregation, when available.

- Reference

- RP-223549		New WID on Expanded and Improved NR Positioning

- R1-2306238	RAN1 agreements for Rel-18 WI on Expanded and Improved NR Positioning

- R1-2306237	R18 Pos Higher Layer Params

- R2-2307010	LS to RAN2 on SRS bandwidth aggregation for positioning (R1-2306214; contact: ZTE)