3GPP TSG-RAN WG2 Meeting #123	R2-2307430

Toulouse, France, August 21st-25th, 2023

Source: 	vivo

Title:  	Remaining issues on SPR

Agenda Item:	7.13.4

Document for:	Discussion and Decision

Introduction

In the last meeting, the following agreements were achieved regarding SON for successful PSCell addition/change report [1].

=> intra-NR SHR and Inter-RAT SHR from LTE to NR will be deprioritized in RAN2 for R18. => SPR except the critical issues will not be further enhanced from this meeting until the end of R18. => Send LS RAN3 the above conclusion is acceptable for RAN3 (Huawei# 579).

Agreements:

SPR

1	 For values of triggering conditions of SPR, Percentage based threshold variables for SHR (T310/T312/T304) can be reused for SPR is applied.

In addition, an LS [2] to RAN2 on inter-RAT SHR and SPR is raised after RAN3#120, there are several questions in the LS that need RAN2’s feedback.

In this contribution, we will first discuss the remaining issues unresolved on SPR and give our proposals, then analyze and give our answers to the question in the LS.

Discussion

Remaining issues on SPR

Reviewing the previous agreements and running CR regarding SPR, there are some remaining issues on the content of and the configuration of SPR, mainly including the following two topics.

- Issue 1: What other information can be included in SPR

- Issue 2: When to release the SPR configuration

Next, the issues above will be resolved one by one in this section.

- Issue 1: What other information can be included in SPR

In order to enable the network to optimize the configuration of PSCell addition/change, the node that retrieves the SPR should forward the corresponding information to the network that optimizes the configuration. Regarding forwarding of SPR, in RAN3#119bis-e [3], RAN3 agreed that the “new node” should send the SPR to the “old node” and the “old node” is responsible for forwarding to the respective node(s) which should perform the SPR optimization. And CGI of the PCell which sent the SPR configuration is to assist in the forwarding of SPR from RAN3 perspective. The agreements from RAN3 are shown as follows. To align with RAN3 agreements, RAN2 to agree that CGI of the PCell which sent the SPR configuration should be included in SPR.

Furthermore, upon “old node” receiving the SPR, the “old node” should forward the information in SPR to the respective node(s) which should perform the SPR optimization. If the old node receives the SPR long after it sends the PSCell addition/change configuration was sent, this configuration may already be released when it receives the SPR, so the “old node” does not know whether or which network node to forward the SPR. For PSCell addition and MN-initiated PSCell change, MN is the network node that needs SPR and need not forward the SPR to other nodes; for SN-initiated PSCell change, MN should forward the received SPR to SN. To assist old MN node to decide whether or which network node to forward the SPR, an indication is needed to indicate the type of PSCell addition/change (i.e., PSCell addition, MN-initiated PSCell addition/change or SN-initiated PSCell change). This also aligns with the work assumption in RAN3 [3]. UE can know the type of PSCell addition/change implicitly. For CPAC, UE can know whether the PSCell change procedure is initiated by MN or SN according to the signaling structure of configuration for the execution event; for ordinary PSCell change, since RAN2 agreed that different signaling structures are used for MN-initiated PSCell change and SN-initiated PSCell change, i.e., for the MN-initiated PSCell Change/Addition, MN sends the SPR configuration directly to the UE; for the SN-initiated PSCell Change, the source-SN sends the SPR configuration within the container through MN.

Agreements: In case the SPR is retrieved in a “new node” (different from the node that sent the SPR configuration to the UE i.e., “old MN”), the SPR is always sent from the “new node” to the “old MN” which then forwards to the respective node(s) which should perform the SPR optimization. Agreements: To assist in the forwarding of SPR, UE may include the following in SPR - CGI of the PCell which sent the SPR configuration (presence of this IE is to be discussed) - WA: Indication whether the PSCell change was MN-initiated or SN-initiated (RAN3 should discuss how the UE knows whether the PSCell change as MN-initiated or SN-initiated and will check with RAN2 on the mechanism)

Proposal 1: The following information should be included in the SPR:

- CGI of the PCell which sent the SPR configuration.

- An indication to indicate the type of PSCell addition/change, i.e., PSCell addition, MN-initiated change or SN-initiated PSCell change.

- Issue 2: When to release the SPR configuration

According to the editor’s note in the running CR for Rel-18 SON MRO, when to release the SPR configuration is needed to be specified. According to the RRC specification, UE will release SHR configuration (i.e., successHO-Config) when initiating RRC connection re-establishment, initiating RRC connection resume or SHR is generated. Take SHR as a baseline, when initiating RRC connection re-establishment, initiating RRC connection resume, SPR is generated or SCG failure occurs due to PSCell addition/change, UE shall release the SPR configuration. Furthermore, we think UE should release all the SPR configurations including configured directly by MN and configured by MN through the container since the configuration of the PSCell addition/change is released when the one of previously mentioned events occurs.

Proposal 2: If one of the following events occurs, UE shall release all the SPR configurations:

- Initiate RRC connection re-establishment;

- Initiate RRC connection resume;

- SPR is generated;

- SCG failure due to PSCell addition/change.

Answer to RAN3’s question in R2-2307030

The content of this LS on inter-RAT SHR and SPR [2] is excerpted below

1. Overall description: RAN3 discussed how to retrieve mobility information related to a UE in the nodes involved in the mobility procedure when Successful Handover Report (SHR) and Successful PSCell Change Report (SPR) related optimizations are to be performed in the network nodes involved in the mobility procedure. One solution discussed was whether UE can report the source C-RNTI and time since receiving HO/PSCell change/addition command and retrieving SHR/SPR to assist in the network node receiving the SHR/SPR identifying information related to a UE (e.g. UE related configuration of interest, mobility strategies). But since the possibility to retrieve information related to a specific UE depends on gNB's implementation. e.g. whether it can store the UE related configuration of interest during the wanted time (up to 48 hours) after a successful HO or PSCell change/addition, RAN3 discussed an alternative solution to enable the retrieval of UE related information without requiring this to be stored on a per UE basis. RAN3 is therefore studying whether another solution can be defined for newly defined scenarios since Rel-18 as well as intra-RAT SHR, whenever the configuration used by the UE needs to be retrieved in order to perform SON optimizations. One potential solution RAN3 discussed is as follows: - The node involved in mobility procedure (via implementation) create references to a configuration or a set of parameters used by one or a group of UEs and can send this “Configuration Information” to the UE in order to assist in the analysis of SHR or SPR or other SON reports (if needed). - this “Configuration Information” can be optionally sent to the UE in dedicated signaling (e.g., together with the SHR/SPR configuration or in any other RRCReconfiguration). If received, UE should then store this “Configuration Information” together with the SON report(s) and UE should report it back to the gNB along with the SON report(s) (e.g., SHR/SPR). - How to encode this “Configuration Information” is up to RAN2, taking into account RAN3 guidance related to the size of this information. One example to do this would be to encode this as an OCTET STRING (e.g., 32 bits) as is done for Mobility Information in XnAP (i.e., TS 38.423). - This solution shall cover all existing mobility scenarios, including dual-connectivity scenarios. RAN3 therefore have the following questions to RAN2: Q1: Whether RAN2 sees any issues in defining a solution for “Configuration Information” as described above? Q2: For SHR/SPR, is there any issue to include this “Configuration Information” in the RRC Reconfiguration message with sync containing Handover Command or PSCell change command? Q3: In cases when this “Configuration Information” is not configured by the network to the UE, RAN3 discussed whether UE can include the source cell C-RNTI and the time between the event that triggered the report and the sending of the report to the network. RAN3 wants to check with RAN2 if it’s feasible in the above scenario? Further, RAN3 discussed the following information to be reported in the SPR to assist in the forwarding of SPR over network interfaces: - CGI of the PCell which sent the SPR configuration (already agreed in RAN3); - Indication whether the PSCell change was MN-initiated or SN-initiated (WA but not agreed in RAN3). Explicit or implicit indicator can be decided by RAN2. Q4: RAN3 kindly asks RAN2 to confirm RAN3’s understanding on CGI of the PCell and update their specifications if feasible, and also take the above information for indication on whether the PSCell change was MN-initiated or SN-initiated into account and provide feedback. 2. Actions: RAN3 respectfully asks RAN2 to take the above information into account, provide feedback to the above questions and update their specification if needed.

Q1 to Q3 in the LS [3] are about the feasibility of including “Configuration Information” in RRC signaling. For Q1 and Q2, we don’t see any issues on the network providing “Configuration Information” through the RRC Reconfiguration message with sync containing handover command or PSCell change command since this is allowed in terms of signaling structure. For Q3, UE can determine the source cell C-RNTI when determining the content of SHR or SPR, UE can decide whether to include source cell C-RNTI based on the presence of “Configuration Information” in the handover command or PSCell change command since these configurations were still stored on the UE at that time. However, UE determines the time between the event that triggered the report and the sending of the report to the network at the time reporting the SHR or SPR to the network, the handover command or PSCell change command may be released at that time, so UE cannot decide whether to include source cell C-RNTI based on the presence of “Configuration Information”. Therefore, we think the time between the event that triggered the report and the sending of the report to the network can be always included in SHR or SPR, whether to use this information can be left to the NW implementation.

Proposal 3: The following should be included in the reply LS regarding the solution for “Configuration Information”:

- RAN2 does not see any issues in defining a solution for “Configuration Information”;

- RAN2 does not see any issues on including “Configuration Information” in the RRC Reconfiguration message with sync containing Handover Command or PSCell change command;

- The scenario in Q3 is feasible for including the source cell C-RNTI but not for including the time between the event that triggered the report and the sending of the report to the network.

For Q4 in the LS, as per the analyses in section 2.1 in the contribution, to assist the node to forward the corresponding information to the node that optimizes the configuration, CGI of PCell should be included in SPR, UE shall indicate whether the PSCell change was MN-initiated or SN-initiated explicitly.

Proposal 4: The following should be included in the reply LS regarding the content of SPR:

- RAN2 thinks CGI of PCell and explicit indication of the type of PSCell addition/change (i.e., PSCell addition, MN-initiated change or SN-initiated PSCell change) is needed.

Furthermore, to express our proposal more clearly, a draft reply LS regarding is displayed in the appendix.

Conclusions

Based on the analyses given above, we have the following proposals:

Proposal 1: The following information should be included in the SPR:

- CGI of the PCell which sent the SPR configuration.

- An indication to indicate the type of PSCell addition/change, i.e., PSCell addition, MN-initiated change or SN-initiated PSCell change.

Proposal 2: If one of the following events occurs, UE shall release all the SPR configurations:

- Initiate RRC connection re-establishment;

- Initiate RRC connection resume;

- SPR is generated;

- SCG failure due to PSCell addition/change.

Proposal 3: The following should be included in the reply LS regarding the solution for “Configuration Information”:

- RAN2 does not see any issues in defining a solution for “Configuration Information”;

- RAN2 does not see any issues on including “Configuration Information” in the RRC Reconfiguration message with sync containing Handover Command or PSCell change command;

- The scenario in Q3 is feasible for including the source cell C-RNTI but not for including the time between the event that triggered the report and the sending of the report to the network.

Proposal 4: The following should be included in the reply LS regarding the content of SPR:

- RAN2 thinks CGI of PCell and explicit indication of the type of PSCell addition/change (i.e., PSCell addition, MN-initiated change or SN-initiated PSCell change) is needed.

Reference

- Chairman_notes of 3GPP TSG RAN WG2 meeting #122

- R2-2307030 LS on SHR and SPR

Appendix

1. Overall Description:

- RAN2 thanks RAN3 for their LS on inter-RAT SHR and SPR.

- RAN2 would like to offer the following answers to the questions raised by RAN3:

Q1: Whether RAN2 sees any issues in defining a solution for “Configuration Information” as described above?

[RAN2 answer]: No.

Q2: For SHR/SPR, is there any issue to include this “Configuration Information” in the RRC Reconfiguration message with sync containing Handover Command or PSCell change command?

[RAN2 answer]: No.

Q3: In cases when this “Configuration Information” is not configured by the network to the UE, RAN3 discussed whether UE can include the source cell C-RNTI and the time between the event that triggered the report and the sending of the report to the network. RAN3 wants to check with RAN2 if it’s feasible in the above scenario?

[RAN2 answer]: It is feasible for including the source cell C-RNTI but is not for including the time between the event that triggered the report and the sending of the report to the network in such a scenario. The reason is that UE determines the time between the event that triggered the report and the sending of the report to the network at the time reporting the SHR or SPR to the network, UE cannot know whether the “Configuration Information” is present since the handover command or PSCell change command may be released at that time. But we think UE can always include the time between the event that triggered the report and the sending of the report to the network in SHR or SPR, whether to use this information can be left to the NW implementation.

Q4: RAN3 kindly asks RAN2 to confirm RAN3’s understanding on CGI of the PCell and update their specifications if feasible, and also take the above information for indication on whether the PSCell change was MN-initiated or SN-initiated into account and provide feedback.

[RAN2 answer]: RAN2 thinks CGI of the PCell and explicit indication of the type of PSCell addition/change (i.e., PSCell addition, MN-initiated change or SN-initiated PSCell change) is needed.

2. Actions:

To RAN3.

ACTION: 	RAN2 kindly asks RAN3 to take the above answers into account for their work.

3. Date of Next RAN2 Meetings:

TSG-RAN WG2#123bis            2023-10-09 to 2023-10-13		 Xiamen, CN

TSG-RAN WG2#124              2023-11-13 to 2023-11-17		 Chicago, US