3GPP TSG-RAN WG2 Meeting #123		R2-2307434

Toulouse, France, August 21-25, 2023

Source:	vivo, Qualcomm Incorporated

Title:          Remaining issues of data collection for model training at server

Agenda Item:	7.16.2.2

Document for:	Discussion and Decision

- Introduction

At the RAN2#121 meeting, the typical characteristics of existing data collection methods were summarized (see Annex A). The following data collection methods are considered, and the collected data are terminated at the network side, i.e., gNB, OAM or LMF.

- Logged MDT

- Immediate MDT

- L3 measurements

- L1 measurement (CSI reporting)

- UAI

- Early measurements

- LPP

At the RAN2#122 meeting, some working assumptions were made about the data termination entity. For the model training, data can be generated by UE/gNB and terminated at gNB/OAM/OTT server/LMF.

- P5a: For the data generation entity and termination entity deployed at different entities, RAN2 assumes: For CSI enhancement and beam management use cases: - For model training, training data can be generated by UE/gNB and terminated at gNB/OAM/OTT server. - For NW-sided model inference, input data can be generated by UE and terminated at gNB. - For UE-side model inference, input data/assistance information can be generated by gNB and terminated at UE. - For model monitoring at NW side, performance metrics can be generated by UE and terminated at gNB. For positioning enhancement use case: - For model training, training data can be generated by UE/gNB and terminated at LMF/OTT server. - For NW-sided model inference, input data can be generated by UE/gNB and terminated at LMF and/or gNB. - For UE-side model inference, input data/assistance information can be generated by LMF/gNB and terminated at the UE. - For model monitoring at NW side, performance metrics can be generated by UE/gNB and terminated at LMF.

It can be observed that how the OTT server acquires the training data has not been considered yet. In this contribution, we will further discuss the data collection methods for model training at server.

- Discussion

At the RAN1#112 meeting, the following agreements were made regarding different collaboration levels.

- For Level y, i.e., without model transfer from NW to UE, the training function of the UE-sided model can be performed at the UE-side or the neutral site.

- For Level z, i.e., with model transfer from NW to UE, the model can be trained at the UE-side/neutral site (level z1 and z3) or NW-side (level z2,z4 and z5) and stored in 3GPP network.

Agreement To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models. Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top; Outside 3gpp Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format; 3GPP Network; NW-side. z3; model transfer in open format; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE; 3GPP Network; NW-side.

Existing considered data collection methods, such as MDT, L1/L3 measurement, UAI, early measurement and LPP, are all characterized by UE collecting and reporting data to NW. Thus, these methods can facilitate the model training at NW-side.

For model training at UE-side, as data volume for model training may be huge, in practical NW deployment, it is not realistic to perform model training at UE itself due to limited storage/capability. If the UE-sided model is not transferred from NW, model training can be performed at the UE-side server or third-party neutral server. In this case, how training data are collected and delivered to UE-side server or third-party neutral server, should be considered.

Proposal 1: RAN2 to discuss how to enable data collection for model training at UE-side server or third-party neutral server.

To our understanding, potential options include:

- Option 1: UE may generate the required data and potential data from NW, e.g., assistance information, and then the required data and/or assistance information can be further delivered to the server. The interactions between UE and server can be left to implementation. However, user consent may still be required to be compliance with regulatory, which may not be possible to obtain using this solution.

- Option 2: NW may collect the required data reusing existing considered methods, e.g., MDT, and then the required data, potentially associated with assistance information, can be further delivered to the server.  This does not allow the collection of data that has not been standardized, which is very limiting. The interactions between NW and server is actually a multiple-parties-coordination involving the UE vendors, RAN vendors, OAM vendors, and the Operator.

- Option 3: Using other specified methods, e.g., EVEX, to directly collect data at the server.

The initial analysis of the options is as follows:

| option | Involved network entity | Collected Data type | Data reporting size | Data reporting solution | User consent enforcement |
| --- | --- | --- | --- | --- | --- |
| Option 1 | None, network is not aware of server | - Metrics collected are left to implementation - Maybe challenging to collect assistant information from NW | No size limit | UP solution, or even WIFI, which has limited/no impact on air interface |  |
| Option 2 (e.g., MDT) | OAM | - Standardized metrics related to air interface, e.g., RSRP | Limited by RRC message size | CP solution, i.e., RRC, which has a significant impact on the existing signaling transmission. | Reuse operator mechanism |
| Option 3 (e.g., EVEX) | DCAF | - Application layer data - Currently no support for data collection related to air interface is specified - Support collection of both standardized and non-standardized data.. | No size limit | UP solution, or even WIFI, which has limited/no impact on air interface | Reuse operator mechanism |

To our understanding, Option 1 is totally transparent to 3GPP/operators. However, without the operator’s involvement, the user consent concern is difficult to be addressed.

Option 2 may require the involvement of multiple parties and only yields data that has gone through the standardization and commercialization processes, which can take years.

Option 3 resolves the issues of options 1 and 2. It is also suitable for training data collection at the server, which can reuse operator mechanisms to ensure user consent and has limited/no impact on the air interface. Therefore, we propose:

Proposal 2: A standardized data collection mechanism can leverage existing 3GPP procedures (e.g., EVEX) for compliance with regulatory requirements, such as user consent, for model training purposes at UE-side server or third-party neutral server.

Note that, the current mechanisms such, EVEX framework only collects the data from the application layer and is not able to collect data related to AS layer, such as L1 measurement. Current mechanisms can be enhanced to adapt to different data collection requirements, such as modem data. Such enhancement exercise would require, at least, SA2 involvement.

Therefore, we propose:

Proposal 3: LS to SA2/SA5 about the analysis and requirements of the candidate data collection methods for model training purposes at UE-side server or third-party neutral server from RAN2’s perspective, e.g., collecting data related to AS layer.

- Conclusion

Proposal 1: RAN2 to discuss how to enable data collection for model training at UE-side server or third-party neutral server.

Proposal 2: A standardized data collection mechanism can leverage existing 3GPP procedures (e.g., EVEX) for compliance with regulatory requirements, such as user consent, for model training purposes at UE-side server or third-party neutral server.

Proposal 3: RAN2 to send an LS informing SA2/SA5 about the analysis and requirements of data collection methods for model training purposes at UE-side server or third-party neutral server from RAN2’s perspective, e.g., collecting data related to AS layer.

- Reference

- Annex

- Annex A: Characteristics of existing data collection methods

|  | Involved Network entity | RRC state to generate data | Max payload size per reporting* | Contents to be collected | End-to-End report latency** | Report type | Security and Privacy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Logged MDT | TCE/OAM (It can be utilized by gNB) | RRC_IDLE/RRRC_INACTIVE | <9kbyte | L3 cell/beam measurements, location info, sensor info, timing info | - Procedure latency***: - Latency to enter CONNECTED state - Latency to receive gNB request signaling (~20ms) - Air interface signaling latency****: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message, Privacy via user consent |
| Immediate MDT | TCE/OAM (It can be utilized by gNB) | RRC_CONNECTED | <9kbyte | L3 cell/beam measurements, location info, sensor info | - Procedure latency: - Report interval: - l20ms~30min for periodic report - TTT for event triggered report - Air interface signaling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | Event triggered report, Periodic reporting | AS security via RRC message, Privacy via user consent |
| L3 measurements | gNB | RRC_CONNECTED | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Report interval: - l20ms~30min for periodic report - TTT for event triggered report - Air interface signaling latency: - 20ms (RRC) | Event triggered report, Periodic reporting | AS security via RRC message. |
| L1 measurement (CSI reporting) | gNB | RRC_CONNECTED | <1706bit in PUCCH, <3840bit in PUSCH | L1 CSI measurement | - Procedure latency: - Report interval: - 4-320 slot for periodic report and semi-persistent report - 0-32 slot after reception of DCI for aperiodic report - Air interface signaling latency: - 1 TTI (PUCCH) | Aperiodic report, Semi-persistent report, Periodic report | No AS security |
| UAI | gNB | RRC_CONNECTED | <9kbyte | Assistance information to show UE preference | - Procedure latency: - Upon generation of UE's preference - Air interface signaling latency: - ~20ms (RRC) | Up to UE implementation when to report | AS security via RRC message |
| Early measurements | gNB | RRC_IDLE/RRC_INACTIVE | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Latency to enter CONNECTED state - Latency to receive gNB request signaling (~20ms) - Air interface signaling latency: - ~20ms (RRC) | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message |
| LPP | LMF | RRC_CONNECTED | <9kbyte | Location info | - Procedure latency: - Latency to get upper layer trigger (for UE triggered) - Or latency to receive NW request message (~20ms) - Air interface signaling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and LMF | UE-triggered, NW-triggered | AS security via RRC message |