3GPP TSG-RAN WG2 Meeting #123		R2-2307440

Toulouse, France, 21st – 25th Aug 2023

Source:	vivo

Title:	Discussion on Configuration of Enhanced Type 3 HARQ-ACK Codebook for PUCCH group

Agenda Item:	6.1.3

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the per-HARQ configuration of Enhanced Type 3 HARQ-ACK Codebook to support 32 HARQ processes.

- Discussion

RAN1#112bis-e meeting has sent out a LS[1] about HARQ issues as,

First of all, RAN1 observed that according to current TS 38.331 specification, enhanced type 3 HARQ-ACK codebook supports up to 16 HARQ process numbers per serving cell since bitmap size allocated for a serving cell equals to 16 as highlighted below. PDSCH-HARQ-ACK-EnhType3-r17 ::= SEQUENCE { pdsch-HARQ-ACK-EnhType3Index-r17 PDSCH-HARQ-ACK-EnhType3Index-r17, applicable-r17 CHOICE { perCC SEQUENCE (SIZE (1..maxNrofServingCells)) OF INTEGER (0..1), perHARQ SEQUENCE (SIZE (1..maxNrofServingCells)) OF BIT STRING (SIZE (16)) }, pdsch-HARQ-ACK-EnhType3NDI-r17 ENUMERATED {true} OPTIONAL, -- Need R pdsch-HARQ-ACK-EnhType3CBG-r17 ENUMERATED {true} OPTIONAL, -- Need S ... } However, if a UE is provided with nrofHARQ-ProcessesForPDSCH-v1700 for a serving cell, a maximum of 32 HARQ processes for the serving cell can be used for the downlink. Therefore, RAN1 respectfully request RAN2 to update 331 specification for enhanced type 3 HARQ-ACK codebook, taking into account that up to 32 HARQ processes can be configured for a serving cell.

In order to figure out the definition of perHARQ, we traced back to RAN1#107-e meeting[2], it is agreed that the granularity of the list of enhanced Type 3 HARQ-ACK codebooks is per PUCCH cell group.

Agreement The list enhanced Type 3 HARQ-ACK codebooks is configured per PUCCH cell group (i.e., separately configurable for primary and secondary PUCCH cell group).

Observation 1: RAN1 agreed that the list of enhanced Type 3 HARQ-ACK codebooks is configured per PUCCH cell group.

Based on the LS mentioned above, RAN2 accordingly modified TS 38.331 as follow,

PhysicalCellGroupConfig ::= SEQUENCE { <SKIPPED THE IRRLEVANT PART> -- start of enhanced Type3 feedback pdsch-HARQ-ACK-EnhType3ToAddModList-r17 SEQUENCE (SIZE(1..maxNrofEnhType3HARQ-ACK-r17)) OF PDSCH-HARQ-ACK-EnhType3-r17 OPTIONAL, -- Need N pdsch-HARQ-ACK-EnhType3ToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofEnhType3HARQ-ACK-r17)) OF PDSCH-HARQ-ACK-EnhType3Index-r17 OPTIONAL, -- Need N pdsch-HARQ-ACK-EnhType3SecondaryToAddModList-r17 SEQUENCE (SIZE(1..maxNrofEnhType3HARQ-ACK-r17)) OF PDSCH-HARQ-ACK-EnhType3-r17 OPTIONAL, -- Need N pdsch-HARQ-ACK-EnhType3SecondaryToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofEnhType3HARQ-ACK-r17)) OF PDSCH-HARQ-ACK-EnhType3Index-r17 OPTIONAL, -- Need N pdsch-HARQ-ACK-EnhType3DCI-FieldSecondaryPUCCHgroup-r17 ENUMERATED {enabled} OPTIONAL, -- Cond twoPUCCHgroup pdsch-HARQ-ACK-EnhType3DCI-Field-r17 ENUMERATED {enabled} OPTIONAL, -- Need R -- end of enhanced Type3 feedback <SKIPPED THE IRRLEVANT PART> PDSCH-HARQ-ACK-EnhType3-r17 ::= SEQUENCE { pdsch-HARQ-ACK-EnhType3Index-r17 PDSCH-HARQ-ACK-EnhType3Index-r17, applicable-r17 CHOICE { perCC SEQUENCE (SIZE (1..maxNrofServingCells)) OF INTEGER (0..1), perHARQ SEQUENCE (SIZE (1..maxNrofServingCells)) OF BIT STRING (SIZE (16)) }, pdsch-HARQ-ACK-EnhType3NDI-r17 ENUMERATED {true} OPTIONAL, -- Need R pdsch-HARQ-ACK-EnhType3CBG-r17 ENUMERATED {true} OPTIONAL, -- Need S ..., [[ perHARQ-Ext-r17 SEQUENCE (SIZE (1..maxNrofServingCells)) OF BIT STRING (SIZE (32)) OPTIONAL -- Need R ]] } <SKIPPED THE IRRLEVANT PART> } PDSCH-HARQ-ACK-EnhType3 field descriptions. pdsch-HARQ-ACK-EnhType3CBG When configured, the DCI format 1_1 or DCI format 1_2 can request the UE to include CBG level A/N for each CC with CBG level transmission configured of the enhanced Type 3 HARQ-ACK codebook. When not configured, the UE will report TB level A/N even if CBG level transmission is configured for a CC.. pdsch-HARQ-ACK-EnhType3NDI When configured, the DCI format 1_1 or DCI format 1_2 can request the UE to include NDI for each A/N reported of the enhanced Type 3 HARQ-ACK codebook.. perCC Configures enhanced Type 3 HARQ-ACK codebook using per CC configuration.. perHARQ, perHARQ-Ext Configures enhanced Type 3 HARQ-ACK codebook using per HARQ process and CC configuration. perHARQ-Ext is present only when nrofHARQ-ProcessesForPDSCH-v1700 is present in pdsch-ServingCellConfig of at least one serving cell in this cell group. If perHARQ-Ext is present, the UE ignores perHARQ..

The current specification describes the IE perHARQ-Ext, which is contained in the codebook element IE PDSCH-HARQ-ACK-EnhType3, as conditionally present. The condition is that the cell group rather than the PUCCH group contains at least one serving cell, whose PDSCH parameter is configured with nrofHARQ-ProcessesForPDSCH-v1700.

nrofHARQ-ProcessesForPDSCH-v1700 is contained in The IE PDSCH-ServingCellConfig, configuring UE-specific PDSCH parameters that are common across the UE's BWPs of one serving cell.

Observation 2: nrofHARQ-ProcessesForPDSCH-v1700 is configured per cell in a cell group.

Since PDSCH-HARQ-ACK-EnhType3 can be used to represent a codebook in both pdsch-HARQ-ACK-EnhType3ToAddModList-r17 and pdsch-HARQ-ACK-EnhType3SecondaryToAddModList-r17, the wording of “perHARQ-Ext is present when…” can be confusing. The highlight part can be interpreted in two ways.

- Way 1: “in this cell group” is “in this MCG or SCG”

When UE is enabled with 32 HARQ processes through PDSCH configuration in at least one serving cell, all the cells belonging to the same cell group (i.e. MCG or SCG) shall prolong the “per-HARQ” indication for enhanced Type 3 HARQ-ACK codebook from 16-bit to 32-bit. There is no differentiation between the primary PUCCH group and the secondary PUCCH group for the involving cell group.

Observation 3: Based on the current description, there is no such situation that one PUCCH group is configured with per-HARQ and the other is with per-HARQ-Ext-r17.

- Way 2: “in this cell group” is “in this PUCCH cell group”

When UE is enabled with 32 HARQ processes through PDSCH configuration in at least one serving cell, the cells belonging to the same involving PUCCH group (i.e. the primary PUCCH group or the secondary PUCCH group respectively) shall prolong the “per-HARQ” indication for enhanced Type 3 HARQ-ACK codebook from 16-bit to 32-bit; while the other PUCCH group shall keep 16-bit “per-HARQ” indication.

If the interpretation goes to Way 2, to avoid the misunderstanding, it is straightforward to revise from the “cell group” to the “PUCCH group”. To clarify the scope of function for maximum number of HARQ processes, there is no way to configure per-HARQ-Ext by mistake.

On the other hand, Way 1 may be the common understanding. Since Release-17 has frozen for some time, a new IE can be introduced in order to capture RAN1’s agreement instead. Besides, this feature is not backward compatible in view of the design of ASN.1, a new UE capability indication can also be introduced to handle the NBC issue.

Proposal: RAN2 to discuss how to capture the configuration of enhanced Type 3 HARQ-ACK codebook supporting up to 32 HARQ processes in PUCCH group with two perspectives:

- Revise the field description to specify the scope of function in support of maximum number of HARQ processes is PUCCH group as Annex;

- Owing to NBC issue, endorse CR R2-2307438 [4] and R2-2307439 [5] to introduce 32-bit per-HARQ indication for PUCCH group and a UE capability indication for such feature.

- Conclusion

We make the following observations based on the above discussion,

Observation 1: RAN1 agreed that the list of enhanced Type 3 HARQ-ACK codebooks is configured per PUCCH cell group.

Observation 2: nrofHARQ-ProcessesForPDSCH-v1700 is configured per cell in a cell group.

Observation 3: Based on the current description, there is no such situation that one PUCCH group is configured with per-HARQ and the other is with per-HARQ-Ext-r17.

We hence propose:

Proposal: RAN2 to discuss how to capture the configuration of enhanced Type 3 HARQ-ACK codebook supporting up to 32 HARQ processes in PUCCH group with two perspectives:

- Revise the field description to specify the scope of function in support of maximum number of HARQ processes is PUCCH group as Annex;

- Owing to NBC issue, endorse CR R2-2307438 and R2-2307439 to introduce 32-bit per-HARQ indication for PUCCH group and a UE capability indication for such feature.

- Reference

- R1-2304099	LS to RAN2 on scheduling and HARQ issues for FR2-2

- R1-2111142	Moderator summary #1 on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT	Moderator (Nokia)

- TS 38.331		Radio Resource Control (RRC) protocol specification (Release 17)

- R2-2307438	Change Request on UE Capability of Enhanced Type 3 HARQ-ACK Codebook supporting 32 HARQ processes for PUCCH group

- R2-2307439	Correction on Configuration of Enhanced Type 3 HARQ-ACK Codebook for PUCCH group

# Annex

PDSCH-HARQ-ACK-EnhType3 field descriptions

pdsch-HARQ-ACK-EnhType3CBG When configured, the DCI format 1_1 or DCI format 1_2 can request the UE to include CBG level A/N for each CC with CBG level transmission configured of the enhanced Type 3 HARQ-ACK codebook. When not configured, the UE will report TB level A/N even if CBG level transmission is configured for a CC.

pdsch-HARQ-ACK-EnhType3NDI When configured, the DCI format 1_1 or DCI format 1_2 can request the UE to include NDI for each A/N reported of the enhanced Type 3 HARQ-ACK codebook.

perCC Configures enhanced Type 3 HARQ-ACK codebook using per CC configuration.

perHARQ, perHARQ-Ext Configures enhanced Type 3 HARQ-ACK codebook using per HARQ process and CC configuration. perHARQ-Ext is present only when nrofHARQ-ProcessesForPDSCH-v1700 is present in pdsch-ServingCellConfig of at least one serving cell in the PUCCH group.