3GPP TSG-RAN WG2 Meeting #123	R2-2307450

Toulouse, France, August 21-25, 2023

Agenda Item:	7.17.2

Source:	Huawei, HiSilicon

Title:	Discussion on early MUSIM indication

Document for: Discussion and Decision

Introduction

RAN2 discussed the early MUSIM indication in RAN2-121bis [1] and made the following agreements:

Support “early indication” from UE to network during RRC connection setup/resume procedure. FFS how to indicate this and in which message. The indication will tell network that UE capabilities are temporarily restricted. FFS on details (i.e. when UE can indicate this, what does it indicate, how does it relate to UAI, etc.)

In this contribution, we discuss the details for early MUSIM indication.

Discussion

RAN2 has agreed to support “early MUSIM indication” from UE to network during RRC connection setup/resume procedure, the “early MUSIM indication” indicates that UE capabilities are temporarily restricted. Considering that not much information can be carried during RRC connection setup/resume procedure, the indication can be in a simple manner, e.g., single-bit indication. Based on this indication, the gNB understands that the stored UE capability is not valid currently and the UE’s capabilities are temporarily restricted, so the gNB can configure the UE with a conservative configuration in the first RRC reconfiguration procedure if the UE is setting up the RRC connection or change the UE’s configuration to a conservative one if the UE is resuming the RRC connection. The conservative configuration can be, for example, 1 UL/DL max MIMO layer without CA/DC.

RRCReconfiguration (enable MUSIM UAI)

UE

gNB

Early MUSIM indication

Decision on conservative configuration

RRCReconfigurationComplete

MUSIM UAI

Proposal 1: Using a 1-bit as the “early MUSIM indication” from the UE to NW during RRC connection setup/resume procedure to indicate that the UE capabilities are temporarily restricted.

Figure 1: Early MUSIM indication and the following procedure

The temporary UE capability update is under gNB’s control, so whether the UE can send above “early MUSIM indication” should be configured by the gNB via the system information. Only when the gNB indicates that “early MUSIM indication” is allowed, the UE can send early MUSIM indication.

Proposal 2: gNB indicates whether sending the early MUSIM indication during the RRC connection setup/resume procedure is allowed via the System information. It is FFS which SIB is used to carry such indication.

Early MUSIM indication is a coarse-grained indication which only inform the NW  that the capabilities are temporarily restricted due to MUSIM operation, but the gNB has no idea which specific capability is restricted. So, after the UE enters RRC_CONNECTED state, i.e. RRC connection setup/resume procedure is complete, the gNB can enable the MUSIM UAI procedure to acquire the detailed temporary capabilities restriction from the UE.

In [2], we discussed the proactive and reactive solution for the temporary capability restriction. In our understanding, the early MUSIM indication can only combined with the proactive solution. After sending the MUSIM early indication, if the NW A enables the UE to report the MUSIM UAI for temporary capability indication, the UE can proactively report the potential capability restriction in NW A based the current configuration in the NW B, regardless of whether the current configuration in NW A has beyond the capability.  However, if the UE decides to use reactive solution, it means it is acceptable for the UE that the current configuration exceeds the temporary capabilities after the RRC setup or RRC resume procedure for a short period of time. In this case, the UE can decide not to send the early MUSIM indication. If the NW A’s configuration exceeds the capabilities used by UE in NW A, the UE  indicate to the NW A via UAI of temporary capabilities restriction which is based on the current NW A’s configuration. So whether the UE needs to send the early MUSIM indication should be up to the UE implementation.

Proposal 3: It is up to UE implementation whether to send early MUSIM indication during the RRC connection setup/resume procedure if the gNB enable it. After UE sends early MUSIM indication, the gNB can enable the UE to provide MUSIM UAI for temporary capabilities reporting.

As for how to indicate this and in which message, we further discuss the details for RRC connection setup procedure and RRC connection resume procedure separately as below.

RRCReconfiguration (enable MUSIM UAI)

UE

gNB

RRCSetup

RRCSetupComplete (early MUSIM indication)

Decision on conservative configuration

RRCSetupRequest

RRCReconfigurationComplete

MUSIM UAI

Details for RRC connection setup procedure

Figure 2: Procedure for RRC setup

During the RRC setup procedure, since the gNB has not obtained the UE capability from the core network, the configuration in RRCSetup will be conservative. After the RRC setup procedure, the first configuration in RRCReconfiguration message may exceed the current temporary UE capabilities, e.g. the CA/DC combination or MIMO layers since the gNB will configure the UE based on full capabilities (stored in CN) forwarded by the CN. Thus, the early MUSIM indication should be carried when the UE sends the RRCSetupComplete message. Considering there is no restriction on the size of this message, the 1-bit can be added in it.

Proposal 4: The 1-bit Early MUSIM indication is introduced in RRCSetupComplete message for RRC connection setup procedure.

One issue with the early indication in RRC setup procedure is the risk of security. According to the previous RAN2 understanding, it is not expected to report the UE capability before AS security is activated, which could be captured and modified by the attacking node. One scenario is, if the gNB misunderstands the UE with a restricted capability but the UE actually not, the gNB will continue with a conservative configuration. However, the UE has no idea about the reason, so no UAI is initiated. In this case, there will be a downgrading of system performance. Similar issue was discussed when the UL RRC segmentation capability is introduced in Msg5, which is used to help with NW on capability enquiry. It was agreed that by including the UL segmentation capability in UE capability container could help to provide more security protection. We understand compared with the segmentation capability, a fake Msg5 for early indication of capability restriction brings more security risk.

Then to solve the security issue for MUSIM, one possible solution is the UE double confirms the indication of capability restriction in an encrypted message, e.g. RRCReconfiguratoinComplete message. After receiving the early MUSIM indication in RRCSetupComplete message, if no further indication is included in RRCReconfiguratoinComplete message, the NW can consider the full capability, as reported in UECapabilityInformation, is valid and revert the configuration to the one that match with the UE’s normal capability.

Proposal 5: To avoid security issue, the early MUSIM indication is also introduced in RRCReconfigurationComplete message in RRC connection setup procedure for confirmation.

RRCReconfiguration (enable MUSIM UAI)

UE

gNB

RRCResume

RRCResumeComplete

Decision on conservative configuration

RRCResumeRequest (early MUSIM indication)

RRCReconfigurationComplete

MUSIM UAI

Details for RRC connection resume procedure

Figure 3: Procedure for RRC resume

According the current procedure, the UE shall release the configurations from otherConfig if the UE is moved from connected state to inactive state, and RRCResume message cannot configure the otherConfig for the UE. When the UE is in RRC_INACTIVE state, the stored configurations in the Inactive AS context may exceed its current temporary capability. After receiving the RRCResume message, the UE needs to restore these configurations which would have problem, the stored configurations in the UE’s AS context may exceed the UE’s temporary capability. Thus, the early MUSIM indication should be carried when the UE sends the RRCResumeRequest message. After receiving the early MUSIM indication, the gNB can change the UE’s configuration to a conservative one in the RRCResume.

Proposal 6: Early MUSIM indication is carried when the UE sends the RRCResumeRequest message for RRC connection resume procedure.

However, currently there is only one spare bit reserved in RRCResumeRequest message, and we don’t think we have strong reason to use it for early MUSIM indication, so how the RRCResumeRequest message carries early MUSIM indication needs to be further considered. For example, to reuse the similar way as msg3-based RedCap UE early identification, i.e. using new LCID for the RRCResumeRequest message if the UE has temporarily capability restriction when initiating the RRC resume procedure.

Proposal 7: If Proposal 6 is agreed, RAN2 to discuss how the RRCResumeRequest message carries early MUSIM indication.

Conclusion

In this contribution, we further discussed the details for early MUSIM indication and have the following proposals:

Proposal 1: Using a 1-bit as the “early MUSIM indication” from the UE to NW during RRC connection setup/resume procedure to indicate that the UE capabilities are temporarily restricted.

Proposal 2: gNB indicates whether sending the early MUSIM indication during the RRC connection setup/resume procedure is allowed via the System information. It is FFS which SIB is used to carry such indication.

Proposal 3: It is up to UE implementation whether to send early MUSIM indication during the RRC connection setup/resume procedure if the gNB enable it. After UE sends early MUSIM indication, the gNB can enable the UE to provide MUSIM UAI for temporary capabilities reporting.

Proposal 4: The 1-bit Early MUSIM indication is introduced in RRCSetupComplete message for RRC connection setup procedure.

Proposal 5: To avoid security issue, the early MUSIM indication is also introduced in RRCReconfigurationComplete message in RRC connection setup procedure for confirmation.

Proposal 6: Early MUSIM indication is carried when the UE sends the RRCResumeRequest message for RRC connection resume procedure.

Proposal 7: If Proposal 6 is agreed, RAN2 to discuss how the RRCResumeRequest message carries early MUSIM indication.

References

- RAN2 Chair’s Notes RAN2#121bis.

- R2-2307454, Discussion on proactive and reactive approaches, Huawei, HiSilicon.