3GPP TSG-RAN WG2 Meeting #123	R2-2307451

Toulouse, France, August 21-25, 2023

Agenda Item:	7.17.3

Source:	Huawei, HiSilicon

Title:	Details of allowed MUSIM temporary capability restrictions

Document for: Discussion and Decision

Introduction

UE capability restrictions were discussed in RAN2#121bis meeting [1] and made the following agreements with FFS:

1: For Rel-18 MUSIM dual active operation, the maximum MIMO layer may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B. 3: For Rel-18 MUSIM dual active operation, the measurement gap requirement may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B. 4: For Rel-18 MUSIM dual active operation, the measurement gap requirement change is reported for each serving cells, and for target bands or all supported NR bands depending on whether target bands are configured by the NW. FFS on whether the reporting can reuse the current needForGapInfoNR in RRC reconfiguration complete or extend the similar function in UAI. FFS if this is only for NW A or also NW B. 8: The maximum UL power may be changed due to Rel-18 MUSIM dual active operation, but there is no need to introduce any new UE behavior for reporting this change. 6: UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. FFS how/whether this works for the proactive case. 9: RAN2 should avoid duplicating all the capabilities that UE reports via the UECapabilityInformation in the UAI for R18 MUSIM purpose. RAN2 can discuss P2, P5 and P7 from R2-2304397 during RAN2#123.

In this contribution, we discuss the open issues from the above agreements.

Discussion

General procedure

For temporary UE capabilities, e.g. Scell(s)/SCG release, maximum MIMO layers and measurement gap requirement which are agreed to be updated, it is FFS if the temporary UE capabilities indication is only for NW A or also NW B. It is assumed that both NW A and NW B are NR networks (it was agreed that the temporary UE capabilities indication only happens in NR network), the following scenarios are considered.

- Scenario 1: UE is in RRC_CONNECTED state in NW A and UE is moving from RRC_IDLE/INACTIVE to RRC_CONNECTED in NW B

- Scenario 2: UE is moving from RRC_IDLE/INACTIVE to RRC_CONNECTED in NW A and UE is in RRC_CONNECTED state in NW B

- Scenario 3: UE is in RRC_CONNECTED state in both NW A and NW B

In scenario 1, part of the UE capabilities will be occupied for NW B, so the UE can indicate temporary UE capabilities restriction in NW A. Similarly, in scenario 2, the UE can indicate temporary UE capabilities restriction in NW B. In scenario 3, the reconfiguration (e.g. including SCell/SCG addition, handover, etc) in a NW may lead to the available UE capabilities change, thus, the UE can indicate temporary UE capabilities restriction in another NW. Thus, the temporary UE capabilities indication can be transmitted in either NR NW A or NR NW B or both.

Proposal 1: For the NR + NR case, the UE can indicate the temporary UE capabilities to either NR NW A or NR NW B or both if the corresponding NW enables the temporary UE capabilities reporting.

Allowed MUSIM temporary capability restrictions

    - Which UE capabilities can be impacted

SRS capability

The RF chains will be changed in MUSIM scenario which impacts the SRS capability. However, the temporary restrictions on maximum UL/DL MIMO layers has already been supported and the temporary restrictions on SRS capability can be implicitly known from the maximum UL/DL MIMO layers. In RAN2#122 meeting, the CRs for clarification on UAI for UL MIMO layers were agreed in [2][3][4] with the following NOTE added. That means, if the maximum UL MIMO layers are reduced, the maximum number of SRS ports will be regarded as to be reduced to the same number.

NOTE 1: The field may also indicate the UE’s preference on reduced configuration corresponding to the maximum number of SRS ports (i.e. nrofSRS-Ports) of each serving cell operating on the associated frequency range.

Additionally, for SRS Tx switching capability srs-TxSwitch, the temporary restrictions on SRS Tx switching capability can be implicitly known based on the maximum UL/DL MIMO layers and legacy srs-TxSwitch-v1610 reported by the UE in the UE capability information message. This was introduced to match the reduced maximum UL/DL MIMO layers for power saving purpose [5], and the same mechanism can be reused for MUSIM purpose. For example, if the original srs-TxSwitch capability is t1r4, if the maximum DL MIMO layers are restricted to 2, the restricted SRS Tx switching capability is t1r2 based on t1r1-t1r2-t1r4 from srs-TxSwitch-v1610.

srs-TxSwitch, srs-TxSwitch-v1610 Defines whether UE supports SRS for DL CSI acquisition as defined in clause 6.2.1.2 of TS 38.214 [12]. The capability signalling comprises of the following parameters: - supportedSRS-TxPortSwitch indicates SRS Tx port switching pattern supported by the UE, which is mandatory with capability signalling. The indicated UE antenna switching capability of ′xTyR′ corresponds to a UE, capable of SRS transmission on ′x′ antenna ports over total of ′y′ antennas, where ′y′ corresponds to all or subset of UE receive antennas, where 2T4R is two pairs of antennas. supportedSRS-TxPortSwitch-v1610, which is optional to report, indicates downgrading configuration of SRS Tx port switching pattern. If the UE indicates the support of downgrading configuration of SRS Tx port switching pattern using supportedSRS-TxPortSwitch-v1610, the UE shall report the values for this as below, based on what is reported in supportedSRS-TxPortSwitch. supportedSRS-TxPortSwitch; supportedSRS-TxPortSwitch-v1610. t1r2; t1r1-t1r2. t1r4; t1r1-t1r2-t1r4. t2r4; t1r1-t1r2-t2r2-t2r4. t2r2; t1r1-t2r2. t4r4; t1r1-t2r2-t4r4. t1r4-t2r4; t1r1-t1r2-t2r2-t1r4-t2r4.

Thus, there is no need to introduce additional signalling to indicate the temporary restrictions on SRS capability for MUSIM purpose.

Proposal 2: No need to introduce additional signalling to indicate the temporary restrictions on SRS capability for MUSIM purpose.

Bandwidth

The motivation of reduced aggregated bandwidth introduced in overheating/power saving assistance information is power saving, the power consumption can be reduced according to the reduced bandwidth. However, from UE capability perspective, we don’t see the case that the supported bandwidth is restricted. Some companies mentioned that this is to solve band conflict issue, it can be addressed by release/recovery of SCells/SCG, so bandwidth is unnecessary for MUSIM purpose.

Proposal 3: Do not introduce the signalling to indicate the temporary restrictions on bandwidth for MUSIM purpose.

Band conflict

For band conflict case, it can be covered by release/recovery of SCells/SCG, for example, the UE indicates the release of the SCells/SCG if band conflict occurs. Thus, no additional signalling procedure is needed specially for band conflict.

Proposal 4: No additional signalling procedure is needed specifically for band conflict since it can be addressed by release/recovery of SCells/SCG.

    - The granularity of impacted capabilities

Maximum MIMO layers

It was agreed that maximum MIMO layers may be changed and the change can be indicated to the NW, however, the granularity for maximum MIMO layer is still FFS. Considering the scenarios, if the SIM B is in RRC_IDLE/RRC_INACTIVE, SIM A can use full capability, e.g. all RF chains will be occupied by the SIM A. But if the SIM B enters RRC_CONNECTED, some of the RF chains will be occupied by the SIM B, so the RF chains that can be used by SIM A will be restricted. RF chains impact the scheduled maximum UL/DL MIMO layers. For example, assuming the UE with capability of 2 UL MIMO layers and 4 DL MIMO layers, if the SIM B is in RRC_IDLE/RRC_INACTIVE, the UE can use 2 UL MIMO layers for transmission and 4 DL MIMO layers for reception in NW A. After the SIM B switches to RRC_CONNECTED, the UE may split the capability between NW A and NW B, e.g. the UE may only use 1 UL MIMO layer for transmission and 2 DL MIMO layers for reception in NW A. After the SIM B switched from RRC_CONNECTED state to RRC_IDLE/INACTIVE state again, the UE will resume its capability to support 2 UL MIMO layers and 4 DL MIMO layers in NW A.

Proposal 5: Both UL and DL maximum MIMO layers can be changed and are supported to be indicated to the NW separately.

In overheating/power saving assistance information, UE preferred maximum UL/DL MIMO layers is per frequency range, this is enough for power saving purpose. But for MUSIM, the maximum UL/DL MIMO layers supported for different cells/carriers may not be the same. Thus, the UE can indicate the temporary maximum UL/DL MIMO layers based on currently configured serving cell(s) separately. Moreover, for the proactive solution [6], the UE indicates the temporary supported BCs, the maximum UL/DL MIMO can be updated per CC per band per BC.

Proposal 6: The temporary maximum UL/DL MIMO layers can be indicated per serving cells/CCs.

Other capabilities

For release/recovery of SCells/SCG, it was agreed that the UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose.

For band combinations related to CA/DC, UE indicates temporary supported band combinations [6].

For measurement gap requirement, it was agreed that the measurement gap requirement change is reported for each serving cells, and for target bands or all supported NR bands depending on whether target bands are configured by the NW.

For above capabilities, no need to consider the granularity.

Conclusion

In this contribution, we further discussed MUSIM temporary capability restrictions and have the following proposals:

General procedure

Proposal 1: For the NR + NR case, the UE can indicate the temporary UE capabilities to either NR NW A or NR NW B or both if the corresponding NW enables the temporary UE capabilities reporting.

Which UE capabilities can be impacted

Proposal 2: No need to introduce additional signalling to indicate the temporary restrictions on SRS capability for MUSIM purpose.

Proposal 3: Do not introduce the signalling to indicate the temporary restrictions on bandwidth for MUSIM purpose.

Proposal 4: No additional signalling procedure is needed specifically for band conflict since it can be addressed by release/recovery of SCells/SCG.

The granularity of impacted capabilities

Proposal 5: Both UL and DL maximum MIMO layers can be changed and are supported to be indicated to the NW separately.

Proposal 6: The temporary maximum UL/DL MIMO layers can be indicated per serving cells/CCs.

References

- R2-2304397, Report of [AT121bis-e][230][MUSIM] UE capability restrictions (vivo), vivo.

- R2-2306824, Clarification on UAI for UL MIMO layers (R15), Huawei, HiSilicon.

- R2-2306825, Clarification on UAI for UL MIMO layers (R16), Huawei, HiSilicon.

- R2-2306826, Clarification on UAI for UL MIMO layers (R17), Huawei, HiSilicon.

- R1-1909422, Enable downgrading SRS antenna switching from 2T4R to 1T2R, OPPO, CMCC, ZTE.

- R2-2307454, Discussion on proactive and reactive approaches, Huawei, HiSilicon.