3GPP TSG-RAN WG2 Meeting #123	 R2-2307459

Toulouse, France, August 21-25, 2023

Agenda Item:	7.11.2.1

Source: 	NEC

Title:  	Discussion on control plane for Multicast reception in RRC_INACTIVE

Document for:	Discussion and Decision

- 1 Introduction

In RAN2#122 meeting [1], the following agreements are made:

The multicast MCCH configuration takes the broadcast MCCH configuration structure (i.e., mcch-Config-r17) as baseline.

To notify the multicast MCCH change, change notification mechanism for Rel-17 broadcast MCCH is the baseline.

Working assumption (to be confirmed by RAN1 via pending reply LS): One bit in the MCCH DCI is used to notify the change of the multicast MCCH. We reuse the bit used for MCCH change indication from Rel-17 MBS broadcast. This does not cover session deactivation which is FFS.

It is not supported to provide the PTM configuration of intra-gNB neighbour cells in the dedicated signalling.

For PTM configuration structure on the multicast MCCH, Rel-17 broadcast PTM configuration structure is taken as baseline.

As a baseline, The PTM configuration in the RRCRelease message with suspendconfig has the same structure as the PTM configuration in multicast MCCH.

FFS how existing MRBs are handled.

Introduce a new indication per tmgi in the group paging which informs Rel-18 UEs having a valid PTM configuration to receive the multicast in RRC_INACTIVE.

MCCH is used for notifying MC session deactivation for multicast reception in RRC_INACTIVE to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI.

This is assumed to have no/minor impact on RAN1/PHY

Last meeting, we made a further progress on control plane including multicast MCCH structure and session activation/deactivation notification. In this contribution, we analyzed more details based on the agreements also discussed leftover issue, e.g., service continuity.

- 2 Discussion

- 2.1 PTM configuration structure

MRB handling

It is agreed in last RAN2#122 meeting that the Rel-17 broadcast structure will be reused. But still there is an FFS that how the existing MRBs are handled.

For the legacy multicast (used in RRC_CONNECTED) MRB configuration, it follows the traditional RadioBearerConfig procedure / structure, i.e., UE performs Multicast MRB addition/modification and release based on multicast MRB ID. However it is different from the structure of MCCH. In MCCH, each session can be mapped to one G-RNTI also one LCID (within RLC config), there is no explicit MRB ID in MCCH configuration. Thus UE performs MRB establishment and release based on event like start of MBS session or stop of MBS session.

Based on the description above, we can see that it is a little complicated to reuse multicast-connected MRB into RRC_INACTIVE. Therefore we suggest that the UE could suspend the multicast configuration (including MRB) used for RRC_CONNECTED when enters RRC_INACTIVE and apply a new multicast MRB for the reception in RRC_INACTIVE based on MCCH configuration or RRCRelease (it is noted that the PTM configuration in RRCRelease has the same structure as MCCH’s).

Proposal 1: The UE suspends the multicast configuration (including MRB) used for RRC_CONNECTED when enters RRC_INACTIVE and apply a new multicast MRB for the reception in RRC_INACTIVE based on multicast MCCH configuration or RRCRelease.

The applicability of PTM configuration between RRCRelease and MCCH

On the other hand, both RRCRelease and Multicast MCCH can provide UE with PTM configuration for multicast reception in RRC_INACTIVE. More detail is that RRCRelease can provide the initial PTM configuration when UE enters RRC_INACTIVE, this method avoids UE additionally reading MCCH and can also indicate the enable of multicast reception. As for MCCH, it can ensure RRC_INACTIVE UE to acquire the subsequent (possible updated) PTM configuration also deactivation notification. Regarding which PTM configuration to be applied, it is understood that only one PTM configuration can be applied, thus the UE behaviour should be specified. For example the Multicast MCCH information acquisition procedure overwrites any previously stored PTM configuration (i.e., delta configuration is not applicable).

Proposal 2: The Multicast MCCH information acquisition procedure overwrites any previously stored PTM configuration.

- 2.2 Details of MCCH handling by the UE

Based on the agreements achieved for MCCH, multicast MCCH can provide UE with a subsequent updated PTM configuration as well as deactivation notification for each multicast session received in RRC_INACTIVE. And only one bit is used to notify UE of MCCH change in MCCH-DCI.

It is admitted that due to MCCH mechanism, UE could easily update its PTM configuration without entering RRC_CONNECTED also be aware of the session state (e.g., session deactivation). However, it requires UE to continuously monitor MCCH even though there is no any updated MCCH information, which causes unnecessary power consumption as shown in Fig.1, we need to consider this issue.

Fig.1 unnecessary MCCH monitoring

First, let’s review how legacy MCCH mechanism works [2]:

- Change of MCCH information only occurs at specific radio frames, i.e. the concept of a modification period is used. Within a modification period, the same MCCH information may be transmitted a number of times, as defined by its scheduling (which is based on a repetition period).

- When the network changes (some of) the MCCH information, it notifies the UEs about the change starting from the beginning of the MCCH modification period via PDCCH which schedules the MCCH in every repetition in that modification period.

Fig.2 MCCH mechanism

The MCCH modification mechanism is somehow similar to SI change modification, regarding SI change modification, a RRC_CONNECTED UE is not required to monitor every paging occasions during modification period, that is to say, UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period. Here, as the same MCCH information is transmitted repeatedly during one modification period, same mechanism as SI change can be used, the UE is not required to monitor every MCCH occasions during one modification period, i.e., UEs receiving multicast in RRC_INACTIVE shall monitor for MCCH change information in any MCCH occasion at least once per MCCH modification period, so that not only the updated MCCH information is available but also power consumption from MCCH monitoring can be saved.

Proposal 3: UEs receiving multicast in RRC_INACTIVE shall monitor for MCCH change information in any MCCH occasion at least once per MCCH modification period.

One more thing is that what the UE behaviour is after acquiring MCCH information, including updated PTM configuration and deactivation notification.

After the reception of updated PTM configuration, since the UE has already got the latest PTM configuration and MCCH information is transmitted repeatedly during one modification period. The UE actually could stop the subsequent MCCH occasions monitoring if the same modification period is not finished yet.

After the reception of session deactivation, since the UE only based on the paging – session activation to start monitoring the session again, there is no need to monitor MCCH anymore, the UE actually could stop MCCH monitoring, until the corresponding session is (re) activated again (by group paging).

Thus we suggest:

Proposal 4: If updated PTM configuration is received, the UE receiving multicast session in RRC_INACTIVE may stop monitoring the subsequent MCCH PDCCH occasions for this modification period.

Proposal 5: If session deactivation is received, the UE receiving multicast session in RRC_INACTIVE may stop monitoring the subsequent MCCH PDCCH occasions, until the corresponding session is (re)activated.

- 2.3 Service continuity during mobility and state transition

- Details of frequency prioritization and multicast NCL

RAN2 has agreed that frequency prioritization and multicast NCL (neighbour cell list) are introduced for better mobility. To our understanding, frequency priority is used before the cell is reselected and NCL is used after the cell is reselected. As long as UE is able to move to other cells, it is anyway helpful to provide frequency priority and NCL. And how to use is FFS.

In legacy MBS broadcast frequency prioritization, UE considers one frequency to be the highest priority, when the frequency is the only one to let UE camp on receiving MBS broadcast, and the selected cell contains SIB20 or FSAI/USD indicate this frequency mapping for the concerned service. However only setting one frequency as highest priority is not sufficient to support multicast reception during RRC_INACTIVE, the situation is a little different, as how to setting frequency priority is not only based on which one is supporting multicast reception in RRC_INACTIVE but also based on network congestion. Thus it is somehow a little tough to use specific mechanism to make the best cell reselection decision (e.g., based on FSAI/USD), it is desired to leave it to network implementation to determine which frequency is preferred for a UE receiving multicast in RRC_INACTIVE to perform cell reselection, i.e., network directly provide CellReselectionPriority to UE, and the further details is that both RRCRelease and SIB can be used to provide UE with this multicast specific frequency priority.

Proposal 6: Network (based on NW implementation) directly provides multicast specific frequency priority (i.e., CellReselectionPriority dedicated for Multicast reception in RRC_INACTIVE capable UE), and both RRCRelease and SIB can be used to provide UE with this multicast specific frequency priority.

For NCL, the legacy mechanism can be used, that is to say, it can be used by UE to determine resuming RRC connection without reading MCCH in the re-selected cell if the concerned service is not available in this cell by previously checking NCL.

Proposal 7: Use multicast specific NCL similarly to Rel-17 NCL mechanism in MBS broadcast.

RRC resumption based on reception quality

Multicast scheduling is unified based on amount of RRC_CONNECTED UEs feedback, but now since RRC_INACTIVE UE can still receive multicast service, they should be responsible for its own multicast scheduling during RRC_INACTIVE. If RRC_INACTIVE UE is experiencing poor signal channel and no cell reselection is triggered, RRC connection resumption is required in this case.

To our understanding, the QoS requirement of multicast reception in RRC_INACTIVE is not too high, we don’t have to design a very precise criterion based on QoS. Instead, we can just simply reuse the common criterion quantity -  RSRP/RSRQ or even SINR to reflect UE’s situation. For example, a new threshold threshold_Mcast_RSRP/RSRQ can be introduced, when the receptions of quality of the serving cell fulfils RSRP/RSRQ < threshold_Mcast_RSRP/RSRQ, the UE may initiate RRCResumeRequest.

Proposal 8: Use RSRP/RSRQ to define the reception quality, when reception quality of the serving cell fulfils RSRP/RSRQ < threshold_Mcast_RSRP/RSRQ, the UE may initiate RRCResumeRequest.

RRC resume cause

In case that UE receiving multicast in RRC_INACTIVE can not acquire the subsequent PTM configuration no matter it is caused by cell reselection or MCCH is not available, UE could initiate RRC connection resumption. But in this case, UE actually does not have to go back to RRC_CONNECTED as long as network can provide PTM configuration (e.g., via RRCRelease as response).

Therefore, a new RRC resume cause (e.g., multicast-acquisition) can be introduced, the UE can transmit RRCResumeRequest carrying resume cause - multicast-acquisition to notify network that the reason of RRC resumption is to request a PTM configuration. And network may, based on implementation, resume this UE or send a RRCRelease with a PTM configuration as a response to this UE. So that UE can continue receiving multicast in RRC_INACTIVE in a low power consumption.

Fig.3 PTM acquisition procedure for RRC_INACTIVE UE

Proposal 9 : Introduce a new resumeCause – multicast-acquisition for PTM configuration request when initiating RRCResumeRequest.

Proposal 10 : Network may send RRCRelease with PTM configuration as the response to RRC resumption which carries resumeCause – multicast-acquisition.

- 3 Conclusions

Proposal 1: The UE suspends the multicast configuration (including MRB) used for RRC_CONNECTED when enters RRC_INACTIVE and apply a new multicast MRB for the reception in RRC_INACTIVE based on multicast MCCH configuration or RRCRelease.

Proposal 2: The Multicast MCCH information acquisition procedure overwrites any previously stored PTM configuration.

Proposal 3: UEs receiving multicast in RRC_INACTIVE shall monitor for MCCH change information in any MCCH occasion at least once per MCCH modification period.

Proposal 4: If updated PTM configuration is received, the UE receiving multicast session in RRC_INACTIVE may stop monitoring the subsequent MCCH PDCCH occasions for this modification period.

Proposal 5: If session deactivation is received, the UE receiving multicast session in RRC_INACTIVE may stop monitoring the subsequent MCCH PDCCH occasions, until the corresponding session is (re)activated.

Proposal 6: Network (based on NW implementation) directly provides multicast specific frequency priority (i.e., CellReselectionPriority dedicated for Multicast reception in RRC_INACTIVE capable UE), and both RRCRelease and SIB can be used to provide UE with this multicast specific frequency priority.

Proposal 7: Use multicast specific NCL similarly to Rel-17 NCL mechanism in MBS broadcast.

Proposal 8: Use RSRP/RSRQ to define the reception quality, when reception quality of the serving cell fulfils RSRP/RSRQ < threshold_Mcast_RSRP/RSRQ, the UE may initiate RRCResumeRequest.

Proposal 9 : Introduce a new resumeCause – multicast-acquisition for PTM configuration request when initiating RRCResumeRequest.

Proposal 10 : Network may send RRCRelease with PTM configuration as the response to RRC resumption which carries resumeCause – multicast-acquisition.

- 4 References

[1] R2-2306547	Report from MBS breakout session	RAN#122		Dawid

[2] TS 38.331		MCCH information validity and notification of changes, clause 5.9.1.3